Source:	Philips International B.V., Qualcomm Incorporated

Title:	IVAS Characterization - Room Acoustics Test Statistical Analysis

Document for:	Agreement

Agenda Item:	7.5

# Introduction

The IVAS-8b test plan for the characterization phase [1] lacks details regarding statistical analysis of the room acoustics test results.

The test methodology for room acoustics is a multi-stimulus test, roughly based on concepts known from BS.1534 [2]. The BS.1534 document recommends performing parametric statistics using analysis of variance (ANOVA).

Following the BS.1534 recommendation, this contribution proposes using ANOVA for the analysis of statistical significance followed by Tukey’s post-hoc test for pairwise comparisons between different rendering modes.

# Proposed statistical analysis

The Analysis of Variance (ANOVA) allows for determining statistically significant factors that contribute to the differences between conditions. In this scope, a two-way ANOVA can be used to evaluate the influence of two factors that are considered independent, such as rendering modes and test signal types, as well as the combined interaction of those factors. An example of ANOVA results is shown in Table 1.

Table 1. Example of ANOVA variance analysis

|  | sum_sq | df | F | PR(>F) |
| --- | --- | --- | --- | --- |
| C(Signal) | 1234.54 | 11.0 | 0.77 | 0.67 |
| C(Renderer) | 154214.04 | 3.0 | 352.08 | 4.46e-66 |
| C(Signal):C(Renderer) | 3049.08 | 33.0 | 0.63 | 0.94 |
| Residual | 21024.0 | 144.0 |  |  |

Where:

- sum_sq – sum of squares as a variance measure per factor,

- df – degrees of freedom – number of levels per factor,

- F – F-statistic – the ratio of variance explained by the factor to unexplained (residual) variance,

- PR(>F) – p-value – the probability of observing such F-statistic if the null hypothesis is true (no effect of the factor).

In this example,

- the signal factor C(Signal) is not statistically significant (high p-value, low F-statistic) – different test signals do not affect listener scores,

- the renderer factor C(Renderer) is highly significant (very low p-value, high F-statistic) – strong influence of rendering mode on listener scores,

- the interaction between signal and renderer C(Signal):C(Renderer) is not significant (high p-value, low F-statistic) – the rendering mode influence on listener scores is consistent across signals.

While ANOVA answers the question which factors significantly influence the listeners’ scores, and over which non-significant factors results may be pooled, the post-hoc statistical tests allow to further analyze the differences between the values of such significant factors.

Following [1], the sources propose using Tukey’s Honest Significant Difference (HSD) test on ANOVA results considered significant (i.e., the renderer). The Tukey’s test performs pairwise comparison based on:

- mean values,

- standard error within each renderer type,

- range distribution based on 95% confidence.

The combination of ANOVA and Tukey’s HSD post-hoc analysis provides a robust framework for identifying and interpreting statistically significant differences in listener scores across test conditions. This helps understanding listener response patterns and ensures that the conclusions are meaningful and correct.

# Proposal

It is proposed to update clause 3.3.6.2 of [1] by adding the following text at the end of the clause.

For the room acoustics tests (Annex I) the statistical tests will be based on ANOVA parametric variance analysis followed by Tukey’s HSD post-hoc test for pairwise comparisons between different rendering modes. The reference (anchor) will be the IVAS rendering with anechoic HRTF (IVAS_Binaural).

Results of the tests will be presented for individual signal types as shown in Figure 1, and averaged for rendering modes, as shown in Figure 2. The markers above the plots in Figure 2 indicate pairwise comparisons considered as statistically significant (p < 0.05).

Figure 1. Detailed results per signal type

Figure 2. Average results per rendering mode

Next to that, the results of Tukey Honest Squares Difference post-hoc test will be provided as shown in Table 1. The ‘Significant’ column indicates statistically significant differences between the rendering modes. This shows whether there are statistically significant differences in perception of different rendering modes with room acoustics.

Table 2, Tukey HSD post-hoc test results

| Group 1 | Group 2 | Mean diff | p-adj | Lower | Upper | Significant |
| --- | --- | --- | --- | --- | --- | --- |
| IVAS_Binaural | IVAS_BinauralRoomIR | 66.8958 | 0.0 | 60.7568 | 73.0349 | True |
| IVAS_Binaural | IVAS_BinauralRoomReverb | 64.625 | 0.0 | 58.4859 | 70.7641 | True |
| IVAS_Binaural | ReferenceRenderer_BinauralRoomIR | 64.7292 | 0.0 | 58.5901 | 70.8682 | True |
| IVAS_BinauralRoomIR | IVAS_BinauralRoomReverb | -2.2708 | 0.7729 | -8.4099 | 3.8682 | False |
| IVAS_BinauralRoomIR | ReferenceRenderer_BinauralRoomIR | -2.1667 | 0.797 | -8.3057 | 3.9724 | False |
| IVAS_BinauralRoomReverb | ReferenceRenderer_BinauralRoomIR | 0.1042 | 1.0 | -6.0349 | 6.2432 | False |

# Conclusions

In this contribution the sources propose updates to the IVAS Permanent Document IVAS-8b: Test Plan for Characterization Phase, filling the gaps regarding IVAS room acoustics synthesis test statistical analysis. The sources propose to use the combination of ANOVA and Tukey’s HSD post-hoc analysis as a robust framework for listening test results analysis.

# References

- S4-251138 – IVAS Permanent Document IVAS-8b: Test Plan for Characterization Phase v.0.7.0

- Recommendation ITU-R BS.1534-3 (10/2025), Method for the subjective assessment of intermediate quality level of audio systems, International Telecommunication Union