Agenda item: 	10.8

Source: 	Qualcomm Inc.

Title: 	[AvCall-MED] Network animation and rendering

Document for	Discussion and Agreement

# Introduction

In this contribution, we propose content to specify the network animation and rendering scenario.

# Proposed Changes

### 6.5.4	Network Animation and Rendering

When an AR-MTSI client initiates or receives a call with avatar media, the IMS Application Server (IMS-AS) shall evaluate whether network-based avatar animation and rendering is required. The IMS-AS forwards avatar-related INVITE requests to the Avatar-capable AR AS for capability assessment and media routing decisions.

The AR AS shall invoke network-based animation and rendering through an MF when:

- the receiving MTSI client has not registered the "+g.3gpp.3d-avatar-animation" feature tag,

- the receiving MTSI client's registered capabilities indicate insufficient resources for avatar animation,

- the offered animation frameworks are not supported by the receiving MTSI client, or

When network rendering is invoked, the AR AS shall allocate an MF capable of real-time avatar rendering and configure it with the appropriate rendering parameters based on the receiving UE's video capabilities. The AR AS shall modify the SDP to route avatar animation data to the MF instead of the receiving UE, effectively inserting the MF into the media path between the sending and receiving MTSI clients.

The sending AR-MTSI client shall establish a data channel with the MF for avatar animation parameters, while the MF shall establish a video stream with the receiving MTSI client using standard video codecs as specified in [2].

The MF performing network-based avatar rendering shall fetch the ARF container from the BAR using the reference provided in the SDP and load the avatar model for rendering. The MF shall receive animation streams through the data channel from the sending AR-MTSI client and apply these animation samples to the avatar model in real-time. The animated avatar shall be rendered as a 2D video stream or stereoscopic 3D video stream based on the receiving UE's capabilities.

For the receiving MTSI client, the avatar data channel media description shall be replaced with a video media description including standard video codec negotiation as specified in [2], and avatar-specific attributes that are not applicable to video streams shall be removed.

The MF shall ensure lip-sync between avatar animation and associated audio streams while maintaining consistent frame timing and low latency for interactive communication. The AR AS should consider the computational load on the MF and may distribute avatar rendering across multiple MF instances for scalability.

# Proposal

We propose to agree the content of section 2 and add it to the base CR.