3GPP Audio Telco Post TSG SA WG4 #133	S4-251303

Online, July 18-25, 2025                                                                       Revision of S4aA250083

# Title:	Scenarios for RAN simulation calibration for ULBC over NTN

Source:	Qualcomm, vivo, Xiaomi, Huawei Technologies Co. Ltd

Agenda item:	7.9 FS_ULBC

# Document for:	Information

# 1 Introduction

The RAN simulation parameters are agreed in S4aA250069. To calibrate companies’ simulators to be used for the RAN simulation, some initial scenarios and the expected outputs are proposed in this contribution.

# 2 Scenarios

The following parameters are common across all scenarios:

- Channel model: NTN-TDL-C, Table 6.9.2-3 of [38811]

- Modulation: QPSK

- Voice bundling period: 80ms

- Doppler spread: 5 Hz

- TBS value: 144 and 256

- Number of base station antenna: 1

The scenarios are as follows:

Scenario 1.1: Uplink (TBS144)

- Subcarrier Spacing (SCS): 3.75kHz

- Number of tones: 1

- Number of RU’s: 2, i.e., 64ms NPUSCH

- No repetition

- Number of UE antennas: 1

- SNR points: -1, 0, 1, 2, 3, 4, 5 and 2.6, 5.6 dB

- DMRS bundling: across all slots in an RU and across RUs

Note: For UE with 23dBm, DMRS bundling seems essential; for UE with higher power it may not be needed. For calibration purpose, DMRS bundling is assumed.

Scenario 1.2: Uplink (TBS256)

- Subcarrier Spacing (SCS): 3.75kHz

- Number of tones: 1

- Number of RU’s: 2, i.e., 64ms NPUSCH

- No repetition

- Number of UE antennas: 1

- SNR points: 5, 6, 7, 8, 9, 10, 11dB

- DMRS bundling: across all slots in an RU and across RUs

Note: For UE with 23dBm, DMRS bundling seems essential; for UE with higher power it may not be needed. For calibration purpose, DMRS bundling is assumed.

Scenario 2.1.1: Downlink with 1 UE antenna (TBS144)

- SCS: 15kHz

- Number of tones: 12

- Number of subframes: 4, i.e., 4ms NPDSCH

- Number of repetitions: 2. Therefore, in total 8ms DL resource.

- Number of UE antennas: 1

- SNR points: -9, -7, -5, -4, -3, -2, -1, 0, 1 and -3.3 dB

- NRS bundling

Note: For calibration purpose, NRS bundling is assumed.

# Scenario 2.1.2: Downlink with 2 UE antenna (TBS144)

This scenario is the same as scenario 2.1.1 except with 2 UE receive antennas.

- Antenna correlation: uncorrelated.

Scenario 2.2.1: Downlink with 1 UE antenna (TBS256)

- SCS: 15kHz

- Number of tones: 12

- Number of subframes: 8, i.e., 8ms NPDSCH

- Number of repetitions: 1. Therefore, in total 8ms DL resource.

- Number of UE antennas: 1

- SNR points: -5, -4, -3, -2, -1, 0, 1 and -3.3 dB

- NRS bundling

Note: For calibration purpose, NRS bundling is assumed.

# Scenario 2.2.2: Downlink with 2 UE antenna (TBS256)

This scenario is the same as scenario 2.2.1 except with 2 UE receive antennas.

- Antenna correlation: uncorrelated.

# 3 Outputs

# The expected outputs are:

- BLER vs SNR curve, in both figure and text file

- The magnitude of the channel vs time spanning [100] bundling periods for a given random seed used for 2 different scenarios that differ only in TBS values (e.g., Scenario 1.1 and Scenario 1.2), in figure. This is for channel consistency check.

  - The sampling rate should be higher than the OFDM symbol rate to be able to show phase discontinuity if there is any. This makes sure the channel correlation is preserved at bundling boundaries.

  - Identical channels help with codec evaluation.

- Loss traces, a sequence of binary numbers, each corresponding to a TB, where 1 means loss and 0 means success