3GPP TSG SA WG4#133-e	Tdoc S4-251320

Online, 18 - 25 July 2025

Source:	China Mobile Com. Corporation

Title:	[FS_Beyond2D] pCR on Evaluation Results for the Scenario 1

Spec:	3GPP TR 26.956 1.0.0

Agenda item:	9.6

Document for:	Agreement

1. Introduction

During the SA4-(AH) Video SWG post 132 meeting, the evaluation results for  Scenario 1 was agreed for PD. This document is to include the same content in the TR.

2. Proposal

It is proposed to agree the following changes to the 3GPP draft TR 26.956 V1.0.0

* * * First Change * * * *

9.x	 Scenario 1: UE-to-UE Stereoscopic Video Live Streaming

9.x.1	Evaluation Overview

This section presents an overview of the evaluation process for the UE-to-UE stereoscopic video live streaming scenario. MV-HEVC and simulcast HEVC are detailed as candidate solutions. It includes objective evaluation results.

9.x.2	Reference Sequences

The evaluation has been performed on the sequences presented in Annex C.3.2, C.3.3, C.3.4, C.3.5, C.3.6, and C.3.7. Only the first 150 frames were used of each test sequence.

9.x.3	Performance Metrics

Only Peak-Signal to Noise Ratio PSNR(Y) is provided.

9.x.4	Candidate Solutions

9.x.4.1	Solution 1: Simulcast HEVC

9.x.4.1.1	Introduction

This clause provides the evaluation process for Scenario 1 using simulcast HEVC, based on the test conditions defined in Clause 7.2.8.

9.x.4.1.2	Reference Software

The reference software for HEVC is called HM (HEVC Test Model). The source code can be downloaded from the GitLab: https://vcgit.hhi.fraunhofer.de/jvet/HM.

After compiling the source code, the following executable files were generated:

-	TAppEncoderStatic / TAppEncoderStaticd: The encoder application. It converts raw video data (e.g., YUV) into an HEVC bitstream. It implements all encoding tools as defined by the HEVC standard, configurable via command-line arguments.

-	TAppDecoderStatic / TAppDecoderStaticd: The decoder application. It decodes an HEVC bitstream back into raw video data (YUV). It is primarily used for verifying the correctness of the encoded bitstream.

-	TAppDecoderAnalyserStatic / TAppDecoderAnalyserStaticd: An enhanced decoder with analysis capabilities. In addition to decoding, it provides detailed statistics on the decoding process, such as information per CU/PU/TU, distribution of motion vectors, prediction modes, and quality metrics (e.g., PSNR, SSIM).

-	TAppExtractorStatic / TAppExtractorStaticd: A tool for extracting specific information from an HEVC bitstream, such as NAL unit information, parameter sets (VPS, SPS, PPS), or data for specific frames/slices.

-	TAppRendererStatic / TAppRendererStaticd: A tool to render decoded raw video data directly to the screen for visual inspection, often using graphics APIs to convert YUV to RGB for display.

-	annexBbytecountStatic / annexBbytecountStaticd: A utility to analyze and count bytes of NAL units within an Annex B formatted HEVC bitstream. Useful for validating stream structure and gathering NAL unit size statistics.

-	convert_NtoMbit_YCbCrStatic / convert_NtoMbit_YCbCrStaticd: A utility for converting the bit-depth of YCbCr data (e.g., from 8-bit to 10-bit) or for converting between chroma subsampling formats (e.g., YUV420 to YUV422).

The following Python scripts were used to calculating objective metrics:

-	The script for calculating MV-HEVC bitrate and Y-PSNR is under ./metrics/analyze_mvhevc.py

-	The script for calculating Simulcast HEVC bitrate and Y-PSNR is under ./metrics/analyze_hm_logs.py

-	The script for calculating BD-Rate is under ./metrics/compare.py

9.x.4.1.3	Parameter Settings

- Run HM TAppEncoderstatic to encode each YUV file using the fixed QP values that are defined in clause 7.2.8.2, the

- same QP values are used for all sequences and all encoder conditions.

All sequences have been encoded using the configurations in ./cfg/HM/encoder_lowdelay_main_rext.cfg.

Encoding was performed by running the shell script with appropriate parameters. The shell scripts are in the attached sh files. For example:

echo "streetView_captured_left: QP=17"

./TAppEncoderStatic -c ~/HTM-16.3/cfg/HM/encoder_lowdelay_main_rext.cfg -i ~/yuvfiles/streetView_captured_left.yuv -o output0703.bin --SourceWidth=1920 --SourceHeight=1080 --FrameRate=30 --FramesToBeEncoded=150 --QP=17

echo "streetView_captured_right: QP=17"

./TAppEncoderStatic -c ~/HTM-16.3/cfg/HM/encoder_lowdelay_main_rext.cfg -i ~/yuvfiles/streetView_captured_right.yuv -o output0703.bin --SourceWidth=1920 --SourceHeight=1080 --FrameRate=30 --FramesToBeEncoded=150 --QP=17

9.x.4.1.4	Evaluation Results

The evaluation results are provided in the attached logs files, under ./logs/HEVC,and summarize in Table in clause 9.x.5.

9.x.4.1.5		Network Requirements

Simulcast HVEC serves as a baseline for performance, the evaluation results indicate that to achieve very high objective quality (PSNR > 41 dB), simulcast HEVC requires significant high bandwidth. For exmaple, the complex scenes such as Street View and Cute Dog, the bitrate demand ranges from 30,000 kbps to 83,000 kbps. Even at lower quality settings (QP 27, 32, 37), Simulcast HEVC still requires notable bandwidth demands for complex scenes, ranging from 1,500 kbps to 14,500 kbps.

9.x.4.2	Solution 2: MV-HEVC

9.x.4.2.1	Introduction

This clause provides the evaluation process for Scenario 1 using MV-HEVC, based on the test conditions defined in Clause 7.2.8.

9.x.4.2.2	Reference Software

The reference software for Multiview High Efficiency Video Coding (MV-HEVC) is an extension of the HEVC (H.265) standard that supports multiple views. The latest version of the MV-HEVC reference software (HTM) is available on: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/branches/HTM-16.3-fixes/.

9.x.4.2.3	Parameter Settings

- Run HM TAppEncoderstatic to encode each YUV file using the fixed QP values that are defined in clause 7.2.8.2, the

- same QP values are used for all sequences and all encoder conditions.

All sequences have been encoded using the configurations in ./cfg/MV-HEVC/baseCfg_2view. The configuration files are in the attached cfg files.

Encoding was performed by running the shell script with appropriate parameters. The shell scripts are in the attached sh files. For example:

echo "dog_ai_generated: QP=17"

./TAppEncoderStatic -c ./cfg/baseCfg_2view_movegirl_ai_generated.cfg --QP=17 -wdt 1920 -hgt 1080 -fr 30 -f 150

echo "dog_ai_generated: QP=22"

./TAppEncoderStatic -c ./cfg/baseCfg_2view_movegirl_ai_generated.cfg --QP=22 -wdt 1920 -hgt 1080 -fr 30 -f 150

echo "dog_ai_generated: QP=27"

./TAppEncoderStatic -c ./cfg/baseCfg_2view_movegirl_ai_generated.cfg --QP=27 -wdt 1920 -hgt 1080 -fr 30 -f 150

echo "dog_ai_generated: QP=32"

./TAppEncoderStatic -c ./cfg/baseCfg_2view_movegirl_ai_generated.cfg --QP=32 -wdt 1920 -hgt 1080 -fr 30 -f 150

echo "dog_ai_generated: QP=37"

./TAppEncoderStatic -c ./cfg/baseCfg_2view_movegirl_ai_generated.cfg --QP=37 -wdt 1920 -hgt 1080 -fr 30 -f 150

9.x.4.2.4	Evaluation Results

The evaluation results are provided in the attached logs files, under ./logs/MV-HEVC,and summarize in Table in clause 9.x.5.

9.x.4.2.5		Network Requirements

MV-HEVC is designed to improve coding efficiency by leveraging redundancy between different views. The evaluation results show significant lower network bandwidth requirements compared to a Simulcast HEVC approach for stereoscopic video. The bandwidth savings are substantial, ranging from 20% to over 50% depending on the content and quality level. For example, the "Cute Dog - Generated" sequence at R2 (QP 22), MV-HEVC achieved a bitrate of 16.5 Mbps, representing a 50% reduction compared to Simulcast HEVC's 32.8 Mbps.

- x.5	Summary of Evaluation

The evaluation results are summarized in Table 9.x.5-1.

Table 9.x.5-1: Evaluation Results with Simulcast HEVC and MV-HEVC for Scenario 1

| Test Sequence | Rate points | Simulcast HEVC |  | MV-HEVC |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Bitrate [kbps] | PSNR [dB] | Bitrate [kbps] | PSNR [dB] |
| Street View - captured | R1 (17) | 82597.98 | 45.46 | [77934.46] | [42.79] |
|  | R2 (22) | 40906.65 | 41.19 | [29811.63] | [37.89] |
|  | R3 (27) | 12828.06 | 36.93 | [9759.37] | [35.08] |
|  | R4 (32) | 4591.49 | 34.35 | [3882.06] | [32.86] |
|  | R5 (37) | 1655.49 | 31.69 | [1568.76] | [30.38] |
| Street View - Generated | R1 (17) | 79587.76 | 45.59 | [63793.59] | [42.05] |
|  | R2 (22) | 37996.06 | 41.31 | [22728.87] | [38.38] |
|  | R3 (27) | 11355.45 | 37.19 | [7199.21] | [35.87] |
|  | R4 (32) | 3988.24 | 34.69 | [3105.62] | [33.83] |
|  | R5 (37) | 1506.74 | 32.03 | [1387.66] | [31.37] |
| Cute Dog - Captured | R1 (17) | 60282.85 | 46.58 | 50309.66 | 43.54 |
|  | R2 (22) | 31296.69 | 42.86 | 21219.06 | 40.21 |
|  | R3 (27) | 13854.47 | 39.02 | 8549.64 | 37.45 |
|  | R4 (32) | 7691.15 | 35.89 | 4030.95 | 34.97 |
|  | R5 (37) | 4361.23 | 32.99 | 2146.72 | 32.38 |
| Cute Dog - Generated | R1 (17) | 64623.85 | 46.53 | 40674.14 | 43.26 |
|  | R2 (22) | 32782.40 | 42.69 | 16506.89 | 40.33 |
|  | R3 (27) | 14509.13 | 38.79 | 6686.76 | 37.67 |
|  | R4 (32) | 7981.78 | 35.69 | 3274.28 | 35.22 |
|  | R5 (37) | 4504.21 | 32.82 | 1771.73 | 32.65 |
| Moving Girl - Captured | R1 (17) | 9100.29 | 51.28 | 8944.59 | 49.49 |
|  | R2 (22) | 3319.19 | 48.92 | 2661.69 | 47.36 |
|  | R3 (27) | 1485.50 | 46.75 | 947.79 | 45.72 |
|  | R4 (32) | 818.86 | 44.34 | 475.70 | 43.77 |
|  | R5 (37) | 480.74 | 41.72 | 250.63 | 41.41 |
| Moving Girl - Generated | R1 (17) | 9788.79 | 51.19 | 7,297.06 | 49.41 |
|  | R2 (22) | 3630.03 | 48.73 | 2300.78 | 47.26 |
|  | R3 (27) | 1523.01 | 46.49 | 822.27 | 45.59 |
|  | R4 (32) | 825.61 | 44.14 | 422.32 | 43.75 |
|  | R5 (37) | 478.95 | 41.56 | 250.63 | 41.58 |

Table 9.x.5-2 shows the Bjøntegaard Delta (BD) bitrates reductions obtained by MV-HEVC in comparison to Simulcast HEVC.

Table 9.x.5-2: BD-rate reduction of MV-HEVC relative to Simulcast HEVC for Scenario 1

| Test Sequence | BD-rate reduction of MV-HEVC [%] relative to Simulcast HEVC |
| --- | --- |
| Street View - captured | [61.580523] |
| Street View - Generated | [22.565416] |
| Cute Dog - Captured | -8.714002 |
| Cute Dog - Generated | -37.240038 |
| Moving Girl - Captured | -2.813581 |
| Moving Girl - Generated | -20.596453 |

* * * Next Change * * * *