Agenda item: 	10.7

Source: 	Nokia, Samsung Electronics Co., Ltd, InterDigital Canada.

Title: 	[AvCall-MED] Avatar Selection and Negotiation Flow

Document for	Discussion and Agreement

# Introduction

In this contribution, we propose content for the Avatar Selection and Negotiation Flow for Avatar Communication, as well as the updated call flow on call setup and capability negotiation.

In TR 26.813, clause 8.3.1, it is indicated that BAR (Base Avatar Repository) stores the Base Avatar Representations and their associated Avatar IDs. Furthermore, One or more Base Avatars may be stored for a user, and each Base Avatar is identified with an Avatar ID. Accordingly, in clause 8.3.2.1, when UE sends capability negotiation requests using the application data channel through MF to the DC AS. The message for capability negotiation requests carries parameters including an avatar id chosen by UE. The current call flows start from the point where the UE has obtained the avatar ID without detailed description.

In TS 23.228, AC.11.3, it is indicated that the avatar ID list can be downloaded via application data channel or pre-configured in the UE locally.

Also, in TR 33.790 clause 7.2, as part of the Conclusions for Key Issue #2: Security of IMS based Avatar Communication: Avatar ID sent by the UE1 should be verified by the IMS network about whether it belongs to the UE1.

Furthermore, in TS 23.228, AC 11.3.3 Network Centric Procedures, it was indicated that network centric modes include MF rendering and DC AS rendering, which also need further alignment in SA4 call flows.

Therefore, it is necessary to address the related procedures and details in the call flow during the call setup and capability negotiation, and to align with SA2 and SA3 related work.

# Proposed Changes

#### 2.1 Proposed changes to TS 26.264

It is proposed to include detailed procedures to the main CR to TS 26.264 under subclauses of the Annexas below.

CHANGE STARTS

##### A.1.6.3	 Avatar Selection and Negotiation Flow

Figure X: Avatar Selection and Negotiation Flow

The capability negotiation procedure is based on the avatar type (2D or 3D) and the capability information of sender/receiver UEs and MF. The capability information includes the animation data type(s) (e.g., text, expression data and motion signals for joints) supported by either UEs or MF. For network centric mode, after capability negotiation, the IMS AS instructs MF to download UE1’s base avatar from BAR, generate animation data from the source data received from UE1, and animate UE1’s base avatar using the animation data received from UE1 or generated by MF itself.

A.0. (optional) An Avatar ID List is downloaded through the Bootstrap Data Channel (see details in AC 11.3.1 in TS 23.228 [23]) or an Avatar ID is pre-configured in UE1.

NOTE: 	The step 0 is optional. The Avatar ID List is provisioned or downloaded to the UE before a data channel for avatar call is setup. The UE and the BAR may interact by means out of the scope of 3GPP. The step 0 also can be included in step Z in the general call flows.

A.1: An audio/video session is established between UE1 and UE2.

A.2: The bootstrap and application data channels are established between UE1, the MF, and the DC AS.

A.3:	UE1 selects an avatar representation to be used for the avatar call. The list of available avatar representations may already be known to UE1.

A.4: UE1 sends a capability negotiation request using the application data channel through the MF to the DC AS. The message carries parameters which may include an avatar ID associated with the selected avatar representation selected in step 3 (if the avatar ID is downloaded or pre-configured in UE1),animation data types (e.g., text, expression data, or motion signals for joints) supported by UE1, and related rendering requirement or capability information of the network.

If UE1 does not have any downloaded Avatar ID List or pre-configured avatar ID, it may request to obtain Avatar ID List through the established application data channel via a capability negotiation request.

A.5. (optional): If the message in step 3 carries parameters including, e.g., an avatar ID downloaded or pre-configured in step 0, then the avatar ID needs to be further verified by the BAR through a verification request from the DC AS to the BAR. If UE1 requests to obtain an Avatar ID List via a capability negotiation request, the DC AS sends the request to the BAR.

A.6. (optional) According to the request received in step 4, the BAR verifies the avatar ID and sends response to the DC AS. If the avatar ID does not pass the verification by the BAR, an error message is sent to the UE1. If the BAR receives an Avatar ID List request, the BAR (generates and) sends the Avatar ID List to the DC AS.

NOTE:	Steps A.5 and A.6 are optional. Whether and which user identity(ies) should be used by the user of the sending UE (UE1) and/or the receiving UE (UE2) for the download of the Avatar Representations in the case of a receiving UE rendering mode will be decided by SA WG3 and the procedure will be aligned with SA WG3 decision.

A.7: The DC AS sends an avatar capability request to the serving MF via DCSF and IMS AS. The request may include rendering requirement or capability information per UE1’s requested.

A.8: The MF sends a response with its avatar capability information to the DC AS for further DC AS to make decision, e.g. if network-centric rendering is supported, and if the MF rendering mode or DC AS rendering.

NOTE1: If the serving MF is capable of rendering the avatar, the MF indicates its avatar capability information in the response. If the serving MF is not capable of rendering the avatar, the MF will also indicate that in the response with a description of reasoning in the response. The need for DC AS interacting with the MF (e.g. querying) for the MF’s avatar capability is related to SA2’s decision on a possible interface between DC AS and MF.

NOTE2: The service of avatar capability provided by MF will be further defined in CT1/CT4 if needed.

A.9: (optional) Through an established P2A application data channel, MF/DC AS sends a capability negotiation request to the UE2. The message may include the same information as in described in step 4

A.10: (optional) UE2 sends the capability negotiation response to MF/DC AS. The message carries the capability negotiation result related to UE2’s preference.

A.11: The DC AS gets the avatar type (2D or 3D, from base avatar retrieved from BAR or to be generated by the MF) associated with the avatar ID and confirms the capability negotiation result based on the avatar type and the capabilities supported by UE1,MF and UE2. The capability negotiation result includes the rendering mode (e.g., MF rendering mode or DC AS rendering mode) and animation method (e.g., by audio, text or expression data and motion signals for joints).

A.12: The DC AS sends the capability negotiation response to UE1 through MF. The message carries the capability negotiation result.

A.13: (optional) Depending on capability negotiation results for the selected avatar representation, media re-negotiation may take place between UE1 and UE2 in order to establish the necessary media streams. If the received capability negotiation response fulfils UE1’s rendering request, the subsequent steps continue.

A.14: The subsequent procedure continues.

CHANGE ENDS

# Proposals

We propose adding the above content into the base CR on Avatar Communication to TS 26.264.