3GPP TSG-SA WG4#133e	S4-251388

Online, July 18-25,  2025                                                              revision of S4aA250075

Source:	Dolby Laboratories, Inc., Nokia, Novamint, EDF

Title:	[FS_ULBC] On ULBC bit rates

Agenda item:	7.9

Document for:	Agreement

- Introduction

For IMS Voice Call over GEO, in TR 22.887, a “Transmission data rate” of [1-3] kbit/s is assumed. While “Transmission data rate” is not defined in TR 22.887, discussions are ongoing as to what available source (or codec) bit rates this would correspond to depending on, e.g., the chosen transport formats, the bundling time, the potential packet loss resilience of a future ULBC codec, and RTP/UDP/IP packetization overhead. More specifically, given the characteristics of the GEO NTN NB-IoT channel assumed for transmission of IMS voice call data and the ULBC properties such as rate-distortion and packet loss, an important question is, at what operation bit rate the best possible voice quality can be achieved. In simple terms, at a higher bit rate, the ULBC would give higher voice quality however at the cost of less robust transmission causing more packet losses and resulting quality degradations. Thus, the best possible trade-off between bit rate on the one hand and packet loss rate and the resulting quality impairments on the other hand must be found.

This contribution addresses the question of how to find this ‘optimal’ ULBC bit rate. To be more illustrative, an example is provided describing how to compare two rate options of an assumed ULBC candidate. This example can be extended to compare any ULBC candidate against another candidate codec or a reference codec to be used to define performance requirements.

- Past practice

The question of potentially setting required bit rates was discussed and decided in several 3GPP speech codec standardization exercises.

In case of EVS, design constraint bit rates were based on TBSs (Transport Block Sizes) originally designed for the UTRAN system to allow optimized transmission of AMR and AMR-WB speech codec frames. IVAS codec design constraint bit rates followed the EVS codec bit rates.

It is thus worth understanding how source coding bit rates for AMR and AMR-WB were decided. As can be seen from AMR-WB design constraints [1] and performance requirements [2], no specific bit rates were required in either of these Pdocs. Absent deeper insights than what codec proponents had with their specific domain knowledge, finding the best possible bit rates was considered part of the design freedom that each proponent had when developing its candidate codec. The task was to provide a codec that offered the best possible overall quality in the GSM system across a set of relevant carrier-to-interference (C/I) ratios. The set of relevant C/I conditions was determined based on GSM system measurements and ranged from 4 dB, which was a challenging channel condition for the EFR (Enhanced Full Rate) reference codec, to 19 dB and ideal channel.

The outcome of the AMR/AMR-WB standardization efforts is well-known. It resulted in multi-mode/multi-rate speech codecs and a system allowing for fast bit rate adaptation in case of changing channel conditions.

- Teachings for ULBC case

Due to substantial differences in the target systems, the example of how the AMR/AMR-WB bit rates were found is not directly applicable. However, there are some teachings that suggest how ULBC bit rates can be determined. In the following differences and commonalities are discussed:

- AMR/AMR-WB standardization included the specification of channel coding. It involved the use of a radio simulator that included all relevant components of the GSM transmission channel including multipath/Rayleigh fading channel modelling. In the ULBC case, physical uplink and downlink channel coding (and modulation) procedures are fully specified. A specific Transport Format - implying the selection of a channel coding and modulation scheme - is chosen by correspondingly setting relevant parameters such as Bundling time, TBS, SCS, MCS and NRep. The presently envisioned Transport Formats allow a large range of PHY bit rates and corresponding source bit rates. This leaves design freedom for codec developers to find most suitable codec bit rate(s) that maximize(s) the quality for the relevant receive SNR conditions.

- The target system for AMR/AMR-WB was GSM (circuit-switched), while ULBC will be operated over IMS over NTN NB IoT. The main difference is that the transmission of ULBC frames will be packet-oriented, where some packetization overhead for the header information reduces the available source bit rate. The current assumption is that the overhead may be 7 bytes per packet.

- Another difference is that the AMR/AMR-WB decoder was designed to handle residual bit errors. In the ULBC case, it is assumed that the decoder only receives an indication if a frame is received without residual error or, if it is lost.

- AMR/AMR-WB was standardized together with a link adaptation scheme. The result was a multi-mode/multi-rate codec. In the ULBC case, the need and possibility of link adaptation has not yet been identified, but it should however be considered. This would imply that a solution could support multiple or scalable bit rates and that the selection of a specific operation bit rate could be based on a suitable adaptation technique either per fixed configuration, per calls or during a call.

- In the AMR/AMR-WB case, the codec was developed with the objective of offering the best possible performance over a range of relevant (average) C/I conditions after multipath reception. The corresponding objective in the ULBC case is to provide a codec that offers the best possible performance over a set of relevant receiver SNR conditions after fading based on NTN-TDL-C [3] channel modelling.

- In the AMR/AMR-WB case, the most suitable source bit rate(s) or any requirements to handle certain frame were upfront unknown. What was known were the relevant receive conditions in terms of a set of C/I ratios for which the overall voice call quality should be optimized. For ULBC, the situation is similar. The most suitable source bit rate is unknown and cannot be known since it depends on the specific properties such as intrinsic quality (without frame losses) and frame loss resiliency of each potential ULBC candidate. What is known is a set of relevant receive SNRs, for which the voice call quality should be maximized.

- In the standardization of AMR/AMR-WB, no specific target frame loss rates were defined for codec optimization. The reason is straightforward: for a given carrier-to-interference (C/I) ratio, the frame loss rate depends on the balance between error protection and the source bit rate. A solution with a higher source bit rate inherently has less room for error protection, resulting in a higher frame loss rate at the same C/I ratio compared to a solution with a lower source bit rate and stronger error protection.

Since the source bit rates were unspecified or unknown during standardization, it would not have been feasible to fairly compare candidate solutions using a fixed frame loss rate. Applying the same frame loss rate across different candidate solutions would have unfairly penalized those with lower source bit rates and better error protection.

The same logic applies to the ULBC case. Therefore, using a fixed target BLER (e.g., 2%) based on a notion of ‘TBS feasibility’ for all codec candidates is problematic. This approach would effectively force all candidate solutions to operate at the same source bit rate, which contradicts the fact that the optimal source bit rates depends on the specific codec designs that, to date, are unknown. As a result, such a method would likely lead to the choice of a suboptimal source bit rate and in consequence to suboptimal performance of a standardized solution.

- Proposal

To give ULBC proponents the design freedom to deliver the most optimized candidate codec proposals, the following steps are needed:

- Agree on a set of relevant operation points in terms of maximum achievable receive SNRs for which ULBC candidates will be evaluated. This set should cover the range from marginal operation to good and essentially error-free operation and include fading based on NTN-TDL-C [3] channel modelling. Care should be taken that this covers the full range of receive SNRs for which IMS Voice Call over GEO is expected to operate.

- Define performance requirements for each of the receive SNR operation points.

- For each of the potential bundling times out of the set of 80, 160 and 320 ms, agree on a set of potential source bit rates that candidate solutions could use. This set should be based on presently discussed transport formats in terms of the following transport format parameters TBS, SCS, MCS, NRep. 
Note: With the presently discussed transport format parameters, a range of potential source bit rates from 825 – 4650 bits/s is achieved. However, the granularity of possible bit rates in that range appears insufficient and unequal. The source addresses this point in a separate contribution [4].

- For each potential source bit rate, determine the optimum transport format parameter combination SCS, MCS, NRep that lead to best possible transmission performance at that bit rate. This could be done based on BLER vs SNR curves like those presented in [5].

- Produce packet loss patterns for each bundling time and each potential source bit rate for the set of relevant maximum achievable receive SNRs (after channel modelling). For a codec selection, it should be made sure that the specific loss patterns used in a comparative evaluation are unknown to any proponent.

- Compare ULBC candidates with each other on the basis of performance requirements for the relevant receive SNRs. Based on the choice of source bit rate and bundling time, a given candidate codec will be subjected to the corresponding packet loss patterns.

Note:

Although the ULBC codec selection should primarily be done for GEO NTN Nb IoT (as prioritized in the SID), it may be beneficial to ensure that the candidate codecs are additionally tested for other conditions/use cases than NTN NB IoT where the BLER can assumed to be independent of the source bit rate, e.g., Terrestrial IMS (1% BLER target usually), OTT (ideal channel (0% BLER)) and, potentially, extreme conditions (e.g. 10% BLER or typical long losses due to blockage).

- Example

The following example illustrates how a ULBC proponent could find the best possible bitrate for its solution such that the voice quality is optimized.

Assume that the proponent has a design that allows operation at two net (codec) bitrates, at 0.95 kbps and 3.4 kbps. The proponent seeks to determine whether the low or the high bitrate option would lead to better voice quality.

Additionally, assume that the study is done for UL and that the bundling time is 160 ms. In that case, and further assuming that the IP overhead (after header compression) is 7 bytes, a TBS of 208 bits would be chosen for the low-rate option, while a TBS of 600 bits would be chosen for the high-rate option.

Furthermore, the UE tx power assumptions and link budget assumptions according to Tdoc S4aA250058 [5] are taken, namely, leading to the following receive CNR/SNR values based on the link budget analysis of this document:

Table 5-1 [5]: CNR results under different parameter assumptions

|  | SCS/BW | UE power | UL CNR |
| --- | --- | --- | --- |
| Config. 1 | 3.75kHz/3.75kHz | 23dBm | 8.66 dB |
| Config. 2 | 15kHz/15kHz | 23dBm | 2.64 dB |
| Config. 3 | 3.75kHz/3.75kHz | 26dBm | 11.66 dB |
| Config. 4 | 15kHz/15kHz | 26dBm | 5.64 dB |
| Config. 5 | 3.75kHz/3.75kHz | 29dBm | 14.66dB |
| Config. 6 | 15kHz/15kHz | 29dBm | 8.64dB |
| Config. 7 | 3.75kHz/3.75kHz | 31dBm | 16.66dB |
| Config. 8 | 15kHz/15kHz | 31dBm | 10.64dB |

To facilitate a comparison of the achievable voice quality with the two ULBC bitrate options, BLER patterns are generated using NTN-TDL-C [3] channel modelling in the following steps.

Initially, among the available transport format parameter configurations, the one that offers best transmission performance is chosen.

For TBS = 208 bits, the choice is made among the configurations shown in the following table (with tx-time =128 ms) :

Table 5-2: Possible transport format configurations for TBS=208 bits and 160 ms bundling

| Configuration | TBS | SCS | MCS | Number of RU | Repetition number | PHY bitrate(kbps) |
| --- | --- | --- | --- | --- | --- | --- |
| #1 | 208 | 15 | 0 | 8 | 2 | 1.3 |
| #2 | 208 | 15 | 3 | 4 | 4 | 1.3 |
| #3 | 208 | 15 | 12 | 1 | 16 | 1.3 |
| #4 | 208 | 3.75 | 3 | 4 | 1 | 1.3 |
| #5 | 208 | 3.75 | 12 | 1 | 4 | 1.3 |

For TBS = 600 bits, the choice is made among the configurations shown in the following table (with tx-time =128 ms):

Table 5-3: Possible transport format configurations for TBS=600 bits and 160 ms bundling

| Configuration | TBS | SCS | MCS | Number of RU | Repetition number | PHY bitrate(kbps) |
| --- | --- | --- | --- | --- | --- | --- |
| #1 | 600 | 15 | 15 | 2 | 8 | 3.75 |
| #2 | 600 | 3.75 | 15 | 2 | 2 | 3.75 |

Subsequently, for the selected configurations that are found most suitable, generate the following BLER patterns using an NTN-TDL-C [3] channel simulator:

Table 5-4: BLER patterns to be generated for low and high rate for 4 relevant UE tx power settings

| UE power | Low rate (TBS=208 bits) | High rate (600 bits) |
| --- | --- | --- |
| 23 dBm | ep-ul-208-23dBm | ep-ul-600-23dBm |
| 26 dBm | ep-ul-208-26dBm | ep-ul-600-26dBm |
| 29 dBm | ep-ul-208-29dBm | ep-ul-600-29dBm |
| 31 dBm | ep-ul-208-31dBm | ep-ul-600-31dBm |

Next, prepare proponent codec simulation such that it can be operated with inserted packet loss using the BLER patterns generated in the previous step.

Proceed to run codec simulation using these patterns, encoding suitable test speech material. The input speech data is assumed to be called ‘inp_speech.wav’. The simulation generates output speech data files for the different applied BLER patterns. The following output files are generated:

Table 5-5: Output speech files for low and high rate options for the different UE tx power settings

| Low rate (TBS=208 bits) | High rate (600 bits) |
| --- | --- |
| out_speech-ul-208-23dBm.wav | out_speech-ul-600-23dBm.wav |
| out_speech-ul-208-26dBm.wav | out_speech-ul-600-26dBm.wav |
| out_speech-ul-208-29dBm.wav | out_speech-ul-600-29dBm.wav |
| out_speech-ul-208-31dBm.wav | out_speech-ul-600-31dBm.wav |

Lastly, evaluate the quality of the output speech, e.g. in a listening test. The quality evaluation will have to consider that the four tx power settings may result in different respective quality levels. To get to an overall voice quality judgement, it will be needed to make some (weighted) averaging of the obtained opinion scores.

Notes:
It is important to note that the quality is expected to increase with higher UE tx power since fewer packet losses are expected to occur. Moreover, it may be expected that the high rate codec option may show relatively degraded quality especially for low UE tx power. On the other hand, the low rate option of the codec may be limited due to low bit rate and that bitrate option may not benefit from higher UE tx power conditions since the transmission for low UE tx power may already be essentially error-free. The evaluation would provide deeper insight and let the proponent choose the best option overall.

Furthermore, note that speech files of a reference system to be used as performance requirements could be generated in a very similar fashion. Assuming that such a reference codec would operate at e.g. 1.5 kbps, the task would be to identify the TBS and transport format configuration that would be most suitable to transport payloads of this reference codec over the GEO NTN NB-IoT channel, generate the corresponding BLER patterns for the relevant UE tx power settings and then generate the respective output speech files to be used as performance requirement.

- Conclusion

It is proposed to agree the preceding paragraphs 4 and 5 and to document them in the TR.

References

[1] Tdoc S4/SMG11 (00)0087: Permanent project document WB-4: Design Constraints, v.1.0

[2] Tdoc S4/SMG11 (00)00321: AMR wideband performance requirements (WB-3) version 2.2

[3] 3GPP TR 38.811: Study on New Radio (NR) to support non-terrestrial networks

[4] Tdoc S4-251390: [FS_ULBC] On ULBC bit rate spacing

[5] Tdoc S4aA250058: Simulation results for FS_ULBC, Beijing Xiaomi Mobile Software