3GPP TSG-SA4 Meeting #133-e	 																		S4-251393

Online, 18th Jul - 25th Jul 2025																		rev of S4-251386

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.253 | CR | 0020 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Playout configuration status PI data for IVAS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Panasonic Holdings Corporation, Nokia, Philips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IVAS_Codec |  |  |  |  |  | Date: |  |  | 2025-07-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Practical feature (context adaptation) is missing for which Annex A does not specify the RTP handling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RTP handling is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of context adaptation may lead to suboptimal immersive quality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE 1

### A.3.5.5	Supported PI data types

Supported PI types are listed in tables A.3.5.5-1 and A.3.5.5-2 and described in the following subsections. Table A.3.5.5-1 lists PI types for forward direction signalling. Table A.3.5.5-2 lists additional PI types.

Table A.3.5.5-1 : Supported forward direction PI types in an IVAS session.

| Type bits | Forward direction PI type | Description | SDP indication | Size (bytes) | Described in clause |
| --- | --- | --- | --- | --- | --- |
| 00000 | SCENE_ORIENTATION | Describes the orientation of a spatial audio scene in unit quaternions. | fsco | 8 | A.3.5.6.1.2 |
| 00001 | DEVICE_ORIENTATION_COMPENSATED | Describes the orientation of a device in unit quaternions. The orientation is compensated in the transmitted audio. | fdoc | 8 | A.3.5.6.1.3 |
| 00010 | DEVICE_ORIENTATION_UNCOMPENSATED | Describes the orientation of a device in unit quaternions. The orientation is not compensated in the transmitted audio. | fdou | 8 | A.3.5.6.1.3 |
| 00011 | ACOUSTIC_ENVIRONMENT | Selects and optionally describes the acoustic environment. | face | 1,5 or 8 | A.3.5.6.2 |

Table A.3.5.5-1A: Supported reverse direction PI types in an IVAS session.

| Type bits | Reverse direction PI type | Description | SDP indication | Size (bytes) | Described in clause |
| --- | --- | --- | --- | --- | --- |
| 11011 | PLAYOUT_CONFIGURATION_STATUS | Describes the change in playout configuration status. | rpcs | 1 | A.3.5.7.1 |

Table A.3.5.5-2: Additional PI types in an IVAS session.

| Type bits | PI type | Description | SDP indication | Size (bytes) |
| --- | --- | --- | --- | --- |
|  | Reserved | - | - | - |
| 11111 | NO_PI_DATA | Indicates an empty PI data frame. | nopi | 0 |

NO_PI_DATA PI data type can be used to indicate empty PI data sections. The PM marker bits for a NO_PI_DATA PI data type shall be set as PM=10, see table A.3.5.2-2. For example, if an IVAS RTP payload includes multiple audio frames, and some of the audio frames do not have associated PI data, NO_PI_DATA PI type can be used.

CHANGE 2

### A.3.5.7	Reverse direction PI data types

#### A.3.5.7.1	Playout Configuration Status

A receiving device may signal its current terminal status to the media sender by transmitting Playout Configuration Status (PCS) PI data. This PCS PI data conveys the media receiver’s present playout context—by signalling whether the playout configuration is loudspeaker (mono, stereo, or any multi-channel layout), binaural (with or without room effect), or default with no playout indication. Based on this information, the media sender may respond with a bitstream adapted to the media receiver's updated configuration. The size of the PCS PI data is 1 byte, with the PCS value accounting for 3 bits and the remaining 5 bits are zero padded to ensure byte alignment. The structure of the PCS PI data frame is illustrated in Figure A.3.5.7.1, and the PCS bit field definitions are defined in Table A.3.5.7.1.

By signalling the playout configuration, context-aware capture optimization and fine-tuning at the sender side may be achieved—leading to an enhanced overall audio experience.

If the PCS PI data indicates a loudspeaker configuration (PCS=001) and the media sender is currently operating in binaural mode, the transmitter may adjust the capture settings (e.g., capture front end), should stop sending data related to binaural rendering (e.g., scene or device orientation PI data), should stop applying signal processing techniques that emulates binaural audio and/or switch to a suitable configuration. In contrast, if the PCS PI data indicates loudspeaker configuration (PCS=001) and the media sender is already operating in non-binaural mode, no change in processing is required.

If the PCS PI data indicates change in playout from loudspeaker (PCS=001) or default (PCS=000) configuration to binaural with/without room effect configuration, the media sender, depending on its capability and microphone configuration and/or spacing, may apply signal processing techniques to the input audio signal to emulate binaural audio . The media sender may also start sending data related to binaural rendering (e.g., scene or device orientation PI data).

Additionally, if the media receiver indicates a change in the playout from binaural with room effect (PCS=011) to binaural without room effect (PCS=010) or to loudspeaker (PCS=001) configuration, the media sender should stop transmitting AE data to the media receiver. In case the configuration is changed to binaural with room effect (PCS changes to 011), the media sender may resume to transmit AE data. The default PCS value 000 may be used to indicate that the playout configuration of the media receiver is unspecified. When PCS PI data is negotiated for the session, the playout configuration for the media receiver is expected to be no preference (PCS=000) at the beginning of the session.

0 1 2 3 4 5 6 7
+-+-+-+-+-+-+-+-+
| PCS |0 0 0 0 0|
+-+-+-+-+-+-+-+-+

Figure A.3.5.7.1: PCS PI data frame

Table A.3.5.7.1 - PCS bit field definitions

| PCS | Definition |
| --- | --- |
| 000 | Default |
| 001 | Loudspeaker |
| 010 | Binaural (without room effect) |
| 011 | Binaural (with room effect) |
| 100 | Reserved |
| 101 | Reserved |
| 110 | Reserved |
| 111 | Reserved |

END OF CHANGES