3GPP TSG-SA WG4 Meeting #133-e	S4-251399

Online, 18 – 25 July 2025

Source:	Xiaomi

Title:	Pseudo-CR on Coded Video Layer in system integration

Spec:	3GPP TS 26.265

Agenda item:	9.5

Document for:	Decision

1. Introduction

This contribution addresses the open issue of better integrating CVL into the clause 7. This issue is tracked here:

https://forge.3gpp.org/rep/sa4/vops/work-item-management/-/issues/1

2. Reason for Change

Better integrated text in clause 7.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 26.265 1.2.0.

* * * First Change * * * *

# 7	Common System Integration

## 7.1	Introduction

This clause documents general functionalities that are relevant for integration of video codecs into delivery systems to support common APIs on encoders and decoders.

Editor’s NOTE: This text may be removed. Updates are needed.

[A more system-centric architecture is provided in Figure 4.2-3. The workflow addresses the generation of a transport stream from a video signal using a video encoder and a packager. The packager may include for example timing and metadata information. The video signal may be encoded in the form of one or more CVL. The de-packaging and decoding of the transport stream by a de-packager and a video decoder, respectively, allows for providing the resulting video signal as well as associated metadata to a rendering and display process. Again, the packager/encoder as well as the de-packager/decoder may be configured to certain operations indicated by APIs in Figure 4.2-2.

Figure 4.2-3 Reference architecture for system Operation points and capabilities

In addition, on system level the following terms are defined:

-	System Operation Point: A collection of different possible video formats including spatial and temporal resolutions, colour mapping, transfer functions, CVL configuration, etc., a video encoding and a packaging format.

-	Transport Stream: A packaged media bitstream that conforms to a particular video coding and packaging specification/format and one or more Operation Points.

-	System Receiver: A receiver that can de-package and decode any system bitstream, including adaptive selection to one or more CVLs, that is conforming to a particular System Operation Point, and optionally render it

NOTE: 	A reference architecture for multiple decoders is for further study.

System Operation Points are not defined in this specification but are left for mappings to specific delivery protocols such as CMAF/DASH for 5G Media Streaming, or ISO BMFF for Messaging Services. However, this specification provides mapping principles to delivery protocols.

]

* * * Second Change * * * *

## 7.2	Functional Definitions

### 7.2.1	General

##### 7.2.1.1	Summary

This clause defines functional definitions for system integration in Table 7.2.1.1-1. The remainder of this

Table 7.2.1.1-1	Functional Definitions

| Term | Summary | Details |
| --- | --- | --- |
| Codec String | A single value identifying the codec indicated to render the content in the Bitstream as defined in IETF RFC 6381. | 7.2.1.2 |
| Decoder Configuration | a data structure storing essential parameters needed for decoding and rendering a video stream. | 7.2.1.3 |
| Random Access Point | A byte position in the Bitstream, for which in combination with the Decoder Configuration, the Bitstream can be randomly accessed, i.e. in decoding order the Bitstream carries sufficient information to access the media in the stream. | 7.2.1.4 |
| Access Unit (AU) | See Clause 3.1 |  |
| Coded access unit (CAU) | bits corresponding to an Access Unit | 7.2.1.5 |
| Random Access CAU | A CAU that starts with a random access point | 7.2.1.6 |
| Coded Video Layer (CVL) | See Clause 3.1 | 7.2.1.7 |

* * * Third Change * * * *

##### 7.2.1.7	Coded Video Layer

A CVL represents a component of a video signal (e.g., luma, chroma, auxiliary data).

- Layer Identification : Each CVL is identified with a unique layer ID in the bitstream and decoder configuration.

- Dependency Signaling : CVL dependencies for decoding purpose, if any, is typically declared in the Video Parameter Set (VPS) or SEI messages.

##### Random Access : A Random Access Point (RAP) in a CVS shall enable decoding of all CVLs starting from that point.

Different types of Coded Video Layer exist:

-	Independent CVL is a CVL which does not depend on any other CVL in the CVS for prediction purposes.

-	Output CVL is a CVL whose coded pictures are output after decoding.

-	Base CVL is an Independent CVL and Output CVL and it is the first CVL in the CVS.

-	Dependent CVL is a CVL that depends on another CVL for prediction purposes.

-	Auxiliary CVL is an Output CVL that is not the first CVL of the CVS.

Editor’s Note: This needs to be completed.

* * * End of Changes * * * *