3GPP TSG-SA WG4#133-e	S4-251540

revision of S4-251260

revision of S4aA250078

Online, 18-25 July 2025	revision of S4aA250045

Source:	Skylo Technologies

Title:	On parameters for simulation model

Spec:	3GPP TR 26.940

Agenda item:	1.9

Document for:	Agreement

1. Introduction

It was agreed in S4-251154, with an Editor’s Note, that companies may propose candidate TBS values. In line with this, we recommend adding TBS of 536 bits for 160ms bundling in Table 2 and TBS of 808 bits for 320ms bundling in Table 3.

For 320ms bundling configuration, the proposed TBS value of 808 bits (current TBS options: 328, 776, 1000, 1544 bits) fits well between the existing 776 and 1000 bits. This supports practical deployment, with IRU=6 and ITBS=6, accommodating 8 RUs with a total transmission duration of 256ms in the case of 3.75KHz SCS.

For 160ms bundling configuration, the proposed TBS value of 536 bits (current TBS options: 208, 424, 600, 808 bits) fits well between the existing 424 and 600 bits. This supports practical deployment, with IRU=3 and ITBS=8, accommodating 4 RUs with a total transmission duration of 128ms in the case of 3.75KHz SCS.

In addition, repetition plays an important role in IoT NTN scenarios where UE transmit power is limited and channel conditions are challenging. The number of repetitions, such as 1, 2, 4, and 8 should be considered in simulations for a subset of TBS values.

2. Reason for Change

Including intermediate TBS values in simulation enables validation across a wider range of application-layer payload sizes and allows for more thorough evaluation of optimal payload sizes under varying traffic and channel conditions.

In addition, the number of repetitions should be considered in simulations for a subset of TBS values. The number of repetition levels 1, 2, 4 and 8 reflect realistic coverage enhancement requirements in NTN scenarios. They are critical for evaluating performance under varying link budgets, especially where UE transmit power is limited, and to ensure robustness in challenging satellite channel conditions. Including these repetition levels enables a more comprehensive performance assessment across diverse deployment conditions.

The channel model is updated with NTN-TDLC5 as specified in TS 36.102.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 26.940 or to the permanent document.

* * * First Change * * * *

### 5.2.2	Simulation Model to derive error traces

The NTN link consists of a service link (between the UE and the satellite) and a feeder link (between the satellite and the ground station). The bottleneck is the service link due to the limited TX power and small antenna at the UE. The feeder link is typically characterized by large capacity and high reliability and can be abstracted as an ideal link in the end-to-end simulation. The RAN simulation addresses the service link only.

The objective is to generate multiple loss traces for a combination of frame loss rate (target BLER), raw bitrate (TBS), voice bundling period and Doppler spread, while maintaining channel consistency among different combinations.

The multiple loss traces are the result of using multiple random seeds, and the number is 10. For each combination, all 10 seeds are used in generating the error traces.

Each trace represents a duration of 400 seconds (or 6.67 minutes). Therefore, for 80ms bundling, there are 5000 TBs, and for 160ms bundling there are 2500 TBs.

The following parameters are for the uplink of the service link.

Channel model: NTN-TDL-C [38811]

Modulation: QPSK

SCS: 3.75kHz, 15kHz

Number of tones: 1

Number of repetitions: company to report in simulation

Voice bundling period: 80ms, 160ms, 320ms

NOTE: the 40ms bundling is not considered because for SCS 3.75kHz the minimum time-domain allocation is 32ms and it leaves insufficient time for downlink data (NPDSCH) and control (NPDCCH) transmissions in the same 40ms time interval.

Doppler spread: 1Hz, 5 Hz

Target BLER: 1%, 2%, 6%, 10%

Target SNR:  Company report, 3GPP SET-1 SNR +[X] dB, X is TBD, where 3GPP SET-1 parameters are defined in [38821] and the [X] dB accounts for better performance of commercial satellites.

TBS values and PHY bitrates: The TBS values are selected from table 16.5.1.2-2 for NB-IoT for NPUSCH in TS36.213 and the corresponding PHY bitrates and codec bitrate (assuming 8 bytes of packet header with RoHC) are calculated for each bundling period.

[

Table 1 TBS and PHY bitrate for 80ms bundling

| TBS (bits) | 144 | 256 | 328 | 424 |
| --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | 1.8 | 3.2 | 4.1 | 5.3 |
| Codec bitrate (kbps) | 1.0 | 2.4 | 3.3 | 4.5 |

Table 2 TBS and PHY bitrate for 160ms bundling

| TBS (bits) | [208] | 424 | 536 | 600 | 808 |
| --- | --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | [1.30] | 2.65 | 3.35 | 3.85 | 5.05 |
| Codec bitrate (kbps) | [0.9] | 2.25 | 2.95 | 3.45 | 4.65 |

Table 3 TBS and PHY bitrate for 320ms bundling

| TBS (bits) | [328] | 776 | 808 | 1000 | 1544 |
| --- | --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | [1.025] | 2.425 | 2.525 | 3.125 | 4.825 |
| Codec bitrate (kbps) | [0.8025] | 2.225 | 2.325 | 2.925 | 4.625 |

]

Editor’s NOTE: Company can report candidate values of TBS.

NOTE: A PHY bitrate is considered feasible if the required SNR to meet a target BLER is lower than the target SNR.

Channel consistency: The same set of channel realizations are used across all combinations.

* * * End of Changes * * * *