3GPP TSG-SA WG4 Meeting #134	S4-251698

# Dallas, US, 17 – 21 November 2025

Source:	Nokia, Fraunhofer HHI

Title:	On Compression of AI/ML data in IMS

Agenda item:	10.5

Document for:	Discussion and agreement

# 1. Introduction

In this contribution, we argue for considering efficient compression techniques for the transport of AI/ML data in IMS services, and, in particular, for specifying MPEG’s Neural Network Coding standard ISO/IEC 15938-17 (NNC) [1] as a representation format.

# 2. Discussion

Given the rapid growth of AI/ML model sizes, the need for solutions for frequent AI/ML data exchange is increasingly critical. Use cases identified by SA1 [2] and targeted by SA4 [3] demonstrate this:

- Model delivery for local UE inference presents significant challenges. Several use cases require not just a one-time download of a single AI/ML model, but multiple downloads depending on context, such as location, time, or specific task requirements. Limited local storage on UEs further complicates this scenario, as models on the device may be discarded and re-downloaded [2].

- Beyond the download of entire models, some applications require sending incremental AI/ML model data updates, either unidirectional for continuously updating an AI/ML model on a UE, or multidirectional, e.g., for co-learning of multiple UEs and edge nodes.

- Split inferencing heavily depends on the frequent exchange of intermediate data between edge devices and UEs.

In all use cases, the advantages of compression are:

- Bandwidth Optimization: The reduced size of compressed models minimizes data transfer needs, allowing for lower operational costs.

- Reduced Latency: By minimizing data size, compressed models can be transmitted faster to UEs and edge devices, enhancing performance in real-time applications.

- Broader Accessibility: Efficiently compressed AI/ML data enables usage of AI/ML applications even in networks with reduced bandwidth.

Beyond compression, a standardized data format for exchanging AI/ML models can ensure seamless interoperability, which is important for scenarios that involve multiple parties. For example, language translation applications may use AI/ML models from different third-party providers in different regions, or applications may transmit a generic AI/ML model to an application server for execution.

MPEG’s Neural Network Coding standard ISO/IEC 15938-17 (NNC) is well-suited to address these needs. The technology has been evaluated in SA4 [4] and MPEG [5][6], demonstrating a compression to 0.1% to 20% of original AI/ML data size with transparent performance, depending on the application scenario. Beyond compression of AI/ML data, NNC provides several advanced features that enhance the transmission of AI/ML data:

- Topology Signalling: Encodes the AI/ML model’s architecture using generic syntax.

- Random Access: Allows independent decoding of tensors and thus parallel decoding.

- Parameter Update Signalling: Encodes metadata indicating the dependencies and relations of incremental AI/ML data updates.

- Robustness and Error Resilience: Enables parameter sets and tensor data to be packetized into distinct data units which allows configurable prioritization or error-protection levels; Enables detection of missing parameter updates.

- Performance Indicator: Signals the performance of the encoded AI/ML model, such as accuracy.

- Encapsulation Flexibility: Facilitates integration of existing AI/ML formats, such as PyTorch, ONNX, NNEF, and TensorFlow, and enables generic support for other formats.

For more details on the capabilities, syntax, and advantages of the ISO/IEC 15938-17 NNC standard, including its second edition enhancements for differential updates and federated learning, refer to the annex of this document.

In conclusion, compression is essential to enabling performant AI/ML services over IMS. MPEG’s NNC standard not only delivers efficient compression but also specifies a versatile high-level syntax that facilitates the transmission of AI/ML data.

# 3. Proposal

We propose to take into account the above discussion and consider the inclusion of NNC-based compression in IMS-based AI/ML services.

# References

- "ISO/IEC 15938-17:2024 Information technology - Multimedia content description interface - Part 17: Compression of neural networks for multimedia content description and analysis”, Edition 2, ISO/IEC, January 2024.

- 3GPP TS 22.261: “Service requirements for the 5G system”.

- 3GPP TR 26.927: “Study on Artificial Intelligence and Machine Learning in 5G media services”

- 3GPP TR 26.847: "Evaluation of Artificial Intelligence and Machine Learning in 5G media services".

- ISO/IEC JTC 1/SC 29/AG 03 N139, “White Paper on Neural Network Coding”, January 2024.

- ISO/IEC JTC 1/SC 29/WG 04 N0696, “Application and Verification of NNC in Different Use Cases”, Daejeon, KR, July 2025.

# Annex: NNC capabilities and syntax

## Data components

NNC provides syntax elements for compressed and uncompressed representations of neural network (NN) model parameters, tensors in AI-based media processing (e.g. intermediate feature maps), NN topology data, meta data, and performance data of the represented NN. NNCs carries these syntax elements in neural network representation (NNR) units of different types. An overview of general bitstream syntax elements is given in ISO/IEC 15938-17 (NNC) [1] clause 6.2.

### Payload types

NNC specifies the representation of compressed and uncompressed payload types in NNR compressed data units (NNR_NDU). As shown in Table 1, the payload type can be specified by the syntax element nnr_compressed_data_unit_payload_type. Payload of type NNR_PT_RAW_FLOAT is not compressed. For other payload types, NNC applies context-adaptive entropy coding using DeepCABAC.

| Payload identifier | Compressed parameter type ID | Payload |
| --- | --- | --- |
| NNR_PT_INT | - | integer parameter tensor |
| NNR_PT_FLOAT | - | float parameter tensor |
| NNR_PT_RAW_FLOAT | - | uncompressed float parameter tensor |
| NNR_PT_BLOCK |  | float parameters that form a block in the model architecture: |
|  | NNR_CPT_DC (0x01) | weight tensor is decomposed |
|  | NNR_CPT_LS (0x02) | local scaling parameters are present |
|  | NNR_CPT_BI (0x04) | biases are present |
|  | NNR_CPT_BN (0x08) | batch norm parameters are present |

Table 1: Payload types of NNR compressed data units

When the payload is of type NNR_PT_BLOCK, the syntax element compressed_parameter_types specifies the signalled parameters. The value of compressed_parameter_types is given by OR-combination of the compressed parameter IDs as show in in Table 1.

For float and integer parameter tensors, NNC supports various bit depths, which can be specified by the syntax element nnr_decompressed_data_format. Furthermore, NNC can also represent pre-quantized float parameter tensors.

### Topology data

NNC signals topology data describing the AI/ML data topology in NNR topology units (NNR_TPL). The storage format of topology data and optional compression are signaled in the NNR_TPL unit header with the syntax elements topology_storage_format, and topology_compression_format. The topology data is signaled in the topology_data syntax element as byte sequence (typically a null‑terminated UTF‑8 string for textual formats). Depending on topology_compression_format, the byte sequence can be uncompressed or deflated according to RFC 1950.

NNR compressed data units (NNR_NDU) comprise the syntax element topology_elem_id or topology_elem_id_index to specify their topology element.

### Meta data

NNC allow to signal meta data describing the coded tensors in the NNR compressed data units (NNR_NDU). Syntax elements tensor_dimensions_flag and tensor_dimension_list( ) specify the tensor dimensions. The syntax element scan_order defines the mapping of tensor parameter values to the tensor dimensions. The syntax elements bit_offset_delta1 and bit_offset_delta2 specify entry points to the bitstream for decoding of individual tensors.

NNC specifies several syntax elements for incremental coding of tensor updates. The syntax elements mps_parent_signalling_enabled_flag and parent_node_id_present_flag specify that an NNR compressed data unit (NNR_NDU) is coded as an increment to a previously coded one. In this context, NNR compressed data units (NNR_NDU) are referred to as nodes in a parameter update tree (PUT). For node identification, NNC specifies the syntax element parent_node_id_type. Depending on parent_node_id_type, nodes are either identified by syntax elements device_id, parameter_id, and put_node_depth, which enumerate device contexts, parameters, and updates, respectively, or by syntax elements parent_node_payload_sha256 and parent_node_payload_sha512, which allow identification based on hash values.

Additionally, NNR model parameter set data units (NNR_MPS) signal global NN meta data and information, including the syntax element base_model_id to indicate the model to which the update parameters are related.

### Performance data

NNC allows to indicate the performance of the coded AI/ML model in NNR model parameter set unit (NNR_MPS) and/or NNR layer parameter set units (NNR_LPS).

The syntax elements validation_set_performance_present_flag, metric_type_performance_map_valid_flag, and performance_metric_type, specify the presence and type of performance metrics of the NN model. Depending on these syntax elements, the syntax element validation_set_performance indicates the performance metric on a validation set.

The syntax structures, sparsification_performance_map( ), pruning_performance_map( ), unification_performance_map( ), and decomposition_performance_map( ) indicate how the decoded NN would perform after applying different variants of sparsification, pruning, unification, and decomposition, respectively, on the decoded tensor parameters.

### Encapsulation of NNEF, ONNX, PyTorch, TensorFlow data

An NNC conformant bitstream can encapsulate NNEF, ONNX, PyTorch, and TensorFlow data. Annexes A to D of NNC specify how to transmit topology and quantization meta data related to these formats in NNR topology data units and NNR quantization data units, respectively.

## Coding tools

### For increased compression performance, NNC enables coding tools by signalling syntax as described in the following.

### Parameter reduction methods

### NNC supports several parameter reduction methods. For these methods, NNC signals data for tensor reconstruction, including enabling flags, parameters, and additional tensors.

### As discussed in clause 1.1.1, NNR compressed data units with payload type NNR_PT_BLOCK can signal additional parameters for local scaling adaptation, batch norm folding, and tensor decomposition. Additionally, syntax elements decomposition_rank and g_number_of_rows specify the dimensions of the decomposed tensors. These parameters can be used to reconstruct the coded tensors.

### When signalling incremental NN updates, NNR model parameter set units signal the syntax element nnr_pre_flag, which specifies whether predictive residual encoding (PRE) is enabled. When predictive residual encoding is enabled, coded tensor parameters represent the difference between the current parameter update and a previously coded parameter update.

### The quant_tensor syntax structure, which comprises the coded tensor parameters, signals the syntax element row_skip_enabled_flag. When enabled, the row-skipping mechanism signals row_skip_list syntax elements that specify which rows of a tensor are entirely zero.

### Quantization and codebook

The syntax elements lps_quantization_method_flags, mps_quantization_method_flags, and codebook_present_flag specify whether the tensor parameters signalled in the quant_tensor( ) syntax structure are quantized or coded using a codebook.

The syntax element dq_flag in the quant_tensor( ) syntax structure specifies whether uniform or dependent quantization is applied. The quantization step size is specified by syntax elements qp_value, lps_qp_density, and/or mps_qp_density. If dependent quantization is enabled, the syntax element dq_state_list specifies the initial state of the dependent quantization at the beginning of entry points.

When the syntax element codebook_present_flag signals that a codebook is present in the bitstream, tensor parameters are mapped to a different set of integer values, which is signalled in the integer_codebook( ) syntax structure.

### Entropy coding

NNC encodes payloads of types not equal to NNR_PT_RAW_FLOAT using context adaptive binary arithmetic coding (DeepCABAC). DeepCABAC binarizes the tensor values to syntax elements sig_flag, sign_flag, abs_level_greater-flags, and abs_remainder. The syntax element cabac_unary_length specifies how the binarization is performed. For entropy coding, DeepCABAC estimates the probabilities of the binarized syntax elements. The syntax element shift_idx_minus_1 specifies parameters for the estimation model’s initialization and update. Furthermore, syntax elements scan_order, bit_offset_delta1, and cabac_offset_list allow to signal a scan order, entry points for random access to the bitstream, and the state of DeepCABAC at the entry points, respectively.

When NNC carries incremental NN updates, entropy coding supports additional coding modes.  The syntax element temporal_context_modeling_flag specifies whether DeepCABAC probability estimation depends on the tensor parameters of the previously coded tensor. The syntax element hist_dep_sig_prob_enabled_flag specifies whether DeepCABAC probability estimation depends on the tensor parameters of several previously coded tensors.