3GPP TSG-SA WG4 Meeting #134	S4-251752

Dallas, US, 17 – 21 November 2025

3GPP TSG-SA Meeting #110	SP-25nnnn

Baltimore, USA, December 09 – 12, 2025

Title:	Summary for ATIAS_Ph2

Type:	Work Item Summary

Agenda Item:	21.5 (SA)

Source:	Jan Reimes

Contact:	jan.reimes@head-acoustics.com

Summary based on the input provided by HEAD acoustics GmbH in S4-251752.

| Unique ID | Name | Acronym | WID | WI Rapporteur name |
| --- | --- | --- | --- | --- |
| 1050113 | Terminal Audio quality performance and Test methods for Immersive Audio Services, Phase 2 | ATIAS_Ph2 | SP-241314 | Jan Reimes, HEAD acoustics GmbH, |

Introduction

The work item ATIAS_Ph2 continued and expanded upon previous work on Terminal Audio quality performance and Test methods for Immersive Audio Services (ATIAS). The focus was on test setups/infrastructure, test methods and requirements needed to evaluate assessment of immersive audio terminals supporting IVAS codec. The work addressed gaps remaining from Rel-18 and concluded with updated specifications that are highly relevant for the industry.

Description

ATIAS_Ph2 developed and consolidated test methods in TS 26.260 [1] and performance requirements in TS 26.261 [2] for immersive UEs across several technical areas:

Test setups and UE configurations: Clarifications on UE types, bitrate settings and simultaneous capture modes/capabilities.

Acoustic echo control for immersive communication: Initial work in this area introduced test methods evaluating acoustic echo control for immersive scenarios. Methodologies for terminal coupling loss were adapted from principles in related standards on mono telephony. Provisional performance requirements were specified for all send/receive UE types.

Headtracking and rendering capabilities: Substantial progress was achieved for evaluating headtracking performance of immersive UEs. Measurement setup, test methods and requirements involving motorized Head and Torso Simulators (HATS) were specified to quantify the relationship between head movement and associated audio scene adaptation in terms of motion-to-sound latency.

Ambient sound field transmission: Efforts in this area resulted in new procedures for testing the transparent capture of transmitted ambient sound fields. Test cases and requirements addressed loudness, frequency response and temporal characteristics. Emerging developments in IVAS_Ph2 regarding new SDP parameters to control noise suppression functionalities in the UE were taken into account [3].

Enhanced testing for single sound source: To accommodate an increased spatial resolution, the existing test methods on half-circular planar capture capabilities were extended for full-circular planar and 3D spherical capabilities. In addition, new performance requirements and objectives for SBA and MASA were introduced to accompany this.

References

[1]	3GPP TS 26.260: "Objective test methodologies for the evaluation of immersive audio systems"

[2]	3GPP TS 26.261: "Terminal audio quality performance requirements for immersive audio services"

[3]	3GPP TS 26.253: "Codec for Immersive Voice and Audio Services; Detailed Algorithmic Description incl. RTP payload format and SDP parameter definitions"