Source:	Beijing Xiaomi Mobile Software

Title:	Simulation results for FS_ULBC

## Document for:	Discussion

## Agenda Item:	7.9

# Introduction

During the June 4, 2025 Audio SWG post 132 meeting, there were a lot of discussions on “Link budget analysis” and T“TBS” for the simulation model, many remaining parameters are still for further discussion, however, in order to save time, interest company can start to simulate the common numbers of the proposals listed in black text in S4aA250038[1], and the simulations will follow the NTN-TDL-C channel model as specified in TS 36.102[2]. The source has done some simulations for several cases with assumed simulation parameters based on the discussions. The details are described in the following clauses.

Based on the previous simulation results, this document adds the missing simulation results for NTN-TDL-C 10 NPUSCH and adds the simulation results for NTN-TDL-C 10 NPDSCH.

# Discussion

## Channel model

When performing the link budget calculation, we assume the satellite elevation is 12.5 degrees. When conducting the LLS simulation, the channel parameters of the channel model, including the delay and power of each path for the channel model of NTN-TDL-C, should also be determined according to the satellite elevation of 12.5 degrees (which can be approximated as 10 degrees).

## Link budget analysis

The CNR is calculated as follows, refer to TR38.821[3]

CNR dB=EIRP [dBW]+GT[dB/K]-k [dBW/K/Hz]-PLFS [dB]-PLA [dB]-PLSM [dB]-PLSL[dB]-PLAD [dB]-B [dBHZ]

For the link budget, according to the incoming LS from RAN1[4], the following CNR values are taken as the baseline:

- UL CNR = 2.6dB, 0dBi UE antenna gain, 3.75kHz SCS, 1 tone, UE maximum TX power 23dBm

- DL CNR=-3.3dB, 0dBi UE antenna gain, 15kHz SCS, 12 tones, 1 UE receive antenna, noise figure of 7dB

Regarding the UL CNR, in addition to the value above, we can also obtain other values corresponding to different subcarrier spacings (SCS) and/or different UE transmission powers. The specific details are shown in the Table 2.2-1.

Table 2.2-1 UL CNR results under different assumptions

|  | SCS | UE power | UL CNR |
| --- | --- | --- | --- |
| Configuration 1 | 3.75kHz | 23dBm | 2.6 dB |
| Configuration 2 | 15kHz | 23dBm | -3.42 dB |
| Configuration 3 | 3.75kHz | 26dBm | 5.6 dB |
| Configuration 4 | 15kHz | 26dBm | -0.42 dB |
| Configuration 5 | 3.75kHz | 31dBm | 10.6 dB |
| Configuration 6 | 15kHz | 31dBm | 4.58 dB |

As for the DL CNR, in addition to the value specified above, we can also obtain the values outlined in Table 2.2-2 for other G/T values that can be supported in commercial implementations (e.g., -28.6 dB) or for the DL CNR with 2 receive antennas (up to 3 dB performance gain). It should be noted that consensus has not been reached on other G/T values during the RAN1 meeting discussions, and these values may could be further deliberated in this meeting.

Table 2.2-2 DL CNR results under different assumptions

|  | Number of Rx | G/T value | UL CNR |
| --- | --- | --- | --- |
| Configuration 1 | 1 | -31.6 | -3.3 dB |
| Configuration 2 | 2 | -31.6 | -0.3 dB |
| Configuration 3 | 1 | -28.6 | -0.3 dB |
| Configuration 4 | 2 | -28.6 | 2.7 dB |

## Simulation results

### NPUSCH

The common simulation parameters are in Table 2.3.1-1

Table 2.3.1-1 Common simulation parameters

|  | Parameter | value |
| --- | --- | --- |
| scenario | orbit | GEO |
|  | Elevation angle | 12.5 degree (10 degree) |
| Channel and impairments | carrier frequency | 2GHz |
|  | Channel model | NTN-TDL-C The channels from different UE are independent. |
|  | Delay spread | 5ns |
|  | TO/FO | Not assumed |
| transmitter | Physical channel | NPUSCH format 1 |
|  | SCS | 3.75KHz and 15KHz |
|  | Number of tones | Single tone |
|  | Waveform | SC-FDMA |
|  | Antenna number | 1 |
|  | MIMO scheme | SISO |
|  | DMRS configuration | OS#4 per slot for 3.75kHz OS#3 per slot for 15kHz |
|  | Velocity of UE | 3km/h |
|  | #symbols per slot | 7 |
|  | #Slots per RU | 16 |
|  | Modulation order | Pi/4-QPSK |
| receiver | Antenna number | 1 |
|  | Receiver algorithm | MMSE |
|  | Channel estimation | Real channel estimation |
| KPI | SNR at target BLER | Target BLER: 1%, 2%, 6%, 10% |

In this simulation, the assumption of the determination of repetition number is to ensure the total transmission delay between uplink and downlink meeting the bundling time constraint, while allocating specific time-domain resources for downlink transmission, under this assumption to get the maximum repetition number for uplink.

- 80ms bundling time

  - 144 bits

Table 2.3.1-2 Combinations of TBS 144bit w/ 80ms bundling time

| scheme | TBS | SCS | MCS | Number of RU | Repetition number | PHY bitrate(kbps) | Codec bitrate(kbps) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | 144 | 15 | 2 | 4 | 2 | 1.8 | 1.0 |
| Case 2 |  | 15 | 2 | 2 | 1 |  |  |
| Case 3 |  | 3.75 | 10 | 1 | 2 |  |  |
| Case 4 |  | 15 | 10 | 1 | 4 |  |  |

Table 2.3.1-3 SNR at target BLER with TBS 144bit w/ 80ms bundling time

| BLER | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| SNR(case1) | -4.97 | -4.83 | -4.54 | -4.35 |
| SNR(case2) | 1.8 | 1.9 | 2.3 | 2.7 |
| SNR(case3) | 1.5 | 1.71 | 2.11 | 2.30 |
| SNR(case4) | -4.64 | -4.46 | -4.07 | -3.90 |

  - 256 bits

Table 2.3.1-4 Combinations of TBS 256bit w/ 80ms bundling time

| scheme | TBS | SCS | MCS | Number of RU | Repetition number | PHY bitrate(kbps) | Codec bitrate(kbps) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 5 | 256 | 15 | 3 | 5 | 1 | 3.2 | 2.4 |
| Case 6 |  | 3.75 | 8 | 2 | 1 |  |  |
| Case 7 |  | 3.75 | 8 | 2 | 1 |  |  |
| Case 8 |  | 15 | 8 | 2 | 4 |  |  |

Table 2.3.1-5 SNR at target BLER with TBS 256bit w/ 80ms bundling time

| BLER | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| SNR(case5) | -0.89 | -0.77 | -0.5 | -0.33 |
| SNR(case6) | 5.1 | 5.5 | 5.8 | 5.9 |
| SNR(case7) | 5.02 | 5.18 | 5.51 | 5.73 |
| SNR(case8) | -2.84 | -2.71 | -2.45 | -2.28 |

  - 328 bits

Table 2.3.1-6 Combinations of TBS 328bit w/ 80ms bundling time

| scheme | TBS | SCS | MCS | Number of RU | Repetition number | PHY bitrate(kbps) | Codec bitrate(kbps) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 9 | 328 | 15 | 4 | 5 | 1 | 4.1 | 3.3 |
| Case 10 |  | 3.75 | 10 | 2 | 1 |  |  |
| Case 11 |  | 15 | 10 | 2 | 4 |  |  |

Table 2.3.1-7 SNR at target BLER with TBS 328bit w/ 80ms bundling time

| BLER | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| SNR(case9) | 0.43 | 0.66 | 0.90 | 0.95 |
| SNR(case10) | 8.26 | 8.53 | 9.00 | 9.45 |
| SNR(case11) | -1.53 | -1.34 | -1.09 | -1.01 |

  - 424 bits

Table 2.3.1-8 Combinations of TBS 424bit w/ 80ms bundling time

| scheme | TBS | SCS | MCS | Number of RU | Repetition number | PHY bitrate(kbps) | Codec bitrate(kbps) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 12 | 424 | 15 | 5 | 5 | 1 | 5.3 | 4.5 |

Table 2.3.1-9 SNR at target BLER with TBS 424bit w/ 80ms bundling time

| BLER | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| SNR(case12) | 1.44 | 1.58 | 1.88 | 2.05 |

- 160ms bundling time

  - 208 bits

Table 2.3.1-10 Combinations of TBS 208bit w/ 160ms bundling time

| scheme | TBS | SCS | MCS | Number of RU | Repetition number | PHY bitrate(kbps) | Codec bitrate(kbps) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 13 | 208 | 15 | 2 | 6 | 2 | 1.3 | 0.9 |
| Case 14 |  | 3.75 | 4 | 3 | 1 |  |  |
| Case 15 |  | 15 | 4 | 3 | 4 |  |  |

Table 2.3.1-11 SNR at target BLER with TBS 208bit w/ 160ms bundling time

| BLER | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| SNR(case13) | -5.56 | -5.4 | -5.04 | -4.89 |
| SNR(case14) | 0.91 | 1.07 | 1.36 | 1.53 |
| SNR(case15) | -5.41 | -5.21 | -4.9 | -4.76 |

  - 424 bits

Table 2.3.1-12 Combinations of TBS 424bit w/ 160ms bundling time

| scheme | TBS | SCS | MCS | Number of RU | Repetition number | PHY bitrate(kbps)） | Codec bitrate(kbps) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 16 | 424 | 15 | 5 | 5 | 2 | 2.65 | 2.25 |

Table 2.2-13 SNR at target BLER with TBS 424bit w/ 160ms bundling time

| BLER | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| SNR(case16) | -1.95 | -1.85 | -1.65 | -1.52 |

  - 600 bits

Table 2.3.1-14 Combinations of TBS 600bit w/ 160ms bundling time

| scheme | TBS | SCS | MCS | Number of RU | Repetition number | PHY bitrate(kbps) | Codec bitrate(kbps) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 17 | 600 | 15 | 6 | 6 | 2 | 3.75 | 3.35 |

Table 2.3.1-15 SNR at target BLER with TBS 600bit w/ 160ms bundling time

| BLER | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| SNR(case17) | -1.38 | -1.28 | -1.08 | -0.97 |

  - 808 bits

Table 2.3.1-16 Combinations of TBS 808bit w/ 160ms bundling time

| scheme | TBS | SCS | MCS | Number of RU | Repetition number | PHY bitrate(kbps) | Codec bitrate(kbps) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 18 | 808 | 15 | 6 | 8 | 2 | 5.05 | 4.65 |
| Case 19 |  | 15 | 8 | 6 | 2 |  |  |

Table 2.3.1-17  SNR at target BLER with TBS 808bit w/ 160ms bundling time

| BLER | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| SNR(case18) | -1.42 | -1.35 | -1.18 | -1.07 |
| SNR(case19) | -0.15 | -0.03 | 0.12 | 0.21 |

- 320ms bundling time

  - 328 bits

Table 2.3.1-18 Combinations of TBS 328bit w/ 320ms bundling time

| scheme | TBS | SCS | MCS | Number of RU | Repetition number | PHY bitrate(kbps) | Codec bitrate(kbps) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 20 | 328 | 3.75 | 3 | 6 | 1 | 1.025 | 0.8250 |
| Case 21 |  | 15 | 3 | 6 | 4 |  |  |
| Case 22 |  | 3.75 | 4 | 5 | 1 |  |  |
| Case 23 |  | 15 | 4 | 5 | 4 |  |  |
| Case 24 |  | 3.75 | 10 | 2 | 4 |  |  |
| Case 25 |  | 15 | 10 | 2 | 8 |  |  |

Table 2.3.1-19 SNR at target BLER with TBS 328bit w/ 320ms bundling time

| BLER | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| SNR(case20) | -0.74 | -0.63 | -0.38 | -0.22 |
| SNR(case21) | -6.80 | -6.69 | -6.47 | -6.32 |
| SNR(case22) | 0.21 | 0.32 | 0.56 | 0.72 |
| SNR(case23) | -5.93 | -5.83 | -5.62 | -5.49 |
| SNR(case24) | -1.47 | -1.31 | -0.98 | -0.84 |
| SNR(case25) | -4.61 | -4.47 | -4.17 | -3.99 |

  - 776 bits

Table 2.3.1-20 Combinations of TBS 776bit w/ 320ms bundling time

| scheme | TBS | SCS | MCS | Number of RU | Repetition number | PHY bitrate(kbps) | Codec bitrate(kbps) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 26 | 776 | 3.75 | 9 | 5 | 1 | 2.425 | 2.225 |
| Case 27 |  | 15 | 9 | 5 | 4 |  |  |

Table 2.3.1-21 SNR at target BLER with TBS 776bit w/ 320ms bundling time

| BLER | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| SNR(case26) | 6.02 | 6.12 | 6.33 | 6.46 |
| SNR(case27) | -2.48 | -2.35 | -2.08 | -1.94 |

  - 1000 bits

Table 2.3.1-22 Combinations of TBS 1000bit w/ 320ms bundling time

| scheme | TBS | SCS | MCS | Number of RU | Repetition number | PHY bitrate(kbps) | Codec bitrate(kbps) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 28 | 1000 | 15 | 6 | 10 | 2 | 3.125 | 2.925 |
| Case 29 |  | 3.75 | 10 | 6 | 1 |  |  |
| Case 30 |  | 15 | 10 | 6 | 4 |  |  |

Table 2.3.1-23 SNR at target BLER with TBS 1000bit w/ 320ms bundling time

| BLER | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| SNR(case28) | -1.5 | -1.44 | -1.31 | -1.23 |
| SNR(case29) | 6.97 | 7.09 | 7.33 | 7.47 |
| SNR(case30) | -1.95 | -1.89 | -1.75 | -1.66 |

  - 1544 bits

Table 2.3.1-24 Combinations of TBS 1544bit w/ 320ms bundling time

| scheme | TBS | SCS | MCS | Number of RU | Repetition number | PHY bitrate(kbps) | Codec bitrate(kbps) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 31 | 1544 | 15 | 9 | 10 | 2 | 4.825 | 4.625 |

Table 2.3.1-25 SNR at target BLER with TBS 1544bit w/ 320ms bundling time

| BLER | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| SNR(case31) | 0.48 | 0.58 | 0.69 | 0.76 |

### NPDSCH

The common simulation parameters are in Table 2.3.2-1

Table 2.3.2-1 Common simulation parameters

|  | Parameter | value |
| --- | --- | --- |
| scenario | orbit | GEO |
|  | Elevation angle | 10 degree |
| Channel and impairments | carrier frequency | 2 GHz |
|  | Channel model | NTN-TDL-C |
|  | Doppler spread | 5 Hz |
|  | FO/TO | Not assumed |
| transmitter | Physical channel | NPDSCH |
|  | SCS | 15 kHz |
|  | Number of tones | 12 |
|  | Waveform | OFDM |
|  | Antenna number | 1 |
|  | MIMO scheme | SISO |
|  | #symbols per subframe | 14 |
|  | Number of subframes | 4 |
|  | Number of repetitions | 2 |
|  | TBS | 144 bits |
|  | Voice bundling period | 80ms |
|  | Modulation order | QPSK |
| receiver | Antenna number | 1 |
|  | Receiver algorithm | MMSE |
|  | NRS-bundling | Assumed |
|  | Channel estimation | Real channel estimation with all slots NRS-bundling |
| KPI | Target BLER | 1%, 2%, 6%, 10% |

TBS, number of subframes per repetition, number of repetitions, and MCS are in the following tables.

- 80ms bundling time

  - Table 2.3.2-1: 144 bits

| Scheme | TBS | SCS | MCS | Number of subframe | Repetition number | DL duration time | DL+guard duration time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 0a | 144 | 15 | 1 | 4 | 2 | 8 | 16 |

  - Table 2.3.2-2: 256 bits

| Scheme | TBS | SCS | MCS | Number of subframe | Repetition number | DL duration time | DL+guard duration time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 0b | 256 | 15 | 1 | 8 | 1 | 8 | 16 |

Table 2.3.2-3 SNR at target BLER for 80 ms bundling time

| Required SNR | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| Case 0a (1R) | -6.6 | -6.3 | -5.8 | -5.3 |
| Case 0a (2R) | -9.3 | -9.1 | -8.4 | -8.1 |
| Case 0b (1R) | -4.3 | -4.1 | -3.4 | -3.1 |
| Case 0b (2R) | -7.1 | -6.8 | -6.2 | -6.1 |

  - Table 2.3.2-4: 328bits

| Scheme | TBS | SCS | MCS | Number of subframe | Repetition number | DL duration time | DL+guard duration time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | 328 | 15 | 2 | 8 | 4 | 32ms | 40ms |
| Case 2 |  |  | 3 | 6 | 2 | 12ms | 16ms |

  - Table 2.3.2-5: 424bits

| scheme | TBS | SCS | MCS | Number of subframe | Repetition number | DL duration time | DL+guard duration time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 3 | 424 | 15 | 2 | 10 | 2 | 20 ms | 40ms |
| Case 3b | 424 | 15 | 2 | 10 | 4 | 40 ms | 40ms |

Table 2.3.2-6 SNR at target BLER for 80 ms bundling time

| Required SNR | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| Case1(1R) | -9.1 | -9.0 | -8.3 | -8.1 |
| Case1(2R) | -11.8 | -11.4 | -11.0 | -10.8 |
| Case2(1R) | -5.0 | -4.5 | -4.1 | -4.0 |
| Case2(2R) | -7.8 | -7.5 | 7.1 | -7.0 |
| Case3(1R) | -6.1 | -5.7 | -5.2 | -5.0 |
| Case3(2R) | -9.0 | -8.6 | -8.1 | -8.0 |
| Case3b(1R) | -8.8 | -8.5 | -8.1 | -7.8 |
| Case3b(2R) | -11.6 | -11.3 | -11.0 | -10.5 |

- 160 ms bundling time

  - Table 2.3.2-7: 208bits

| scheme | TBS | SCS | MCS | Number of subframe | Repetition number | DL duration time | DL+guard duration time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 4 | 208 | 15 | 1 | 6 | 8 | 48ms | 64 ms |

  - Table 2.3.2-8: 424bits

| scheme | TBS | SCS | MCS | Number of subframe | Repetition number | DL duration time | DL+guard duration time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 5 | 424 | 15 | 2 | 10 | 4 | 40ms | 80 ms |
| Case 5b | 424 | 15 | 2 | 10 | 8 | 80ms | 80 ms |

  - Table 2.3.2-9: 600bits

| scheme | TBS | SCS | MCS | Number of subframe | Repetition number | DL duration time | DL+guard duration time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 6 | 600 | 15 | 6 | 6 | 8 | 48ms | 64 ms |

  - Table 2.3.2-10: 808bits

| scheme | TBS | SCS | MCS | Number of subframe | Repetition number | DL duration time | DL+guard duration time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 7 | 808 | 15 | 6 | 8 | 2 | 16ms | 32 ms |
| Case 7b |  |  | 6 | 8 | 4 | 32ms | 32ms |
| Case 8 |  |  | 6 | 8 | 4 | 32ms | 64 ms |
| Case 8b |  |  | 6 | 8 | 8 | 64ms | 64 ms |

Table 2.3.2-11 SNR at target BLER for 160 ms bundling time

| Required SNR | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| Case4(1R) | -12.8 | -12.4 | -12.1 | -11.8 |
| Case4(2R) | -15.3 | -15.2 | -14.7 | -14.3 |
| Case5(1R) | -9.1 | -8.7 | -8.2 | -8.0 |
| Case5(2R) | -11.8 | -11.4 | -11.1 | -10.8 |
| Case5b(1R) | -12.0 | -11.6 | -11.1 | -11.0 |
| Case5b(2R) | -14.6 | -14.3 | -14.0 | -13.7 |
| Case6(1R) | -8.2 | -8.1 | -7.4 | -7.2 |
| Case6(2R) | -11.1 | -10.9 | -10.3 | -10.1 |
| Case7(1R) | -2.1 | -2.0 | -1.3 | -1.1 |
| Case7(2R) | -5.1 | -5.0 | -4.3 | -4.1 |
| Case7b(1R) | -4.9 | -4.5 | -4.1 | -3.9 |
| Case7b(2R) | -7.8 | -7.4 | -7.1 | -7.0 |
| Case8(1R) | -5.1 | -5.0 | -4.3 | -4.1 |
| Case8(2R) | -8.1 | -7.8 | -7.2 | -7.1 |
| Case8b(1R) | -8.0 | -7.7 | -7.2 | -7.1 |
| Case8b(2R) | -11.0 | -10.6 | -10.2 | -10.0 |

- 320 ms bundling time

  - Table 2.3.2-12: 328 bits

| scheme | TBS | SCS | MCS | Number of subframe | Repetition number | DL duration time | DL+guard duration time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 9 | 328 | 15 | 2 | 8 | 8 | 64ms | 128 ms |
| Case 9b |  |  | 2 | 8 | 16 | 128 | 128 |
| Case 10 |  |  | 10 | 2 | 64 | 128ms | 160 ms |
| Case 11 |  |  | 2 | 8 | 4 | 32ms | 64 ms |
| Case 11b |  |  | 2 | 8 | 8 | 64ms | 64 ms |

  - Table 2.3.2-13: 776bits

| scheme | TBS | SCS | MCS | Number of subframe | Repetition number | DL duration time | DL+guard duration time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 12 | 776 | 15 | 9 | 5 | 16 | 80ms | 160ms |
| Case 12b | 776 | 15 | 9 | 5 | 32 | 160ms | 160ms |

  - Table 2.3.2-14: 1000bits

| scheme | TBS | SCS | MCS | Number of subframe | Repetition number | DL duration time | DL+guard duration time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 13 | 1000 | 15 | 11 | 5 | 16 | 80ms | 128ms |

  - Table 2.3.2-15: 1544bits

| scheme | TBS | SCS | MCS | Number of subframe | Repetition number | DL duration time | DL+guard duration time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 14 | 1544 | 15 | 9 | 10 | 8 | 80ms | 160 ms |
| Case 14b |  |  | 9 | 10 | 16 | 160ms |  |

Table 2.3.2-16 SNR at target BLER for 320 ms bundling time

| Required SNR | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| Case9(1R) | -12.1 | -12.0 | -11.3 | -11.1 |
| Case9(2R) | -15.0 | -14.6 | -14.1 | -14.0 |
| Case9b(1R) | -15.0 | -14.7 | -14.1 | -14.0 |
| Case9b(2R) | -17.7 | -17.4 | -17.1 | -16.9 |
| Case10(1R) | -14.5 | -14.2 | -14.0 | -13.4 |
| Case10(2R) | -17.2 | -17.1 | -16.5 | -16.2 |
| Case11(1R) | -9.1 | -9.0 | -8.3 | -8.1 |
| Case11(2R) | -12.0 | -11.7 | -11.1 | -11.0 |
| Case11b(1R) | -12.0 | -11.7 | -11.2 | -11.0 |
| Case11b(2R) | -14.8 | -14.4 | -14.1 | -13.8 |
| Case12(1R) | -9.0 | -8.7 | -8.2 | -8.1 |
| Case12(2R) | -12.0 | -11.6 | -11.1 | -11.0 |
| Case12b(1R) | -11.9 | -11.5 | -11.1 | -11.0 |
| Case12b(2R) | -14.8 | -14.4 | -14.1 | -14.0 |
| Case13(1R) | -7.3 | -7.2 | -6.8 | -6.4 |
| Case13(2R) | -10.3 | -10.1 | -9.6 | -9.2 |
| Case14(1R) | -6.0 | -5.6 | -5.1 | -5.0 |
| Case14(2R) | -8.9 | -8.5 | -8.1 | -8.0 |
| Case14b(1R) | -8.8 | -8.5 | -8.1 | -8.0 |
| Case14b(2R) | -11.7 | -11.4 | -11.1 | -11.0 |

# Conclusion

It is recommended to consider the above simulation results for determining the design constraints for ULBC.

# References

[1] S4aA250038, Qualcomm, “Calibration of contributions on RAN simulation assumptions for ULBC”.

[2] TS 36.102,” Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception for satellite access”.

[3] TS 38.821," Solutions for NR to support non-terrestrial networks (NTN)".

[4] R1-2506541, Reply LS on the RAN simulation assumptions for ULBC

# ANNEX A : Figures of relationship between BLER and SNR

A.1

A.1b

A.2

A.2b

A.3

A.4

A.5

A.6

A.7

A.8

A.9

A.10

A.11

A.12

A.13

A.14

A.15

A.16

A.17