3GPP TSG-SA WG4 Meeting #134	S4-251814

Dallas, US, 17- 21 November 2025

Source:	China Mobile Com. Corporation

Title:	[FS_ULBC] Discussion of FS_ULBC Objective Speech Quality Assessment Method

Agenda item:	 7.9

Document for:	Discussion and Agreement

- Background

In the recent SA4 meeting, challenges related to speech quality assessment of ultra-low bitrate speech codecs were discussed. Several subjective testing methods for speech quality evaluation were reviewed, along with factors to be considered regarding test materials and criteria. It was agreed that subjective testing remains the benchmark for ULBC codec selection.

As noted in [5], "Subjective listening remains the most important and direct method for evaluating codec performance, although such tests are time- and resource-intensive. In addition to codec quality assessment, objective models that predict perceptual speech quality are essential for various other tasks."

To enable more convenient and efficient quality verification of encoded speech, certain objective speech evaluation methods can serve as predict tools to subjective testing. This document discusses several subjective testing tools that can be considered for predicting speech quality during the intermediate processes of ULBC testing and parameter adjustment.

- Discussion

  - Overview of Existing Speech Objective Quality Evaluation Method(Tools)

| Method/Tool | Standard | Bitrate Range | Reference Type | Strengths | Limitations |
| --- | --- | --- | --- | --- | --- |
| P.863 | ITU-T P.863 | Variable | Full-reference | - Widely adopted in ITU 3GPP - Supports NB, WB, SWB | - |
| P.862 | ITU-T P.862 | ≥ 4 kbps[8] | Full-reference | - Classical standard - Applicable to NB and WB codecs | - Not accurate for ultra-low bitrates[8] - No support for SWB |
| P.563 | ITU-T P.563 | Variable | No-reference | - No need for clean reference - Suitable for real-time | Evaluation of extreme noise or complex distortion may not be as accurate as with full-reference methods |
| ViSQOL | Open Source | Variable[2] | Full-reference | - Performs well for low bitrates[6] | - not standardized - Still under evaluation for 3GPP usage |
| STOI/ESTOI | Open Source | Variable | Full-Reference | -Focus on speech intelligibility, suitable for low bit rate | Not adopted by ITU and 3GPP |
| SCOREQ | - | Variable | No-reference | -no-reference speech-quality access model with strong cross-domain robustness[9] | Not adopted by ITU and 3GPP |

  - Capabilities and Limitations of Existing Speech Objective Quality Evaluation Method for ULBC

P.863 (ITU-T P.863): The most widely adopted ITU-T standard method,  wide range support for narrowband (NB) to super-wideband (SWB). Studies show that P.863 maintains performance below 4kbps in SWB model[7].

P.862 (ITU-T P.862): The traditional full-reference metric for narrowband (NB) and wideband (WB) speech quality assessment, It is suggested not be used to compare similar speech codecs that have vastly different algorithms, especially when the algorithms use a wide variety of nonlinear signal processing[8]. now superseded by POLQA in many scenarios due to its limitations in handling super-wideband (SWB) signals and modern codecs.

P.563: A no-reference method useful for real-time applications but less adaptability to non-linear distortions (e.g. neural codecs)

ViSQOL: An open-source, full-reference , for low bitrates speech under 8 kbps, the correlation coefficient between ViSQOL and MOS still maintains good consistency[2],  all though it is not formally standardized by ITU-T or 3GPP.

STOI/ESTOI: Focuses on speech intelligibility assessment. provides a computationally efficient solution with high correlation to subjective tests in noisy conditions [3]. Its extended version (ESTOI) further improves robustness to nonlinear distortions (e.g., neural codecs)  [4].But they’re  not formally standardized by ITU-T or 3GPP.

SCOREQ : is a novel no-reference speech-quality prediction approach that addresses the domain-generalisation shortcomings of existing metrics. [9].Evaluation across multiple speech domains demonstrates improved robustness to out-of-domain data and better correlation with human judgments compared to prior state-of-the-art models. [9].

Based on the analysis and the mentioned (though not exhaustive) list,  a diverse range of objective assessment tools are available. Therefore, after excluding several methods unsuitable for ultra-low bitrate scenarios from the aforementioned objective testing approaches, we recommend considering P.863, ViSQOL, ESTOI  as potential objective quality assessment method for ULBC.

- Proposal

We propose to document the consideration of optional objective test method for ultra-low bit rate codecs provided above in a pCR to TR 26.940 or PD file as shown below.

- pCR to 26.940

* * * First Change * * * *

# 9	Test methodologies

Editor’s Note:

5b. Identify appropriate test methodologies, regarding speech quality, intelligibility, conversational quality, in particular taking into account:

-	Clean speech and noisy speech

-	Tandeming with existing IMS voice codecs

-	Clean channel and GEO channel conditions

## 9.1	General

### 9.1.1	Typical quality impairments of ultra-low bit rate speech coding

Speech codecs operating at ultra-low bit rates may impact aspects of speech communication in at least the following categories

-	Loss of listening-only audio quality

-	Audio bandwidth loss

-	Impaired intelligibility

-	Impaired speaker identifiability

-	Prosodic impairments

-	Hallucination, i.e. word and phone confusions

A related quality impairment category is sensitivity to non-speech input. Non-speech input in this context may mean any non-clean speech input such as background noise, music, but also noisy speech, interfering talker speech, reverberant speech.

Additionally, speech codecs operating at ultra-low bit rates may imply speech enhancement algorithms, such as noise suppression, gain normalization etc.

Editor’s note: Investigate impairment relevance in context of use case, e.g. emergency call.

### 9.1.2	Challenges of quality assessment of ultra-low bit rate speech codecs

Testing of ULBC introduces new challenges in comparison to signal processing-based codecs that may necessitate additional or alternative evaluation methods compared to previous 3GPP SA4 speech coding development activities.

Conversation-opinion tests are intended to reproduce, in a laboratory situation, the actual service conditions experienced by telephone customers. [ITU-T P.800 Section 6]. For practical reasons, and due to the need of strict test control, listening-opinion tests are often employed as an alternative in speech coding development.

According to ITU-T Rec. P.800, the recommended test method for listening-only tests is the "Absolute Category Rating" (ACR) method described in Annex B of ITU-T P.800 (see ITU-T P.800 Clause 6.2). An alternative to the Absolute Category Rating method is the Degradation Category Rating (DCR) method which is described in detail in Annex D of ITU-T P.800. As this is a comparative method it is suitable when the impairment (especially digital impairments) is small.

Based on the principles of ITU-T Rec. P.800, 3GPP established a practice of listening-only evaluations in the context of the standardization of its voice codecs. AMR, AMR-WB and EVS codecs were all evaluated using P.800 ACR and (modified) DCR test methodologies, the latter also applied for tests with larger impairments such as noisy speech over poor channel conditions. ACR was generally used for clean speech tests while P.800 DCR was used for SWB clean speech, mixed-bandwidth,  speech + background noise and mixed/music quality evaluations. Other aspects relevant to speech communication, such as impaired intelligibility, impaired speaker identifiability and prosodic impairments were not the focus of testing in AMR, AMR-WB and EVS codec standardization.

For ULBC, these other aspects may need to be addressed more directly, through dedicated tests. Hallucination, for example, is a category that plays only a role in ML-based coding systems but not for signal-processing based codecs, which AMR, AMR-WB and EVS are.

While P.800 recommends ACR as the listening-only test method, it may not be the optimal test to quantify all potential impairments in speech communication such as hallucination, impaired intelligibility and prosodic impairments. These impairments are possible in ML-based coding systems and could be covered by alternative tests such as automatic speech recognition methods, modified rhyme tests and DCR tests (see below).

In contrast, DCR methodology generally focuses on differences to a reference. If such differences are small and pertain to prosodic differences, these may not directly impact the conversational capability of a communication system but can be relevant for other aspects such as identity recognition.

Besides the usage of ACR and DCR, further subjective or objective test methodologies are available that are specifically designed for the evaluation the above-listed impairment categories of ultra-low-rate speech codecs. A non-exhaustive list of such methods is:

-	Diagnostic Rhyme Tests (DRT)

-	Modified Rhyme Tests (MRT)

-	MOS testing for speaker similarity

-	Speaker verification/identification tests

-	Prosodic naturalness MOS tests

-	Intonation recognition tests

-	Transcription tests involving testing for word and semantic equivalence

-	Phoneme recognition tests

-	Automatic speech recognition tests

Subjective listening remains the most important and direct method for evaluating codec performance; however, these tests require significant time and resources[5]. In addition to codec quality assessment, objective speech quality evaluation methods such as P.863, ViSQOL, ESTOI etc. can be considered as optional tools for predicting speech quality during the the process of ULBC simulation testing and parameter optimization.

Speech enhancing tool, which may be part of ultra-low bitrate-codecs, are typically evaluated by multi-dimensional rating scales provided by P.835. Here, the quality of speech and the noise suppression capability are evaluated separately.

Editor’s note: Other options for listening methods are invited for documentation.

* * * End of Changes * * * *

- POLQA 2015 (V2.4) Investigated Technical White Paper

- Hines, A., Skoglund, J., Kokaram, A., & Nair, N. (2015). "ViSQOL: An Objective Speech Quality Model."

- Taal, C. H., Hendriks, R. C., Heusdens, R., & Jensen, J. (2011). "An Algorithm for Intelligibility Prediction of Time–Frequency Weighted Noisy Speech."

- Andersen, A. H., de Haan, J. M., Tan, Z.-H., & Jensen, J. (2018). "Refinement and Validation of the Extended Short-Time Objective Intelligibility Measure." IEEE

- Thomas Muller, Stéphane Ragot, Vincent Barriac, Pascal Scalart. EVALUATION OF OBJECTIVE QUALITY MODELS ON NEURAL AUDIO CODECS. 18th International Workshop on Acoustic Signal Enhancement (IWAENC 2024), IEEE signal processing society, Sep 2024, Aalborg, Denmark. hal-04921783

- Hines, A.,Eoin Gillen, Damien Kelly, Jan Skoglund, and Anil Kokaram "ViSQOLAudio: An objective audio quality metric for low bitrate codecs"

- Jan Holub  (2011)."Validation of ITU-T P.863 for low bit-rate coded speech quality measurements"

- Antony W. Rix, John G. Beerends, Doh-Suk Kim, (2006)"Objective Assessment of Speech and Audio Quality—Technology and Applications"

- A. Ragano, J. Skoglund and A. Hines, “SCOREQ: Speech Quality Assessment with Contrastive Regression,” in Proc. of the 38th Neural Information Processing Systems (NeurIPS) 2024.