3GPP SA4 WG4 meeting #134	S4-251821

# Dallas, Texas, 17-21 November 2025

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR | 0015 | rev | 0 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Energy_Ph2_MED] Solution for KI #6 Client-driven management of media delivery service energy optimisation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Energy_Ph2_MED |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addtion of a solution to Key Issue #6 on Client-driven management of media delivery service energy optimisation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a clause 7.14 adding a solution to Key Issue #6 on Client-driven management of media delivery service energy optimisation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | KI#6 may remain unresolved. |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1, 7.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1ST Change

## 7.1	Mapping of Solutions to Key Issues

Table 7.1-1: Mapping of solutions to Key Issues

| Solutions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 |
| #1 |  |  | X |  |  |  |
| #2 |  |  | X |  |  |  |
| #3 |  | X |  |  |  |  |
| #4 | X |  |  |  |  |  |
| #5 | X |  |  |  |  |  |
| #6 | X | X |  |  |  |  |
| #7 | X |  |  |  |  |  |
| #8 |  | X |  |  |  |  |
| #9 | X |  |  |  |  |  |
| #10 |  |  |  |  |  | X |
| #11(a) |  |  |  |  | X |  |
| #11 (b) |  |  |  |  | X |  |
| 12 |  |  |  |  |  | X |

These solutions are simply candidate solutions. Their inclusion in the following clauses does not imply that they have been agreed upon or endorsed. Any decisions and work to be done for the normative work will be detailed in the conclusions of this Technical Report.

2nd change
(All new text)

## 7.14	Solution #12: Client-driven management of media delivery service energy optimisation

### 7.14.1	Key Issue mapping

This solution candidate addresses Key Issue #6 (Client-driven management of media delivery service energy optimisation) described in clause 6.6.

### 7.14.2	Functional description

#### 7.14.2.1	Introduction

Mobile networks may prioritize energy efficiency in either network or UE or both depending upon various situations. While energy-saving techniques aim to optimise energy consumption without compromising service quality, trade-offs may sometimes be necessary. To identify potential energy-saving opportunities, networks may target specific user devices consuming excessive energy and offer incentives for reducing energy consumption in exchange for service adjustments. The network should assess whether target users would accept proposed service adjustments that may result in QoS degradation. Not all users may be willing to tolerate such changes, especially if their current services or content received by service at special time or location are sensitive to performance reductions. If a user agrees to the adjustments, the UE and network can explore potential mitigation strategies to minimize the impact on service experience and QoE.

To this end, the following issues have been identified:

- How can a user's tolerance for reduced QoS and QoE be determined?

- How can the AF propose the strategies to minimize the impacts on service experience and QoE on the user-end?

- How can the UE prioritize strategies to minimize the impacts on the QoE?

This candidate solution proposes method by which AF can assess the QoS degradation (in collaboration with the 5GC and/or the Application Service Provider (ASP)) and notifies a UE application when QoS degradation is imminent due to either network or UE energy saving or both. This enables the UE to proactively adjust QoE metrics, minimizing the impact on user experience. This candidate solution is based on the following:

- Triggering the energy saving toward AF

Authorized entities, such as 5GC or Application Service Providers (ASP) can trigger the AF to request a UE to reduce its QoE. This can occur due to network and/or UE energy-saving initiatives. For instance, 5GC might decide to reduce QoS (e.g. allocated bandwidth) and trigger energy saving through the AF along with the proposed, either QoS or QoE ranges. Additionally, application service providers (ASPs) with contracts with cloud providers or 5GC may initiate energy-saving towards AF. Another example is an application provider with a UE contract for green media streaming could trigger energy saving due to energy status of cloud and 5GC resources.

- Assessing the User tolerance to QoE degradation

Based on this candidate solution, when an AF receives an energy-saving request (in collaboration with the 5GC and/or the Application Service Provider (ASP)), it assesses the potential impact on the user's QoE. By comparing the requested QoE parameters to predefined thresholds, the AF determines the acceptable levels of QoE degradation. If the request includes specific QoS ranges instead of QoE, the AF may request the NWDAF to estimate QoE impacts. The AF may check the user's subscription for agreed-upon QoE levels. If these levels align with the estimated/requested QoE degradation, no user approval is needed, but user feedback is collected. Otherwise, the AF requests user approval, specifying the potential QoE impact.

- UE Controlling the QoE

Upon receiving a request for potential QoE degradation, the UE should assess its current status and determine how to respond in a way that minimizes the impact on its service experience. In order for this to happen, a UE needs to decide whether its application could tolerate a degradation in its current QoE or not. And when it does, the application should be able to manage its current QoE without too much degradation (i.e. reducing the qoe below the certain pre-defined threshold).

### 7.14.3	Procedures

Figure 7.14.3-1 below details the different steps for Client-driven management of media delivery service energy optimisation.

Figure 7.14.3-1: Procedures for Client-driven management of media delivery service energy optimisation

The working assumptions are:

-	initial connection set-up is established between the UE and the Application Server (Provisioning, configuration, Service Announcement and content discovery).

Call flows steps:

Step 1 to 8: These steps follow the usual step-up while establishing a downlink media delivery session between the UE and the media application server. The media application requests a downlink media session, following which an downlink media session is established, and the media is transferred to the UE client from the Application Provider. Downlink media session starts at Step 8.

Step 9: In step 9, the Energy related characteristics/information of the network, which is collected, stored in EIF is shared to the ASP and or to the 5GC operator. This is now shared between the EIF of the 5GC and the EIAF of the AF.

Steps 10 to 15 relate to Client management of energy information received from the EIF/EIAF:

Step 10.a : At a particular time window, say between 12 noon and 14h, which is peak hours data traffic period, the Application Service Provider (ASP) decides to reduce the energy consumption of its Applications Servers. The application service provider energy saving trigger contains the possible ranges of QoE.

[Note: Why does the Application Service Provider (ASP) responsible for reducing the energy consumption? To answer this question, text from KI#2 description of TR 26.942 is placed here fore reference: “For instance, when a Mobile Network Operator (MNO) deploys a communication service to fulfil application service requirements, such as those of a gaming application, it is crucial for the customer – whether an Application Service Provider (ASP) or an industry vertical – to ensure that the application service minimizes energy consumption for both end users and the data network”.

Step 10.b: The request may come from 5GC (EIF) to the AF (EIAF) to inform the EIC to request UE media player to degrade the QoE as a result of QoS degradation due to network and/or UE energy saving. How the 5GC can determine the possible QoE ranges are based on the following steps in 5GC:

- UE subscription QoE reduction authorization is added to UDM.

- SMF will select the specific UE for a specific application to reduce the QoS.

- SMF will register to Observed Service Experience analytics for the specific UE with candidate QoS parameter sets and Application ID from NWDAF.

- [Note: There was a proposed solution in the August meeting in SA2, which the consumer NF may provide candidate QoS parameter Sets and Application IDs for Observed Service Experience analytics are determined in S2-2408611]

- NWDAF will provide the possible predicted QoE to the SMF.

- SMF optionally can checks UDM (UE subscription QoE reduction authorization) that if UE agrees already for such QoE degradations.

- SMF will send the energy saving trigger AF with the QoE ranges.

Figure 7.14.3-2: 5G core procedure to trigger the energy saving towards AF

Step 11: the Application Function (AF) receives this request and activates the energy saving mode.

[Note: The AF shall decide which UEs under the control of this particular AF could be activated to energy saving mode based on the received request energy saving trigger.]

Step 12: The AF requests all or some UEs (which are under its control) to report their current QoE metrics.

[Note: QoE configuration and reporting can already optionally be specified by the QoE Measurement Collection (QMC) functionality in TR 26.942 KI#6. QoE metrics reporting shall be based on the existing standardized practices such as QMC. It would be good to reuse the QMC functionality e.g. based on TS 26.114]

Step 13: The UE reports its current QoE. The UE shall decide to limit this for a duration of X ms.

[Note: On the topic of UE consent in order to have its QoE being reported, text from KI#2 description of TR 26.942 is placed here for reference: “NOTE 1: It is presumed that the user has granted consent for its UE data to be collected, reported and subsequently exposed by means outside the scope of the study or following normative work.”]

Step 14: As soon as the AF receives the QoE metrics reported by the UE, the AF starts processing the recevied QoE metrics.

[Note: The received QoE metrics shall be compared against a standard value which are already pre-defined by the Application Service Provider (ASP) within the AF. For instance, the preferred video resolution quality of all (or some) users between 12 noon and 14h which are connected to a particular AF shall be limited to 720p only. or it is possible that AF receives the energy saving request with possible QoS degradations. Hence, the AF based on the receieved QoS degradation can request the NWDAF for the possible QoE for that specific UE based on the requested QoS ranges.]

Step 15:  After such comparison, the AF now requests all (or some) of users to reduce their QoE (for instance, video resolution), if they are using higher QoE metrics (for intance 4K resolution).

[Note: AF before requesting the UE to reduce the QoE may be able to check with the 5GC or ASP about the user subscriptions. In this scenario, if UE already has agreed to have such QoE degradtion, AF will reduce the QoE without asking UE approval and it will continue from step 13. Or it is possible that 5GC or ASP before triggering AF for energy saving, they have already checked the UE susbscription info and they will address that in their request.]

There are two possible scenarios proposed here:

Step 16 & 17: The UE agrees to reduce its QoE, it will activate the buffer control, which is present in the Media Access Function (MAF).

[Note: There could be a timer in the AF after sending the request to UE, If the UE response has not receievd by the AF after specific time (i.e. X min), it means that UE does not agree with the QoE degradtion and it will continue from step 23.]

Step 18: The on-going media session is required to be modifed between MAF and MSH.

Step 19: The MSH requests now the AF to modify its on-going media session.

Step 20 & 21: The AF thus modifies the on-going downlink media delivery session.

Step 22 is optional: This information could be optionally shared by the AF with the NWDAF (which will allow to retrive this information in the future). Here the accepted QoE ranges by the users will be stored which can be used by the ASP or 5GC for analytics.

Step 23 & 24: If  the AF informs the ASP and 5GC about the users who decided to modify (and reduce) their QoE, the ASP or 5GC then provides incentives to those users who reduce their QoE. The AF will inform about such incentives to the slected users.

Step 25 and 26: The ASP continues to provide media to the UE, which are reduced in QoE when they reach the UE from the MAF.

Step 27 to 30: The UE does not agree to reduce its QoE, it will not activate the buffer control and therefore the on-going downlink media delivery session is continues between MAF and MSH. Media streams continue to be downloaded by the UE.

### 7.13.4	Summary

This candidate solution defines a mechanism enabling an Application Function (AF) to manage energy-saving operations that may cause QoS (Quality of Service) and QoE (Quality of Experience) degradation. The AF collaborates with the 5GC (5G Core) and/or Application Service Providers (ASPs) to assess user tolerance to QoE degradation and coordinate adaptive strategies between the network and the User Equipment (UE) to minimize service impact.

Key Functional Aspects

- Energy-Saving Trigger Toward AF

  - Energy-saving actions can be triggered by authorized entities such as:

    - 5GC: Initiates QoS adjustments (e.g., bandwidth reduction) for network energy saving.

    - ASPs: Initiate energy-saving requests based on service agreements or cloud resource status.

  - The AF receives these requests along with proposed QoS/QoE ranges and initiates downstream procedures to manage QoE impacts.

- Assessment of User Tolerance to QoE Degradation

  - Upon receiving an energy-saving request, the AF evaluates the potential QoE impact:

    - Compares requested QoE parameters with predefined QoE thresholds or user-specific subscription data.

    - If only QoS parameters are provided, the AF may use the NWDAF (Network Data Analytics Function) to estimate QoE degradation.

  - Decision logic:

    - If degradation is within user’s pre-agreed QoE range → proceed automatically, with optional feedback collection.

    - If degradation exceeds the allowed range → user approval is required, specifying the expected QoE impact.

- UE-Controlled QoE Adaptation

  - The UE application evaluates whether it can tolerate the requested QoE degradation.

  - If acceptable, the UE adjusts application behavior to minimize QoE impact (e.g., adaptive bitrate reduction, buffering control, or resolution adjustment).

  - The UE ensures QoE remains above predefined thresholds, maintaining acceptable user experience despite lower resource allocation.

This candidate solution:

- Enables dynamic coordination between the AF, 5GC, ASPs, and UE for energy-efficient operation.

- Provides a framework for user-aware energy-saving decisions based on tolerance to QoE degradation.

- Allow the network and UE to jointly optimize energy consumption while maintaining service continuity and acceptable QoE.

End of changes