3GPP TSG-SA4 Meeting #134	S4-251833

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.260 | CR | 0007 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding source directions for spatial directivity tests in send direction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Dolby Laboratories Inc., Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATIAS_Ph2 |  |  |  |  |  | Date: |  |  | 2025-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Additional sound source directions are needed for testing of full-circular planar and 3D spherical capture |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add sound source directions to single-source tests |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The directional single-source capture performance of UEs offering full-circular planar and 3D spherical capture cannot be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 26.261 CR 0001 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

### 5.6.4	Directional information (single source)

#### 5.6.4.1	Test method

This test method applies to the spatial capture mode.

The default arrangement for a single sound source is used, as defined in clause 5.4.2 for each UE type that supports spatial capture and for each UE orientation supporting spatial capture according to this clause. The decoded and rendered output format shall be the same as the IVAS audio format used by the UE. L  directions shall be evaluated, as indicated in clause 5.6.4.3, Table 5. In case the setup is realized with a turntable, the device shall be rotated around the vertical center of the UE.

For each sound source direction (ϕi, θi) and i = 1, ..., L, specified in clause 5.6.4.3, the following procedure shall be used:

1)	The test signal to be used for the measurements shall be the British-English single talk sequence described in clause 7.3.2 of Recommendation ITU-T P.501 [14], calibrated to an active speech level according to clause 5.5.1.

2)	The UE under test and the reference client are connected and configured as described in the clause 5.3.2.

3)	Acoustical interface: The UE is mounted as described in clause 5.4.2 and the acoustic source is positioned such that the source direction under test is met. The test signal is played via the acoustic source.

Electrical interface: The test signal is generated by virtually placing the acoustic source such that the source direction under test is met as described in clause 5.4.2.7.

4)	The output format-dependent directional metric calculations shall be done as defined in the following clause for the tested format.

#### 5.6.4.2	IVAS format specific definitions

Stereo

The estimated source directions in the stereo panorama ζϕ shall be calculated as follows:

1)	The left and right channel signals (sLk, sRk) are recorded by the test equipment.

2)	The inter-channel time difference (ICTDϕi=Δτ) is determined as described in Annex C between the signals sLk and sRk.

3)	The inter-channel level difference (ICLD) is determined as follows:

a)	The active speech level according to Recommendation P.56 [10] is calculated separately for slk and srk, resulting in LL and LR.

b)	The ICLD for source direction ϕi is calculated as:

ICLDϕi=LL-LR

4)	The equivalent level difference Δϕi for source direction ϕi is calculated as:

Δϕi=ICLDϕi+ICTDϕi⋅17.3 dBms    [dB]

5)	The estimated source directions in the stereo panorama ζϕi is calculated according to [34] as:

ζϕi=100⋅Δϕi13.5 forΔ≤6.75 dB-32ΔϕidB3+288ΔϕidB2+20736ΔϕidB-6561⋅signΔϕi273375for6.75 dB<Δ<18 dBsignΔϕiforΔ≥18 dB    [%]

Scene-based audio

Direction of arrival analysis with scene-based audio shall be conducted as follows:

1)	From the SBA signal decoded by the reference client, only the first four Ambisonics component channels (FOA) in time-domain B-format representation are used, i.e., higher order Ambisonics components are discarded. These four FOA components are referred to as W, Y, Z, X in ACN notation.

2)	Each channel is segmented into time-domain frames of 20 ms length. The frames are referred to by frame index m, leading to a sequence of frames wm, ym, zm, xm.

3)	Intensity parameters ix, iy, iz for the three catesian directions are calculated as follows:

ixiyiz=-mgwmxm'ym'zm'wm.

The gating function gwm equals 1, if the input frame wm to the function is larger than -48 dBov; otherwise, it equals 0.

4)	The direction of arrival estimation ϕi,θi is calculated based on the intensity parameter using the equations:

ϕi=atan2iy, ix⋅180°π,

θi=atan2(iz, ix2+iy2)⋅180°π

The traditional arctangent function (arctan(x/y)) is not in itself sufficient to uniquely resolve the correct quadrant, thus the two-argument version "atan2" is used:

5)	The estimated sound source direction of arrival ϕi,θi is compared to the direction of the sound source (ϕi, θi).

Metadata-assisted spatial audio

Direction of arrival analysis with metadata-assisted spatial audio shall be done as follows:

1)	The decoded MASA signal is captured by the test equipment. The decoded MASA metadata includes estimated direction indices and direct-to-total energy ratio quantities rdirk,n,m for each subframe n and MASA spatial metadata frequency band k in every metadata frame m. Direction indices are further converted to the direction angular values ϕ'k,n,m,θ'k,n,m as described in clause 5.5.2.2 of 3GPP TS 26.253 [29]. The energy Ek,n,m of the transport channel(s) is estimed for each subframe and MASA spatial metadata frequency band in every frame. The MASA format time-frequency resolution and metadata parameters are further described in 3GPP TS 26.258 Annex A [31].

2)	 Estimated direction angular values (in radians) are mapped into Cartesian coordinate vectors X, Y and Z over all subframes N=4 in every frame M and MASA spatial metadata frequency bands K=20 as:

X=mMn4k20(cos(ϕ'(k,n,m))⋅cosθ'k,n,m⋅rdir(k,n,m)⋅E(k,n,m))

Y=mMn4k20sinϕ'k,n,m⋅cosθ'k,n,m⋅rdirk,n,m⋅Ek,n,m,

Z=mMn4k20(sinθ'k,n,m⋅rdir(k,n,m)⋅E(k,n,m))),

where k is the index of the MASA spatial frequency band, n is the index of the subframe and m is the index of the frame.

3)	The sound source direction estimation ϕi,θi in degrees is calculated based on the mapped Cartesian coordinate vectors using the equations:

ϕi=arctan2Y, X⋅180°π,

θi=arctan2(Z, X2+Y2)⋅180°π

See previous definition of the function "atan2".

4)	The estimated sound source direction ϕi,θi is compared to the direction of the sound source θi,φi.

#### 5.6.4.3	Sound source directions for single-source directional tests

##### 5.6.4.3.1	Sound source directions for testing half-circular (planar) capture

The L  = 7 sound source directions applicable for testing half-circular (planar) capture are specified in Table 5.

Table 5: Additional source directions to cover half circle

| i | ϕi [°] | θi [°] |
| --- | --- | --- |
| 1 | -90 | 0 |
| 2 | -60 | 0 |
| 3 | -30 | 0 |
| 4 | 0 | 0 |
| 5 | 30 | 0 |
| 6 | 60 | 0 |
| 7 | 90 | 0 |

##### 5.6.4.3.2	Sound source directions for testing (full)-circular (planar) capture

Sound source directions applicable for testing full-circular (planar) capture are specified in Table 5 above and Table 6 below. Including the directions mentioned in Table 5, this increases L to 12 source directions in the listening plane.

Table 6: Extended source directions to cover full circle

| i | ϕi [°] | θi [°] |
| --- | --- | --- |
| 8 | 120 | 0 |
| 9 | 150 | 0 |
| 10 | 180 | 0 |
| 11 | 210 | 0 |
| 12 | 240 | 0 |

##### 5.6.4.3.3	Sound source directions for testing 3D spherical capture

Sound source directions applicable for testing 3D spherical capture are specified in Tables 5 and 6 above and Table 7 below. Including the directions mentioned in Tables 5 and 6 above increases L to [20]  source directions, with an additional set of [8] non-zero elevation directions.

Table 7: Extended source directions to cover elevated plane

| i | ϕi [°] | θi [°] |
| --- | --- | --- |
| [13 | -90 | 30] |
| [14 | -45 | 30] |
| [15 | 0 | 30] |
| [16 | 45 | 30] |
| [17 | 90 | 30] |
| [18 | 135 | 30] |
| [19 | 180 | 30] |
| [20 | 225 | 30] |

==============End of change==============