Source:	Dolby Laboratories Inc., Nokia, Novamint

Title:	[FS_ULBC] Updated proposal on methodology for error trace generation

Agenda item:	7.9

Document for:	Discussion and Agreement

### Introduction

Various inputs to the methodology for generating error traces have been discussed in previous Audio SWG meetings on FS_ULBC. While the recent FS_ULBC adhoc meeting in Erlangen made progress, time constraints prevented resolution of all differing viewpoints. One topic that has recently gained prominence is ULBC system capacity—specifically, the number of simultaneous ULBC connections supported on a 180 kHz NB-IoT carrier.

This contribution builds upon the latest proposal from the source regarding error trace generation methodology [1]. It broadens the scope to address system capacity as a central consideration and proposes using it as the starting point for the error trace generation procedure. The capacity topic is addressed in a separate contribution [2] as the cornerstone of the error trace generation methodology.  Beyond using capacity as the starting point for the error trace generation procedure, this contribution introduces a number of clarifications compared to [1].

Below is an updated proposal on methodology for error trace generation. It comprised two parts, namely the algorithmic descriptions of the methodology and a description where the methodology is reduced to practice.

It is proposed for inclusion in the ULBC TS as clause 5.2.3.

### 5.2.3 Methodology to generate error traces

#### 5.2.3.1 Algorithmic steps of the methodology

Loop: For each of the potential bundling times of 80, 160 and 320 ms do the following steps:

Note: It would be desirable to reduce the number of potential bundling times upfront. Contribution [2] indicates such a possibility.

Step 0: Agree on a set of scheduling scheme configurations (number of tones, SCS, ISF, IRU, Nrep) that achieves an agreed-upon target capacity in UL and DL.

Note1: The agreed-upon target capacity should strike a good balance between number of supported simultaneous ULBC call sessions and link-level performance. According to [2], the general approach should be to balance the available transmission resources between DL and UL such that voice quality on UL and DL is about similar while maximizing system capacity.

Note2: For each bundling time, one or more scheduling scheme configurations may use an equal amount of transmission resources. For example, configurations with identical products of NSF∙ NRep and NRU∙ NRep for DL and UL, respectively, result in equivalent transmission resource usage. With reference to Tables 16.5.1.2-2: Transport block size (TBS) table for NPUSCH of TS 36.213 and Table 16.4.1.5.1-1 applicable for NDDSCH, each TBS in the column identified by a specific ISF or IRU value is equivalent in terms of resource utilization. E.g., NPDSCH with 12 subcarriers using ISF = 2 along with NRep=2 and ISF = 1 along with NRep=2 are equivalent in terms of resource utilization.

|  | IRU/SF |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1000/1032* |
| 7 | 104 | 224 | 328 | 472 | 584 | 712/680* | 1000/968* | 1224 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 1096 | 1384/1352* |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1256 | 1544 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1000/1032* | 1384 | 1736 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1608 | 2024 |
| 12 | 208 | 440 | 680 | 1000/904* | 1128 | 1352 | 1800 | 2280 |
| 13 | 224 | 488 | 744 | 1032 | 1256 | 1544 | 2024 | 2536 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 |  |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 |  |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | 2536 |  |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 |  |  |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 |  |  |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2536 |  |  |
| 20 | 440 | 904 | 1384 | 1864 | 2344 |  |  |  |
| 21 | 488 | 1000 | 1480 | 1992 | 2536 |  |  |  |

* The differences of available TBS in UL and DL are indicated by two-number entries. The lower of the two numbers would be available for transmission should the corresponding entry in the MCS tables be selected. The unused bits available in the respective opposite link direction could be zero-padded.

Step 1: Agree on a set of candidate TBS (from Tables 16.5.1.2-2 and 16.4.1.5.1-1 in TS 36.213) associated with the agreed set of scheduling scheme configurations to be evaluated.

Note: The set of candidate TBS from Table 16.5.1.2-2 and 16.4.1.5.1-1 in TS 36.213 corresponds to a set of potential net bit rates. Accordingly, they may be confined within reasonable upper and lower limits. The set may preferably be contiguous, i.e. containing all TBSs of a column of the TBS table within the limits. See example with green cells in above table for DL. When confining the set of candidate TBS, also to be considered are possible limitations in terms of the available modulation schemes for NTN NB-IoT, where only those entries can be used that rely on BPSK or QPSK modulation. Such limitations are indicated as greyed out in the above TBS/MCS table. Details on such constraints are given in [2].

Step 2: Generate the BLER vs SNR curves per candidate TBS for the different agreed scheduling scheme configurations.

Step 3: Calculate link budget results with different UE parameters (e.g. tx power classes, number of rx in DL), i.e., achievable UL/DL CNRs, and view them as the selected SNR values (e.g. SNR1, SNR2)

Figure 1: Example with 2 selected SNR values according to step 3

SNR1

SNR2

TBS1

TBS2

SNR

BLER

Step 4: For each candidate TBS, agree on the scheduling scheme configuration that leads to best possible transmission performance minimizing BLER at given SNR. Regard them as the selected TBS configurations (e.g. select solid curves, see Figure 2)

Step 5: For each selected TBS configuration lookup target BLER corresponding to selected SNR values (e.g. BLER11 and BLER12 for TBS1 and BLER11 and BLER12 for TBS2)

Step 6: Discard TBSs that lead to a target BLER > [20%].

Figure 2: Example with selected TBS configurations showing target BLER corresponding to selected SNR values according to step 5

SNR1

SNR2

TBS1

TBS2

BLER12

BLER22

BLER11

BLER21

SNR

BLER

Step 7: Generate traces: Generate packet loss patterns for the selected TBS configurations for identified target BLERs.

Note: BLER11 with TBS1 and BLER21 with TBS2 are equivalent in terms of capacity and link budget (i.e. SNR) and can be compared on a fair basis.

#### 5.2.3.2 Practical example

The methodology for error pattern generations is illustrated as follows for the example of 160 ms bundling time. Step 0 including the underlying assumptions refer to [2].

Step 0: Agree on radio resource balance as identified in section 4.3.2 of [2], i.e. using 128 ms as UL transmission time resource and 12 ms as DL transmission time resource. This leads to a set of scheduling scheme configurations that make use of this time resource distribution.

Step 1: Agree on a set of candidate TBS (from Tables 16.5.1.2-2 and 16.4.1.5.1-1 in TS 36.213) that is compatible with the identified set of scheduling scheme configurations for DL and UL. In the present example, this could be the following set of candidate TBS values: [176, 208, 256, 328], identified along with potential scheduling scheme configurations in the table below (blue highlight: UL, green highlight: DL).

|  | IRU/SF |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1000/1032* |
| 7 | 104 | 224 | 328 | 472 | 584 | 712/680* | 1000/968* | 1224 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 1096 | 1384/1352* |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1256 | 1544 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1000/1032* | 1384 | 1736 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1608 | 2024 |
| 12 | 208 | 440 | 680 | 1000/904* | 1128 | 1352 | 1800 | 2280 |
| 13 | 224 | 488 | 744 | 1032 | 1256 | 1544 | 2024 | 2536 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 |  |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 |  |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | 2536 |  |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 |  |  |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 |  |  |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2536 |  |  |
| 20 | 440 | 904 | 1384 | 1864 | 2344 |  |  |  |
| 21 | 488 | 1000 | 1480 | 1992 | 2536 |  |  |  |

Step 2: Generate the BLER vs SNR curves per candidate TBS for the different scheduling scheme configurations.

Note that BLER vs. SNR curves for TBS=208 have already been provided [4]. What would be required are just 7 additional BLER vs SNR curves for the other considered TBS values.

Step 3: Calculate link budget results with different UE parameters (e.g. tx power classes, number of rx in DL), i.e., achievable UL/DL CNRs, and view them as the selected SNR values (e.g. SNR1, SNR2).

Note that this step is pending a decision in SA4 how many different UE parameters to consider. According to the assumptions in [2], the minimum required UE capabilities should be considered which would lead to a single selected SNR value for each UL and DL.

Step 4: For each candidate TBS, agree on the scheduling scheme configuration that leads to best possible transmission performance minimizing BLER at given SNR. Regard them as the selected TBS configurations.

Note that the best possible scheduling scheme configuration for TBS=208 has already been determined [4]. The only task left in this step would be to determine whether the DL transmission of TBS=256 and 328, respectively should use NSF=3 and Nrep=4 or NSF=6 and Nrep=1. This step results in 4 selected TBS configurations for each DL and UL.

Step 5: For each selected TBS configuration lookup target BLER corresponding to selected SNR values.

Note that this leads to 4 target BLER values for each DL and UL.

Step 6: Discard TBSs that lead to a target BLER > [20%].

Note 1: The result of this step would depend on the actual BLER vs SNR curves.

Note 2: As stated during the oral discussions, the value of 20% may be subject to discussions.

Step 7: Generate traces: Generate packet loss patterns for the selected TBS configurations for identified target BLERs.

Note that this step would involve the generation of at maximum 4 packet loss patterns for each DL and UL, where the number might be reduced if a TBS is discarded in step 7.

### References

[1] S4aA250271: [FS_ULBC] Consolidated methodology for error trace generation

[2] S4-251863: [FS_ULBC] On ULBC system capacity as cornerstone for error trace generation

[3] S4aA250116: Multiuser SPS Capacity Analysis and TBS/MCS suitability

[4] S4aA250112: [FS_ULBC] Detail methodology for error trace generation, Vivo