Source:	Ericsson LM

Title:	Ericsson Listening Lab report for IVAS Characterization

Agenda item:	7.5

Document for:	Approval

# Introduction

In accordance with the IVAS test plan for Characterization [1], Ericsson was assigned to run four listening test experiments, namely P800-16 (OSBA, 3-4 objects), P800-18 (OMASA, 1-2 objects), BS1534-6 (MC 5.1), and BS1534-20 (6-DoF and directivity). The test results are delivered via the Global Analysis Lab (GAL) in accordance with the test plan. This report provides additional information on the performance of the experiments.

# P.800 experiments

## Test setup

The tests were set up in the Ericsson Audio lab, fulfilling requirements as specified in [2][3][4]. The tests were conducted using three test rooms, with two listening subjects seated in each room. The playback and voting were done using internally developed software. The listening test software does not enforce voting, which means there are occasionally missing votes in the delivered raw scores. The two tests took 5 days to complete, including complementing listening sessions for listeners with high number of missing votes and re-scheduled listening sessions.

The playback order was implemented by converting the updated randomization sheets, distributed by Stefan Bruhn on 16th of September 2025, converted to the format accepted by the internal listening test software. Each listening panel/group was divided into 3 listening sessions of 25 minutes duration with 10-minute breaks between the sessions. Two randomization blocks were covered in each session.

The audio playback was done using Audient iD14 sound cards and PreSonus HP4 headphone amplifiers connected to Beyerdynamic DT 770 Pro 250 Ohm closed-back headphones.

## Test subjects and instructions

The listener test subjects were recruited from Ericsson employees and interns, the invite stating the requirements to have normal hearing and Swedish as native language. The instructions were translated to Swedish from the example instructions in [1] and included below for reference.

Lyssnarinstruktion

I det här experimentet kommer du att utvärdera system som skulle kunna användas för telekommunikation med omslutande/spatialt ljud. Spatialt ljud innebär att du kan lokalisera olika ljudkällor omkring dig. Till exempel kan en första talare befinna sig på din vänstra sida och en andra talare på din högra sida, en talare kan röra på sig, etc.

Du kommer att få lyssna på par av ljudsampel, uppspelade i hörlurar. Varje par består av ett referenssampel följt av ett testsampel. Testsamplet har samma innehåll som referenssamplet, men kan ha försämrats när det har passerat ett telekommunikationssystem.

Din uppgift är att bedöma den sammantagna försämringen av det andra samplet jämfört med det första samplet, där bedömningen tar hänsyn till både försämring i ljudkvalitet (såsom brusighet, klick eller annan distorsion) och/eller förändring av den spatiella ljudbilden (t.ex. ljudkällors position, avstånd, upplevd storlek, rörelse, etc.).

Lyssna noga på båda ljudsamplen och välj därefter det alternativ som bäst beskriver din bedömning av försämring i det andra samplet relativt det första:

5 - Ingen försämring

4 - Liten försämring

3 - Måttlig försämring

2 - Stor försämring

1 - Mycket stor försämring

Notera att nivåerna av försämring i testsamplen förväntas spänna upp hela skalan under experimentet.

Testet består av en övningsdel med 12 par av ljudsampel och en huvuddel som är uppdelad i tre lyssningssessioner om ca 25 minuter. Mellan lyssningssessionerna är det 10 minuters paus.

Var vänlig diskutera inte dina åsikter om testmaterialet med någon annan deltagare i testet. Fråga testledaren om det är något du undrar över.

Tack för din medverkan!

## Encountered issues

Our playback system required raw audio files, which meant all wav headers needed to be stripped from the audio data. MD5 checksums were verified on the downloaded wav files on the playback computers before converting to raw pcm format.

The total test time was a little bit long, where the effective test time excluding breaks exceeded 75 minutes for the main test. Listener subjects expressed getting a bit tired.

# BS.1534 experiments

## Test setup

The tests were set up in Ericsson Audio lab based on [5]. There was a total of 14 experienced listeners for each test after post screening.

For BS1534-6 (MC 5.1) a test signal was played out to test that sound came from the correct speaker and subwoofer. The speaker placement was 0, -30, +30, -110, +110 degrees from the center with a radius of 2 meters. It was made sure that the distance from the speaker to the walls was at least 1 meter. All the test participants used the same room and setup. The playback was done with internally developed software and an Orion32 sound card connected to Genelec 8040A loudspeakers and a Genelec 7360A subwoofer. The speakers were calibrated prior to the test by playing pink noise on each speaker and adjusting the gain such that the sound pressure level in the listening spot became 73 dB for the speaker and 79 dB for the subwoofer.

BS1534-20 (6-DoF and directivity) was set up in two identical rooms. The audio playback was done using Audient iD14 sound cards and PreSonus HP4 headphone amplifiers connected to Sennheiser HD 650 open-back headphones.

## Test subjects and instructions

The test instructions below were given to all test participants.

Listener instructions

Training phase

The first step in the listening tests is to become familiar with the testing process. This phase is called a training phase, and it precedes the main test.

The purpose of the training phase is to allow you to:

- to become familiar with typical sound excerpts under test and their quality level ranges; and

- learn how to use the test interface and the grading scale.

During the training phase you should be able to learn how you interpret the audible impairments in terms of the grading scale. You should not discuss your personal interpretation of the scale with the other assessors at any time during the training phase.

No grades given during the training phase will be taken into account in the reported tests results.

Main test - blind grading phase

The purpose of the blind grading phase is to invite you to assign your grades using the quality scale. Your grades should reflect your subjective judgement of the quality level for each of the sound excerpts presented to you. Each trial will contain [6/8]* signals to be graded. Each of the items is approximately 10 s long. You may listen to the signals in any order, any number of times.

Use the slider for each signal to indicate your opinion of its quality. When you are satisfied with your grading of all signals you should click on the “Next trial” button at the bottom of the screen.

You will use the following quality scale when assigning your grades. The grading scale is continuous from “excellent” to “bad”. A grade of 0 corresponds to the bottom of the “bad” category, while a grade of 100 corresponds to the top of the “excellent” category.

In evaluating the sound excerpts, please note that you should not necessarily give a grade in the “bad” category to the sound excerpt with the lowest quality in the test. However, one or more excerpts must be given a grade of 100 because the unprocessed reference signal is included as one of the excerpts to be graded. Note that you need to move all sliders to actively set a grading for each sound excerpt.

* BS1534-6 had 8 signals per trial and BS1534-20 had 6 signal per trial.

## Encountered issues

We did not encounter any issues during the tests.

# Summary

The experiments P800-16, P800-18, BS1534-6 and BS1534-20 of the IVAS Characterization tests were conducted in the Ericsson Audio lab. A preliminary analysis of each experiment was conducted, showing reasonable results given the tested conditions. The raw scores from the listening tests were delivered to the Global Analysis Lab (GAL) in time, using the supplied templates. The preliminary analysis results were included in case they could be useful for validation purposes.

# References

Tdoc S4-251565, IVAS-8b: Test Plan for Characterization Phase, Version v.1.1.0

ETSI EG 202 396-1, Speech Processing, Transmission and Quality Aspects (STQ); Speech quality performance in the presence of background noise; Part 1: Background noise simulation technique and background noise database

ITU-T P.800, Methods for Subjective determination of Transmission

ITU-R BS.1116-1 Methods for the Subjective Assessment of Small Impairments in Audio Systems Including Multichannel Sound Systems.

ITU-R recommendation BS.1534, Method for the subjective assessment of intermediate quality level of audio systems