3GPP TSG SA WG 4 Meeting #134 	S4-251877

17-21 November 2025, Dallas, Texas, USA

Source:	Tencent

Title:	[FS_3DGS_MED] Pseudo-CR on 3DGS content generation

Spec:	3GPP TR 26.958 v0.0.1

Agenda item:	9.7

Document for:	Agreement

1. Introduction

This pseudo-change request proposes to update the TR document of the FS_3DGS_MED study, and more specifically section 7 related to the content generation.

2. Reason for Change

Provide a first version of this section.

3. Proposal

It is proposed to agree the following changes to the 3GPP Draft TR 26.958.

* * * First Change * * * *

# 7	3DGS content creation

[Editor’s note: Placeholder for the description of the 3DGS content creation processes]

## 7.1	3DGS generic workflow

As an example, a 3DGS model construction from the capture of 2d data follows the production workflow as illustrated in figure 1:

Figure 1: 3DGS model production workflow from a 2d video

The workflow consists of three parts. First, the capture phase. During this phase, we acquire numerous views of a real-world object or scene from different positions, typically as photos or video frames.

Next, a 3D motion reconstruction (SfM) step estimates the camera's intrinsic parameters and poses and reconstructs a sparse 3D structure, which serves as initialization for a set of Gaussians roughly aligned with the object. If the captures were performed using 3D scanning, this step can use depth information to improve accuracy.

Finally, during training, we compare the rendered image of the current Gaussian scene to the corresponding real-world image for each view and minimize the photometric error. As optimization progresses, the Gaussians are split or pruned, resized, reoriented, and their color/opacity is adjusted, progressively densifying and refining the scene to produce an accurate and consistent 3D representation with viewpoints.

## 7.2	Capture

Content acquisition for 3D Gaussian Splats (3DGS) relies on capturing accurate 3D data from real-world objects and environments. Primary methods include:

-	2D image and video capture: video sequences or sets of 2D images capture from various positions and orientations offer coverage of dynamic or complex environments. Sequential frames can provide dense colour and positional cues for downstream 3D reconstruction and Gaussian splat generation, making it practical for capturing motion or large-scale scenes efficiently.

3D Scanning: High-precision laser or LiDAR scanners provide accurate spatial measurements of objects and scenes, capturing dense geometric detail suitable for Gaussian splat representation. These devices enable large-scale and high-fidelity acquisition, though they require specialized equipment and controlled capture conditions.

-	Multi-Image Capture: Still images taken from multiple viewpoints allow detailed reconstruction of object surfaces. Overlapping images ensure sufficient coverage and colour information for later conversion to 3DGS format. This approach is cost-effective and flexible, particularly for small to medium scale scenes.

These capture methods focus on acquiring raw visual and spatial data, forming the foundation for 3DGS content generation.

## 7.3	Structure from Motion

The Structure from Motion (SfM) step consists in operating the feature extraction and matching and retrieving the camera parameters when unknown. After image alignment, the process creates a sparse point cloud that is further densified with depth calculation methods.

Camera parameters, often known by the capturing system include the focal length and centre and its distortion model from the lens.

Even if the focal parameters may be estimated from the captured video from the SfM software, the lens distortion model if ignored may result in an anamorphic 3d model.

Popular SfM software include COLMAP, ReCap and Meshroom.

## 7.4	Training

The second step is about the creation of the gaussian splats associated to each 3D point with a dedicated AI/ML generation model that will search for the splats that match as much as possible the source video for a given pose (position + orientation) by optimizing the size, shape, colour and transparency of the splats.

The extrinsic parameters such as the pose will help the alignment of the splat rendering with the exact video frame used by the model.

Many tools are available: Brush, Gsplat, 3DGStream…

* * * End of Changes * * * *