3GPP TSG SA WG 4 Meeting #134 	S4-251908

17-21 November 2025, Dallas, Texas, USA

Source:	Fraunhofer IIS

Title:	On noise suppression for ULBC

Spec:	3GPP TR 26.940

Agenda item:	7.9

Document for:	Discussion

1. Introduction

This document aims to provide clarification and proposals for the ULBC design constraint “Potential use of noise suppression as part of the codec”.

2. Discussion

2.1 Recent proposals

At the Ad-Hoc in Erlangen, detailed evidence on the benefits of noise suppression for low bitrate speech coding was provided and it was agreed to include S4aA250262 into TR 26.940 including the following conclusions:

### 7.3.3	Conclusions

This clause presents two different listening tests demonstrating that performance of speech coders in noisy conditions can be significantly enhanced by ensuring a dedicated high SNR, e.g. by combining noise suppression and speech coding. Current neural speech coders seem sensitive to noisy environments and may benefit more from noise suppression than traditional speech coders. Ensuring a dedicated high SNR enables improved performance at very low bitrates under both high and low signal-to-noise ratio (SNR) condition.

Generally, using noise suppression methods might have an impact on overall delay and complexity, which is for further study. However, complexity and delay aspects  do not affect the results of the subjective evaluation presented and the observations done thereafter.

NOTE: 	While speech is considered the primary information, removing all parts of the background audio scene may in some cases not be desirable, e.g. when there is no active speech, as the background may contain relevant information, e.g. in the case of emergency calls.

Editor’s note: It is TBD whether the design constraints and performance requirements should reflect the note above.

The present document aims to provide further information on the NOTE and explain the requirements for eCall senarios.

2.2. eCall

One scenario of eCalls are so called “silent calls” as for instance described in [eCall_2]:

The term ”silent call“ describes an emergency call for the PSAP side without voice communication. This is an important aspect already today since emergency calls can be initiated from mobile phones without necessarily unlocking the device. Silent calls (see also table 1) may also appear

- in situations with manual eCalls from vehicles, when driver respectively passengers have left the car to provide assistance as first aider,

- in automatically generated eCalls, when driver and passengers left the vehicle without being seriously injured - as it is recommended by the police, e.g. for accidents on highways,

- or when driver or passengers are unconscious in the car or bikers (or first aiders) are in far distance from the bike after a crash.

The only chance for the PSAP side to judge the severity of a call is to amplify the transmitted signal at the PSAP side as much as possible and gather the information from the transmitted acoustic environment. The transmitted ambient noise carries important information for the PSAP operator in this case.

Requirements on silent call performances can be found in [eCall_1] and ITU-T P.1140. An exact measurement description for vehicles is given in ITU-T P.1140 clause 9.11.

2.3	 ULBC design constraints

The silent call scenario implies some requirements on the background noise transmission, i.e. in case no active speech is present, the background noise need to be transmitted within certain limits. Consequently, in case of active speech, noise suppression can be used for ULBC to enhance speech quality.

3. Proposal

It is proposed to adapt the following changes to TR 26.940.

* * * First Change * * * *

## 6.2	Design Constraint Parameter

Table 6.2-1 List of ULBC design constraint parameter

| Parameter | Design Constraint | Note |
| --- | --- | --- |
| Bit rates |  |  |
| Sample rate and audio bandwidth |  |  |
| Frame length |  |  |
| Complexity and memory demands |  |  |
| Algorithmic delay |  | The algorithmic delay is defined as the frame size buffering delay plus any other delays inherent in the codec algorithm (e.g., look-ahead, sample-rate conversion, and decoder post-processing) |
| Packet loss concealment (PLC) |  |  |
| Potential use of noise suppression as part of the codec | For active speech, noise suppression within ULBC is allowed. |  |
| Discontinuous transmission including voice activity detection and comfort noise |  |  |
| Robustness to non-speech input |  | Editor’s note: May need to be in performance requirement |

* * * End of Change * * * *

[eCall_1]	U. Müsch et al., eCall Testing and Certification – Lessons Learned, DAGA, 2019

[eCall_2]	F. Ketler et al, Speech Communication in Emergency Call Scenarios for Motorcycles, DAGA, 2017