3GPP TSG-S4 Meeting  #134	S4-251930

# Dallas, USA, 17th November – 21st November 2025

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | 0036 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD_Ph2] Network Assistance for multi-access media delivery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics Co. Ltd., |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 5G-MAG requested support for network assistance with uplink streaming use cases when using multi-access media delivery. Rel-19 TR 26.804 also documents gaps on this topic. Please see attached discussion document for more information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a key issue description on supporting network assistance for multi-access media delivery over multiple access networks |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multi-access media delivery feature is incomplete |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.1.4, 5.18.1.5 (new), 5.18.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | .. |  |  |  |  |  |  |  |  |

CHANGE-1

#### 5.18.1.4	Dynamic Policies in 5G Media Streaming

##### 5.18.1.4.1	Overview

TS 26.501 [15] and TS 26.510 [108] specify the stage-2 procedures and stage-3 data model definitions for the Dynamic Policy feature in 5G Media Streaming. Specified as part of a high-level procedure for service provisioning in clause 5.3.2 of TS 26.501 [15], when the dynamic policy feature is offered and selected, the 5GMSd Application Provider specifies a set of policies which can be invoked for the media streaming session. The data model for the PolicyTemplate resource in specified in clause 8.7.3 of TS 26.510 [108]. The Media Session Handler becomes aware of the selected policies in the form of a list of valid Policy Template Ids listed in the Service Access Information it obtains from the 5GMS AF at reference point M5 or by private means via reference point M8.

When the Media Session Handler intends to activate a Dynamic Policy for a media streaming session, the Media Session Handler sends a Dynamic Policy API request to the 5GMS AF. As specified in clause 5.7.3 of TS 26.501 [15], the request includes at least the Provisioning Session identifier, the Service Data Flow Description(s) of the relevant application flow(s) and the Policy Template identifier to be applied to the described application flow(s). The details of the Dynamic Policy API are specified in clause 9.3 of TS 26.510 [108], and the data model for the DynamicPolicy resource is specified in clause 9.3.3 of TS 26.510 [108].

The applicationFlowBindings property in the DynamicPolicy data model specifies the bindings between application flows at reference point M4 managed within the scope of the Dynamic Policy Instance and their network QoS requirements. The ApplicationFlowBinding object referenced by this property includes three sub-properties that allow for specification of the application flows to which the dynamic policy QoS specification is to be applied:

-	componentIdentifier that references a particular service component in the Policy Template

-	applicationFlowDescription which provides a specification of an application flow to be used by the 5G Core for application traffic identification purposes.

-	qosSpefication that provides network QoS requirements for the application flow(s) described by applicationFlowDescription.

The ApplicationFlowDescription type referenced by the applicationFlowDescription property is specified in clause 7.3.3.2 of TS 26.510 [108] and includes the following sub-properties:

-	filterMethod of type SdfMethod (specified in clause 7.3.4.2 of TS 26.510 [108]) provides details about how to identify packets belonging to this application flow.

-

-	domainName describes the application flow in terms of the FQDN of a Media AS.

-	mediaType describes the type of media carried by this application flow.

-	mediaTransportParameters of type ProtocolDescription (specified in clause 5.5.4.13 of TS 29.571 [29571]) describes the set of media transport protocol parameters to be used by the 5G Core for the purpose of PDU Set identification and/or end of data burst detection on this application flow.

##### 5.18.1.4.2	Key Issue objectives

When the UE and the network agree to use a Multi-Access PDU Session (as described in clause 5.18.1.2.1 of the present document) for a 5G Media Streaming session, it is not clear how the Dynamic Policy feature specified in TS 26.501 [15] and TS 26.510 [108] is activated and implemented for application flows over multiple access networks.

Specifically, the following issues need to be studied:

-	If M4 application flows are carried over two access networks, what does "activate dynamic policy with QoS requirements" mean – whether the requested network QoS is applicable to one, or more, or all access paths.

-	Is it feasible to request QoS for a subset of access paths over specific access networks?

-	Are any enhancements to the ApplicationFlowDescription type described in TS 26510 [108] needed to support identification of M4 application flows over multiple access networks?

CHANGE-2 (ALL NEw TEXT)

#### 5.18.1.5	Network Assistance with multi-access media delivery

##### 5.18.1.5.1	Overview

Clause 4.0.5 of TS 26.501 [15] describes the high-level procedures for network assistance feature in 5G Media Streaming. Network assistance enables the 5GMS Client in the UE to interrogate the network Quality of Service for an ongoing media streaming session. Two methods for obtaining network assistance are defined in TS 26501 [15]:

-	AF-based network assistance, where in the 5GMS Client requests the network side component of the 5GMS system, the 5GMS AF, for assistance, and the 5GMS AF interacts with 5G System network components, such as the PCF, to provide the requested assistance to the 5GMS Client.

-	ANBR-based network assistance, which is based on signalling interactions between the UE modem and RAN control/user plane entities. The RAN control plane entities interact with 5G System network components to provide network assistance to the requesting UE

With either of the above two methods above, the following network assistance facilities may be available to the 5GMS Client:

-	Bit rate recommendation, where in a 5GMS Client requests an estimate, from 5GMS AF, of a bit rate that can be offered to the current 5G Media Streaming session. The 5GMS AF, based on this request from the 5GMS Client, queries 5G System network components such as the PCF to obtain this information, and informs the 5GMS Client of the same. The 5GMS Client may use this information to adjust its own session parameters – e.g., the streaming bit rate to fit within the QoS that the network is able to offer.

-	Delivery boost, where in a 5GMS Client requests for temporary boost of bit rate for the current media streaming session from the network, and the 5GMS AF interacts with 5G System components such as the PCF to facilitate the boost

When a 5G Media Streaming session is conveyed over a Single Access PDU Session, the 5GMS Client may receive network assistance using the above assistance procedures from the network. However, when a 5G Media Streaming session is conveyed over a Multi-Access PDU Session, it is not clear whether the information procured using currently specified network assistance procedures is useful or enough for the 5GMS Client.

##### 5.18.1.5.2	Key Issue objectives

When the UE and the network agree to use a Multi-Access PDU Session (as described in clause 5.18.1.2.1 of the present document) for a 5G Media Streaming session, the following issues need to be studied to specify network assistance for multi-access media delivery:

-	If M4 application flows are carried over multiple access networks, and the 5GMS Client intends to gather network assistance for adapting its behaviour, whether the current bit rate recommendation and delivery boost network assistance facilities enough to support the 5GMS Client, or enhancements to assistance information is required?

-	Whether and how the 5GMS Client use current bit rate recommendation information from the 5GMS AF to distribute application flows at reference point M4 over one or more access networks

-	Whether the 5GMS Client is able to request, or receive, Network Assistance over a specific access network when the application flows at reference point M4d are using the Multi-Access PDU Session spanning multiple access networks as described in clause 5.18.1.3 of the present document?

-	Whether the 5GMS Client is able to request, or receive, Network Assistance for application flows at reference point M4d over a specific access network?

-	Whether and how the network assistance information from the network to the 5GMS Client include the information such as below:

-	Split recommendation of application flows at reference point M4 over one or more access networks (e.g., 30% on one access and 70% over another access)

-	Split rate recommendation of application flows at reference point M4 over one or more access networks based on type of media being sent (e.g., audio over one access, video on another access)

-	Split recommendation of application flows at reference point M4 over one or more access networks based on stream priority

-	Initial recommended throughput with associated latency over one or more access networks

-	Other information such as upcoming network conditions, upcoming network handover etc.

CHANGE-3

### 5.18.5	Gap analysis and requirements

#### 5.18.5.1	Multi-access downlink media streaming using CMMF

#### 5.18.5.2	Multi-Access media delivery using ATSSS

##### 5.18.5.2.1	Gaps in multi-access delivery path management

The following potential open issues are identified in the specific case where multipath delivery is not transparent to the 5GMS-Aware Application:

- There is no specification related to informing the 5GMS-Aware Application of multiple paths when the UE is using ATSSS-based multipath media delivery.

##### 5.18.5.2.2	Gaps in Dynamic Policy procedures

Existing procedures for Dynamic Policies do not allow the 5GMS Client to request policy treatment over a specifc access network if the M4 flows are exchanged using multiple access networks. This is due to the fact that the currently specified data model parameters for dynamic policy procedures (as described in clause 5.18.1.4 of the present document) do not allow for identification of specific paths if multiple paths are possible between the UE and the 5GMS AS. Specifically, the sourceAddress and destinationAddress parameters of IPPacketFilterSet used in the Application‌Flow‌Description type (as described in clause 5.18.1.4 of this present document) both use the IP addresses of the Multi-Access PDU Session (as described in clause 5.18.1.3.1 of present document) assigned to the UE and UPF Steering Functionalities.

For identification of specific path, it is required that either the MPTCP link-specific multipath addresses or the MPQUIC link-specific multipath addresses be used, but these addresses are not routable via N6 (see clause 5.18.1.3.1 of present document), so these IP addresses are not candidates for use during the Dynamic Policy instantiation request that is to be sent to the 5GMS AF over N6.

This is an issue if one of the paths is a problematic path in a multipath environment as described in clause 5.18.1.3.2 of the present document. However, according to clause 5.32.4 of TS 23.501 [23], traffic splitting for GBR QoS Flows is not supported. So, if M4 media flows are transported as GBR QoS Flows, then traffic splitting of M4 media flows using ATSSS is not supported in this release, and the study needs to be revisited in a future release.

NOTE:	The Rel-18 ATSSS specification does not support traffic splitting for GBR QoS Flows between 3GPP and Non-3GPP accesses. The ATSSS specification work in future releases is to be monitored, and referenced here when there is an update.

##### 5.18.5.2.4	Gaps with network slicing

As described in clause 5.18.1.3 of the present document, if the UE requests a network slice instance with the Multi-Access PDU Session, the same S-NSSAI is allowed to span both access networks. Clause 5.15.19 of TS 23.501 [23] specifies support for Network Slice Replacement, a feature that is used to temporarily replace an S-NSSAI with an Alternative S-NSSAI when the originally selected S-NSSAI becomes unavailable or congested in the 5G network. There is no specification text in clause 5.15.19 of TS 23.501 [23] describing the impact of Network Slice replacement on a Multi-Access PDU Session.

end of CHANGEs