3GPP TSG-SA WG4 Meeting #134

Dallas, USA, 17th November–21st November 2025

Title:	[FS_AMD_Ph2]: Network Assistance for multi-access media delivery

Agenda Item:

Source:	Samsung Electronics Co Ltd.

1. Introduction

During the SA4#131 meeting, there was an LS from 5G-MAG to 3GPP SA4 group in S4-250002. As part of this LS, 5G-MAG informed 3GPP SA4 about a then recently published report “Uplink media delivery: Architectures and Features”. The report is available at https://www.5g-mag.com/post/uplink-media-delivery-architectures-features [1]. As part of this LS, 5G-MAG communicated some findings to 3GPP SA4 for potential consideration in future improvements of the 5GMSu and RTC architectures, specific to network assistance enhancements with uplink media streaming and multi-access media delivery.

Clause 5.18.5.2.3 of TR 26.804 also describes gaps in network assistance procedures in relation to multi-access media delivery. This contribution discusses findings by 5G-MAG on multi-access delivery and network assistance in relation to uplink streaming. The contribution further proposes some study aspects on this topic. Please see the accompanying CR attached to this contribution.

2. Background

This clause presents background work on 5G-MAG report on uplink media delivery – architectures and features; and current support for network assistance in relation to multi-access media delivery in TR 26804.

2.1 5G-MAG report on uplink media delivery

In Page 12 of 5G-MAG report on uplink media delivery [1], they discuss on the issue of traffic steering and multipath architectures.

5G-MAG introduces on the limitations of cellular bonding approaches which use multiple SIM cards (UEs) to access multiple connectivity paths in parallel. They then talk about the recently specified 3GPP aspects on multi-access procedures such as Dual Steer and ATSSS. They present some scenarios and propose that those scenarios may replace some of the cases requiring cellular bonding. 5G-MAG also mentions that it might be useful to get assistance from the 5G System in order to efficiently split traffic across multiple available access networks. More specifically, the exposure of following information is requested:

-	Split recommendation based on type of media being sent (e.g., audio on link A, video on link B)

-	Split rate recommendation (e.g., 30% of traffic on link A, 70% on link B)

-	Split recommendation based on stream priority

-	Support for aggregation of multiple links into a unique media delivery session

Multi-band traffic steering: On the topic of multi-band traffic steering, 5G-MAG proposes that “certain scenarios may benefit from the availability on different radio carriers in different frequency bands. A camera may obtain connecion from one carrier (e.g., mid-band) but allocate intensive traffic on a carrier with larger capacity (e.g., mmWave). The ability to instruct the camera to make use of such additional capacity for intensive data traffic could be investigated.”

Network Assistance features for multipath: On this topic, 5G-MAG presents “ .. [cellular bonding] devices embed multiple SIM cards, advanced Network Assistance can be useful to better drive the bit rate allocation across multiple links and avoid retransmission quality degradation”. 5G-MAG proposes that exposing following information to the Media Client (e.g., 5GMS Client or RTC Client) is required:

-	Initial recommended throughput with associated latency

-	Upcoming network conditions

-	Upcoming network handover

2.2 Network assistance with multi-access media delivery in TR 26804

In Rel-19, as part of multi-access media delivery work, SA4 investigated on collaboration scenarios for multi-access media delivery using ATSSS, architecture mapping of 5GMS with ATSSS, and enhancing Application API for configuration of multi-access/multi-path transport session parameters. No work was performed on investigating the impact of 5GMS network assistance procedures with multi-access media delivery. At the end of FS_AMD study, some gaps related to network assistance and multi-access media delivery were identified and documented in clause 5.18.5.2.3. Below is the extract of the gaps as documented in TR 26804:

Clause 4.0.5 of TS 26.501 [15] describes the high-level procedure for Network Assistance. As specified in that clause Network Assistance enables the 5GMS Client to interrogate or manipulate the network Quality of Service for an ongoing media streaming session. Clause 5.4.4 of TS 26.510 [108] specifies the procedures used by a 5GMS Client to request a bit rate recommendation and a delivery boost respectively from the Media AF. For both these requests, it is unclear whether the5GMS Client is able to request, or receive, Network Assistance over a specific access network when the application flows at reference point M4 are using the Multi-Access PDU Session spanning multiple access networks as described in clause 5.18.1.3 of the present document.

3. Proposal

To support the enhancements requested by 5G-MAG for network assistance with multi-access media delivery, the following issues need to be studied. Please see the accompanying CR for the requested changes. They are included below.

When a UE and the network agree to use a Multi-Access PDU Session for a 5G Media Streaming session to use one or more access networks, following topics for network assistance are to be studied:

-	If M4 application flows are carried over multiple access networks, and the 5GMS Client intends to gather network assistance for adapting its behaviour (similar to case for Single Access PDU Session), whether the current bit rate recommendation and delivery boost network assistance facilities enough to support the 5GMS Client, or enhancements to assistance information is required?

-	Whether and how the 5GMS Client use current bit rate recommendation information from the 5GMS AF to distribute application flows at reference point M4 over one or more access networks

-	Whether the 5GMS Client is able to request, or receive, Network Assistance over, and for, a specific access network when the application flows at reference point M4d are using the Multi-Access PDU Session spanning multiple access networks as described in clause 5.18.1.3 of the present document?

-	Whether and how the network assistance information from the network to the 5GMS Client include the information such as below:

-	Split recommendation of application flows at reference point M4 over one or more access networks (e.g., 30% on one access and 70% over another access)

-	Split rate recommendation of application flows at reference point M4 over one or more access networks based on type of media being sent (e.g., audio over one access, video on another access)

-	Split recommendation of application flows at reference point M4 over one or more access networks based on stream priority

-	Initial recommended throughput with associated latency over one or more access networks

-	Other information such as upcoming network conditions, upcoming network handover etc.

4. References

[1]	“Uplink media delivery in 5G: Architecture and FEatures”, https://www.5g-mag.com/post/uplink-media-delivery-architectures-features, Nov 2024