3GPP TSG-SA4 Meeting #134	S4-251988

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025	revision of S4-251641

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.445 | CR | 0070 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correcting non-existing clause reference |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Dolby Laboratories Inc., Nokia, VoiceAge Corporation, Fraunhofer IIS, Ericsson LM, NTT, Orange, Panasonic Holdings Corporation, Philips International B.V., Huawei, Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | EVS_codec-SA4 |  |  |  |  |  | Date: |  |  | 2025-11-04 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The specification contains cross-references to non-existing clause 5.1.16. Other cross-references are incorrect. In addition, editorial mistakes were found. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the reference to the correct clause 5.1.14 and correct the other erroneous cross-references. Correct the editorial mistakes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Implementers may be confused which may lead to incorrect implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.3.3, 5.1.4, 5.1.13.6, 5.1.13.6.9, 5.3.1, 5.2.2.1.4, 5.6.1, 6.1.1.3.3, 6.3.3.2, 6.8.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Update of the CR header to list all affected clauses and describe the change and its reason more precisely. Adding missing clause header in change 6. |  |  |  |  |  |  |  |  |

#### 5.1.3.3	Conversion of input signals to 16, 25.6 and 32 kHz

If ACELP core is selected for WB, SWB or FB signals at bitrates higher than 13.2 kbps (see subclause 5.1.14), its internal sampling rate is set to 16 kHz rather than 12.8 kHz. If the input signal is sampled at 8 kHz, there is no conversion needed because for NB signals, ACELP core is always operated at 12.8 kHz. If the input signal is sampled at 16 kHz, no conversion is needed either and the input signal is only delayed by 15 samples which corresponds to 0.9375 ms. This is to keep all pre-processed signals aligned regardless of the bitrate or bandwidth. Thus, the input signal is resampled to 16 kHz only if its sampling frequency is 32 or 48 kHz.

The resampling operation is done in the same way as for the case of 12.8 kHz (see subclause 5.1.3.1), i.e. by means of FIR filtering. The coefficients of the LP filter are different but the filtering delay is still the same, i.e. 0.9375 ms.

The resampled signal is denoted  where n=0,..,319.

The input signal is converted to 25.6 kHz at 48 kbps and to 32 kHz at 96 or 128kbps but only for SWB and FB signals. The sampling conversion is again similar as in the case of 12.8 kHz with differences in LP filter coefficients. The resampled signals are denoted  and , respectively.

### 5.1.4	Pre-emphasis

A first-order high-pass filter is used to emphasize higher frequencies of the input signal and it is given by

(20)

where  is the pre-emphasis factor which is set to 0.68. The input signal to the pre-emphasis filter is  and the output signal is denoted .

If ACELP core is selected for WB, SWB or FB signals at bitrates higher than 13.2 kbps (see subclause 5.1.14), its internal sampling rate is 16kHz rather than 12.8kHz. In this case, the pre-emphasis is re-done at the end of the pre-processing chain, on  with . The resulting signal is denoted .

If  MDCT-based TCX is selected for SWB or FB high-rate LPC configurations,  is used as pre-emphasis factor when pre-emphasis is applied to signals at  a sampling rate higher than 16kHz.

#### 5.1.13.6	Speech/music classification

Music signals, in general, are more complex than speech and conform less to any known LP-based model. Therefore, it is of interest to distinguish music signals (generic audio signals) from speech signals. Speech/music classification then allows using a different coding approach to such signals. This new approach has been called the Generic audio Signal Coding mode (GSC), or the Audio Coding (AC) mode.

The speech/music classification is done in two stages. The first stage of the speech/music classifier is based on the Gaussian Mixture Model (GMM) and performs the best statistically based discrimination of speech from generic audio. The second stage has been optimized directly for the GSC mode. In other words, the classification in the second stage is done in such a way that the selected frames are suitable for the AC mode. Each speech/music classifier stage yields its own binary decision,  and  which is either 1 (music) or 0 (speech or background noise). The speech decision and the background noise decision have been grouped together only for the purposes of the speech/music classification. The selection of the IC mode for inactive signals incl. background noise is done later in the codec and described in subclause 5.1.13.5.7.

The decisions of the first and the second stage are refined and corrected for some specific cases in the subsequent modules, described below. The final decision about the AC mode is done based on  and  but the two flags are also used for the selection of the coder technology which is described in subclause 5.1.14.

##### 5.1.13.6.9	Decision about AC mode

The decisions in the first and in the second stage of the speech/music classifier, refined and corrected by the modules described so far are used to determine the usage of the AC mode. As mentioned before, in the AC mode GSC technology is used to encode the input signal. The decision about the AC mode is done always but the GSC technology is used only at certain bitrates. This is described in subclause 5.1.14.

Before making decision about the AC mode, the speech/music classification results are overridden for certain noisy speech signals. If the level of the background noise is higher than 12dB, i.e. when , then . This is a protection against mis-classification of active noisy speech signals.

For certain unvoiced SWB signals, GSC technology is preferred over the UC or GC mode which would normally be selected. In order to override the selection of the coding mode, there is a flag denoted  which is set to 1 under the following condition

(390)

where  is the raw coding mode, described in subclause 5.1.13.2.

The AC mode is selected if  or if .

### 5.3.1	General description

The decision algorithm for selecting the MDCT Coding mode, instead of CELP, is described in subclause 5.1.14. In general, the MDCT coding mode is selected whenever a signal is not speech or the bit rate for encoding is high enough for the transform coder. It always operates on full input signal independent of sample rate and selected audio bandwidth.

The EVS codec supports two major MDCT coding modes, the MDCT based TCX (TCX) and the High Quality MDCT coder (HQ). Both operate with different configurations depending on the bit rate for encoding. For some operating points, the MDCT coders are switched for optimized efficiency.  The two MDCT selector algorithms are described in subclauses 5.1.14.2 and 5.1.14.3. The following table describes the used mode configurations depending on bit rate and bandwidth.

Table 76: Overview of supported MDCT coding modes

| Bitrate (kbps) | Bandwidth | MDCT Classifier | MDCT mode |
| --- | --- | --- | --- |
| 7.2, 8 | NB |  | HQ low-rate |
| 9.6 | NB, WB,SWB |  | TCX low-rate |
| 13.2 | NB, WB, SWB | MDCT Selector | Switching HQ low-rate and TCX mid-rate |
| 16.4 | NB, WB, SWB FB |  |  |
| 24.4 | NB, WB |  | TCX mid-rate |
| 32 | WB |  | HQ high-rate |
| 24.4 | SWB,FB | MDCT Selector II | Switching HQ high-rate and TCX mid-rate |
| 32 | SWB,FB |  |  |
| 48 | WB,SWB,FB |  | TCX high-rate |
| 64 | WB,SWB,FB |  | HQ high-rate |
| 96,128 | WB,SWB,FB |  | TCX high-rate |

The next subclauses describe the common time-to-frequency transformation for both major MDCT modes. In the following the TCX and the HQ coder are described including their modules and configurations.

##### 5.2.2.1.4 LSF quantizer structure

A safety net, predictive or switched safety-net predictive multi-stage vector quantizer (MSVQ) is used to quantize the full length frame-end LSF vector for all modes except voiced mode at 16 kHz internal sampling frequency. The last stage of the MSVQ is a multiple scale lattice vector quantizer (MSLVQ) [22]. For each coding mode number of 1 to 4 unstructured VQ stages are used followed by a MSLVQ stage. The number of stages, number of bits per each stage and the codebook names for each coding mode are detailed in table 25. The codebook names are mentioned to illustrate how some of the codebooks are reused between modes.

Table 25: Optimized codebooks and their bit allocation for LSF quantizers

| Coding mode | Bits VQ safety net | Bits in VQ stages – safety net | Codebooks | Bits VQ predictive mode | Bits in VQ stages predictive mode | Codebooks |
| --- | --- | --- | --- | --- | --- | --- |
| Inactive NB | - | - | - | 5 | 5 | IAA_MA1 |
| Unvoiced NB | - | - | - | 8 | 4+4 | UVD_MA1 UVD_MA2 |
| Voiced NB | 8 | 4+4 | SVNB_SN1 SVNB_SN2 | 6 | 3+3 | GESVNB_AR1 GESVNB_AR2 |
| Generic NB | 9 | 5+4 | GETRNB_SN1 GETRNB_SN2 | 6 | 3+3 | GESVNB_AR1 GESVNB_AR2 |
| Transition NB | 9 | 5+4 | GETRNB_SN1 GETRNB_SN2 | - | - | - |
| Audio NB | 4 | 4 | AUNB_SN1 | 0 | 0 | - |
| Inactive WB | - | - | - | 5 | 5 | IAA_MA1 |
| Unvoiced WB | - | - | - | 12 | 4+4+4 | UVD_MA1 UVD_MA2 UVWB_MA3 |
| Voiced WB | 8 | 4+4 | SVWB_SN1 SVWB_SN2 | 6 | 3+3 | GESVWB_AR1 GESVWB_AR2 |
| Generic WB | 9 | 5+4 | GETRWB_SN1 GETRWB_SN2 | 6 | 3+3 | GESVWB_AR1 GESVWB_AR2 |
| Transition WB | 9 | 5+4 | GETRWB_SN1 GETRWB_SN2 | - | - | - |
| Audio WB | 4 | 4 | AUWB_SN1 | 0 | 0 | - |
| Inactive WB2 | - | - | - | 5 | 5 | IAA_MA1 |
| Unvoiced WB2 | - | - | - | - | - | - |
| Voiced WB2 | - | - | BC-TCVQ | - | - | BC-TCVQ |
| Generic WB2 | - | - | - | 5 | 5 | GEWB2_MA1 |
| Transition WB2 | 8 | 4+4 | TRWB2_SN1 TRWB2_SN2 | - | - | - |
| Audio WB2 | - | - | - | 5 | 5 | AUWB2_MA1 |
| CNG | 4 | 4 | CNG_SN1 | - | - | - |
| Generic WB >= 9.6kbps | - | - | - | 5 | 5 | GEWB_MA1 |

The WB2 voiced mode is using BC-TCVQ technology detailed in subclause 5.2.2.1.5.

Overall the optimized VQ codebooks use 14,368 kBytes and the MSLVQ parameters use 9.304 kBytes, including CNG mode.

The remaining LSF quantizer bits are used for the MSLVQ stage. The quantization in all the stages is done such that it minimizes a weighted Euclidean distortion. The calculation of the weights is detailed in subclause 5.2.2.1.1. The search in the multi-stage quantizer is done such that at most 2 candidates are kept per stage. For each candidate obtained in the search in the unstructured optimized VQ, a residual LSF vector is formed by subtracting from the LSF vector the codevectors obtained in each unstructured VQ stage. If there is one optimized VQ stage two residual LSF vectors are obtained, if there are two optimized VQ stages, 4 candidates are obtained and so on.

Each residual LSF vector is split into two 8-dimensional sub vectors. Each sub vector is coded as follows. The lattice codebook obtained through the reunion of three D8+ lattice truncations differently scaled. Each lattice truncation has a different number of leader classes. The leader classes contained in the lattice truncations are given in table 26.

Table 26: Lattice leader class vectors

| Leader class index | Leader class vector | Leader class index | Leader class vector |
| --- | --- | --- | --- |
| 0 | 1.0, 1.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0 | 25 | 3.0, 1.0, 1.0, 1.0, 1.0, 1.0, 0.0, 0.0 |
| 1 | 0.5, 0.5, 0.5, 0.5, 0.5, 0.5, 0.5, 0.5 | 26 | 3.0, 2.0, 1.0, 0.0, 0.0, 0.0, 0.0, 0.0 |
| 2 | 1.0, 1.0, 1.0, 1.0, 0.0, 0.0, 0.0, 0.0, | 27 | 1.5, 1.5, 1.5, 1.5, 1.5, 1.5, 0.5, 0.5 |
| 3 | 2.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0 | 28 | 2.5, 1.5, 1.5, 1.5, 0.5, 0.5, 0.5, 0.5 |
| 4 | 1.5, 0.5, 0.5, 0.5, 0.5, 0.5, 0.5, 0.5 | 29 | 2.5, 2.5, 0.5, 0.5, 0.5, 0.5, 0.5, 0.5 |
| 5 | 1.0, 1.0, 1.0, 1.0, 1.0, 1.0, 0.0, 0.0 | 30 | 3.5, 0.5, 0.5, 0.5, 0.5, 0.5, 0.5, 0.5 |
| 6 | 2.0, 1.0, 1.0, 0.0, 0.0, 0.0, 0.0, 0.0 | 31 | 2.0, 2.0, 2.0, 1.0, 1.0, 1.0, 1.0, 0.0 |
| 7 | 1.5, 1.5, 0.5, 0.5, 0.5, 0.5, 0.5, 0.5 | 32 | 2.0, 2.0, 2.0, 2.0, 0.0, 0.0, 0.0, 0.0 |
| 8 | 1.0, 1.0, 1.0, 1.0, 1.0, 1.0, 1.0, 1.0 | 33 | 3.0, 1.0, 1.0, 1.0, 1.0, 1.0, 1.0, 1.0 |
| 9 | 2.0, 1.0, 1.0, 1.0, 1.0, 0.0, 0.0, 0.0 | 34 | 3.0, 2.0, 1.0, 1.0, 1.0, 0.0, 0.0, 0.0 |
| 10 | 2.0, 2.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0 | 35 | 4.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0 |
| 11 | 1.5, 1.5, 1.5, 0.5, 0.5, 0.5, 0.5, 0.5 | 36 | 1.5, 1.5, 1.5, 1.5, 1.5, 1.5, 1.5, 0.5 |
| 12 | 2.5, 0.5, 0.5, 0.5, 0.5, 0.5, 0.5, 0.5 | 37 | 2.5, 1.5, 1.5, 1.5, 1.5, 0.5, 0.5, 0.5 |
| 13 | 2.0, 1.0, 1.0, 1.0, 1.0, 1.0, 1.0, 0.0 | 38 | 2.5, 2.5, 1.5, 0.5, 0.5, 0.5, 0.5, 0.5 |
| 14 | 2.0, 2.0, 1.0, 1.0, 0.0, 0.0, 0.0, 0.0 | 39 | 3.5, 1.5, 0.5, 0.5, 0.5, 0.5, 0.5, 0.5 |
| 15 | 3.0, 1.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0 | 40 | 2.0, 2.0, 2.0, 2.0, 1.0, 1.0, 0.0, 0.0 |
| 16 | 1.5, 1.5, 1.5, 1.5, 0.5, 0.5, 0.5, 0.5 | 41 | 3.0, 2.0, 1.0, 1.0, 1.0, 1.0, 1.0, 0.0 |
| 17 | 2.5, 1.5, 0.5, 0.5, 0.5, 0.5, 0.5, 0.5 | 42 | 3.0, 2.0, 2.0, 1.0, 0.0, 0.0, 0.0, 0.0 |
| 18 | 2.0, 2.0, 1.0, 1.0, 1.0, 1.0, 0.0, 0.0 | 43 | 3.0, 3.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0 |
| 19 | 2.0, 2.0, 2.0, 0.0, 0.0, 0.0, 0.0, 0.0 | 44 | 4.0, 1.0, 1.0, 0.0, 0.0, 0.0, 0.0, 0.0 |
| 20 | 3.0, 1.0, 1.0, 1.0, 0.0, 0.0, 0.0, 0.0 | 45 | 1.5, 1.5, 1.5, 1.5, 1.5, 1.5, 1.5, 1.5 |
| 21 | 1.5, 1.5, 1.5, 1.5, 1.5, 0.5, 0.5, 0.5 | 46 | 2.5, 1.5, 1.5, 1.5, 1.5, 1.5, 0.5, 0.5 |
| 22 | 2.5, 1.5, 1.5, 0.5, 0.5, 0.5, 0.5, 0.5 | 47 | 2.5, 2.5, 1.5, 1.5, 0.5, 0.5, 0.5, 0.5 |
| 23 | 2.0, 2.0, 1.0, 1.0, 1.0, 1.0, 1.0, 1.0 | 48 | 3.5, 1.5, 1.5, 0.5, 0.5, 0.5, 0.5, 0.5 |
| 24 | 2.0, 2.0, 2.0, 1.0, 1.0, 0.0, 0.0, 0.0 | 49 |  |

Given the bitrate available for the lattice codebook, the codebook is thus defined by a set of three integers representing the number of leader vectors for each truncation and three positive real number representing the scale for each lattice truncation. For instance a multiple scale lattice structure is defined by the number of leaders (20, 14, 5, 16, 10, 0) and the scales (1.057, 1.794, 2.896, 1.154, 1.860, 0.0). It means that the first subvector is quantized with a structure having three lattice truncations having 20, 14, and 5 leader classes respectively, which are scaled with the scales 1.057, 1.794, 2.896, respectively. The second subvector has only two truncations having 16 and 10 leader classes respectively. The truncations are ordered such that, for each subvector, their number of leader classes is descendingly ordered.

The difference in number of bits between the total number of bits for LSF end encoding, the prediction bit if needed and the number of bits used for unstructured VQ is used for the MSLVQ stage. The quantization in all the stages is done such that it minimizes a weighted Euclidean distortion. The calculation of the weights is detailed in subclause 5.2.2.1.1.

Suppose  is the current LSF 8-dimensional sub vector and w its corresponding weight vector. The vector  is normalized, i.e. component wise multiplied with the inverse of the off line estimated standard deviation.  The resulting vector  is further sorted in descending order based on the absolute value of its components and the weights vector is arranged following the same order. Let  be the vector of descendingly sorted absolute values of  and  the correspondingly sorted weights vector. The weighted distance to the best codevector of each leader class corresponds to:

(482)

where is the leader vector corresponding to class  and  is the scale of the truncation . Each lattice codebook has at most 3 truncations with their corresponding scales. Each truncation has a given number of leader vector classes. The sum of cardinalities of the classes for the truncations forming the codebook for the first LSF subvector and for the second subvector are within the number of bits for the considered operating point given by the overall bitrate and bandwidth. Computing in the transformed input space the second and the third terms from equation (482) directly gives a relative measure of goodness for the best codevector from the leader class  and truncation  which may be considered as a potential codevector for the truncation  and the leader class .

(483)

The part of equation (483) that is independent of the scale is calculated only once for all the leader classes from the first truncation, which is the one having the highest number of leader classes. When adding the last term to the first sum of equation (483) the product is considered with negative sign if the parity constraint of the leader  is not obeyed by the signs of the vector . The contribution of the scale values is considered only afterwards in order to obtain the value . The leader class vector  and the truncation j having the smallest  correspond to the codevector of the current input vector. The inverse permutation of the sorting operation on the input vector applied on the winning leader vector  gives the lattice codevector after applying also the corresponding signs. If the parity of leader vector  is 0 the signs are identical to the signs of the input vector. If the parity is 1 the signs are similar to the signs of the input vector with the constraint that the number of negative components is even. If the parity is -1 the signs are similar with signs of the input vector with the constraint that the number of negative components is odd. The final codevector is obtained after multiplication with the scale  and with the inverse of the component-wise off-line computed standard deviation. The standard deviations are individually estimated for each coding mode and bandwidth.

The candidate quantized LSF vectors are obtained by adding each lattice quantized residual to the corresponding candidates from the upper stages. The obtained candidates are increasingly sorted. For each sorted candidate the weighted Euclidean distortion with respect to the original LSF vector is calculated. The candidate that minimizes this distortion is selected as codevector to be encoded. The indexes corresponding to the first unstructured optimized VQ codebooks together with the index in the lattice codebook are written in the bitstream. The index for the lattice codebook is obtained as described in subclause 5.2.2.1.4.2.

For the CNG mode, using a total of 29 bits for the LSF quantization, the multiple scale lattice codebook structure is specific to each of the 16 codevectors obtained in the first stage. In addition based on the value of the last component of the 16 dimensional LSF vector only part of the first stage codebook is searched. If the last component of  is larger than 6350 then the search is done only for the first 6 codevectors of the first stage and the LSF vector corresponds to internal sampling frequency of 16kHz, otherwise the search is performed within the last 10 codevectors of the first stage.

### 5.6.1	Overview

This subclause describes the discontinuous transmission (DTX) scheme and the comfort noise generation (CNG) algorithm. The DTX/CNG operation, which is activated on a command line, is used to reduce the transmission rate by simulating background noise during inactive signal periods. The regular DTX/CNG modes are supported for bit rates up to 24.4 kbps. For higher bit rates, the EVS codec supports a less aggressive DTX/CNG scheme that only switches to CNG for low input signal power.

The reduction of the transmission rate during inactive periods is achieved by coding the parameters referred to as comfort noise (CN) parameters. These parameters are used at the decoder to regenerate the background noise as well as possible, by respecting the spectral and temporal content of the background noise at the encoder. In the EVS Codec, the CNG algorithm reproduces high quality comfort noise by choosing between a linear prediction-domain based coding mode (LP-CNG) and a frequency-domain based coding mode (FD-CNG), according to the input characteristics. Each of the two coding modes utilizes a different set of CN parameters. In the LP-CNG mode, four CN parameters are analyzed and encoded: the low-band excitation energy, the low-band signal spectrum, the low-band excitation envelope and the high-band energy, where the high-band energy is only encoded for SWB/FB input. In the FD-CNG mode, the CN parameters consisting of global gain and spectral energies grouped in critical bands. Those parameters are encoded by a vector quantizer for transmission.

When the codec is operated with the DTX/CNG operation, the signal activity detector (SAD) is used to analyse the input signal to determine whether the signal comprises an active or inactive signal (see SAD decision in subclause 5.1.12). Based on its analysis, the SAD generates a SAD flag, , whose state indicates whether the signal is active (= 1) or merely a background noise (= 0). When = 1, the regular encoding and decoding process is performed, as in the default option. When = 0, DTX functions are run at the encoder that transmit either a silence insertion descriptor (SID) frame or a NO_DATA frame. The SID frame contains the CN parameters, which are used to update the statistics of the background noise at the decoder, whereas the NO_DATA frame is empty. The SID frame is always encoded using 48 bits regardless the actual CNG mode operating.

Further, hangover logic, as described in subclause 5.1.12.8, is used to enhance the quality of SID frames. The hangover logic in the SAD algorithm is such that the encoder waits for a certain number of frames before switching from the active signal to inactive signal. If the background noise contains transients that force the encoder to switch from inactive signal to active signal and then back to inactive signal in a very short time period, no hangover is used.

##### 6.1.1.3.3	Pitch enhancer

A pitch enhancer scheme modifies the total excitation of voiced signals by filtering the algebraic codebook excitation through an innovation filter. The filter frequency response emphasizes the higher frequencies and reduces the energy of the low-frequency portion of the innovative codevector. The filter coefficients are related to the periodicity of the signal. Therefore, the pitch enhancer is not applied to excitation in UC at low bit rates, i.e. bitrates < 9600 b/s.

A filter of the form

(1480)

is used where  if Hz and  if Hz, with being a periodicity factor given in equation (1475). The filtered algebraic codebook vector in the current subframe is given by

(1481)

where the out-of-subframe samples and are set to zero. The updated post-processed excitation is given by

(1482)

The above procedure can be done in one step by updating the excitation as follows

(1483)

whereis the modified algebraic codebook gain from equation (1479).

#### 6.3.3.2	CELP coding mode to HQ MDCT coding mode

When the previous frame is CELP and the current frame is to be coded by HQ MDCT, the current frame is a transition frame in which two types of decoding are used:

- Constrained CELP coding and (when required) simplified time-domain BWE coding

- HQ MDCT coding with a modified window

Constrained CELP decoding means here that CELP is restricted to decode only a subset of CELP parameters, to reuse parameters (LPC coefficients) from the previous CELP frame, and to cover only the first subframe of the current frame. These constraints are set to minimize the bit budget taken by continuing CELP decoding in the current frame, this bit budget being taken out of HQ MDCT decoding.

As shown in Figure 112b, the transition frame includes at the decoder side a gap between the previous output frame (decoded by CELP) and the decoded signal with only the contribution from HQ MDCT. The length of this gap at the decoder  is 4.375 ms, which corresponds to 10-5.625 ms (10 ms for ¼ of MDCT window support – 5.625 ms which is the length of the zero segment at the beginning of the ALDO synthesis window). In addition, an overlap period of 1.825 ms is used to attenuate discontinuities between CELP and HQ MDCT decoded signal. The total transition region between CELP and HQ MDCT in the decoder (grey zone decoder  in Figure 112b)  is 4.375+1.825 = 6.25 ms.

Figure 112b: Modified MDCT synthesis window in the transition frame (CELP to HQ MDCT).

##### 6.8.3.1.2	Estimation of sub-frame gains based on LP spectral envelopes

The signal in low-band (0-6400 Hz) is generated based on a source-filter model, where the filter is given by  the synthesis filter . Similarly, as shown in subclause 6.8.3.3, the signal in high-band (above 6400 Hz) is generated based on a source-filter model; the filter in high-band is an linear predictive (LP) filter  derived from the LP filter in low-band.

Since the low and high-band are combined in the final synthesis, a preliminary equalization step is performed to match the levels of the two LP filters at a given frequency. At 6400 Hz the shape of  is already too decreasing, therefore a frequency of 6000 Hz has been chosen for this equalization frequency point.

In each sub-frame, the frequency response of the LP filter  in the low-band and the LP filter   in the high-band are computed at the frequency of 6000 Hz:

,	(2024)

and

, 	(2025)

where =0.9 at 6.6 kbit/s and 0.6 at other modes (from 8.85 to 23.85 kbit/s)

These values are computed efficiently using the following pseudo-code:

px = py = 0

rx = ry = 0

for i=0 to 16

px = px + Ap[i]*exp_tab_p[i]

py = py + Ap[i]*exp_tab_p[33-i]

rx = rx + Aq[i]*exp_tab_q[i]

ry = ry + Aq[i]*exp_tab_q[33-i]

end for

P = 1/sqrt(px*px+py*py)

R = 1/sqrt(rx*rx+ry*ry)

where Aq[i]= are the coefficients of , Ap[i]=are the coefficients of , sqrt() corresponds to the square root operation and the tables exp_tab_p and  exp_tab_q  of size 34 contain the real and imaginary parts of complex exponentials at 6000 Hz:

exp_tab_p[i] = 	(2026)

and

exp_tab_q[i] = 	(2027)

The ratio  provides an estimated gain to be used in each sub-frame to align at the given frequency point (6000 Hz) the level of LP spectral envelopes in two different bands. This value is further refined to optimize overall quality.

To avoid over-estimating the sub-frame gain in high-band which could result in too high enrgy in the high hand, an additional LP filter of lower order is also computed based on the lower-band LP filter. An LP filter of order 2 is derived by truncating the filter  decoded in low band to an order of 2 (instead of an order of 16). The stability of this truncated filter is ensured by the following steps:

- The filter is initialized as: , i=1, 2

- Reflection coefficients are computed: ,

- Filter stability and control of resonance is forced by applying the following conditions:

(2028)

(2029)

- The coefficients of the LP filter of order 2 are then given by: ,

The frequency response of the resulting LP filter of order 2 is computed as follows:

,	(2030)

which can be computed efficiently using a similar pseudo-code with tables exp_tab_p and  exp_tab_q  It was found that, for some signals, using the value  instead of the value  takes better into account the influence of spectral tilt in the actual signal spectrum and therefore avoids the influence of spectral peaks or valleys near the reference frequency point (6000 Hz) which could bias the value .

The optimized gain to shape the excitation in high-band is then estimated based on , , .

Before the gain is estimated, an unvoiced flag is determined first so that the gain estimation will be different for unoiced speech and voiced speech. An unvoicing parameter is defined as,

(2031)

wherein   is a smoothed voicing parameter of  .  The unvoicing parameter is first smoothed by

(2032)

Then, it is further smoothed by,

(2033)

A relative difference parameter is now defined as

(2034)

An initial unvoiced flag is decided by the following procedure,

(2035)

A final unvoiced flag is limited to

(2036)

The gain computed is performed according to the voicing of the signal:

If the sub-frame is classified as unvoiced

(2037)

where the smoothed value in the current sub-frame of index  is computed as

(2038)

and

(2039)

and

(2040)

Otherwise, if the sub-frame is not classified as unvoiced:

(2041)

where the smoothed value in the current sub-frame of index  is computed as

(2042)

with  if  and ,  otherwise, and

(2043)

and where

(2044)

and

(2045)

- *** End of changes ***