3GPP TSG-S4 Meeting  #134	S4-252036

# Dallas, USA, 17th November – 21st November 2025

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR | 0017 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Energy_Ph2_MED] Updating key issue description on Application Service Provider provisioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics Co. Ltd., |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Energy_Ph2_MED |  |  |  |  |  | Date: |  |  | 2025-11-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A document S4-251923 was treated in MBS session during SA4#134 that documented options for consideration for potential solutions. This document proposes to incorporate those options into the TR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a framework of options to Key Issue #4 (Energy-related configuration by the Application Service Provider for media delivery services) that potential solutions can refer to while defining solutions for key issue on energy related configuration by the application service provider. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Study on energy management for media streaming would be incomplete |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | .. |  |  |  |  |  |  |  |  |

Key Issue #4
Energy-related configuration by the Application Service Provider for media delivery services
(All New Text)

### 6.4.3	Framework for solutions

#### 6.4.3.1	Different energy services with their requirements

Editor’s Note: Document framework for solutions related to use case on energy saving service for UE specified in clause 5.1 of present document.

#### 6.4.3.2	Dynamic service adjustment support in the network based on energy information

NOTE:	The title for the Use Case 5.2 "dynamic service adjustment support in the network based on energy information" in TR 22.883 [22883] is slightly misleading because the scope includes "dynamic service adjustment at both user and network levels".

##### 6.4.3.2.1	Media service reactions due to dynamic QoS adjustment by the network because of energy considerations

The following options are considered while defining solutions to this Key Issue.

NOTE:	The options presented are not exhaustive. More options may be added.

Table 6.4.3.2.1-1: Options for media service operations based on possible media delivery provisioning information with dynamic QoS adjustment

| Option | Media Delivery provisioning information | Network behaviour | Media Delivery operations | What needs to be defined/enhanced? |
| --- | --- | --- | --- | --- |
| 1 | - No energy configuration at Media AF. Configuration of energy information/constraints is performed outside the scope of Media Delivery configuration Impact to Media Delivery Provisioning: None | - Network decides to adjust bearer service. No information is notified to any of the Media Delivery architecture entities | None | None |
| 2 | - No energy configuration at Media AF. Configuration of energy information/constraints is performed outside the scope of Media Delivery configuration Impact to Media Delivery Provisioning: None. | - Network decides to adjust bearer service. - The Media AF is notified that one or more application sessions are going to be impacted because of a network decision to degrade the quality of underlying bearer connections as a result of energy conservation tasks. | - The Media AF, may inform other Media Delivery architecture entities, including the Media Client of possible impact. - The Media Client may inform the Media-aware Application of the same | - How is the Media AF informed of impacted application sessions? - How the Media AF notifies other Media Delivery architecture entities e.g., the Media Client of possible impact. - How the Media Client informs Media-aware Application of possible impact. |
| 3 | - Energy parameters are provisioned in the Media Delivery System. - The configured energy information is minimal, for instance, type of energy source (renewable/non-renewable), enable energy rationing in the network etc. Impact to Media Delivery architecture Provisioning: Minimal. | - Network entities are provided with energy information of application sessions by the Media AF. - Network uses the energy information to monitor their underlying bearer connections. - Network decides to adjust bearer service of one or more of application sessions. - The Media AF is notified that one or more application sessions are going to be impacted because of a network decision to degrade the quality of underlying bearer connections as a result of energy conservation tasks. | - The Media AF may inform other Media Delivery architecture entities, including the Media Client of possible impact. - The Media Client may inform the Media-aware Application of the same. | - Format of energy information to be provisioned at the Media AF. - How is the Media AF informed of impacted application sessions? - How the Media AF notifies other Media Delivery architecture entities e.g., the Media Client of possible impact. - How the Media Client informs Media-aware Application of possible impact. |
| 4 | - Energy parameters are provisioned in the Media Delivery System. The energy provisioning information may include: - Types of energy sources (renewable/non-renewable etc) - Enable energy rationing. - Time periods for energy rationing. - Limits (or parameter values) of drop-off in application quality due to energy conservation actions. - Expected Media Client behaviour when drop off in quality is observed (e.g., some Policy Templates may not be applicable, limits on network boost during network assistance etc.). - More. Impact to Media Delivery architecture Provisioning: Some | - Network entities are provided with energy information of application sessions by the Media AF. - Network uses the energy information to monitor their underlying bearer connections Network decides to adjust bearer service of one or more of application sessions. - The Media AF is notified that one or more application sessions are going to be impacted because of a network decision to degrade the quality of underlying bearer connections as a result of energy conservation tasks. | - The Media AF, may inform other Media Delivery architecture entities, including the Media Client of possible impact. - The Media Client may inform the Media-aware Application of the same. - The Media AF may invoke necessary operations to support expected Media Client behaviour during the adjustment period. | - Format of energy information to be provisioned at the Media AF - How is the Media AF informed of impacted application sessions? - How the Media AF notifies other Media Delivery architecture entities e.g., the Media Client of possible impact - How the Media Client informs Media-aware Application of possible impact. - Operations to support Media Client behaviour. |

##### 6.4.3.2.2	Media service reactions due to energy configuration information without QoS adjustment by the network

#### 6.4.3.3	Tolerance to QoS degradation due to network energy savings

NOTE:	The title for the Use Case 5.3 "tolerance to QoS degradation due to network energy savings" in TR 22.883 [22883] is slightly misleading because its scope is "to optimize energy consumption without degrading the network QoS".

##### 6.4.3.3.1	Media service reactions due to QoS degradation by the network

The following options are considered while defining solutions to this Key Issue.

NOTE:	The options presented are not exhaustive. More options may be added.

Table 6.4.3.3.1-1: Options for media service operations based on possible media delivery provisioning information with QoS degradation

| Option | Media Delivery architecture provisioning information | Network behaviour | Media Delivery architecture operations | What needs to be defined/enhanced? |
| --- | --- | --- | --- | --- |
| 1 | - No tolerance configuration at Media AF. Configuration of tolerance information is performed outside the scope of Media Delivery architecture configuration Impact to Media Delivery architecture Provisioning: None. | - Network decides to degrade QoS of underlying bearer because of energy saving actions. No information is notified to any of the Media Delivery architecture entities. | None | None |
| 2 | - No tolerance configuration at Media AF. Configuration of tolerance information is performed outside the scope of Media Delivery architecture configuration. Impact to Media Delivery architecture Provisioning: None. | - Network decides to degrade QoS of underlying bearer because of energy saving actions. - The Media AF is notified that one or more application sessions are going to be impacted because of a network decision to degrade QoS of the underlying bearers as a result of energy conservation tasks. | The Media AF, may inform other Media Delivery architecture entities, including the Media Client of possible impact. - The Media Client may inform the Media-aware Application of the same | - How is the Media AF informed of impacted application sessions? - How the Media AF notifies other Media Delivery architecture entities e.g., the Media Client of possible impact. - How the Media Client informs Media-aware Application of possible impact. |
| 3 | - Energy parameters are provisioned in the Media Delivery architecture System. The energy provisioning information may include: - Types of energy sources (renewable/non-renewable etc). - Offsets or absolute parameter values of drop off in application QoS/QoE due to energy saving actions. - More. Impact to Media Delivery architecture Provisioning: Some. | - Network entities are provided with tolerance information of application sessions by the Media AF. - Network uses the tolerance information to monitor their underlying bearer connections. - Network decides to degrade QoS of underlying bearer because of energy saving actions. - The Media AF is notified that one or more application sessions are going to be impacted because of a network decision to degrade QoS of underlying bearers as a result of energy conservation tasks. | - The Media AF, may inform other Media Delivery architecture entities, including the Media Client. - The Media AF may check if QoS/QoE of application degraded beyond configured tolerance. - The Media AF may wait to hear from Media Client about degradation in QoS/QoE - The Media Client may inform the Media-aware Application of the same. | - Format of tolerance information to be configured at the Media AF. - How is the Media AF informed of impacted application sessions? - How the Media AF notifies other Media Delivery architecture entities e.g., the Media Client. - How the Media Client informs Media-aware Application. - How does the Media AF check/infer if QoS/QoE of application is degraded beyond configured tolerance. - Any Media Delivery architecture operations to restrict because of exceeding configured tolerance. |

##### 6.4.3.3.2	Media service reactions due to energy configuration information without QoS degradation

end of CHANGEs