3GPP TSG-SA WG4 Meeting #134	S4-251878

# Dallas, US, 17 – 21 November 2025

Source:	Fraunhofer IIS, Dolby Laboratories Inc.

Title:	Registration of xHE-AAC as a WebCodec

Agenda item:	8.4

Document for:	Agreement

# Introduction

This document analyzes the gaps in the W3C WebCodecs registration for the AAC family of codecs. While AAC WebCodecs Registration already contains the registration for 3GPP’s AAC, AAC+ and eAAC+, xHE-AAC is missing out, despite support for xHE-AAC in many browser implementations.

# Potential modifications of AAC WebCodecs registration

The following excerpt from AAC WebCodecs Registration with change marks outlines how it would need to be amended to support xHE-AAC.

## 1. Fully qualified codec strings

This codec has multiple possible codec strings:

- "mp4a.40.2" — MPEG-4 AAC LC

- "mp4a.40.02" — MPEG-4 AAC LC, leading 0 for Aud-OTI compatibility

- "mp4a.40.5" — MPEG-4 HE-AAC v1 (AAC LC + SBR)

- "mp4a.40.05" — MPEG-4 HE-AAC v1 (AAC LC + SBR), leading 0 for Aud-OTI compatibility

- "mp4a.40.29" — MPEG-4 HE-AAC v2 (AAC LC + SBR + PS)

- "mp4a.40.42" — xHE-AAC (MPEG-D USAC + MPEG-D DRC)

- "mp4a.67" — MPEG-2 AAC LC

## 2. EncodedAudioChunk data

If the bitstream is in adts format, the [[internal data]] of EncodedAudioChunks are expected to be an ADTS frame, as described in section 1.A.3.2 of [iso14496-3].

If the bitstream is in aac format, the [[internal data]] of EncodedAudioChunks are expected to be a raw AAC frame (syntax element raw_data_block()), as described in section 4.4.2.1 of [iso14496-3]. xHE-AAC shall be in aac format.

## 3. AudioDecoderConfig description

If description is present, it is assumed to be an AudioSpecificConfig as defined in [iso14496-3] section 1.6.2.1, Table 1.19, and the bitstream is assumed to be in aac.

If the description is not present, the bitstream is assumed to be in adts format.

The sampleRate and numberOfChannels members are ignored.

### Informative References

[ISO14496-3]

ISO/IEC 14496-3:2019 - Information technology — Coding of audio-visual objects — Part 3: Audio. 2019-12. URL: https://www.iso.org/standard/76383.html

[WEBCODECS-CODEC-REGISTRY]

Paul Adenot; Bernard Aboba. WebCodecs Codec Registry. URL: https://w3c.github.io/webcodecs/codec_registry.html

# Summary and Proposal

The sources identified the need for extending  AAC WebCodecs Registration for xHE-AAC. Since this document is under W3C control, it is proposed to liaise to W3C Media Working Group to request inclusion of the changes as highlighted above.