Source:		Qualcomm, Inc.

Title:		Comparison of “True” SNRi with G.160 APP2 AMD2 SNRi

Document for:	Discussion

Agenda Item:	8.5

# Introduction

During the Study Group 16 rapporteur’s meeting in November 2010, as reported in [1], there were discussions on a proposal [2] to revise ITU-T Rec. G.160 Appendix II to improve objective measures of noise suppression performance for non-stationary noise suppressor. The discussions resulted in amendment 2 of ITU-T G.160 Appendix 2 [3]. Currently, the applicability of [3] is under study by 3GPP SA4 SQ SWG.

# Discussion

In contribution [4], the G.160 Appendix II AMD 1 is validated for an ideal noise suppressor, which reduces noise level by a constant while maintaining the speech unprocessed. [2 and 5] show that even for ideal noise suppressor, the error for SNRI is sometimes unacceptable due to the speech power which resides in short speech pauses.

The present discussion focus on the metric estimation error for practical noise suppressors, which tend to have higher noise suppression in the short speech pauses than in the speech active regions. The issue is illustrated by the following analysis.

## Calculation of true value of TNLR, NPLR, SNRI, and DSN

The justification for using an ideal noise suppressor is that separated output residual noise and output speech are not readily available. However, for validation of the measures in G.160 Appendix II, we may be able to generate separate output noise and output speech for certain types of noise suppressors. Suppose a noise suppressor which applies spectral gains to the input speech from one microphone. Let S, N, D, H, Y=S and E denote clean speech spectrum, noise spectrum, noisy speech spectrum, diagonal spectral gain matrix, enhanced speech spectrum and error. With additive noise assumption, the error can be represented by:

D=S+N

Y= S=HD=HS+HN

E=S-S=H-IS+HN

, where the first term H-IS is called speech distortion and the second term HN is called residual noise [6].

Suppose that we have clean speech, noise, and noisy speech. As for the noise suppressor which applies spectral gains to one of the microphone signals to produce output and of which source code is available, we can get H for every frame and apply it to the clean speech and noise only signals as well as noisy speech signal. Then, 6 signals would be available - S, N, D, S=HS, N= HN, Y=S=HD. With these 6 signals, we can compute the actual TNLR, NPLR, SNRI and DSN.

G.160 App. II TNLR is defined as

,	(1)

,where

Kpse	is total number of noise frames during speech pauses (frame power of clean speech signal < sp_lvl + th_nh) with active noise (frame power of noisy input signal > cft_lvl);

is a constant that should be set at 8*10–8

cft_lvl	is a comfort noise level constant that will default to –48 dB.

“True” TNLR can be defined as

,	(2)

where

Kpse	is total number of noise frames during speech pauses (frame power of clean speech signal < sp_lvl + th_nh) with active noise (frame power of noisy input signal > cft_lvl);

is a constant that should be set at 8*10–8

cft_lvl	is a comfort noise level constant that will default to –48 dB

using the input noise signal  and output noise signal .

G.160 App. II NPLR is computed as (1) except it is computed for short pauses. “True” NPLR can be calculated as (2) except it is computed for short pauses.

G.160 App. II SNRI is defined as

(3)

(4)

(5)

,where

- Ksph is the total number of frames containing speech of high power;

- Knse is the total number of frames containing noise during short speech pauses;

- is a constant that should be set at 8*10-8 ;

- is a constant that should be set at 0.0631 (equivalent to -12 dB).

The summation with respect to indices k and p is carried out in speech and noise frames of 80 samples, respectively.

SNRI_mij and SNRI_lij are computed correspondingly for medium and low power frames.

The final measure for signal-to-noise ratio improvement is equal to the SNRI for high level speech.

.	(6)

“True” SNRI is defined as

(7)

(8)

(9)

,where

- Ksph is the total number of frames containing speech of high power;

- is a constant that should be set at 8*10-8 ;

- is a constant that should be set at 0.0631 (equivalent to -12 dB).

SNRI_mij and SNRI_lij are computed correspondingly for medium and low power frames.

The final measure for signal-to-noise ratio improvement is equal to the SNRI for high level speech.

(10)

, where input and output speech/noise signals in speech active regions are used. Basically, the definition of “True” SNRI is based on the interpretation that SNRI is the estimation of the SNR improvement during speech active periods, following the intention of G.160 Appendix II AMD 2.

The definition of G.160 App. II DSN is

.		(11)

The definition of the “true” DSN, which is the estimate of the speech level attenuation, is

(12)

.		(13)

## Comparison between “True SNRI and G.160 App. II SNRI

ITU-T P.501 database was used to compare True SNRI and G.160 App. II. P.501 speech data base has 4 files for each of the 10 languages – Chinese, English, American English, Finnish, French, German, Italian, Japanese, Polish, and American Spanish. The four files for each language were concatenated to form one speech file per language which is normally 32 sec long. Also, there are 9 types of monaural noise recordings including cafeteria, in car, street, car interior noise with radio, construction, metro train, office, railway station, and restaurant noise.

All the 10 speech files and 9 noise files were filtered with MSIN filter, and then the active speech level was adjusted to -26 dBov. The RMS level of the noises were adjusted to -32dBov and -38dBov to simulate 2 SNR conditions. So, there were 10 speech * 9 noise * 2 SNR = 180 test files in total.

One example of a single mic noise suppression algorithm was used for the evaluation.

The results are shown in Table 1 and a scatter plot of 180 points with G.160 App. II SNRI on the x-axis and True SNRI on the y-axis is also provided in Figure 1. The Pearson correlation coefficient is 0.7413.

The SNRI numbers can deviate from the true values significantly, with average 4.18 dB error, while the other metrics provided by the method have lower estimation errors. This large value of error indicates that G.160 App. II SNRI is not an accurate estimate of the SNR improvement during speech activity. Moreover, the correlation between True SNRI and G.160 App. II SNRI is less than ideal, meaning that the noise suppression in the speech active region cannot be easily related to the noise suppression in short pauses for certain suppressors. Note that the G.160 SNRI error, which is the estimated value substracted by true value, is positive for a true noise suppressor, while for ideal noise suppressor, the error was found to be negative.

Table 1. G.160 App. II and true values for P.501 DB.

| Speech | Noise | SNR | G.160 App II |  |  |  | True values |  |  |  | Error |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TNLR | NPLR | SNRI | DSN | TNLR | NPLR | SNRI | DSN | TNLR | NPLR | SNRI | DSN |
| Chi | Cafeteria | 6 | 15.44 | 8.07 | 5.38 | -2.69 | 15.50 | 8.19 | 2.60 | -2.80 | -0.06 | -0.11 | 2.78 | 0.11 |
| Eng | Cafeteria | 6 | 20.69 | 8.93 | 6.90 | -2.03 | 20.85 | 10.60 | 5.68 | -1.80 | -0.16 | -1.67 | 1.22 | -0.23 |
| Fr | Cafeteria | 6 | 16.98 | 11.31 | 8.65 | -2.66 | 17.05 | 11.66 | 3.81 | -2.65 | -0.08 | -0.35 | 4.85 | -0.01 |
| Ger | Cafeteria | 6 | 20.28 | 11.70 | 8.56 | -3.14 | 20.30 | 12.20 | 5.17 | -3.10 | -0.01 | -0.50 | 3.39 | -0.04 |
| Ita | Cafeteria | 6 | 13.71 | 7.74 | 6.69 | -1.05 | 13.80 | 9.36 | 3.18 | -1.12 | -0.09 | -1.62 | 3.52 | 0.07 |
| Jap | Cafeteria | 6 | 18.54 | 10.28 | 7.43 | -2.85 | 18.56 | 10.70 | 2.99 | -2.91 | -0.02 | -0.42 | 4.44 | 0.07 |
| Pol | Cafeteria | 6 | 18.72 | 9.08 | 6.17 | -2.91 | 18.73 | 9.26 | 2.68 | -3.24 | -0.01 | -0.18 | 3.49 | 0.33 |
| Spa | Cafeteria | 6 | 19.04 | 11.43 | 8.14 | -3.29 | 19.06 | 11.86 | 3.80 | -3.42 | -0.02 | -0.43 | 4.34 | 0.13 |
| brit | Cafeteria | 6 | 16.46 | 9.51 | 6.83 | -2.68 | 16.47 | 9.80 | 2.93 | -2.76 | -0.01 | -0.29 | 3.90 | 0.07 |
| finn | Cafeteria | 6 | 15.01 | 9.93 | 4.48 | -5.45 | 15.05 | 9.95 | 2.53 | -5.80 | -0.05 | -0.02 | 1.96 | 0.36 |
| Chi | Cafeteria | 12 | 20.60 | 11.74 | 8.34 | -3.41 | 20.70 | 12.47 | 6.75 | -2.88 | -0.10 | -0.72 | 1.59 | -0.52 |
| Eng | Cafeteria | 12 | 16.36 | 13.59 | 8.93 | -4.65 | 16.41 | 13.60 | 4.07 | -4.70 | -0.06 | -0.02 | 4.87 | 0.05 |
| Fr | Cafeteria | 12 | 20.26 | 14.66 | 8.73 | -5.93 | 20.26 | 14.84 | 5.83 | -5.96 | 0.00 | -0.18 | 2.89 | 0.03 |
| Ger | Cafeteria | 12 | 13.75 | 9.76 | 8.33 | -1.43 | 13.80 | 10.47 | 4.02 | -1.54 | -0.04 | -0.71 | 4.31 | 0.11 |
| Ita | Cafeteria | 12 | 18.44 | 12.72 | 7.63 | -5.09 | 18.45 | 12.90 | 3.30 | -5.33 | 0.00 | -0.18 | 4.33 | 0.24 |
| Jap | Cafeteria | 12 | 18.48 | 11.31 | 5.59 | -5.72 | 18.48 | 11.36 | 2.32 | -6.71 | 0.00 | -0.05 | 3.28 | 0.99 |
| Pol | Cafeteria | 12 | 19.14 | 13.96 | 7.39 | -6.57 | 19.15 | 14.10 | 3.51 | -7.13 | -0.01 | -0.14 | 3.88 | 0.56 |
| Spa | Cafeteria | 12 | 16.25 | 11.47 | 6.28 | -5.19 | 16.26 | 11.55 | 2.87 | -5.52 | 0.00 | -0.08 | 3.41 | 0.33 |
| brit | Cafeteria | 12 | 15.67 | 9.88 | 6.34 | -3.54 | 15.73 | 10.06 | 2.26 | -3.92 | -0.06 | -0.17 | 4.08 | 0.38 |
| finn | Cafeteria | 12 | 19.49 | 11.99 | 9.47 | -2.52 | 19.78 | 13.13 | 6.66 | -2.40 | -0.30 | -1.14 | 2.81 | -0.12 |
| Chi | car | 6 | 15.72 | 10.66 | 7.67 | -2.99 | 15.83 | 10.99 | 3.47 | -3.08 | -0.11 | -0.33 | 4.20 | 0.09 |
| Eng | car | 6 | 19.64 | 13.81 | 10.07 | -3.74 | 19.76 | 14.43 | 6.24 | -3.70 | -0.12 | -0.62 | 3.83 | -0.04 |
| Fr | car | 6 | 13.11 | 8.19 | 7.06 | -1.13 | 13.27 | 9.10 | 3.15 | -1.29 | -0.16 | -0.91 | 3.91 | 0.15 |
| Ger | car | 6 | 17.38 | 9.63 | 6.42 | -3.21 | 17.49 | 10.09 | 2.75 | -3.45 | -0.10 | -0.46 | 3.67 | 0.24 |
| Ita | car | 6 | 17.27 | 11.46 | 7.92 | -3.53 | 17.32 | 11.70 | 3.18 | -3.89 | -0.05 | -0.24 | 4.75 | 0.36 |
| Jap | car | 6 | 17.97 | 12.18 | 7.85 | -4.32 | 18.02 | 12.49 | 3.13 | -4.57 | -0.06 | -0.31 | 4.72 | 0.24 |
| Pol | car | 6 | 15.01 | 10.01 | 6.98 | -3.03 | 15.04 | 10.14 | 2.36 | -3.35 | -0.03 | -0.13 | 4.62 | 0.32 |
| Spa | car | 6 | 14.97 | 11.70 | 6.28 | -5.42 | 15.01 | 11.74 | 2.10 | -6.39 | -0.04 | -0.04 | 4.18 | 0.98 |
| brit | car | 6 | 19.56 | 14.57 | 10.69 | -3.88 | 19.68 | 14.96 | 7.67 | -3.61 | -0.12 | -0.39 | 3.03 | -0.26 |
| finn | car | 6 | 16.08 | 13.23 | 7.75 | -5.48 | 16.15 | 13.38 | 3.89 | -5.72 | -0.07 | -0.15 | 3.86 | 0.24 |
| Chi | car | 12 | 20.04 | 17.27 | 10.85 | -6.41 | 20.08 | 17.50 | 7.33 | -6.30 | -0.04 | -0.24 | 3.53 | -0.11 |
| Eng | car | 12 | 13.37 | 9.87 | 8.24 | -1.63 | 13.44 | 10.24 | 3.86 | -1.91 | -0.07 | -0.37 | 4.37 | 0.27 |
| Fr | car | 12 | 17.73 | 12.09 | 6.62 | -5.48 | 17.77 | 12.25 | 3.07 | -6.01 | -0.04 | -0.16 | 3.54 | 0.53 |
| Ger | car | 12 | 17.17 | 13.87 | 7.51 | -6.35 | 17.18 | 13.93 | 3.23 | -7.21 | -0.01 | -0.07 | 4.28 | 0.86 |
| Ita | car | 12 | 18.44 | 15.18 | 7.35 | -7.83 | 18.45 | 15.24 | 2.92 | -8.59 | -0.01 | -0.06 | 4.42 | 0.76 |
| Jap | car | 12 | 14.91 | 12.05 | 6.76 | -5.29 | 14.91 | 12.08 | 2.35 | -6.00 | 0.00 | -0.02 | 4.41 | 0.71 |
| Pol | car | 12 | 15.79 | 9.56 | 6.47 | -3.09 | 15.83 | 9.68 | 2.46 | -3.21 | -0.04 | -0.11 | 4.02 | 0.12 |
| Spa | car | 12 | 19.08 | 10.61 | 8.16 | -2.45 | 19.17 | 11.10 | 5.40 | -2.26 | -0.10 | -0.50 | 2.76 | -0.18 |
| brit | car | 12 | 17.12 | 10.04 | 7.34 | -2.71 | 17.19 | 10.31 | 4.06 | -2.73 | -0.06 | -0.27 | 3.27 | 0.02 |
| finn | car | 12 | 20.06 | 13.44 | 9.54 | -3.90 | 20.08 | 13.56 | 5.71 | -3.78 | -0.02 | -0.12 | 3.82 | -0.12 |
| Chi | con | 6 | 10.66 | 6.48 | 5.35 | -1.13 | 10.77 | 7.29 | 2.72 | -1.21 | -0.12 | -0.80 | 2.64 | 0.08 |
| Eng | con | 6 | 17.60 | 9.81 | 7.09 | -2.73 | 17.64 | 10.16 | 2.54 | -3.02 | -0.04 | -0.35 | 4.55 | 0.30 |
| Fr | con | 6 | 17.84 | 11.26 | 7.45 | -3.81 | 17.85 | 11.34 | 3.04 | -4.06 | -0.01 | -0.08 | 4.41 | 0.25 |
| Ger | con | 6 | 18.90 | 12.29 | 8.03 | -4.26 | 18.92 | 12.46 | 3.51 | -4.32 | -0.02 | -0.17 | 4.52 | 0.06 |
| Ita | con | 6 | 15.68 | 9.45 | 6.49 | -2.97 | 15.69 | 9.60 | 2.66 | -3.16 | -0.01 | -0.14 | 3.83 | 0.20 |
| Jap | con | 6 | 15.48 | 11.83 | 6.44 | -5.39 | 15.51 | 11.84 | 2.19 | -5.86 | -0.02 | -0.01 | 4.24 | 0.47 |
| Pol | con | 6 | 18.95 | 12.54 | 8.53 | -4.01 | 18.99 | 12.70 | 6.13 | -3.68 | -0.04 | -0.15 | 2.40 | -0.34 |
| Spa | con | 6 | 17.29 | 11.63 | 6.73 | -4.90 | 17.33 | 11.73 | 4.26 | -5.08 | -0.04 | -0.10 | 2.47 | 0.19 |
| brit | con | 6 | 20.23 | 16.55 | 9.81 | -6.74 | 20.23 | 16.58 | 6.77 | -6.53 | 0.00 | -0.02 | 3.04 | -0.21 |
| finn | con | 6 | 10.82 | 8.07 | 6.45 | -1.62 | 10.86 | 8.39 | 3.69 | -1.79 | -0.04 | -0.32 | 2.75 | 0.16 |
| Chi | con | 12 | 17.67 | 11.65 | 7.38 | -4.27 | 17.68 | 11.79 | 3.00 | -4.92 | -0.02 | -0.14 | 4.38 | 0.65 |
| Eng | con | 12 | 17.58 | 13.78 | 6.75 | -7.03 | 17.59 | 13.80 | 2.91 | -7.73 | 0.00 | -0.02 | 3.84 | 0.70 |
| Fr | con | 12 | 18.91 | 15.14 | 7.41 | -7.73 | 18.91 | 15.17 | 3.49 | -8.11 | 0.00 | -0.04 | 3.92 | 0.39 |
| Ger | con | 12 | 15.61 | 11.55 | 6.11 | -5.43 | 15.62 | 11.57 | 2.59 | -6.05 | 0.00 | -0.02 | 3.53 | 0.61 |
| Ita | con | 12 | 16.06 | 11.06 | 7.86 | -3.20 | 16.08 | 11.32 | 2.37 | -3.29 | -0.02 | -0.26 | 5.49 | 0.09 |
| Jap | con | 12 | 20.43 | 11.47 | 9.25 | -2.21 | 20.60 | 12.93 | 5.21 | -1.99 | -0.17 | -1.46 | 4.05 | -0.22 |
| Pol | con | 12 | 16.74 | 11.63 | 8.88 | -2.75 | 16.80 | 12.26 | 3.82 | -2.62 | -0.06 | -0.63 | 5.06 | -0.13 |
| Spa | con | 12 | 20.21 | 14.03 | 10.32 | -3.71 | 20.27 | 14.63 | 4.93 | -3.64 | -0.06 | -0.60 | 5.40 | -0.07 |
| brit | con | 12 | 12.60 | 7.10 | 6.07 | -1.04 | 12.74 | 8.31 | 3.21 | -1.12 | -0.14 | -1.21 | 2.85 | 0.08 |
| finn | con | 12 | 18.07 | 10.28 | 7.36 | -2.92 | 18.11 | 10.68 | 2.67 | -3.03 | -0.04 | -0.39 | 4.70 | 0.11 |
| Chi | INCAR | 6 | 18.37 | 10.57 | 7.52 | -3.05 | 18.39 | 10.70 | 2.68 | -3.41 | -0.01 | -0.13 | 4.84 | 0.36 |
| Eng | INCAR | 6 | 18.84 | 12.13 | 8.66 | -3.47 | 18.87 | 12.31 | 3.75 | -3.52 | -0.03 | -0.18 | 4.90 | 0.05 |
| Fr | INCAR | 6 | 16.45 | 11.38 | 8.50 | -2.88 | 16.47 | 11.58 | 2.90 | -2.96 | -0.02 | -0.20 | 5.61 | 0.08 |
| Ger | INCAR | 6 | 15.65 | 12.99 | 7.45 | -5.54 | 15.66 | 13.08 | 2.04 | -5.94 | -0.01 | -0.09 | 5.41 | 0.40 |
| Ita | INCAR | 6 | 20.53 | 14.07 | 10.06 | -4.01 | 20.60 | 14.75 | 6.17 | -3.55 | -0.07 | -0.68 | 3.89 | -0.46 |
| Jap | INCAR | 6 | 16.45 | 13.86 | 9.26 | -4.59 | 16.46 | 14.08 | 4.15 | -4.39 | -0.02 | -0.22 | 5.11 | -0.20 |
| Pol | INCAR | 6 | 20.28 | 16.49 | 10.37 | -6.12 | 20.31 | 16.71 | 5.53 | -6.11 | -0.03 | -0.22 | 4.84 | -0.02 |
| Spa | INCAR | 6 | 12.79 | 9.03 | 7.35 | -1.68 | 12.84 | 9.53 | 4.07 | -1.81 | -0.05 | -0.50 | 3.28 | 0.12 |
| brit | INCAR | 6 | 18.08 | 12.67 | 7.23 | -5.44 | 18.09 | 12.84 | 2.88 | -5.83 | -0.02 | -0.17 | 4.35 | 0.39 |
| finn | INCAR | 6 | 18.34 | 12.41 | 6.67 | -5.74 | 18.34 | 12.44 | 2.18 | -6.81 | 0.00 | -0.03 | 4.50 | 1.08 |
| Chi | INCAR | 12 | 19.05 | 14.28 | 7.65 | -6.63 | 19.05 | 14.31 | 3.42 | -7.00 | 0.00 | -0.04 | 4.23 | 0.37 |
| Eng | INCAR | 12 | 15.72 | 13.18 | 8.04 | -5.14 | 15.72 | 13.23 | 2.71 | -5.48 | 0.00 | -0.04 | 5.33 | 0.34 |
| Fr | INCAR | 12 | 14.88 | 9.40 | 6.61 | -2.79 | 14.93 | 9.55 | 2.08 | -3.07 | -0.05 | -0.15 | 4.52 | 0.28 |
| Ger | INCAR | 12 | 18.81 | 12.15 | 9.62 | -2.53 | 18.98 | 12.81 | 5.39 | -2.48 | -0.17 | -0.66 | 4.23 | -0.05 |
| Ita | INCAR | 12 | 16.98 | 10.32 | 8.05 | -2.27 | 17.10 | 10.71 | 3.92 | -2.38 | -0.12 | -0.39 | 4.13 | 0.11 |
| Jap | INCAR | 12 | 18.50 | 12.39 | 9.00 | -3.40 | 18.56 | 12.66 | 4.67 | -3.41 | -0.06 | -0.27 | 4.33 | 0.01 |
| Pol | INCAR | 12 | 13.18 | 7.80 | 6.63 | -1.17 | 13.38 | 8.44 | 2.44 | -1.26 | -0.20 | -0.64 | 4.19 | 0.09 |
| Spa | INCAR | 12 | 16.27 | 10.86 | 7.97 | -2.89 | 16.36 | 11.20 | 2.43 | -3.07 | -0.09 | -0.34 | 5.54 | 0.18 |
| brit | INCAR | 12 | 16.74 | 8.97 | 5.93 | -3.04 | 16.77 | 9.10 | 2.66 | -3.42 | -0.03 | -0.12 | 3.27 | 0.38 |
| finn | INCAR | 12 | 17.56 | 11.38 | 7.85 | -3.53 | 17.58 | 11.49 | 2.93 | -3.78 | -0.02 | -0.11 | 4.91 | 0.24 |
| Chi | met | 6 | 15.15 | 9.57 | 6.56 | -3.00 | 15.18 | 9.72 | 2.45 | -3.24 | -0.03 | -0.15 | 4.11 | 0.24 |
| Eng | met | 6 | 15.17 | 11.59 | 6.41 | -5.18 | 15.20 | 11.63 | 2.02 | -5.84 | -0.02 | -0.04 | 4.39 | 0.66 |
| Fr | met | 6 | 19.53 | 14.90 | 10.30 | -4.60 | 19.62 | 15.21 | 6.30 | -4.39 | -0.09 | -0.32 | 3.99 | -0.20 |
| Ger | met | 6 | 17.47 | 12.51 | 8.21 | -4.30 | 17.53 | 12.66 | 4.33 | -4.68 | -0.06 | -0.15 | 3.87 | 0.38 |
| Ita | met | 6 | 19.12 | 15.86 | 9.51 | -6.35 | 19.15 | 15.99 | 5.88 | -6.35 | -0.03 | -0.13 | 3.63 | 0.01 |
| Jap | met | 6 | 13.24 | 8.84 | 7.36 | -1.48 | 13.32 | 9.09 | 3.08 | -1.68 | -0.08 | -0.25 | 4.28 | 0.20 |
| Pol | met | 6 | 17.02 | 13.57 | 8.25 | -5.32 | 17.05 | 13.69 | 2.55 | -5.84 | -0.03 | -0.11 | 5.70 | 0.52 |
| Spa | met | 6 | 17.26 | 11.46 | 5.43 | -6.03 | 17.27 | 11.49 | 2.55 | -7.02 | -0.01 | -0.03 | 2.88 | 0.99 |
| brit | met | 6 | 18.25 | 14.21 | 7.15 | -7.07 | 18.26 | 14.24 | 2.88 | -7.71 | 0.00 | -0.03 | 4.27 | 0.64 |
| finn | met | 6 | 15.41 | 11.51 | 6.14 | -5.37 | 15.41 | 11.53 | 2.44 | -5.93 | 0.00 | -0.02 | 3.71 | 0.56 |
| Chi | met | 12 | 14.13 | 9.10 | 6.27 | -2.83 | 14.28 | 9.29 | 2.56 | -3.02 | -0.15 | -0.20 | 3.71 | 0.20 |
| Eng | met | 12 | 17.81 | 12.21 | 10.09 | -2.12 | 18.29 | 13.33 | 6.00 | -1.98 | -0.48 | -1.13 | 4.09 | -0.14 |
| Fr | met | 12 | 14.77 | 11.42 | 8.87 | -2.55 | 14.99 | 11.81 | 3.72 | -2.60 | -0.22 | -0.39 | 5.15 | 0.05 |
| Ger | met | 12 | 18.44 | 14.46 | 10.93 | -3.53 | 18.60 | 14.96 | 5.85 | -3.47 | -0.16 | -0.50 | 5.08 | -0.06 |
| Ita | met | 12 | 11.95 | 8.61 | 7.70 | -0.90 | 12.26 | 9.62 | 3.65 | -0.92 | -0.31 | -1.01 | 4.05 | 0.02 |
| Jap | met | 12 | 16.49 | 11.13 | 7.84 | -3.29 | 16.63 | 11.61 | 3.12 | -3.33 | -0.14 | -0.48 | 4.71 | 0.04 |
| Pol | met | 12 | 16.50 | 12.11 | 8.85 | -3.26 | 16.55 | 12.28 | 3.38 | -3.44 | -0.05 | -0.17 | 5.47 | 0.18 |
| Spa | met | 12 | 16.98 | 12.14 | 8.53 | -3.61 | 17.05 | 12.40 | 3.98 | -3.67 | -0.08 | -0.26 | 4.55 | 0.06 |
| brit | met | 12 | 15.15 | 10.49 | 7.80 | -2.70 | 15.19 | 10.62 | 3.19 | -2.83 | -0.04 | -0.13 | 4.61 | 0.13 |
| finn | met | 12 | 13.61 | 10.45 | 5.58 | -4.87 | 13.71 | 10.51 | 2.33 | -5.48 | -0.10 | -0.06 | 3.25 | 0.60 |
| Chi | off | 6 | 17.66 | 13.64 | 10.13 | -3.51 | 17.93 | 14.17 | 6.92 | -3.22 | -0.27 | -0.53 | 3.21 | -0.29 |
| Eng | off | 6 | 15.14 | 13.36 | 8.80 | -4.56 | 15.27 | 13.51 | 3.94 | -4.84 | -0.13 | -0.15 | 4.86 | 0.28 |
| Fr | off | 6 | 18.87 | 16.87 | 11.01 | -5.86 | 18.93 | 17.07 | 6.55 | -5.83 | -0.06 | -0.20 | 4.46 | -0.04 |
| Ger | off | 6 | 12.30 | 10.41 | 8.96 | -1.45 | 12.45 | 10.93 | 4.46 | -1.48 | -0.15 | -0.52 | 4.50 | 0.03 |
| Ita | off | 6 | 16.93 | 13.34 | 7.89 | -5.44 | 16.97 | 13.46 | 3.31 | -5.66 | -0.04 | -0.12 | 4.58 | 0.21 |
| Jap | off | 6 | 16.73 | 14.36 | 8.20 | -6.15 | 16.74 | 14.40 | 3.13 | -6.78 | -0.01 | -0.05 | 5.07 | 0.62 |
| Pol | off | 6 | 17.74 | 14.71 | 7.76 | -6.95 | 17.76 | 14.80 | 3.83 | -7.28 | -0.02 | -0.09 | 3.93 | 0.33 |
| Spa | off | 6 | 15.41 | 12.18 | 6.96 | -5.22 | 15.42 | 12.20 | 2.95 | -5.79 | -0.01 | -0.02 | 4.00 | 0.56 |
| brit | off | 6 | 15.16 | 9.95 | 6.26 | -3.69 | 15.19 | 10.32 | 2.12 | -3.88 | -0.03 | -0.36 | 4.14 | 0.19 |
| finn | off | 6 | 19.82 | 9.47 | 6.84 | -2.63 | 19.93 | 10.88 | 4.77 | -2.47 | -0.12 | -1.40 | 2.07 | -0.16 |
| Chi | off | 12 | 17.65 | 10.10 | 7.14 | -2.96 | 17.70 | 10.61 | 3.48 | -2.99 | -0.04 | -0.51 | 3.66 | 0.03 |
| Eng | off | 12 | 20.25 | 12.44 | 7.68 | -4.76 | 20.29 | 12.95 | 4.25 | -4.80 | -0.04 | -0.52 | 3.43 | 0.05 |
| Fr | off | 12 | 12.75 | 7.08 | 5.82 | -1.26 | 12.94 | 9.08 | 3.41 | -1.34 | -0.18 | -2.00 | 2.41 | 0.09 |
| Ger | off | 12 | 17.79 | 10.29 | 6.76 | -3.52 | 17.84 | 10.90 | 2.68 | -3.72 | -0.05 | -0.62 | 4.08 | 0.20 |
| Ita | off | 12 | 18.26 | 10.88 | 7.11 | -3.77 | 18.27 | 11.11 | 2.89 | -4.01 | -0.02 | -0.23 | 4.22 | 0.23 |
| Jap | off | 12 | 19.01 | 11.55 | 7.32 | -4.23 | 19.03 | 11.72 | 3.52 | -4.31 | -0.02 | -0.17 | 3.80 | 0.08 |
| Pol | off | 12 | 16.73 | 9.25 | 5.85 | -3.40 | 16.76 | 9.50 | 2.75 | -3.55 | -0.02 | -0.25 | 3.10 | 0.15 |
| Spa | off | 12 | 14.34 | 12.34 | 5.80 | -6.54 | 14.35 | 12.45 | 1.61 | -7.09 | -0.01 | -0.11 | 4.20 | 0.56 |
| brit | off | 12 | 19.86 | 12.22 | 7.75 | -4.48 | 19.92 | 12.93 | 5.60 | -4.06 | -0.05 | -0.71 | 2.14 | -0.42 |
| finn | off | 12 | 17.34 | 12.68 | 7.62 | -5.06 | 17.36 | 12.92 | 3.66 | -5.17 | -0.02 | -0.24 | 3.96 | 0.12 |
| Chi | rai | 6 | 20.45 | 16.48 | 8.76 | -7.72 | 20.46 | 16.64 | 4.90 | -7.90 | -0.01 | -0.16 | 3.86 | 0.19 |
| Eng | rai | 6 | 12.86 | 9.85 | 8.06 | -1.80 | 12.94 | 10.85 | 4.16 | -1.88 | -0.08 | -1.00 | 3.90 | 0.08 |
| Fr | rai | 6 | 17.84 | 12.76 | 6.89 | -5.88 | 17.86 | 12.96 | 3.04 | -6.37 | -0.02 | -0.20 | 3.85 | 0.50 |
| Ger | rai | 6 | 18.04 | 14.46 | 7.45 | -7.02 | 18.04 | 14.50 | 2.48 | -7.77 | 0.00 | -0.04 | 4.97 | 0.76 |
| Ita | rai | 6 | 19.19 | 14.68 | 6.88 | -7.80 | 19.19 | 14.72 | 3.04 | -8.14 | 0.00 | -0.03 | 3.84 | 0.34 |
| Jap | rai | 6 | 16.12 | 11.19 | 5.36 | -5.83 | 16.13 | 11.27 | 2.39 | -6.31 | -0.01 | -0.09 | 2.97 | 0.48 |
| Pol | rai | 6 | 14.90 | 9.39 | 6.21 | -3.18 | 14.95 | 9.70 | 2.49 | -3.43 | -0.05 | -0.32 | 3.72 | 0.25 |
| Spa | rai | 6 | 18.26 | 10.55 | 8.37 | -2.17 | 18.50 | 11.67 | 5.56 | -2.10 | -0.23 | -1.12 | 2.82 | -0.07 |
| brit | rai | 6 | 15.75 | 10.25 | 7.72 | -2.52 | 15.79 | 10.52 | 3.47 | -2.60 | -0.04 | -0.27 | 4.26 | 0.08 |
| finn | rai | 6 | 19.44 | 12.66 | 8.76 | -3.90 | 19.52 | 13.10 | 5.07 | -3.95 | -0.08 | -0.44 | 3.69 | 0.05 |
| Chi | rai | 12 | 13.02 | 6.60 | 5.62 | -0.98 | 13.14 | 7.22 | 2.91 | -1.05 | -0.12 | -0.62 | 2.71 | 0.07 |
| Eng | rai | 12 | 16.90 | 10.00 | 6.83 | -3.17 | 16.95 | 10.30 | 2.69 | -3.34 | -0.05 | -0.31 | 4.13 | 0.17 |
| Fr | rai | 12 | 17.80 | 10.48 | 6.91 | -3.56 | 17.82 | 10.65 | 2.63 | -3.89 | -0.02 | -0.17 | 4.28 | 0.33 |
| Ger | rai | 12 | 18.25 | 11.38 | 7.76 | -3.63 | 18.28 | 11.61 | 3.63 | -3.78 | -0.04 | -0.23 | 4.13 | 0.15 |
| Ita | rai | 12 | 15.29 | 9.51 | 6.60 | -2.91 | 15.32 | 9.72 | 2.88 | -3.14 | -0.03 | -0.21 | 3.72 | 0.24 |
| Jap | rai | 12 | 14.75 | 11.81 | 6.08 | -5.73 | 14.77 | 11.95 | 2.53 | -6.34 | -0.02 | -0.14 | 3.55 | 0.62 |
| Pol | rai | 12 | 18.46 | 13.14 | 9.34 | -3.80 | 18.57 | 13.67 | 6.67 | -3.60 | -0.11 | -0.53 | 2.67 | -0.20 |
| Spa | rai | 12 | 15.96 | 12.47 | 7.82 | -4.65 | 15.98 | 12.55 | 3.85 | -4.81 | -0.01 | -0.08 | 3.97 | 0.16 |
| brit | rai | 12 | 19.83 | 15.96 | 9.43 | -6.53 | 19.85 | 16.14 | 6.12 | -6.71 | -0.03 | -0.18 | 3.30 | 0.17 |
| finn | rai | 12 | 13.07 | 8.04 | 6.72 | -1.32 | 13.13 | 8.34 | 3.89 | -1.42 | -0.06 | -0.30 | 2.83 | 0.10 |
| Chi | res | 6 | 17.20 | 12.22 | 6.67 | -5.55 | 17.22 | 12.35 | 2.87 | -6.03 | -0.02 | -0.13 | 3.80 | 0.48 |
| Eng | res | 6 | 17.90 | 12.92 | 6.31 | -6.61 | 17.91 | 13.00 | 2.26 | -7.54 | -0.01 | -0.08 | 4.05 | 0.93 |
| Fr | res | 6 | 18.71 | 14.07 | 7.05 | -7.02 | 18.72 | 14.14 | 3.37 | -7.51 | -0.01 | -0.07 | 3.68 | 0.49 |
| Ger | res | 6 | 15.48 | 11.72 | 6.19 | -5.53 | 15.49 | 11.80 | 2.73 | -6.20 | -0.01 | -0.08 | 3.46 | 0.67 |
| Ita | res | 6 | 14.21 | 10.05 | 7.14 | -2.90 | 14.25 | 10.23 | 1.94 | -3.28 | -0.05 | -0.18 | 5.21 | 0.38 |
| Jap | res | 6 | 19.25 | 11.42 | 8.95 | -2.47 | 19.66 | 12.89 | 5.33 | -2.40 | -0.42 | -1.47 | 3.62 | -0.06 |
| Pol | res | 6 | 15.58 | 11.60 | 8.74 | -2.86 | 15.72 | 12.16 | 3.40 | -2.92 | -0.14 | -0.56 | 5.34 | 0.06 |
| Spa | res | 6 | 19.22 | 14.02 | 10.28 | -3.74 | 19.42 | 14.85 | 4.61 | -3.87 | -0.21 | -0.84 | 5.66 | 0.13 |
| brit | res | 6 | 12.42 | 7.78 | 6.45 | -1.33 | 12.69 | 8.86 | 2.64 | -1.47 | -0.26 | -1.08 | 3.81 | 0.14 |
| finn | res | 6 | 16.88 | 11.28 | 8.30 | -2.99 | 16.97 | 11.63 | 2.71 | -3.28 | -0.09 | -0.34 | 5.58 | 0.29 |
| Chi | res | 12 | 17.60 | 10.83 | 7.61 | -3.21 | 17.65 | 11.03 | 2.69 | -3.65 | -0.05 | -0.20 | 4.93 | 0.44 |
| Eng | res | 12 | 17.83 | 12.28 | 8.69 | -3.59 | 17.90 | 12.56 | 3.49 | -3.79 | -0.07 | -0.28 | 5.20 | 0.20 |
| Fr | res | 12 | 15.93 | 11.09 | 8.16 | -2.93 | 16.00 | 11.39 | 2.50 | -3.17 | -0.07 | -0.30 | 5.66 | 0.24 |
| Ger | res | 12 | 13.37 | 11.00 | 6.34 | -4.66 | 13.38 | 11.05 | 1.56 | -5.51 | -0.01 | -0.04 | 4.78 | 0.85 |
| Ita | res | 12 | 19.09 | 13.37 | 9.63 | -3.74 | 19.29 | 14.06 | 6.04 | -3.54 | -0.19 | -0.70 | 3.58 | -0.21 |
| Jap | res | 12 | 16.03 | 13.62 | 8.46 | -5.16 | 16.07 | 13.82 | 3.50 | -5.35 | -0.05 | -0.20 | 4.96 | 0.20 |
| Pol | res | 12 | 19.65 | 16.83 | 10.67 | -6.16 | 19.73 | 17.16 | 5.40 | -6.34 | -0.08 | -0.33 | 5.26 | 0.18 |
| Spa | res | 12 | 12.60 | 9.56 | 7.74 | -1.82 | 12.71 | 10.03 | 3.40 | -2.13 | -0.11 | -0.46 | 4.34 | 0.31 |
| brit | res | 12 | 17.03 | 13.59 | 8.37 | -5.22 | 17.05 | 13.67 | 3.02 | -5.74 | -0.02 | -0.08 | 5.36 | 0.52 |
| finn | res | 12 | 17.78 | 12.72 | 7.01 | -5.70 | 17.79 | 12.76 | 2.42 | -6.68 | -0.01 | -0.04 | 4.60 | 0.98 |
| Chi | STREET | 6 | 18.34 | 14.67 | 8.10 | -6.57 | 18.35 | 14.74 | 3.48 | -7.01 | -0.02 | -0.07 | 4.63 | 0.44 |
| Eng | STREET | 6 | 16.18 | 13.35 | 7.81 | -5.54 | 16.19 | 13.40 | 2.54 | -6.00 | -0.01 | -0.05 | 5.27 | 0.46 |
| Fr | STREET | 6 | 13.82 | 9.94 | 6.68 | -3.26 | 14.00 | 10.15 | 1.57 | -3.65 | -0.18 | -0.21 | 5.11 | 0.39 |
| Ger | STREET | 6 | 17.28 | 12.02 | 9.12 | -2.90 | 17.74 | 12.95 | 4.56 | -2.85 | -0.46 | -0.93 | 4.55 | -0.06 |
| Ita | STREET | 6 | 16.03 | 11.28 | 8.13 | -3.14 | 16.24 | 11.57 | 3.02 | -3.32 | -0.21 | -0.30 | 5.11 | 0.17 |
| Jap | STREET | 6 | 18.53 | 14.67 | 10.41 | -4.26 | 18.63 | 14.93 | 3.65 | -4.47 | -0.10 | -0.26 | 6.76 | 0.21 |
| Pol | STREET | 6 | 12.52 | 8.53 | 6.96 | -1.56 | 12.94 | 9.67 | 2.79 | -1.68 | -0.42 | -1.14 | 4.17 | 0.11 |
| Spa | STREET | 6 | 16.50 | 11.26 | 8.12 | -3.13 | 16.63 | 11.61 | 1.71 | -3.35 | -0.12 | -0.35 | 6.42 | 0.22 |
| brit | STREET | 6 | 15.79 | 13.26 | 9.23 | -4.03 | 15.85 | 13.43 | 2.36 | -4.35 | -0.06 | -0.17 | 6.87 | 0.32 |
| finn | STREET | 6 | 17.46 | 13.75 | 9.49 | -4.26 | 17.53 | 13.94 | 2.68 | -4.54 | -0.07 | -0.19 | 6.81 | 0.29 |
| Chi | STREET | 12 | 14.52 | 10.10 | 7.10 | -2.99 | 14.58 | 10.26 | 1.59 | -3.39 | -0.06 | -0.16 | 5.51 | 0.40 |
| Eng | STREET | 12 | 14.05 | 11.11 | 5.55 | -5.56 | 14.17 | 11.18 | 1.30 | -6.46 | -0.12 | -0.07 | 4.25 | 0.89 |
| Fr | STREET | 12 | 17.89 | 14.13 | 9.44 | -4.69 | 18.17 | 14.63 | 5.61 | -4.48 | -0.29 | -0.50 | 3.83 | -0.21 |
| Ger | STREET | 12 | 16.71 | 12.91 | 7.68 | -5.23 | 16.84 | 13.01 | 3.10 | -5.64 | -0.13 | -0.10 | 4.59 | 0.41 |
| Ita | STREET | 12 | 19.29 | 16.81 | 10.02 | -6.79 | 19.32 | 16.90 | 4.37 | -7.30 | -0.03 | -0.09 | 5.65 | 0.51 |
| Jap | STREET | 12 | 12.84 | 9.60 | 7.47 | -2.13 | 13.04 | 10.15 | 3.30 | -2.32 | -0.20 | -0.55 | 4.17 | 0.19 |
| Pol | STREET | 12 | 16.99 | 13.19 | 7.83 | -5.36 | 17.03 | 13.29 | 1.69 | -5.91 | -0.03 | -0.10 | 6.14 | 0.54 |
| Spa | STREET | 12 | 14.79 | 15.30 | 8.04 | -7.26 | 14.80 | 15.34 | 2.04 | -8.05 | -0.01 | -0.04 | 5.99 | 0.78 |
| brit | STREET | 12 | 18.01 | 16.05 | 8.07 | -7.98 | 18.02 | 16.10 | 2.33 | -8.74 | -0.02 | -0.05 | 5.74 | 0.76 |
| finn | STREET | 12 | 14.14 | 12.29 | 6.52 | -5.77 | 14.16 | 12.34 | 1.31 | -6.68 | -0.02 | -0.05 | 5.22 | 0.91 |
| average |  |  | 16.83 | 11.80 | 7.73 | -4.07 | 16.90 | 12.13 | 3.55 | -4.30 | -0.07 | -0.33 | 4.18 | 0.24 |

Figure 1. Scatter plot of G.160 App. II SNRI vs.  True SNRI.

# Conclusion/proposal

This contribution presents discussions on SNRI measures in ITU-T G.160 Appendix II AMD 2. By applying the same spectral gains for clean speech and noise signal as well as noisy speech, the ‘true’ values of TNLR, NPLR, SNRI and DSN could be computed. While NPLR, TNLR and DSN show a low estimate error of the true value, the G.160 App. II SNRI has larger deviations, and correlation is low.

It appears that the G.160 App. II AMD 2 revision still has significant problems in assessing the performance of a practical noise suppressor. Estimation errors > 6dB are observed even when the tool is applied within its newly restricted scope. Additionally, this contribution does not include the effects of acoustical interfaces and inter-lab variability which could further deteriorate the performance of the metric for application to terminals.

It is proposed that these findings are liaisoned to ITU-T SG16/Q16 for discussion.

# References

- TD 130 (WP 1/16), “Report of Q16/16 Rapporteurs Group Meeting (Budapest, Hungary, 3-9 November 2010)”.

- BUD 16-17, “Corrections to G.160 Amendment I (Budapest, Hungary,  3-9 November 2010)”.

- ITU-T G.160 Appendix 2 Amendment 2 - : Revised Appendix II - Objective measures for the characterization of the basic functioning of noise reduction algorithms.

- MTV 16-21, “TNLR+ Algorithm for Accurate Measurement of High-Performance Non-stationary Noise Suppressors (Mountain View, USA, 10-16 June 2009)

- COM 16 – C 630 – E, “G.160 App. II: Considerations regarding the SNRI measure (Geneva, Switzerland, March 2011)

- Y.Ephraim and H.L. Van Trees, “A signal subspace approach for speech enhancement”, IEEE Transactions on Speech and Audio Processing, Vol.3 No. 4, July 1995