3GPP TSG-SA4 Meeting #89 	S4-160648

# Kista, Sweden, 27. Jun - 1. Jul 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.103 | CR | xxxx | rev | - | Current version: | 13.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction and Clarification of Codec Negotiation for EVSoCS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM; |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | EVSoCS |  |  |  |  |  |  | Date: |  | 2016-06-21 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | A compromise solution between Bottom Up Configurations and EVS-SWB Configurations is needed to guarantee TrFO-interworking within one CS-network, between CS-Networks and between CS- and IMS-networks. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Allow one mandatory EVS_Set and one optional, additional EVS-Set in the OoBTC Offer for the "Speech Codec Information Element" for EVS. Clarify the rules, how to construct the OoBTC Offer by the originating node, how to modify it in intermediate nodes and how to select the OoBTC answer in the terminating node, i.e.how to determine the Selected Codec Configuration for UMTS_EVS. Some editorial improvements are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The introduction of EVS in CS networks may lead to market split and unnecessary transcoding. Even interworking issues anc all setup failures are not excluded. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Begin of changes

## 5.7A	EVS Codec Type (UMTS_EVS)

The UMTS_EVS Codec Type [25] … [38] is applied in UTRAN and the CS core network with the Codec IDentification (CoID) code as follows: 
UMTS_EVS_CoID	:= 0x0000.1110.

The UMTS_EVS Codec Type can be used in conversational speech telephony services in a number of different configurations. These allowed configurations for CS networks are defined in Table 5.7A-1.

The combination of the UMTS_EVS Codec Type with one of these allowed configurations for CS networks is abbreviated with "UMTS_EVS (Set x)", e.g. UMTS_EVS (Set 1).

Table 5.7A-1: Allowed Configurations for the UMTS_EVS Codec Type

| Configuration→ (Config-EVS-Code or Set) ↓Codec Rate (kbps) | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Typical SF on downlink | 256 | 128 | 64 | 128 |
| EVS Primary 24.4 |  |  | nb-fb |  |
| EVS Primary 16.4 |  |  | nb-fb |  |
| EVS Primary 13.2 |  | nb-swb | nb-swb | swb |
| EVS Primary 9.6 |  | nb-swb | nb-swb | swb |
| EVS Primary 8 | nb-wb | nb-wb | nb-wb |  |
| EVS Primary 7.2 | nb-wb | nb-wb | nb-wb |  |
| EVS Primary 5.9 VBR (NOTE 2) | nb-wb | nb-wb | nb-wb |  |
| EVS Primary SID | nb-wb | nb-swb | nb-fb | swb |
| EVS AMR-WB IO 12.65 |  | wb | wb | wb |
| EVS AMR-WB IO 8.85 |  | wb | wb | wb |
| EVS AMR-WB IO 6.60 | wb | wb | wb | wb |
| EVS AMR-WB IO SID | wb | wb | wb | wb |
| NOTE 1: Every Speech or SID payload field is complemented by appending the 7-bit EVS-CMR field for Maximum Rate and Bandwidth Control. For details, see TS 26.454 [38]. NOTE 2: From a transmission point of view, the EVS Primary 5.9 VBR mode of operation includes three bit rates of 2.8, 7.2 and 8.0 kb/s to achieve the 5.9 kb/s average bit rate in active speech segments. Together with EVS Primary SID in speech pauses and the EVS-CMR appended in every Speech and SID frame, the total average net bit rate, assuming 40% speech pause and 60% active speech is in the order of 4.4 kb/s. |  |  |  |  |

Table 5.7A-2 defines the coding of the "Single Codec" information element for the UMTS_EVS Codec Type in a BICC-based CS core network.

Table 5.7A-2: Coding of "Single Codec" for the UMTS_EVS Codec Type

| Octet | Parameter | MSB 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 LSB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 m | Single Codec | Single Codec (see ITU-T Q.765.5) |  |  |  |  |  |  |  |
| 2 m | Length Indication | 4 [, 5] |  |  |  |  |  |  |  |
| 3 m | Compat. Info | Compatibility Information |  |  |  |  |  |  |  |
| 4 m | OID | ETSI OID (See ITU-T Q.765.5 [6]) |  |  |  |  |  |  |  |
| 5 m | CoID | UMTS_EVS_CoID |  |  |  |  |  |  |  |
| 6 m* | Config-EVS1 | (spare) | (spare) | (spare) | (spare) | Config-EVS-Code value 1 |  |  |  |
| 7 o* | Config-EVS2 | (spare) | (spare) | (spare) | (spare) | Config-EVS-Code value 2 |  |  |  |

m*= mandatory; o* = optional in the OoBTC Offer, not allowed in the OoBTC Answer.

Config-EVS-Code value 1 in octet Config-EVS1 has the following allowed values: 0, 1, 2 in the Offer and 0, 1, 2, 3 in the Answer. All other values are reserved for future use.

Config-EVS-Code value 2 in octet Config-EVS2 has the following allowed values: 3 in the Offer. This octet Config-EVS2 is not allowed in the Answer. All other values are reserved for future use.

If the originating MSC includes EVS in the OoBTC Offer, then it shall at least provide the Config-EVS1 octet, and it may include the optional Config-EVS2 octet. The offered EVS-Set(s) should reflect the wanted maximum rate at the originating side.

An intermediate MSC, if present, should not (but may) remove Config-EVS2, if present. It may reduce the value of Config-EVS-Code value 1 to a lower value, but not to a higher one and shall not remove Config-EVS1. The forwarded EVS-Set(s) should reflect the wanted maximum rate at the intermediate node.

If the terminating MSC (or terminating network) selects EVS, then it shall select and return exactly one Config-EVS-Code value in the OoBTC Answer in Config-EVS1. The selected EVS-Set should reflect the wanted maximum rate at the originating (and intermediate, if present) and terminating side.

If the OoBTC Offer provides only Config-EVS1, then the terminating MSC shall include in the OoBTC Answer the same or a smaller Config-EVS-Code value, but no bigger one.

If the OoBTC Offer provides in addition Config-EVS2, then the terminating MSC shall include in the OoBTC Answer either the same or a smaller value as received in Config-EVS1, but no bigger one, or exactly the value received in Config-EVS2.

NOTE 3: 	If one or both MSCs (networks) prefer a Bottom Up Configuration (i.e. Set 2 or Set 1 or Set 0), then a Bottom Up Configuration is selected. If both MSCs (networks) prefer the EVS-SWB Configuration (i.e. Set 3), then this is selected. This procedure guarantees always best possible voice quality without transcoding under the given constraints (wanted maximum rates in the whole speech path and/or the wanted audio bandwidth).

Mandatory for UTRAN terminals supporting UMTS_EVS:
If an UTRAN terminal offers Codec Type UMTS_EVS in the Supported Codec List Information Element, then all allowed EVS Configurations for CS networks shall be supported by this UTRAN terminal. The UTRAN terminal shall always apply Discontinuous Transmission (DTX) in uplink direction and shall accept in downlink direction all received EVS Codec Modes and DTX.

NOTE 4:	Mandatory EVS configurations for UTRAN and CS core network infrastructures for UMTS_EVS are for further study.

NOTE 5:	The rules for interworking on user plane between the allowed EVS Configurations for CS networks and the other EVS Configurations (e.g. for MTSI) are defined in TS 26.454 [38]. Also the rules for Maximum Rate and Bandwidth Control in CS networks are defined in TS 26.454 [38].

End of changes