3GPP TSG-SA4 Meeting #89 	S4-160651

# Kista, Sweden, 27. Jun - 1. Jul 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.414 | CR | xxxx | rev | - | Current version: | 13.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EVS Correction and Alignment with TS 26.454 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM; |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | EVSoCS |  |  |  |  |  |  | Date: |  | 2016-06-21 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The current text elements in this specification concerning User Plane handling of the EVS Codec are partly overlapping, partly contradicting and partly complementing the corresponding text elements in TS 26.454, TS 29.415 and TS 29.163. This may lead to interworking issues and call failures. Future maintenance of these four specification documents are complex and error prone. It is therefore proposed to remove these text elements from this specification and place the reference to TS 26.454. Text elements currently not covered in TS 26.454 are integrated into TS 26.454. Some text elements on User Plane handling of AMR and AMR-WB are incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace the text elements for EVS over CS User Plane handling by a reference to TS 26.454 (as far as appropriate). Proposals for correction of text elements for AMR and AMR-WB are included. Some minor editorial corrections are proposed, too. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specifications for EVSoCS in 29- and 26-series would remain. This may lead to interworking issues and call failures. The specifications would be hard to maintain in future. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 2, 7.4.1, 7.4.4.1, 7.4.4.2, 7.4.4.3, 7.4.4.4, 7.4.5, 7.4.6, 7.4.8, 7.4.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Begin of change

# 1	Scope

The present document specifies the bearer data transport and bearer control protocols used between MGWs within the CS core network across the Nb Interface. The present document assumes that the implementation of the split of the call control and the bearer transport and control is as specified in 3GPP TS 23.205 [1] with BICC-based Nc, and in 3GPP TS 23.231 [30] with SIP-I based Nc, see figure 1.

For BICC-based Nc, the User Plane protocol that uses this bearer data transport (Nb UP) is described in 3GPP TS 29.415 [3].

The transport format for the Nb interface with IP transport can optionally allow RTP multiplexing. Note that the present document does not preclude an implementation of a combined MSC Server and MGW.

Figure 1: CS core network logical architecture

# 2	References

The following documents contain provisions, which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 23.205: "Bearer Independent CS Core Network; Stage 2".

[2]	3GPP TS 25.414: "UTRAN Iu Interface Data Transport and Transport Signalling".

[3]	3GPP TS 29.415: "Core Network Nb Interface User Plane Protocols".

[4]	3GPP TS 29.232: "Media Gateway Controller (MGC) – Media Gateway (MGW) Interface; Stage 3".

[5]	ITU-T Recommendation E.164 (5/1997): "Numbering Plan for the ISDN Era".

[6]	ITU-T Recommendation I.361 (2/1999): "B-ISDN ATM Layer Specification".

[7]	ITU-T Recommendation I.363.2 (11/2000): "B-ISDN ATM Adaptation Layer Type 2 Specification".

[8]	ITU-T Recommendation I.363.5 (8/1996): "B-ISDN ATM Adaptation Layer Type 5 Specification".

[9]	ITU-T Recommendation I.366.1 (6/1998): "Segmentation and Reassembly Service Specific Convergence Sublayer for the AAL Type 2".

[10]	ITU-T Recommendation H.248 (6/2000)"Media Gateway Control Protocol".

[11]	3GPP TS 29.205: "Application of Q.1900 Series to Bearer Independent CS Core Network Architecture".

[12]	(Void)

[13]	(Void)

[14]	ITU-T Recommendation Q.2110 (7/1994): "B-ISDN ATM Adaptation Layer-Service Specific Connection Oriented Protocol (SSCOP)".

[15]	ITU-T Recommendation Q.2140 (2/1995): "B-ISDN ATM Adaptation Layer-Service Specific Coordination Function for Support of Signalling at the Network Node Interface (SSCF-NNI)".

[16]	ITU-T Recommendation Q.2150.1 (1999): "B-ISDN ATM Adaptation Layer-Signalling Transport Converter for the MTP3b".

[17]	ITU-T Recommendation Q.2210 (7/1996): "Message Transfer Part level 3 functions and messages using the services of ITU-T Recommendation Q.2140".

[18]	ITU-T Implementor's guide (12/99) for recommendation Q.2210 (07/96).

[19]	ITU-T Recommendation Q.2630.2 (12/2000): "AAL type 2 Signalling Protocol (Capability Set 2)".

[20]	ITU-T Recommendation Q.2931 (2/1995): "Digital Subscriber Signalling System No. 2 (DSS 2) – User-Network Interface (UNI) layer 3 specification for basic call/connection control".

[21]	ITU-T Recommendation X.213 (8/1997): "Information Technology-Open Systems Interconnection-Network Service Definitions".

[22]	IETF RFC 768: "User Datagram Protocol. (UDP)".

[23]	IETF RFC 791: "Internet Protocol (IP)".

[24]	IETF RFC 1889: "RTP: A Transport Protocol for Real Time Applications".

[25]	IETF RFC 1890: "RTP Profile for Audio and Video Conferences with Minimal Control".

[26]	IETF RFC 2327: "SDP: Session Description Protocol".

[27]	IETF RFC 2460: "Internet Protocol, Version 6 (IPv6)".

[28]	3GPP TR 29.814: "Feasibility Study on Bandwidth Savings at Nb Interface with IP transport".

[29]	IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications".

[30]	3GPP TS 23.231: " SIP-I based circuit-switched core network; Stage 2".

[31]	3GPP TS 25.415: "UTRAN Iu Interface User Plane Protocols"

[32]	3GPP TS 26.102: "Adaptive Multi-Rate (AMR) speech codec; Interface to Iu and Uu".

[33]	IETF RFC 3551: "RTP Profile for Audio and Video Conferences with Minimal Control".

[34]	IETF RFC 3555: "MIME Type Registration of RTP Payload Formats".

[35]	IETF RFC 4867: "Real-Time Transport Protocol (RTP) payload format and file storage format for the Adaptive Multi-Rate (AMR) Adaptive Multi-Rate Wideband (AMR-WB) audio codecs".

[36]	3GPP TS 29.007: "General requirements on interworking between the Public Land Mobile Network (PLMN) and the Integrated Services Digital Network (ISDN) or Public Switched Telephone Network (PSTN)" .

[37]	3GPP TS 26.445:"Codec for Enhanced Voice Services (EVS); Detailed Algorithmic Description".

[38]	3GPP TS 26.453: "Codec for Enhanced Voice Services (EVS); Speech codec frame structure".

[39]	3GPP TS 26.454: "Codec for Enhanced Voice Services (EVS); Interface to Iu, Uu, Nb and Mb".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions apply.

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

AAL	ATM Adaptation Layer

AAL2	ATM Adaptation Layer Type 2

AAL5	ATM Adaptation Layer Type 5

AESA	ATM End System Address

ALC	AAL2 Link Characteristics

ARP	Address Resolution Protocol

ATM	Asynchronous Transfer Mode

AVP	Audio Video Profile

BICC	Bearer Independent Call Control

CMR	Codec Mode Request

CN	Core Network

CSRC	Contributing Source

DSS2	Digital Subscriber Signalling 2

EVS	Enhanced Voice Services

EVS-CMR	Codec Mode Request for EVS

IANA	Internet Assigned Numbering Authority

IP	Internet Protocol

IPv4	Internet Protocol version 4

IPv6	Internet Protocol version 6

IPBCP	IP Bearer Control Protocol

ITU-T	International Telecommunications Union-Telecommunication sector

IuFP	Iu Framing protocol

MGW	Media GateWay

MIME	Multi purpose Internet Mail Extension

MTP3b	Message Transfer Part level 3 for Q.2140 [15]

NNI	Network Node Interface

NSAP	Network Service Access Point

PDU	Protocol Data Unit

PVC	Permanent Virtual Circuit

RFC	Request For Comment

RTP	Real-Time Transport Protocol

RTCP	Real-Time Transport Control Protocol

SAR	Segmentation and Reassembly

SCCF-NNI	Service Specific Coordination Function-Network Node Interface

SDP	Session Description Protocol

SDU	Service Data Unit

SID	Silence Insertion Descriptor

SPVC	Switched PVC

SSSAR	Service Specific Segmentation and Re-assembly sublayer

SSCOP	Service Specific Connection Oriented Protocol

SSCS	Service Specific Convergence Sublayer

SSRC	Synchronisation Source

SVC	Switched Virtual Circuit

TrFO	Transcoder Free Operation

UDP	User Datagram Protocol

UNI	User Network Interface

UP	User Plane

VC	Virtual Circuit

# 4	General

The Nb UP shall be transported either over an ATM or IP bearer.

# 5	Transport over ATM for BICC-based Nc

## 5.1	General

Clause 5 is only applicable in combination with BICC-based Nc.

ATM shall be used in the transport network user plane and the transport network control plane according to ITU-T Recommendation I.361 [6]. The structure of the ATM cell header used across the Nb interface shall be the cell header format and encoding at the NNI (see Figure 3 in I.361 [6]).

## 5.2	Transport network user plane

### 5.2.1	General

Figure 2 shows the ATM protocol stack used for the transport network user plane on the Nb interface.

AAL-2 SAR SSCS (I.366.1)

AAL2 (I.363.2)

ATM

Figure 2: ATM protocol stack used for the transport network user plane

ATM AAL2 connections shall be transported over a VC which may either be a PVC, an SPVC or an SVC. For every ATM implementation of the Nb interface, a PVC shall be supported. The support of an SPVC or an SVC is optional. An ATM implementation may either support SPVCs only, SVCs only, or both.

If SPVCs or SVCs are supported, DSS2 signalling [20] shall be used for the establishment and tear down of these VCs. The network element that generated a given switched VC shall be the only network element that is allowed to tear down this VC.

### 5.2.2	ATM Adaptation Layer 2

#### 5.2.2.1	AAL2-Segmentation and Reassembly Service Specific Convergence Sublayer

Service Specific Segmentation and Reassembly (SSSAR) sublayer of ITU-T Recommendation I.366.1 [9] shall be used for the segmentation and reassembly of AAL2 SDUs (i.e. only SSSAR is used from I.366.1 [9]).

#### 5.2.2.2	AAL2-specification

AAL2 shall be used according to ITU-T Recommendation I.363.2 [7].

## 5.3	Transport network control plane

### 5.3.1	General

Figure 3 shows the protocol stack for the transport network control plane on the Nb interface.

AAL2 connection signalling (Q.2630.2)

AAL2 Signalling Transport Converter for MTP3b (Q.2150.1)

MTP3b

SSCF-NNI

SSCOP

AAL5

ATM

Figure 3: ATM protocol stack for the transport network control plane

Tunnelling, as described in 3GPP TS 23.205 [1], is currently not specified to transport the ATM transport network control plane.

### 5.3.2	Signalling protocol

#### 5.3.2.1	AAL2 Signalling Protocol

ITU-T Q.2630.2 [19] shall be used for the establishment of AAL2 connections. The AAL2 transport layer uses the embedded E.164 [5] or AESA variants of the NSAP addressing formats [21]. Native E.164 addressing shall not be used.

The MGW which issues a given ESTABLISH request [19] provides a Binding Reference (see 3GPP TS 23.205 [1]), This binding reference shall be copied into the SUGR parameter of the corresponding ESTABLISH request primitive [19].

The AAL2 Link Characteristics parameter (ALC) in the Establish Request message of the AAL2 signalling protocol shall be used.

### 5.3.3	Signalling transport converter

#### 5.3.3.1	AAL2 MTP3B Signalling Transport Converter

The AAL2 MTP3b Signalling Transport Converter shall be used according to ITU-T Recommendation Q.2150.1 [16].

### 5.3.4	MTP3b

MTP3b shall be used according to ITU-T Recommendation Q.2210 [17 & 18].

### 5.3.5	SSCF-NNI

SSCF-NNI shall be used according to ITU-T Recommendation Q.2140 [15].

### 5.3.6	SSCOP

SSCOP shall be used according to ITU-T Recommendation Q.2110 [14].

### 5.3.7	ATM Adaptation Layer Type 5

AAL5 shall be used according to ITU-T Recommendation I.363.5 [8]

# 6	Transport over IP for BICC based Nc

## 6.1	General

Clause 6 is only applicable in combination with BICC-based Nc.

RTP (RFC 1889 [24] or RFC 3550 [29]) over UDP (RFC 768 [22]) over either IPv4 (RFC 791 [23]) or IPv6 (RFC 2460 [27]) shall be used in the transport network user plane. The present specification takes the role of an RTP profile in describing the transport of the Nb UP protocol by RTP. Figure 4 shows the protocol stack for the transport network user plane on the Nb interface.

RTP

UDP

IPv4 or IPv6

Figure 4: IP Protocol stack for the transport network user plane

Tunnelling, as described in 3GPP TS 23.205 [1], shall be used to transport the IP bearer control protocol IPBCP conform the ITU-T Recommendation Q.1970 "BICC IP Bearer Control Protocol" (IPBCP) (see 3GPP TS 29.205 [11]).

## 6.2	Bearer Transport without multiplexing

### 6.2.0	Introduction

The support of bearer transport without multiplexing shall be supported. It shall be applied unless a bearer transport with multiplexing is negotiated via RTCP, as decribed in sub-clause 6.4.3.

### 6.2.1	IP

Either IPv4 (RFC 791 [23]) or IPv6 (RFC 2460 [27]) shall be used.

One MGW may have several IP interfaces with different IP addresses.

### 6.2.2	UDP

The UDP Protocol (see RFC 768 [22]) shall be applied.

Two consecutive port numbers shall be used at each MGW for the RTP bearer and for the optional RTCP connection that transport a single Nb UP connection. Two such consecutive port numbers are termed "port number block" in what follows. The first port number shall be even and shall be assigned to the RTP protocol. At a given MGW, the same port shall be used to send and to receive RTP PDUs. The next port number shall be assigned to the RTCP protocol. This port shall be reserved even if the optional RTCP protocol is not used.

Each MGW shall administer the port numbers it intends to use for RTP/RTCP port number blocks.

### 6.2.3	RTP

RTP (see RFC 1889 [24] or RFC 3550 [29]) shall be applied.

#### 6.2.3.1	RTP Header

The RTP Header Fields shall be used as described in the following subclauses:

##### 6.2.3.1.1	Version

RTP Version 2 shall be used.

##### 6.2.3.1.2	Padding

Padding shall not be used.

##### 6.2.3.1.3	Extension

The RTP Header shall not have an extension.

##### 6.2.3.1.4	Contributing Source (CSRC) count

There are zero CSRCs.

##### 6.2.3.1.5	Marker Bit

The marker bit is ignored.

##### 6.2.3.1.6	Payload Type

A dynamic Payload Type (see RFC 1890 [25]) shall be used. Values between 96 and 127 shall be used. The value shall be negotiated by means of the bearer control protocol.

##### 6.2.3.1.7	Sequence Number

The sequence number shall be supplied by the source MGW of a RTP PDU. The sink MGW of a RTP PDU may ignore the sequence number or it may use it to obtain statistics about the link quality and / or to correct out-of-sequence delivery, e.g. by dropping out-of-sequence packets.

##### 6.2.3.1.8	Timestamp

The timestamp shall be supplied by the source MGW of a RTP PDU. A clock frequency of 16000 Hz shall be used. The definition of the RTP timestamp is specified in IETF RFC 1889 [24] which states that RTP timestamp is based on the sampling instant of the source encoder. However for the present application the source MGW is not mandated to set the RTP timestamp according to the sampling instant of the payload PDU. Although the RTP timestamp can reflect the sampling instant in some scenarios, the sink MGW cannot rely upon the accuracy of the RTP Timestamp. The sink MGW of a RTP PDU may ignore the timestamp.

NOTE:	An application can use the time based NbFP frame number to obtain end-to-end timing information.

##### 6.2.3.1.9	Synchronisation Source (SSRC)

The source MGW of a RTP PDU shall supply a SSRC. The sink MGW of a RTP PDU may ignore the SSRC if it does not use RTCP.

##### 6.2.3.1.10	CSRC list

This list is empty.

#### 6.2.3.2	RTP Payload

A single UP PDU, as described in 3GPP TS 29.415 [3], shall be transported as a RTP payload.

### 6.2.4	RTCP

RTCP (see RFC 1889 [24]) may be applied. The use of the RTCP protocol is optional.

A MGW may ignore incoming RTCP PDUs.

Figure 5 shows the protocol stack for the transport of RTCP. The above Sections about IP and UDP shall also apply for the transport of RTCP.

RTCP

UDP

IPv4 or IPv6

Figure 5: RTCP Protocol stack for the transport network user plane

## 6.3	Bearer Control Protocol

The ITU-T Recommendation Q.1970 "BICC IP Bearer Control Protocol" (IPBCP) (see 3GPP TS 29.205 [11]) shall be applied.

The use of Iu FP as RTP payload shall be indicated within IPBCP. IuFP shall transport either speech or data in a bearer independent way as described in 3GPP TS 23.205 and 3GPP TS 29.205. The negotiation of the type of payload within IuFP is outside the scope of IPBCP and described in the above specifications.

NOTE:	The IuFP is registered with IANA as the MIME type "VND.3GPP.IuFP"of the "audio" category, however, this registration does not preclude the use of IuFP to transport "data".

### 6.3.1	Transport

IPBCP shall be transported over the Mc and Nc interface by means of the ITU-T Recommendation Q.1990 "BICC Bearer Control Tunnelling Protocol" ( see 3GPP TS 29.205 [11]). The transport of the Q.1990 "BICC Bearer Control Tunnelling Protocol" on the Mc interface is described in 3GPP TS 29.232 [4]. The transport of the "BICC Bearer Control Tunnelling Protocol" on the Nc interface is described in ITU-T Recommendation Q.765.5 (see 3GPP TS 29.205 [11]).

Figure 6: Transport of IPBCP

### 6.3.2	Procedures

The IPBCP procedures shall be used as described in the ITU-T Recommendation Q.1970 "BICC IP Bearer Control Protocol" (IPBCP) (see 3GPP TS 29.205 [11]).

#### 6.3.2.1	Bearer Establishment

The forward and backward RTP bearers used to transport one Nb UP connection shall be set up together after an IPBCP handshake with a Request message and an Accepted message has succeeded.

Each MGW shall signal its peer MGW with the RTP port number. The RTCP port number shall be the next higher number.

If the optional 20 ms packetisation time is supported for PCM encoded speech over Nb and if its use is authorized by local configuration setting, the MGW shall also negotiate with its peer MGW the packetisation time to be used for PCM encoded speech over Nb, even if the codec configured on the bearer is still unknown or differs from PCM. This is done by including the "pcmptime=20" parameter in the IPBCP Request as defined in sub-clause 6.3.3.5. A MGW receiving the "pcmptime=20" parameter ignores it if the 20 ms packetisation time is not supported for PCM encoded speech over Nb. A MGW receiving the "pcmptime=20" parameter shall return the "pcmptime=20" parameter in the IPBCP Accept message only if the 20 ms packetisation is supported for PCM encoded speech over Nb and if its use is authorized by local configuration setting. Receipt of the "pcmptime=20" parameter in the IPBCP Accept shall indicate to the originating MGW that it shall apply the 20 ms packetisation time when PCM encoded speech is sent over the Nb bearer. Otherwise the default 5 ms packetisation time shall be used for PCM encoded speech over Nb. The result of the negotiation is only used if PCM is selected on the bearer.

#### 6.3.2.2	Bearer Modification

A modification of existing RTP bearers is not permitted. The IPBCP Request message shall not be used to modify bearers.

#### 6.3.2.3	Bearer Release

When the H.248 Termination [10] of an Nb UP connection on a MGW is deleted by means of signalling over the Mc interface, which are outside the scope of IPBCP (see 3GPP TS 29.232 [4]), the used resources shall be freed as follows:

The MGW shall discard any packets arriving at the port pair used for the old Nb UP connection until it sets up a new Nb UP connection on these ports. The MGW shall only reuse these ports after a time that is long enough to avoid that packets from the old connection still arrive.

### 6.3.3	Use of IPBCP message fields

The IPBCP message fields shall be used as described in the ITU-T Recommendation Q.1970 "BICC IP Bearer Control Protocol" (IPBCP) (see 3GPP TS 29.205 [11]) and SDP (RFC 2327 [26]). Moreover, the following subclauses shall be applied:

#### 6.3.3.1	Origin

<address> shall be the IP address assigned to the IP interface used for the RTP bearer on the source MGW of the present IPBCP message.

#### 6.3.3.2	Session Name

The source MGW shall supply an arbitrary string as <session name>. The sink MGW shall ignore this string.

#### 6.3.3.3	Connection Data

The <connection address> shall be identical to the above origin <address>

#### 6.3.3.4	Media Announcement

<media> shall always be set to "audio" irrespective of the payload type within IuFP.

<port> shall be set to the port number assigned to the RTP bearer on the source MGW of the present IPBCP message

<transport> shall be set to "RTP/AVP".

<fmt list> shall be set to the chosen dynamic payload number. The MGW that initiates the bearer establishment may choose any value between 96 and 127. The peer MGW shall echo this value.

#### 6.3.3.5	Media Attributes

The following media attribute shall be supplied: "a=rtpmap:<dynamic payload number> VND.3GPP.IUFP/16000", where:<dynamic payload number> is the same dynamic payload type number as in the above media announcement <fmt list>.

The media attribute "a=fmtp:<format> pcmptime=20" shall also be set:

-	in the IPBCP Request if the optional 20 ms packetisation time is supported for PCM encoded speech over Nb and if its use is authorized by local configuration setting;

-	in the IPBCP Accept message if received in the IPBCP Request and if the optional 20 ms packetisation time is supported for PCM encoded speech over Nb and if its use is authorized by local configuration setting.

Other media attributes shall not be used. They shall be ignored in the MGW receiving an IPBCP message.

Example of IPBCP Request / Accept exchange where both MGWs support the 20 ms packetisation time for PCM encoded speech over Nb:

IPBCP Request (MGW1 -> MGW2)

m=audio 49170 RTP/AVP 97

a=rtpmap:97 VND.3GPP.IUFP/16000

a=fmtp:97 pcmptime=20

IPBCP Accept (MGW2 -> MGW1)

m=audio 49320 RTP/AVP 97

a=rtpmap:97 VND.3GPP.IUFP/16000

a=fmtp:97 pcmptime=20

## 6.4	Bearer Transport with multiplexing

### 6.4.1	Introduction

This sub-clause specifies an optional transport format for the Nb interface and IP transport that allows transporting several RTP/NbFP/codec payload PDUs of different user plane connections within one packet, and the corresponding backward compatible signalling extensions required to negotiate the use of this transport option. Use of this transport format saves bandwith in the IP network (bandwidth gains are evaluated in 3GPP TR 29.814 [28]).

Bandwith saving is achieved by multiplexing several NbFP PDUs within one UDP/IP packet. As an option, the RTP header may be compressed in addition. Two transport formats are specified accordingly.

### 6.4.2	Transport format

#### 6.4.2.1	IP

Either IPv4 (RFC 791 [23]) or IPv6 (RFC 2460 [27]) shall be used.

One MGW may have several IP interfaces with different IP addresses.

#### 6.4.2.2	UDP

The UDP Protocol (see RFC 768 [22]) shall be applied. If multiplexing is applied with or without header compression, the source UDP port number shall indicate the local termination used to combine the multiplexed packet and the destination UDP port number shall indicate the remote port number where PDUs are demultiplexed. The negotiation if multiplexing is applied and of the destination UDP port is described in sub-clause 6.4.3. If multiplexing was negotiated for a Nb UP user plane connection, the MGW may apply multiplexing by sending all packets of the user plane connection towards the negotiated destination UDP port.

#### 6.4.2.3	Transport Format for multiplexing without RTP Header Compression

Several RTP/NbFP/codec payload PDUs sent to the same IP address are multiplexed within one single UDP/IP packet over the Nb interface between MGWs. If DiffServ is applied, all multiplexed PDUs also need to share the same Diffserv class. The multiplexing shall only be used with RTP packets. RTCP shall be transported normally by UDP/IP packets.

Use of multiplexing shall be negotiated between MGWs, as specified in sub-clause 6.4.3.

Before each multiplexed RTP/NbFP/codec payload PDU inserted into the UDP/IP packet a Multiplex Header, which identifies the multiplexed packet, shall be inserted.

Figure 7: UDP/IP Packet with multiplexed RTP/NbFP payload PDUs without RTP header compression

The Multiplex Header includes :

-	T bit.

The field has two possible values, 0 for indicating full packet and 1 for indicating compressed packet. Value 0 shall be used for an uncompressed RTP header, as decribed in the present sub-clause.

-	Mux ID, 15 bits.

For identification of different user plane connections. The value shall be the UDP destination port of the corresponding non-multiplexed RTP/NbFP PDU packet divided by two (only even numbered ports are used for RTP sessions).

-	Length Indicator (LI), 8 bits, unsigned integer.

Gives the length of the multiplexed RTP/NbFP PDU packet (RTP header + RTP Payload = RTP header + NbFP header + NbFP payload) in bytes (the last byte of the RTP/NbFP PDU is padded to the next byte boundary if necessary). Maximum length is 255 bytes. This LI allows to calculate where the next Multiplex Header for the next multiplexed RTP/NbFP PDU packet starts.

-	R bit.

Reserved for future use. Shall be set to 0 by the sending entity and be ignored by the receiving entity.

-	Source ID, 15 bits.

For identification of the different connections. The value shall be the source UDP port of the corresponding non-multiplexed RTP/NbFP/codec PDU packet divided by two (only even numbered ports are used for RTP sessions). This information is transferred  to permit the receiving node to optionally detect and filter illegitimate packets (e.g. packets received from the peer termination precedingly associated to the receiving termination).

The multiplexed RTP/NbFP payload PDU shall be inserted in the IP/UDP packet directly after the corresponding Multiplex Header. The multiplexed RTP/NbFP payload PDU shall follow the rules in sub-clause 6.2.3 and consists of the full RTP header, the full NbFP header and the NbFP payload. If the multiplexed RTP/NbFP payload PDU does not end at a byte boundary, then the remaining bits of its last byte shall be padded with zeros.

The multiplexing method does not limit the number of packets being multiplexed and it is thus the data link layer protocol that defines the maximum frame size. E.g. an IP datagram has a maximum length of 65 535 bytes and Ethernet 1 518 bytes. In order to avoid additional delay in the network the packets should not be delayed more than 1 ms to 2 ms, which also effectively limits the number of multiplexed packets and makes the multiplexing-jitter low.

Figure 8: Example of multiplexed packet with two RTP frames

#### 6.4.2.4	Transport Format for multiplexing with RTP header compression

To achieve even better bandwidth savings, the RTP header may optionally be compressed. This is possible since the  RTP header includes many static fields that remain unchanged during an RTP session if NbFP is used as payload (see sub-clause 6.3.2).

Use of RTP header compression shall be negotiated between MGWs, as specified in sub-clause 6.4.3.

At least the first two RTP packets of each RTP session shall be sent with their full RTP header to allow the receiver to store the full header and use it in decompression. RTP packets shall also be sent with their full RTP header till receipt of a RTCP packet from the peer indicating support of RTP header compression. Subsequent packets may be sent with a compressed RTP header. If a MGW does not receive any of the initial RTP packets with a full RTP header, the MGW shall assume that the fields of the RTP header other than those present in the compressed RTP header are set as defined in sub-clause 6.2.3.1, and shall therefore not consider this as an error.

Before each multiplexed RTP/NbFP/codec payload PDU inserted into the UDP/IP packet a Multiplex Header, which identifies the multiplexed packet, shall be inserted

Figure 9: UDP/IP Packet with multiplexed RTP/NbFP payload PDUs with RTP header compression

The Multiplex Header shall be used as described in sub-clause 6.4.2.3. However, the T bit shall be set for a compressed RTP header, as decribed in the present sub-clause. The Length Indicator gives the length of the multiplexed RTP/NbFP PDU packet in bytes (compressed RTP header + RTP Payload ).

The multiplexed RTP/NbFP payload PDU shall be inserted in the IP/UDP packet directly after the corresponding Multiplex Header. The multiplexed RTP/NbFP payload PDU shall consist of the compressed RTP header described below followed by the full NbFP header and the NbFP payload, as described in 3GPP TS 29.415 [3]. If the multiplexed RTP/NbFP payload PDU does not end at a byte boundary, then the remaining bits of its last byte shall be padded with zeros.

The compressed RTP header shall include the following two fields taken from the uncompressed RTP header:

-	Sequence number (SN), 8 bits.

The field changes as the original sequence number (RFC 3550 [29]) but is shortened from 16 bits to 8 bits (256 packets). The least significant byte of the RTP sequence number shall be included. Sub-clause 6.2.3.1.7 is applicable.

-	Timestamp (TS), 16 bits.

The TS field changes as the original timestamp (RFC 3550 [29]) but the length is half of the original resulting in modulo of 4 seconds with 16 kHz clock reference. The least significant two bytes of the RTP timestamp shall be included. Sub-clause 6.2.3.1.8 is applicable.

NOTE:	These RTP fields change during a connection and thus need to be transferred within each packet for NbFP payload. All other RTP fields do not change (see sub-clause 6.2.3).

Figure 10: Example of multiplexed packet with two RTP frames and compressed RTP headers

### 6.4.3	RTCP

#### 6.4.3.1	General

A MGW wishing to apply Bearer Transport with multiplexing shall use RTCP (see RFC 3550 [29]) to negotiate multiplexing.

RTCP shall be used separately for each user plane connection. RTCP shall be transported by UDP/IP packets as described in sub-clause 6.2.4. and not included included in IP/UDP packets using the multiplexing format described above.

#### 6.4.3.2	Multiplex negotiation via RTCP

RTCP shall be used to negotiate the use of multiplexing. A new 3GPP-specific RTCP Multiplexing packet is specified in sub-clause 6.4.3.3 for this purpose, and may be added to compound RTCP packets following the principles of RFC 3550 [29]. The Multiplexing RTCP packet indicates:

-	if multiplexing without RTP header compression is supported ;

-	if multiplexing with RTP header compression is supported ;

-	the local UDP port where to receive multiplexed data streams,

-	if multiplexing is selected..

When setting up a new user plane connection, both peer MGWs of a NbUP connection shall start to send data without applying multiplexing and may indicate their readiness to receive multiplexed data streams by including the new RTCP Multiplexing packet in the initial and all subsequent RTCP packets they send. A MGW shall always announce the same multiplexing capabilities and the same UDP port where to receive multiplexed data streams in all the RTCP Multiplexing packets it sends for a given RTP session. MGWs should preferably send their initial RTCP packet at the very beginning of the RTP session to be able to apply multiplexing as soon as possible. A MGW sending a Multiplexing packet indicating support of multiplexing shall be ready to receive multiplexed packets at the announced UDP port. A single UDP port for multiplexing shall be used per destination IP address.

A MGW receiving an RTCP packet, where the peer indicated its readiness to receive multiplexed data streams, may apply multiplexing to send the corresponding RTP data streams towards the sender of the RTCP packet. If the MGW decides to apply multiplexing, it can immediately start sending multiplexed data streams towards the corresponding UDP multiplexing port announced in the received RTCP Multiplexing packet. The MGW shall indicate in subsequent RTCP Multiplexing packets if it applies multiplexing with or without header compression for the given user connection

A MGW that does not receive RTCP or receives RTCP without the RTCP Multiplexing packet shall continue to send data for the user connection without applying multiplexing.

A MGW that does not support multiplexing will ignore the unknown received RTCP Multiplexing package according to RTCP procedures and will continue to send data for the user connection without applying multiplexing.

Sending of a RTCP Multiplexing packet indicating readiness to receive multiplexed data streams does not necessarily mean that the MGW is ready to send multiplexed data streams, i.e. multiplexing may be applied on a single or on both directions for a given RTP session.

The applicable compressed header format is not negotiated using RTCP, but is determined by the fact if the Nb bearer is controlled via a SIP-I based Nc interface or a BICC based Nc interface. A MGW may support multiplexing of both RTP header compression formats on a same IP interface.

#### 6.4.3.3	RTCP Multiplexing packet

The format of the RTCP Multiplexing packet is specified in figure 11.

Figure 11: RTCP Multiplexing packet

The APP packet header includes :

-	version (V), 2 bits

Identifies the version of RTP, which is the same in RTCP packets as in RTP data packets.  RTP Version 2 shall be used.

-	padding (P), 1 bit

As specified in RFC 3550 [4].

-	subtype, 5 bits.

The following subtype value shall be used :

00001 : RTCP Multiplexing packet

-	packet type (PT), 8 bits.

Shall contains the constant 204 to identify this as an RTCP APP packet

-	length, 16 bits.

As specified in RFC 3550 [4]. The length of this RTCP packet in 32-bit words minus one, including the header and any padding.  (The offset of one makes zero a valid length and avoids a possible infinite loop in scanning a compound RTCP packet, while counting 32-bit words avoids a validity check for a multiple of 4.)

-	SSRC/CSRC, 32 bits.

As specified in RFC 3550 [4].

-	name, 32 bits.

Shall be set to "3GPP"

The application-dependent data includes :

-	multiplexing bit (MUX), 1 bit

Indicates whether multiplexing without RTP header compression is supported or not  by the sender of the RTCP packet : set to 0 if not supported, set to 1 if supported.

-	multiplexing with RTP header compression bit (CP), 1 bit

Indicates whether multiplexing with RTP header compression is supported or not by the sender of the RTCP packet : set to 0 if not supported, set to 1 if supported.

-	Selection bits, 2 bits

Indicates whether the sender of the RTCP packet has selected to apply multiplexing with or without header compression for the user plane packets that it sends on this connection. The following values are defined:

00: no multiplexing is applied

01: multiplexing is applied without RTP header compression

10: multiplexing is applied with RTP header compression

11: reserved

-	Local MUX UDP port, 15 bits :

Local UDP port where the sender demands to receive multiplexed data streams. The value shall be the same as the local MUX UDP port divided by two. This parameter shall be ignored by the receiver of the RTCP Multiplexing packet if the MUX and CP bits indicate that multiplexing is not supported.

-	Reserved bits:

Extension bits may be added in the RTCP Multiplexing packet in future releases. Reserved bits shall be set to 0 in sent RTCP Multiplexing packet of this relaese and shall be ignored in incoming  RTCP Multiplexing packets.

Extension fields may be added in the RTCP Multiplexing packet in future releases. They shall be ignored by a MGW implementing an earlier version of the specification.

# 7	Transport over IP for SIP-I based Nc

## 7.1	General

Clause 7 is only applicable in combination with SIP-I based Nc.

IPv4 (RFC 791 [23]) shall be supported. IPv6 (RFC 2460 [27]) may be supported.

RTP (RFC 1889 [24] or RFC 3551 [29]) over UDP (RFC 768 [22]) over either IPv4 (RFC 791 [23]) or IPv6 (RFC 2460 [27]) shall be used in the transport network user plane. Figure 4 in Clause 6.1 shows the protocol stack for the transport network user plane on the Nb interface.

## 7.2	Bearer Transport without multiplexing

### 7.2.0	Introduction

See 6.2.0

### 7.2.1	IP

See 6.2.1

### 7.2.2	UDP

See 7.2.2

### 7.2.3	RTP

RTP (see RFC 1889 [24] or RFC 3550 [30]) shall be applied.

#### 7.2.3.1	RTP Header

The RTP Header Fields shall be used as described in the following subclauses:

##### 7.2.3.1.1	Version

RTP Version 2 shall be used.

##### 7.2.3.1.2	Padding

Padding shall not be used.

##### 7.2.3.1.3	Extension

The extension bit shall be used as specified for the RTP profile applicable for the used payload.

##### 7.2.3.1.4	Contributing Source (CSRC) count

There are zero CSRCs.

##### 7.2.3.1.5	Marker Bit

The marker bit shall be used as specified for the RTP profile applicable for the used payload.

##### 7.2.3.1.6	Payload Type

The payload type shall be set as negotiated by means of SIP-I and supplied over the Mc interface.

##### 7.2.3.1.7	Sequence Number

The sequence number shall be supplied by the source MGW of a RTP PDU.

##### 7.2.3.1.8	Timestamp

The timestamp shall be supplied by the source MGW of a RTP PDU.

##### 7.2.3.1.9	Synchronisation Source (SSRC)

The source MGW of a RTP PDU shall supply a SSRC.

##### 7.2.3.1.10	CSRC list

This list is empty.

#### 7.2.3.2	RTP Payload

Speech in various encodings packed in corresponding RTP payload types may be transported as RTP Payload, as further specified in TS 26.102 [32]. Data and fax packed in corresponding RTP payload types may also be transported as RTP Payload, as further specified in 3GPP TS 29.007 [36].

NOTE:	The Nb framing protocol specified in 3GPP TS 29.415 [3] is not applicable.

### 7.2.4	RTCP

The use of the RTCP (see RFC 1889 [24] or RFC 3551 [29]) protocol is optional, unless the RTP payload profile of the transported payload requires RTCP.

A MGW may ignore incoming RTCP PDUs, unless the RTP payload profile of the transported payload requires some usage of RTCP

Figure 5 in Clause 6.2.4 shows the protocol stack for the transport of RTCP. The above Sections about IP and UDP shall also apply for the transport of RTCP.

## 7.3	Bearer Transport with multiplexing

### 7.3.1	Introduction

See Clause 6.4.1

### 7.3.2	Transport format

#### 7.3.2.1	IP

See Clause 6.4.2.1

#### 7.3.2.2	UDP

See Clause 6.4.2.2

#### 7.3.2.3	Transport Format for multiplexing without RTP Header Compression

See Clause 6.4.2.3.

However, instead of RTP/NbFP/codec payload PDUs, RTP/codec payload PDUs shall be transported. The RTP header and the RTP payload shall follow the rules in sub-clause 7.2.3.

#### 7.3.2.4	Transport Format for multiplexing with RTP header compression

RTP header compression for Nb bearers with SIP-I based Nc shall be applied as specified in sub-clause 6.4.2.4 for Nb bearers with BICC-based Nc, with the exceptions specified in the present Clause.

The transport format additionally includes the marker bit (bit M) and the payload type (PT), which can very dependent on the transported payload.

Figure 12: UDP/IP Packet with multiplexed RTP/Nb payload PDUs with RTP header compression

The compressed RTP header shall include the following fields taken from the uncompressed RTP header:

- Sequence number (SN), 8 bits.

- Timestamp (TS), 16 bits.

- Marker bit (M), 1 bit.

- Payload Type (PT), 7 bits

The usage of these header fields shall follow Clause 7.2.3

The Transport Format for multiplexing with RTP header compression shall not be applied for RTP payload types that use an RTP header extension. For RTP payload types that use an RTP header extension, this transport format shall not be offered or accepted in the RTCP multiplexing negotiation.

### 7.3.3	RTCP

See Clause 6.4.3

## 7.4	Transcoder-less Interworking with the Iu Framing Protocol

### 7.4.1	Introduction

Figure 13: Transcoder-less Interworking between IuFP and RTP (optional in the event the codecs on both sides are the same or TrFO-compatible)

If no transcoder is inserted, the MGW shall interwork the procedures between the RTP at the Nb interface of the SIP-I based CS CN and the Iu framing protocol (see 3GPP TS 25.415 [31]) at the Iu interface as described in the following Clauses.

### 7.4.2	Initialisation

There is no need to interwork IuFP initialisation procedures between Iu interface and the RTP at the Nb interface of the SIP-I based CS CN.

### 7.4.3	Time alignment

The purpose of the time alignment procedure on the Iu interface is to minimise the buffer delay in the RNC for downlink transmissions by adjusting the vocoder time reference within the network. No such procedure exists within RTP, so the MGW shall return NACK indication time alignment not supported according to 3GPP TS 25.415 [31].

### 7.4.4	Rate control

#### 7.4.4.1	General

The rate control procedure signals to the peer entity the maximum rate among the currently allowed rates at which it can receive codec frames. Rate control only applies to AMR, AMR-WB codec configurations with multiple active modes. For the EVS codec, the rate and mode control procedure signals to the peer entity the maximum bit rate, the maximum audio bandwidth and the mode of operation among the currently allowed modes at which it can receive codec frames. Rate and Mode control applies to EVS even in single-rate EVS Configurations due to the fact, that the EVS AMR-WB IO mode of operation is always included.

#### 7.4.4.2	Rate control for AMR and AMR-WB

For AMR and AMR-WB codecs:

-	On the Iu interface, IuFP provides for rate control via the exchange of RATE CONTROL and RATE CONTROL ACK PDUs.

-	 On the Nb interface of the SIP-I based CS CN, RFC 4867 [35] provides for in-band rate control via the Codec Mode Request (CMR) field of every codec frame (Speech and SID) and in CMR-only frames.

-	 Interworking of rate control procedures at an MGW between an Nb interface and a corresponding Iu interface only applies when the MGW bridges TrFO-compatible codec configurations between the interfaces without applying a transcoding function.

-	 An MGW receiving a CMR from an Nb interface shall initiate the IuFP rate control procedure on the corresponding Iu interface, if the CMR value changed compared to the previous received CMR value.

-	 An MGW receiving a rate control request on an Iu interface shall adjust the CMR field of outgoing speech or SID or CMR-Only frames on the corresponding Nb interface.

#### 7.4.4.3	Rate and Mode control for EVS

For the EVS codec see TS 26.454 [39] for details.

#### 7.4.4.4	Interworking of rate control between compatible AMR-WB and EVS codec configurations

#### For details, see TS 26.454.

### 7.4.5	Frame quality indication

The Iu framing protocol signals frame quality with the Frame Quality Classification (FQC) field of each speech frame PDU. See 3GPP TS 26.102 [32] and 3GPP TS 25.415 [31] for details. The FQC may have possible values: 0=frame_good; 1=frame_bad; 2=frame_bad_due_to_radio; and 3=spare. The RTP for AMR and AMR-WB payload at the Nb interface signals frame quality with the Q bit (frame quality indicator) field of each speech frame, as defined in RFC 4867 [35]. The Q bit may have values: 1=speech_good or sid_good; and 0=speech_bad or sid_bad. For EVS the Q bit does not exist on Nb, for details see TS 26.454 [39].

Tables 1 and 2 provide the mapping between Nb and Iu interfaces.

# Table 1: Mapping of Nb RTP profile Qbit onto Iu FQC

| Nb RTP profile Qbit | Nb RTP profile FT | Iu - FQC |
| --- | --- | --- |
| 1 | x | 0 |
| 0 | x | 1 |

# Table 2: Mapping Iu FQC onto Nb RTP profile Qbit

| Iu - FQC | Nb RTP profile Qbit | Nb RTP profile FT |
| --- | --- | --- |
| 0 | 1 | NC |
| 1 | 0 | NO_DATA |
| 2 | 0 | NC |

### 7.4.6	Framing

Even when the MGW bridges compatible codec configurations for AMR or AMR-WB between the Nb and Iu interfaces, the MGW shall perform translation between the frame formats defined for the two interfaces, since all codec configurations have different framing procedures for the two interfaces. The framing details for Iu are defined in 3GPP TS 26.102 [32] and 3GPP TS 25.415 [31] for AMR and AMR-WB, although they do not describe the framing for ITU-T codecs other than G.711. The framing details for Nb are defined in IETF RFC 4867 [35], IETF RFC 3550 [29], IETF RFC 3551 [33] and IETF RFC 3555 [34]. TS 26.454 [39] defines the framing for EVS on Iu and Nb, as well as the translation between the frame formats on different terminations of the MGWs.

### 7.4.7	Transcoding

Transcoding at the MGW is avoided when the MGW bridges compatible codec configurations between the Nb and Iu interfaces. Otherwise, transcoding is necessary, which eliminates the need to interwork other user plane procedures between the interfaces.

### 7.4.8	Discontinuous transmission

When the MGW bridges compatible codec configurations between the Nb and Iu interfaces, the DTX procedures are normally interworked transparently by translating between the framing formats on the interfaces. AMR, AMR-WB and EVS codecs have configurations that are compatible between the Iu and Nb interfaces. ITU-T Codecs are not applicable on Iu. Only G.711 is applicable on Nb.

For the AMR, AMR-WB and EVS codecs, DTX is always enabled on the CS radio Interface in the uplink direction and the transport channel for DTX in the downlink is always provided.

### 7.4.9	Timing and sequence information

The MGW shall always correct out-of-sequence delivery between Nb and Iu interfaces, either by re-ordering frames, or by dropping frames that are out of sequence.

When the IuFP frame numbers are based on time and if the MGW bridges compatible codec configurations between the Nb and Iu interfaces, it shall either correct jitter before forwarding PDUs or interwork the RTP timestamp (see RFC 3550 [29]) with the IuFP Frame Number (see 3GPP TS 25.415 [31]) so that both the RTP timestamp and IuFP frame number similarly reflect the nominal sampling instant of the user data in the packet.

NOTE:	Correcting jitter may cause additional delay.

The RTP sequence number (see RFC 3550 [29]) is handled independently on Nb, i.e. it is not interworked with the IuFP Frame Number (see 3GPP TS 25.415 [31]). TS 26.454 [39] describes the handling of Jitter for the EVS-Channel-Aware mode of operation. 
End of changes