Agenda Item:	10.7

Source:	Huawei, Hisilicon

Title:	Discussion on VR content impact on QoE

Document for:	Discussion and agreement

- Introduction

This contributions proposes QoE metrics relevant with VR content.

- Discussion

VR content is defined as a new audio-visual media type that shares many characteristics of traditional linear audio-visual content but differs in significant ways, mainly in terms of rendering and display part. Take DASH based VR content for example, the VR content includes media content and media presentation description (MPD). The MPD includes the relevant information on the different encoded versions of the DASH VR content, with different viewport information if viewport-dependent streaming is applied, for example the attributes of codecs, bitrates and resolutions.

As the OMAF DIS is released in December 2017, it provides the base for discussion on VR content impact on QoE. As the present version of OMAF solution supports DASH based VR streaming, MPD will be included that would contain all the information relevant with video/audio quality. This contribution discusses on the information that tells about user experience of VR service. The following cases would be discussed:

- HEVC-based viewport independent streaming: in this case, the MPD would include information for different available versions of video quality for 360 degree video as a whole, e.g. resolution, bitrate etc.

- HEVC-based viewport dependent streaming: in this case, different solutions maybe applied for streaming, e.g. multi-stream or tile based streaming. But generally the attributes of content quality of viewport and non-viewport should be collected. Quality of viewport would be most important in terms of impact on user experience, while quality of non-viewport would also provide clues since when head movement happens, this part of video would be a transient cover before high-quality video could be presented. The quality would be represented separately with information of resolution, bitrate, etc.

Based on the above discussion and take into account the definition in TS26.247[1], the MPD information for VR quality could be defined as:

| Key |  | Type | Description |
| --- | --- | --- | --- |
| MPDInformation |  | Object |  |
|  | representationid | String | Value of Representation@id for the representation addressed by the QoE metrics report. |
|  | viewportindicator | Integer | If viewport-dependent streaming is applied, this element Indicates whether the representation is within the viewport. Possible value: 1 indicates representation within viewport, 0 indicates representation out of viewport. If viewport-independent streaming is applied, the value is NULL. |
|  | subreplevel | Integer | If present, value of SubRepresentation@level for the subrepresentation addressed by the QoE metrics report. If not present, the QoE metrics report concerns the representation as a whole. |
|  | Mpdinfo | RepresentationType | Provides the MPD information for the representation or subrepresentation identified by representationid and subreplevel, if present. The following attributes and elements shall be present within mpdinfo if they are present for the identified representation or subrepresentation and their values shall be identical to those presented in the MPD: @bandwidth, @qualityRanking, @width, @height, @mimeType, and @codecs. |

- Proposal

The detailed proposal is to add relevant content in this contribution in TR26.929, described in pCR S4-180051.

- Reference

[1] TS 26.247 Transparent end-to-end Packet-switched; Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH) v15.0.0;