INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG11 MPEG2016/M42122

January 2018, Gwangju, South Korea

| Source | Qualcomm |
| --- | --- |
| Status | Input |
| Title | Updated demo on Carriage of Web Resources in ISOBMFF (ISO/IEC 23001-15) |
| Author | Niteesh Bhat, Ece Oeztuerk, Waqar Zia, Thomas Stockhammer |

# Overview

M41762 from MPEG120 provided a demo on Web Resources in ISO BMFF. This input provides a few more updates to the demo to emphasize how this web resource track can be used to deliver either planned or unplanned events either via unicast or broadcast or in a hybrid delivery context. The open source code is available at [2] (currently in test branch).

# Demonstration Scenario

## Use Case

A viewer is watching an interactive stream of a major soccer event with interactive notifications enabled. Late into the event suddenly the camera focuses on a spectator who is a legendary soccer star. The viewer can access detailed biography of the star by clicking on his icon image that pops up on the display.

## Realization Architecture

### Sending-end

Using web resource track, the content provider can send interactive information and update the UI not just on planned times but also on unplanned events like the one indicated above.

A web-based UI representing the content provider is realised shown in Figure 1. As soon as the page is opened, a session is created for a demo. The top of the page provides the URL of the interactive application, clicking this opens the application in a new window.

The UI allows the user (representing content provider actor) to select one of a few selectable profiles either at a specific media time or at a configurable time delay from now. Based on this time, a specific sample of the web interactivity track is updated with the required information. The inserted profile is shown on the player-side for 5 seconds.

Server-side scripting (PHP) is used for the demo realization for this purpose. As the content available for demonstration is not live, file crew profiles are made available for unplanned insertion to emulate the demo use case.

Figure 1: Sending-end interface to insert profile

As soon as this sample script is received, the script is added to the DOM on the decode time and the action is played out on the planned time.

### Receiving end

All the data used to create the presentation on the receiver side in the demo is embedded in the web resource or HTML track (i.e. it is in band data) except the actor profile information, which is accessed from the open internet.

#### Realization modules

The current realization focuses on achieving the demonstration use cases as depicted in the Hypothetical Processing Model for Web Engine including Media Engine of [1].

- HTML Track Access is implemented in JS, possibly as a part of the page or as service worker.

- Reads the HTML track samples according to the media decode times and adds them to the DOM.

  - Based on the function provided by the added code, it can either be executed instantaneously (e.g. by providing a method definition and a call (with an arbitrary delay) to the method in the global scope) or it adds itself in the timing model of the main page (e.g. by adding new states to a state-machine of the main page).

- The main DOM is not aware of the media decode times and the samples (fetching issues etc.)

It should be noted that currently the interactive track samples are not encoded in an ISO-BMFF conforming fashion. The demo abstraction makes the decode times as well as locations known to the “HTML Track access” block. In an advanced realization, the decode timing of samples will be embedded in the interactive track and be made known to the application along with the locations via e.g. a manifest/MPD.

The source code of the demo is published here [2].

# Running the Demo on client-side

Upon load, note the original Body and Script elements.

As the user clicks play, a new body replaces this which is loaded as a part of the random access sample, along with initial scripts:

As time passes by, more JS script elements as new differential samples are loaded into the document as can be seen in the debug window.

These scripts serve various roles of adding interactive information to the page to manipulating the video element.

Finally, any unplanned profile inserted by the sending end will appear on the top-right corner of the screen.

A check box is available to enable/disable display of interactivity events.

# References

- N16944, “Working Draft on Carriage of Web Resources in ISOBMFF”

- [online]: https://github.com/waqarz/InteractiveTrack

Hence demo is resource intensive, and the setup public instance for the demo is not meant for frequent public use. The code is open-source based, and interested companies are pointed to run the code from own hosted servers.