3GPP TS 28.662 V12.1.0 (2016-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Generic Radio Access Network (RAN)

Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)

(Release 12)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

NRM, IRP, Converged Management, Generic RAN

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2016, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	6

4	Model	7

4.1	Imported information entities and local labels	7

4.2	Class diagrams	7

4.2.1	Relationships	7

4.2.2	Inheritance	8

4.3	Class definitions	9

4.3.1	SectorEquipmentFunction	9

4.3.1.1	Definition	9

4.3.1.2	Attributes	9

4.3.1.3	Attribute constraints	9

4.3.1.4	Notifications	10

4.3.2	AntennaFunction	10

4.3.2.1	Definition	10

4.3.2.2	Attributes	10

4.3.2.3	Attribute constraints	10

4.3.2.4	Notifications	10

4.3.3	TMAFunction	11

4.3.3.1	Definition	11

4.3.3.2	Attributes	11

4.3.3.3	Attribute Constraints	11

4.3.3.4	Notifications	11

4.3.4	GSMCellPart	12

4.3.4.1	Definition	12

4.3.4.2	Attributes	12

4.3.4.3	Attribute constraints	12

4.3.4.4	Notifications	12

4.3.5	CommonBsFunction	12

4.3.5.1	Definition	12

4.3.5.2	Attributes	12

4.3.5.3	Attribute constraints	12

4.3.5.4	Notifications	12

4.3.6	CellReferences	13

4.3.6.1	Definition	13

4.3.6.2	Attributes	13

4.3.6.3	Attribute constraints	13

4.3.6.4	Notifications	13

4.3.7	RepeaterFunction	13

4.3.7.1	Definition	13

4.3.7.2	Attributes	13

4.3.7.3	Attribute constraints	14

4.3.7.4	Notifications	14

4.4	Attribute definitions	15

4.4.1	Attribute properties	15

4.4.2	Constraints	21

4.5	Common Notifications	21

4.5.1	Alarm notifications	21

4.5.2	Configuration notifications	21

Annex A (informative):	Change history	23

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management; as identified below:

28.661	Generic Radio Access Network (RAN) Network Resource Model (NRM); Integration Reference Point (IRP); Requirements.

28.662	Generic Radio Access Network (RAN) Network Resource Model (NRM); Integration Reference Point (IRP); Information Service (IS).

28.663	Generic Radio Access Network (RAN) Network Resource Model (NRM); Integration Reference Point (IRP); Solution Set (SS) definition.

# 1	Scope

The present document specifies the Generic Radio Access Network (RAN) network resource model (NRM) that can be communicated between an IRPAgent and an IRPManager for telecommunication network management purposes, including management of converged networks.

This document specifies the semantics and behaviour of information object class attributes and relations visible across the reference point in a protocol and technology neutral way.  It does not define their syntax and encoding.

This document specifies equipment that may be shared between BSS in GSM, UTRAN and E-UTRAN.

In order to access the information defined by this NRM, an Interface IRP such as the "Basic CM IRP" is needed (3GPP TS 32.602 [5]). However, which Interface IRP is applicable is outside the scope of the present document.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[3]	3GPP TS 32.102: "Telecommunication management; Architecture".

[4]	3GPP TS 32.150: "Telecommunication management; Integration Reference Point (IRP) Concept and definitions".

[5]	3GPP TS 32.602: "Telecommunication management; Configuration Management (CM); Basic CM Integration Reference Point (IRP) ; Information Service (IS)".

[6]	Void.

[7]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E_UTRA); Base Station (BS) radio transmission and reception".

[8]	Void.

[9]	3GPP TS 25.466: "UTRAN Iuant interface: Application Part".

[10]	3GPP TS 28.661: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM) Integration Reference Point (IRP); Requirements".

[11]	3GPP TS 32.111-2: "Telecommunication management; Fault Management; Part 2: Alarm Integration Reference Point (IRP): Information Service (IS)".

[12]	3GPP TS 28.652: "Telecommunication management; Universal Terrestrial Radio Access Network (UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS) ".

[13]	3GPP TS 28.658: “Telecommunication management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)”.

[14]	3GPP TS 28.655: "Telecommunication management; GSM/EDGE Radio Access Network (GERAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS) ".

[15]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[16]	3GPP TS 32.302: "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP): Information Service (IS)".

[17]	3GPP TS 32.662: "Telecommunication management; Configuration Management (CM); Kernel CM Information Service (IS)".

[18]	3GPP TS 25.106: "Technical Specification Group Radio Access Network; UTRA repeater radio transmission and reception".

[19]	3GPP TS 45.005: "Radio transmission and reception".

[20]	3GPP TS 45.010: "Radio subsystem synchronization".

[21]	3GPP TS 25.104: "Base Station (BS) radio transmission and reception (FDD)".

[22]	3GPP TS 25.105: "Base Station (BS) radio transmission and reception (TDD)".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the definitions given in TR 21.905 [1], TS 32.150 [4], TS 32.101 [2], TS 32.102 [3] and the following apply. The definitions defined in the present document take precedence over those, if any, in TS 32.150 [4], TS 32.101 [2], TS 32.102 [3] and TR 21.905 [1], in that order.

No definition.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BS	Base Station

BSS	Base Station Subsystem

CM	Configuration Management

DN	Distinguished Name

E-UTRAN	Evolved UTRAN

GSM	Global System for Mobile communications

HW	Hardware

IRP	Integration Reference Point

IOC	Information Object Class

IS	Information Service

NE	Network Element

NRM	Network Resource Model

RAN	Radio Access Network

RDN	Relative Distinguished Name

RF	Radio Frequency

SS	Solution Set

TMA	Tower Mounted Amplifier

UTRA	Universal Terrestrial Radio Access

UTRAN	Universal Terrestrial Radio Access Network

# 4	Model

## 4.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| 3GPP TS 28.622 [15], IOC, ManagedFunction | ManagedFunction |
| 3GPP TS 28.652 [12], IOC, UtranGenericCell | UtranGenericCell |
| 3GPP TS 28.658 [13], IOC, EUtranGenericCell | EUtranGenericCell |
| 3GPP TS 28.655 [14], IOC, GSMCell | GSMCell |

## 4.2	Class diagrams

### 4.2.1	Relationships

This subclause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for this IRP. This subclause provides the overview of the relationships of relevant classes in UML. Subsequent subclauses provide more detailed specification of various aspects of these classes.

.

Figure 4.2.1.1: UTRAN/E-UTRAN/GERAN sharing (1/2)

Figure 4.2.1.2: UTRAN/E-UTRAN/GERAN sharing (2/2)

Figure 4.2.1.3: CommonBsFunction

Figure 4.2.1.4: Repeater object Containment/Naming and Association diagram

Figure 4.2.1.5: Repeater related VsDataContainer Containment/Naming and Association diagram

### 4.2.2	Inheritance

This subclause depicts the inheritance relationships.

Figure 4.2.2.1:  Inheritance diagram (1/2)

Figure 4.2.21.2: Inheritance diagram (2/2)

## 4.3	Class definitions

### 4.3.1	SectorEquipmentFunction

#### 4.3.1.1	Definition

This IOC represents a set of cells within a geographical area that has common functions relating to AntennaFunction, TMAFunction and supporting equipment, such as power amplifier.

This IOC is required as part of the capability to satisfy the Requirements statement identified below.

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 28.661 [10] | REQ-GRAN_NRM-CON-001 |  |
| 3GPP TS 28. 661 [10] | REQ-GRAN_NRM- CON-002 |  |

#### 4.3.1.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| fqBand | CM | M | - | - | M |
| eUTRANFqBands | CM | M | M | - | M |
| uTRANFDDFqBands | CM | M | M | - | M |
| uTRANTDDFqBands | CM | M | M | - | M |
| confOutputPower | M | M | M | - | - |
| Attribute related to role |  |  |  |  |  |
| theTMAList | CM | M | - | - | M |
| theAntennaList | CM | M | - | - | M |
| theCellList | CM | M | - | - | M |

#### 4.3.1.3	Attribute constraints

| Name | Definition |
| --- | --- |
| fqBand CM Support Qualifier | Condition: LTE cell is supported AND the attribute eUTRANFqBands contains an empty list. |
| eUTRANFqBands CM Support Qualifier | Condition: LTE cell is supported |
| uTRANFDDFqBands CM Support Qualifier | Condition: UTRAN FDD cell is supported |
| uTRANTDDFqBands CM Support Qualifier | Condition: UTRAN TDD cell is supported |
| theTMAList CM Support Qualifier | Condition: Association between SectorEquipmentFunction and AntennaFunction is absent AND is supporting the UTRAN/E-UTRAN sharing/non-sharing case OR is supporting the GERAN sharing case. In such case, at least one TMAFunction is present. |
| theAntennaList CM Support Qualifier | Condition: Association between SectorEquipmentFunction and TMAFunction is absent AND is supporting the UTRAN/E-UTRAN sharing/non-sharing OR is supporting GERAN sharing case. In such case, at least one AntennaFunction is present. |
| theCellList CM Support Qualifier | Condition: Association between SectorEquipmentFunction and ProxyCellList is present AND is supporting UTRAN/E-UTRAN sharing (and non-sharing) cases. In such case, at least one instance represented by the associated ProxyCell is present. Condition: Association between SectorEquipmentFunction and ProxyCellList is present AND is supporting the GERAN sharing case. In such case, at least one GSMCellPart is present. |

#### 4.3.1.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.2	AntennaFunction

#### 4.3.2.1	Definition

This IOC represents an array of radiating elements that may be tilted to adjust the RF coverage of a cell(s).

This IOC is required as part of the capability to satisfy the Requirements statement identified below.

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 28.661 [10] | REQ-GRAN_NRM-CON-001 |  |
| 3GPP TS 28.661 [10] | REQ-GRAN_NRM-CON-002 |  |

#### 4.3.2.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | IsNotifyable |
| --- | --- | --- | --- | --- | --- |
| retTiltValue | O | M | M | - | M |
| bearing | O | M | M | - | M |
| retGroupName | O | M | M | - | M |
| height | O | M | M | - | M |
| maxAzimuthValue | O | M | M | - | M |
| minAzimuthValue | O | M | M | - | M |
| horizBeamwidth | O | M | M | - | M |
| vertBeamwidth | O | M | M | - | M |
| Attribute related to role |  |  |  |  |  |
| theCellList | CM | M | - | - | M |

#### 4.3.2.3	Attribute constraints

| Name | Definition |
| --- | --- |
| theCellList CM Support Qualifier | Condition: Association between SectorEquipmentFunction and ProxyCell is absent. |

#### 4.3.2.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.3	TMAFunction

#### 4.3.3.1	Definition

This IOC represents a Tower Mounted Amplifier or a number of TMA subunits within one TMA, each separately addressable by a specific index at the application layer.

This IOC is required as part of the capability to satisfy the Requirements statement identified below.

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 28.661 [10] | REQ-GRAN_NRM-CON-001 |  |
| 3GPP TS 28.661 [10] | REQ-GRAN_NRM-CON-002 |  |

#### 4.3.3.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| tmaSubunitNumber | M | M | M | - | M |
| tmaStateFlag | M | M | O | - | M |
| tmaFunctionFlag | M | M | M | - | M |
| tmaMinGain | M | M | - | - | M |
| tmaMaxGain | M | M | - | - | M |
| tmaResolution | M | M | - | - | M |
| tmaGainFigure | M | M | O | - | M |
| tmaNumberOfSubunits | M | M | - | - | M |
| tmaBaseStationId | CO | M | CO | - | M |
| tmaSectorId | CO | M | CO | - | M |
| tmaAntennaBearing | CO | M | CO | - | M |
| tmaInstalledMechanicalTilt | CO | M | CO | - | M |
| tmaSubunitType | CO | M | CO | - | M |
| tmaSubunitRxFrequencyBand | CO | M | CO | - | M |
| tmaSubunitTxFrequencyBand | CO | M | CO | - | M |
| tmaGainResolution | CO | M | CO | - | M |
| Attribute related to role |  |  |  |  |  |
| theCellList | CM | M | - | - | M |

- 4.3.3.3	Attribute Constraints

| Name | Definition |
| --- | --- |
| theCellList CM Support Qualifier | Condition: Association between SectorEquipmentFunction and ProxyCellList is absent. |

| Name | Definition |
| --- | --- |
| The CO support qualifier of the attributes tmaBaseStationId through tmaGainResolution | Condition: The TMA subunit supports the read operation in 3GPP TS 25.466 [9] |
| The CO write qualifier of the attributes tmaBaseStationId through tmaGainResolution | Condition: The TMA subunit supports the write operation in 3GPP TS 25.466 [9] |

#### 4.3.3.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.4	GSMCellPart

#### 4.3.4.1	Definition

A GSM cell can consist of a number of carriers. These carriers can be configured in a number of ways, for example, the carriers can have different propagation properties which are sent with different antenna tilt, with different RF power, different radio band and even possibly different antenna.

The various GSMCellPart instances capture different radio propagation properties allowing different frequency planning schemes, e.g. some GSMCellPart instances can use frequency groups planned for tighter frequency reuse.

Hence, a GSM cell can, and in some cases must, be distributed on more than one SectorEquipmentFunction.

This IOC is required as part of the capability to satisfy the Requirements statement identified below.

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 28.661 [10] | REQ-GRAN_NRM-CON-001 |  |
| 3GPP TS 28.661 [10] | REQ-GRAN_NRM-CON-002 |  |

#### 4.3.4.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | IsNotifyable |
| --- | --- | --- | --- | --- | --- |
| aRFCN | M | M | M | - | M |
| tsc | M | M | M | - | M |
| aTA | M | M | M | - | M |
| theSectorEquipment | M | M | - | - | M |

#### 4.3.4.3	Attribute constraints

None

#### 4.3.4.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.5	CommonBsFunction

#### 4.3.5.1	Definition

This IOC represents common aspects of Base Station (BS) functionality shared by several radio access technologies.

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 28.661 [10] | REQ-GRAN_NRM-CON-001 |  |
| 3GPP TS 28.661 [10] | REQ-GRAN_NRM-CON-002 |  |

#### 4.3.5.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sharedTechnologies | M | M | O | - | M |
| Attribute related to role |  |  |  |  |  |
| theProxyBsList | M | M | - | - | M |

#### 4.3.5.3	Attribute constraints

None

#### 4.3.5.4	Notifications

There is no notification defined.

### 4.3.6	CellReferences

#### 4.3.6.1	Definition

This IOC represents the three references to TMAFunction, SectorEquipmentFunction and AntennaFunction. The references are used by various classes of cells, e.g. UTRANGenericCell.

This is an abstract class.

#### 4.3.6.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attribute related to role |  |  |  |  |  |
| relatedSectorEquipment | CM | M | - | - | M |
| relatedTMAList | CM | M | - | - | M |
| relatedAntennaList | CM | M | - | - | M |

#### 4.3.6.3	Attribute constraints

| Name | Definition |
| --- | --- |
| relatedSectorEquipment CM Support Qualifier | Condition: Association between SectorEquipmentFunction and ProxyCellList is present AND is supporting the GERAN sharing case. In such case, there shall be at least one GSMCellPart present at one end of this association. |
| relatedAntennaList CM Support Qualifier | Condition: Association between SectorEquipmentFunction and ProxyCellList is absent. |
| relatedTMAList CM Support Qualifier | Condition: Association between SectorEquipmentFunction and ProxyCellList is absent. |

#### 4.3.6.4	Notifications

There is no notification defined.

### 4.3.7	RepeaterFunction

#### 4.3.7.1	Definition

This IOC represents the management aspect of a repeater. For the information on repeater see 3GPP TS 25.106 [18].

This IOC is required as part of the capability to satisfy the Requirements statement identified below.

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 28.661 [10] | REQ-GRAN_NRM-CON-003 |  |

#### 4.3.7.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| priority | M | M | M | - | M |
| latitude | M | M | - | - | - |
| longitude | M | M | - | - | - |
| ctrlConnMode | M | M | M | - | M |
| environmentInfo | M | M | - | - | - |
| powerSwitch | M | M | M | - | M |
| ulAttenuation | M | M | M | - | M |
| dlAttenuation | M | M | M | - | M |
| firmwareVer | M | M | - | - | - |
| repeaterType | M | M | - | - | - |
| Attribute related to role |  |  |  |  |  |
| externalUTRANCell | M | M | - | - | M |

#### 4.3.7.3	Attribute constraints

None.

#### 4.3.7.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

## 4.4	Attribute definitions

### 4.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| aRFCN | This attribute (Absolute Radio Frequency Channel Number) defines a pair of Radio Frequency (RF) channel frequencies for uplink and downlink use. See 3GPP TS 45.005 [18] clause 2 for the ARFCN for GSM. ARFCN are based on a 200 kHz channel raster. allowedValues: See 3GPP TS 45.005 [18] clause 2 | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| aTA | This attribute (allowed Timing Advance) defines the signal sent by the BTS to the MS which the MS uses to advance its timings of transmissions to the BTS so as to compensate for propagation delay. allowedValues: See 3GPP TS 45.010 [19] | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| bearing | The bearing in degrees that the antenna is pointing in. Antenna bearing" in Ref. 3GPP TS 25.463 [8]. allowedValues: See "Antenna bearing" in TS 25.463 [8]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| confOutputPower | It defines the allowed total power to use for all cells together in this sector. It may be set by the operator and/or limited by HW limitation or licensed power, e.g.: 20, 40, 60, 80,120 watts allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| ctrlConnMode | Remote communication mode used by a repeater to send and receive control message, such as GSM SMS, WCDMA SMS, Circle Switch Data-CSD, Package Switch Dat-IP, Serial port. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| dlAttenuation | Downlink signal attenuation of the device to change downlink gain. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| environmentInfo | The repeater device is located either in the building or out of the building. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| eUTRANFqBands | This is the list of LTE frequency bands supported by the hardware associated with the SectorEquipmentFunction. The earfcnDl and earfcnUl or earfcn of LTE cells associated with the SectorEquipmentFunction must be assigned with value within one of the specified eUTRANFqBands values. allowedValues: A list of frequency bands expressed as strings. Valid frequency band values are specified in sub-clause 5.7.3 in 36.104 [7]. For HW not supporting LTE frequency bands, the list shall be empty. | type: String multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: True |
| firmwareVer | Version of the device firmware. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| fqBand | This is the LTE frequency band supported by the hardware associated with the SectorEquipmentFunction. The earfcnDl and earfcnUl of cells associated with the SectorEquipmentFunction must be assigned with value within this fqBand value. allowedValues: See section 5 Table 5.2-1 “E-UTRA frequency band” of TS 36.104 [7]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| height | The height of an antenna above sea level. Note: The value of this attribute has no operational impact on the network, e.g. the NE behavior is not affected by the value setting of this attribute. Note as well that this attribute is not supported over the Iuant interface according to Ref. 3GPP TS 25.466 [9]. An integral value representing a number of meters in 0.1 meter increments. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| horizBeamwidth | The 3 dB power beamwidth of the antenna pattern in the horizontal plane. A value of 360 indicates an omni-directional antenna. Note: The value of this attribute has no operational impact on the network, e.g. the NE behaviour is not affected by the value setting of this attribute. Note as well that this attribute is not supported over the Iuant interface according to Ref. 3GPP TS 25.466 [9]. A single integral value corresponding to an angle in degrees between 0 and 360. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
|  |  | type: DN multiplicity: 1..* isOrdered: N/A isUnique: T defaultValue: None isNullable: True |
| latitude | The latitude of the antenna location based on World Geodetic System (1984 version) global reference frame (WGS 84). Positive values correspond to the northern hemisphere. allowedValues: -90.0000 to +90.0000 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| longitude | The longitude of the antenna location based on World Geodetic System (1984 version) global reference frame (WGS 84). Positive values correspond to degrees east of 0 degrees longitude. allowedValues: -180.0000 to +180.0000 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| maxAzimuthValue | The maximum amount of change of azimuth the RET system can support. This is the change in degrees clockwise from bearing. Note: The value of this attribute has no operational impact on the network, e.g. the NE behaviour is not affected by the value setting of this attribute. Note as well that this attribute is not supported over the Iuant interface according to Ref. 3GPP TS 25.466 [9]. A single integral value corresponding to an angle in degrees between 0 and 360 with a resolution of 0.1 degrees. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| minAzimuthValue | The minimum amount of change of azimuth the RET system can support. This is the change in degrees counter-clockwise from bearing. Note: The value of this attribute has no operational impact on the network, e.g. the NE behaviour is not affected by the value setting of this attribute. Note as well that this attribute is not supported over the Iuant interface according to Ref. 3GPP TS 25.466 [9]. A single integral value corresponding to an angle in degrees between 0 and 360 with a resolution of 0.1 degrees. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| priority | The priority of a repeater decided by an operator. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| powerSwitch | Power switch of device which has two status: ON/OFF. allowedValues: ON, OFF | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| relatedAntennaList | This attribute contains the DNs of one or more AntennaFunction. allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: N/A isUnique: T defaultValue: None isNullable: True |
| relatedSectorEquipment | This attribute contains the DN of one SectorEquipmentFunction. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| relatedTMAList | This attribute contains the DNs of one or more TmaFunction. allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: N/A isUnique: T defaultValue: None isNullable: True |
| repeaterType | The repeater type defined by operator, such as wide band, frequency selective, indoor and fiber optic. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| retGroupName | The group name is a textual, alpha-numeric string to define a logical grouping of antennas which may be in different cells. This attribute permits the definition of a logical grouping of the antennas. This may be defined either at installation time, or by management activity to provisioning the group name via the Itf-N. allowedValues: N/A (String size is bounded to 80 characters.) | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| retTiltValue | The electrical tilt setting of the antenna, "Tilt value" in Ref. 3GPP TS 25.466 [9]. allowedValues: See "Tilt value" in Ref. 3GPP TS 25.466 [9]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| sharedTechnologies | This attribute defines the radio access technologies sharing the common functionalities of a Base Station (BS). allowedValues: GSM, UMTS, LTE, or any combination thereof | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| tmaAntennaBearing | A data field defined in Table B.3 of 3GPP TS 25.466 [9]. See definition in TS 25.466 [9]. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| tmaBaseStationId | A data field defined in Table B.3 of 3GPP TS 25.466 [9] allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| tmaFunctionFlag | Defined in 3GPP TS 25.466 [9] allowedValues: N/A | type: Integer multiplicity: isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| tmaGainFigure | Defined in 3GPP TS 25.466 [9] allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| tmaGainResolution | A data field defined in Table B.3 of 3GPP TS 25.466 [9] allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| tmaInstalledMechanicalTilt | A data field defined in Table B.3 of 3GPP TS 25.466 [9] allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| tmaMaxGain | Defined in 3GPP TS 25.466 [9] allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| tmaMinGain | Defined in 3GPP TS 25.466 [9] allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| tmaNumberOfSubunits | Defined in 3GPP TS 25.466 [9] allowedValues: -- | Defined in TS 25.466 [9] type: -- multiplicity: -- isOrdered: -- isUnique: -- defaultValue: -- isNullable: -- |
| tmaResolution | Defined in 3GPP TS 25.466 [9] allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| tmaSectorId | A data field defined in Table B.3 of 3GPP TS 25.466 [9] allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| tmaStateFlag | Defined in 3GPP TS 25.466 [9] allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| tmaSubunitNumber | Defined in 3GPP TS 25.466 [9] allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| tmaSubunitRxFrequencyBand | A data field defined in Table B.3 of 3GPP TS 25.466 [9] allowedValues: See TS 25.466 [9]. | type: Integer multiplicity: 2 isOrdered: True isUnique: True defaultValue: None isNullable: False |
| tmaSubunitType | A data field defined in Table B.3 of 3GPP TS 25.466 [9] allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| tmaSubunitTxFrequencyBand | A data field defined in Table B.3 of 3GPP TS 25.466 [9] allowedValues: See TS 25.466 [9]. | type: Integer multiplicity: 2 isOrdered: True isUnique: True defaultValue: None isNullable: False |
| tsc | This attribute has the same definition as the one used in GsmCell IOC. The presence of GSMCellPart means the tsc attribute in GsmCell IOC instance is irrelevant (not applicable). allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| ulAttenuation | Uplink signal attenuation of the device to change uplink gain. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| uTRANFDDFqBands | This is the list of UTRAN FDD frequency bands supported by the hardware associated with the SectorEquipmentFunction. The arfcnDl and arfcnUl of UTRAN FDD cells associated with the SectorEquipmentFunction must be assigned with value within one of the specified uTRANFDDFqBands values. allowedValues: A list of frequency bands expressed as strings. Valid frequency band values are specified in sub-clause 5.2 of TS 25.104 [20]. | type: String multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: True |
| uTRANTDDFqBands | This is the list of UTRAN TDD frequency bands supported by the hardware associated with the SectorEquipmentFunction. The earfcn of UTRAN TDD cells associated with the SectorEquipmentFunction must be assigned with value within one of the specified uTRANTDDFqBands values. allowedValues: A list of frequency bands expressed as strings. Valid frequency band values are specified in sub-clause 5.2 of TS 25.105 [21]. | type: String multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: True |
| vertBeamwidth | The 3 dB power beamwidth of the antenna pattern in the vertical plane. The value of this attribute has no operational impact on the network, e.g. the NE behaviour is not affected by the value setting of this attribute. This attribute is not supported over the Iuant interface according to Ref. 3GPP TS 25.466 [9]. allowedValues: A single integral value corresponding to an angle in degrees between 0 and 180. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| Attribute related to role |  |  |
| externalUTRANCell | This role (when present) represents repeaterFunction capability to identify one ExternalUtranCell. When present, it shall contain one ExternalUtranCell DN. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True passedById: True |
| theAntennaList | This attribute contains the DNs of one or more AntennaFunction. allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: TruepassedById: True |
| theCellList | This attribute contains the DNs of EUtranGenericCell or UtranGenericCell if association between SectorEquipmentFunction and ProxyCellList, parent of EUtranGenericCell or UtranGenericCell is used. This attribute contains the DNs of GSMCellPart if association between SectorEquipmentFunction and ProxyCellList, parent of GSMCellPartis used. allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: TruepassedById: True |
| theProxyBsList | A CommonBsFunction instance serves a number of ProxyBsFunction instances. This CommonBsFunction role-attribute contains a list of DNs of ENBFunction (TS 28.658 [13]), NodeBFunction (TS 28.652 [12]) and BssFunction (TS 28.655 [14]) that it serves. allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: TruepassedById: True |
| theTMAList | This attribute contains the DNs of one or more TMAFunction. allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: TruepassedById: True |

### 4.4.2	Constraints

None

## 4.5	Common Notifications

### 4.5.1	Alarm notifications

This subclause presents a list of notifications, defined in 3GPP TS 32.111-2 [11], that IRPManager can receive. The notification header attribute objectClass/objectInstance, defined in 3GPP TS 32.302 [16], would capture the DN of an instance of an IOC defined in this IRP specification.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAckStateChanged | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyChangedAlarm | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyClearedAlarm | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyNewAlarm | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyComments | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyAlarmListRebuilt | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyPotentialFaultyAlarmList | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |

### 4.5.2	Configuration notifications

This subclause presents a list of notifications, defined in 3GPP TS32.662 [17], that IRPManager can receive. The notification header attribute objectClass/objectInstance, defined in 3GPP TS 32.302 [16], would capture the DN of an instance of an IOC defined in this IRP specification.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAttributeValueChange | O |  |
| notifyObjectCreation | O |  |
| notifyObjectDeletion | O |  |

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2013-09 | SA#61 | SP-130433 | 0001 | - | F | Add missing Repeater Object IS definitions | 11.1.0 |
| 2014-06 | SA#64 | SP-140359 | 0002 | - | F | remove the feature support statements | 11.2.0 |
| 2014-10 | - | - | - | - |  | Update to Rel-12 version (MCC) | 12.0.0 |
| 2016-06 | SA#72 | SP-160408 | 0004 | 1 | A | Correcting references and reintroducing attributes | 12.1.0 |