3GPP TS 32.792 V10.2.0 (2016-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Generic Radio Access Network (RAN)

Network Resource Model (NRM) Integration Reference Point (IRP);

Information Service (IS)

(Release 10)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

NRM, IRP, RAN

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2016, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions and abbreviations	7

3.1	Definitions	7

3.2	Abbreviations	7

4	Information Object Classes	7

4.1	Imported information entities and local labels	7

4.2	Class diagram	8

4.2.1	Attributes and relationships	8

4.2.2	Inheritance	9

4.3	Information Object Class (IOC) definitions	10

4.3.1	SectorEquipmentFunction	10

4.3.1.1	Definition	10

4.3.1.2	Attributes	10

4.3.1.3	Attribute constraints	10

4.3.1.4	Notifications	10

4.3.2	AntennaFunction	10

4.3.2.1	Definition	10

4.3.2.2	Attributes	11

4.3.2.3	Attribute constraints	11

4.3.2.4	Notifications	11

4.3.3	TmaFunction	11

4.3.3.1	Definition	11

4.3.3.2	Attributes	12

4.3.3.3	Attribute Constraints	12

4.3.3.4	Notifications	12

4.3.4	GSMCellPart	12

4.3.4.1	Definition	12

4.3.4.2	Attributes	13

4.3.4.3	Attribute constraints	13

4.3.4.4	Notifications	13

4.3.5	CommonBsFunction	13

4.3.5.1	Definition	13

4.3.5.2	Attributes	13

4.4	Information relationship definitions	13

4.4.1	A1 (CO)	13

4.4.1.1	Definition	13

4.4.1.2	Roles	13

4.4.1.3	Constraints	13

4.4.2	A2 (CM)	14

4.4.2.1	Definition	14

4.4.2.2	Roles	14

4.4.2.3	Constraints	14

4.4.3	A3 (CO)	14

4.4.3.1	Definition	14

4.4.3.2	Roles	14

4.4.3.3	Constraints	14

4.4.4	A4 (CM)	14

4.4.4.1	Definition	14

4.4.4.2	Roles	14

4.4.4.3	Constraints	14

4.4.5	A5 (CM)	15

4.4.5.1	Definition	15

4.4.5.2	Roles	15

4.4.5.3	Constraints	15

4.4.6	A6 (CM)	15

4.4.6.1	Definition	15

4.4.6.2	Roles	15

4.4.6.3	Constraints	15

4.4.7	A7 (M)	16

4.4.7.1	Definition	16

4.4.7.2	Roles	16

4.4.7.3	Constraints	16

4.5	Information attribute definitions	17

4.5.1	Definition and Legal Values	17

4.6	Common Notifications	18

Annex A (informative):	Change history	19

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management; as identified below:

32.791:	Common Radio Access Technology (RAT) Network Resource Model (NRM) Integration Reference Point (IRP): Requirements.

32.792:	Generic Radio Access Network (RAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS).

32.796:	Generic Radio Access Network (RAN) Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions.

# 1	Scope

The present document specifies the Generic Radio Access Network (RAN) network resource information that can be communicated between an IRPAgent and one or several IRPManagers for network management purposes.

This document specifies the semantics and behaviour of information object class attributes and relations visible across the reference point in a protocol and technology neutral way.  It does not define their syntax and encoding.

This document specifies equipment that may be shared between BSS in GSM, UTRAN and E-UTRAN.

In order to access the information defined by this NRM, an Interface IRP such as the "Basic CM IRP" is needed (3GPP TS 32.602 [5]). However, which Interface IRP is applicable is outside the scope of the present document.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[3]	3GPP TS 32.102: "Telecommunication management; Architecture".

[4]	3GPP TS 32.150: "Telecommunication management; Integration Reference Point (IRP) Concept and definitions".

[5]	3GPP TS 32.602: "Telecommunication management; Configuration Management (CM); Basic CM Integration Reference Point (IRP) ; Information Service (IS)".

[6]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[7]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E_UTRA); Base Station (BS) radio transmission and reception".

[8]	Void .

[9]	3GPP TS 25.466: "UTRAN Iuant interface: Application Part".

[10]	3GPP TS 32.791: "Telecommunication management; Common Radio Access Technology (RAT)  Network Resource Model (NRM) Integration Reference Point (IRP); Requirements".

[11]	3GPP TS 32.111-2: "Telecommunication management; Fault Management; Part 2: Alarm Integration Reference Point (IRP): Information Service (IS)".

[12]	3GPP TS 32.642 "Telecommunication management; Configuration Management (CM); UTRAN network resources Integration Reference Point (IRP); Network Resource Model ( NR)".M

[13]	3GPP TS 32.762 "Telecommunication management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[14]	3GPP TS 32.652 "Telecommunication management; Configuration Management (CM); GERAN network resources Integration Reference Point (IRP); Network Resource Model (NRM)".

[15]	3GPP TS 32.622: "Telecommunication management; Configuration Management (CM); Generic network resources Integration Reference Point (IRP) ; Network Resource Model (NRM)".

[16]	3GPP TS 25.104: "Base Station (BS) radio transmission and reception (FDD)".

[17]	3GPP TS 25.105: "Base Station (BS) radio transmission and reception (TDD)".

[18]	3GPP TS 45.005: "Radio transmission and reception".

[19]	3GPP TS 45.010: "Radio subsystem synchronization".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], TS 32.150 [4], TS 32.101 [2], TS 32.102 [3] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TS 32.150 [4], TS 32.101 [2], TS 32.102 [3] and TR 21.905 [1], in that order.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BS	Base Station

BSS	Base Station Subsystem

CM	Configuration Management

DN	Distinguished Name

E-UTRAN	Evolved UTRAN

GSM	Global System for Mobile communications

HW	Hardware

IRP	Integration Reference Point

IOC	Information Object Class

IS	Information Service

NE	Network Element

NRM	Network Resource Model

RAN	Radio Access Network

RDN	Relative Distinguished Name

RF	Radio Frequency

SS	Solution Set

TMA	Tower Mounted Amplifier

UTRA	Universal Terrestrial Radio Access

UTRAN	Universal Terrestrial Radio Access Network

# 4	Information Object Classes

## 4.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| 3GPP TS 32.622 [15], IOC, ManagedFunction | ManagedFunction |
| 3GPP TS 32.642 [12], IOC, UtranGenericCell | UtranGenericCell |
| 3GPP TS 32.762 [13], IOC, EUtranGenericCell | EUtranGenericCell |
| 3GPP TS 32.652 [14], IOC, GSMCell | GSMCell |

## 4.2	Class diagram

### 4.2.1	Attributes and relationships

Figure4.2.1.1: UTRAN and E-UTRAN sharing

Figure 4.2.1.2: GERAN sharing

Figure 4.2.1.3: CommonBsFunction

### 4.2.2	Inheritance

This clause depicts the IOCs’ inheritance relationships.

Figure 4.2.2.1: CommonBsFunction

## 4.3	Information Object Class (IOC) definitions

### 4.3.1	SectorEquipmentFunction

#### 4.3.1.1	Definition

This IOC represents a set of cells within a geographical area that has common functions relating to AntennaFunction, TmaFunction and supporting equipment, such as power amplifier.

This IOC is required as part of the capability to satisfy the Requirements statement identified below.

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.791 [10] | REQ-GRAN_NRM-CON-001 |  |
| 3GPP TS 32.791 [10] | REQ-GRAN_NRM- CON-002 |  |

#### 4.3.1.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| fqBand | CM | M | -- |
| eUTRANFqBands | M | M | - |
| uTRANFDDFqBands | M | M | - |
| uTRANTDDFqBands | M | M | - |
| confOutputPower | M | M | M |
| theTmaList | M | M | - |
| theAntennaList | M | M | - |
| theCellList | M | M | - |

#### 4.3.1.3	Attribute constraints

| Name | Qualifier | Notes |
| --- | --- | --- |
| The Conditional/Mandatory (CM) support qualifier of the attribute fqBand | CM | This attribute is used if the attribute eUTRANFqBands contains an empty list. |

#### 4.3.1.4	Notifications

The common notifications defined in subclause 4.6 are valid for this IOC, without exceptions or additions.

### 4.3.2	AntennaFunction

#### 4.3.2.1	Definition

This IOC represents an array of radiating elements that may be tilted to adjust the RF coverage of a cell(s).

This IOC is required as part of the capability to satisfy the Requirements statement identified below.

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.791 [10] | REQ-GRAN_NRM-CON-001 |  |
| 3GPP TS 32.791 [10] | REQ-GRAN_NRM-CON-002 |  |

#### 4.3.2.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| retTiltValue | O | M | M |
| bearing | O | M | M |
| retGroupName | O | M | M |
| height | O | M | M |
| maxAzimuthValue | O | M | M |
| minAzimuthValue | O | M | M |
| horizBeamwidth | O | M | M |
| vertBeamwidth | O | M | M |
| theCellList | M | M | - |

#### 4.3.2.3	Attribute constraints

None.

#### 4.3.2.4	Notifications

The common notifications defined in subclause 4.6 are valid for this IOC, without exceptions or additions.

### 4.3.3	TmaFunction

#### 4.3.3.1	Definition

This IOC represents a Tower Mounted Amplifier or a number of TMA subunits within one TMA, each separately addressable by a specific index at the application layer.

This IOC is required as part of the capability to satisfy the Requirements statement identified below.

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.791 [10] | REQ-GRAN_NRM-CON-001 |  |
| 3GPP TS 32.791 [10] | REQ-GRAN_NRM-CON-002 |  |

#### 4.3.3.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| tmaSubunitNumber | M | M | M |
| tmaStateFlag | M | M | O |
| tmaFunctionFlag | M | M | M |
| tmaMinGain | M | M | - |
| tmaMaxGain | M | M | - |
| tmaResolution | M | M | - |
| tmaGainFigure | M | M | O |
| tmaNumberOfSubunits | M | M | - |
| tmaBaseStationId | CO | M | CO |
| tmaSectorId | CO | M | CO |
| tmaAntennaBearing | CO | M | CO |
| tmaInstalledMechanicalTilt | CO | M | CO |
| tmaSubunitType | CO | M | CO |
| tmaSubunitRxFrequencyBand | CO | M | CO |
| tmaSubunitTxFrequencyBand | CO | M | CO |
| tmaGainResolution | CO | M | CO |
| theCellList | M | M | - |

#### 4.3.3.3	Attribute Constraints

| Name | Qualifier | Notes |
| --- | --- | --- |
| The Conditional/Optional (CO) support qualifier of the attributes tmaAdditionalDataFieldNumber through tmaGainResolution | CO | The TMA subunit supports the read operation in 3GPP TS 25.466 [9] |
| The conditional/optional (CO) write qualifier of the attributes tmaAdditionalDataFieldNumber through tmaGainResolution | CO | The TMA subunit supports the write operation in 3GPP TS 25.466 [9] |

#### 4.3.3.4	Notifications

The common notifications defined in subclause 4.6 are valid for this IOC, without exceptions or additions.

### 4.3.4	GSMCellPart

#### 4.3.4.1	Definition

A GSM cell can consist of a number of carriers. These carriers can be configured in a number of ways, for example, the carriers can have different propagation properties which are sent with different antenna tilt, with different RF power, different radio band and even possibly different antenna.

The various GSMCellPart instances capture different radio propagation properties allowing different frequency planning schemes, e.g. some GSMCellPart instances can use frequency groups planned for tighter frequency reuse.

Hence, a GSM cell can, and in some cases must, be distributed on more than one SectorEquipmentFunction.

This IOC is required as part of the capability to satisfy the Requirements statement identified below.

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.791 [10] | REQ-GRAN_NRM-CON-01 |  |
| 3GPP TS 32.791 [10] | REQ-GRAN_NRM-CON-02 |  |

#### 4.3.4.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| aRFCN | M | M | M |
| tsc | M | M | M |
| aTA | M | M | M |
| theSectorEquipment | M | M | - |

#### 4.3.4.3	Attribute constraints

None.

#### 4.3.4.4	Notifications

The common notifications defined in subclause 4.6 are valid for this IOC, without exceptions or additions.

### 4.3.5	CommonBsFunction

#### 4.3.5.1	Definition

This IOC represents common aspects of Base Station (BS) functionality shared by several radio access technologies.

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.791 [10] | REQ-GRAN_NRM-CON-001 |  |
| 3GPP TS 32.791 [10] | REQ-GRAN_NRM- CON-002 |  |

#### 4.3.5.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| sharedTechnologies | M | M | O |

## 4.4	Information relationship definitions

### 4.4.1	A1 (CO)

#### 4.4.1.1	Definition

This association represents the bidirectional relation between TmaFunction and ProxyCell.

#### 4.4.1.2	Roles

| Name | Definition |
| --- | --- |
| theCellList | This role represents the associated ProxyCell instances of a TmaFunction instance. |
| theTmaList | This role represents the associated TmaFunction instances of a ProxyCell instance. |

#### 4.4.1.3	Constraints

Condition: Association A2 is absent.

### 4.4.2	A2 (CM)

#### 4.4.2.1	Definition

This association represents the bidirectional relation between SectorEquipmentFunction and ProxyCell used in UTRAN and E-UTRAN sharing (and non-sharing) cases.

#### 4.4.2.2	Roles

| Name | Definition |
| --- | --- |
| theCellList | This role represents the associated ProxyCell instances of a SectorEquipmentFunction instance. |
| theSectorEquipment | This role represents the associated SectorEquipmentFunction instance of a ProxyCell instance. |

#### 4.4.2.3	Constraints

Condition: SectorEquipmentFunction instance is present and supporting UTRAN and E-UTRAN sharing (and non-sharing) cases.  In these cases, at least one instance represented by the associated ProxyCell must be present.

### 4.4.3	A3 (CO)

#### 4.4.3.1	Definition

This association represents the bidirectional relation between AntennaFunction and ProxyCell.

#### 4.4.3.2	Roles

| Name | Definition |
| --- | --- |
| theCellList | This role represents the associated ProxyCell instances of an AntennaFunction instance. |
| theAntennaList | This role represents the associated AntennaFunction instances of a ProxyCell instance. |

#### 4.4.3.3	Constraints

Condition: Association A2 is absent.

### 4.4.4	A4 (CM)

#### 4.4.4.1	Definition

This association represents the unidirectional relation from SectorEquipmentFunction to TmaFunction.

#### 4.4.4.2	Roles

| Name | Definition |
| --- | --- |
| theTmaList | This role represents the associated TmaFunction instances of a SectorEquipmentFunction instance. |

#### 4.4.4.3	Constraints

Condition: SectorEquipmentFunction instance is present AND is supporting the UTRAN and E-UTRAN sharing (and non-sharing) cases AND A5 is absent.  In this case, at least one TmaFunction is present.

.

### 4.4.5	A5 (CM)

#### 4.4.5.1	Definition

This association represents the unidirectional relation from SectorEquipmentFunction to AntennaFunction.

#### 4.4.5.2	Roles

| Name | Definition |
| --- | --- |
| theAntennaList | This role represents the associated AntennaFunction instances of a SectorEquipmentFunction instance. |

#### 4.4.5.3	Constraints

Condition: SectorEquipmentFunction instance is present AND is supporting the UTRAN and E-UTRAN sharing (and non-sharing) cases AND A4 is absent.  In this case, at least one AntennaFunction is present.

### 4.4.6	A6 (CM)

#### 4.4.6.1	Definition

This association represents the bidirectional relation between SectorEquipmentFunction and GSMCellPart.

#### 4.4.6.2	Roles

| Name | Definition |
| --- | --- |
| theCellPartList | This role represents the associated GSMCellPart instances of a SectorEquipmentFunction instance. |
| theSectorEquipment | This role represents the associated SectorEquipmentFunction instance of a GSMCellPart instance. |

#### 4.4.6.3	Constraints

Condition: SectorEquipmentFunction instance is present and is supporting the GERAN sharing case.  In this case, there shall be at least one GSMCellPart present at one end of this association.

### 4.4.7	A7 (M)

#### 4.4.7.1	Definition

This association represents the association between a ProxyBsFunction and its related CommonBsFunction, as well as between a SCommonBsFunction and the ProxyBsFunctions it serves.

#### 4.4.7.2	Roles

| Name | Definition |
| --- | --- |
| theCommonBs | This role represents the CommonBsFunction that is associated with a ProxyBsFunction. |
| theProxyBsList | This role represents the ProxyBsFunctions that are associated with a CommonBsFunction. |

#### 4.4.7.3	Constraints

| Name | Definition |
| --- | --- |
| ProxyBsCommonBsConstraint | The ProxyBsFunction has an association with a CommonBsFunction. |

## 4.5	Information attribute definitions

### 4.5.1	Definition and legal values

| Attribute Name | Definition | Legal Values |
| --- | --- | --- |
| aRFCN | This attribute (Absolute Radio Frequency Channel Number) defines a pair of Radio Frequency (RF) channel frequencies for uplink and downlink use. | See 3GPP TS 45.005 [18] clause 2 for the ARFCN for GSM. ARFCN are based on a 200 kHz channel raster. |
| aTA | This attribute (allowed Timing Advance) defines the signal sent by the BTS to the MS which the MS uses to advance its timings of transmissions to the BTS so as to compensate for propagation delay. | See 3GPP TS 45.010[19] |
| bearing | The bearing in degrees that the antenna is pointing in. Antenna bearing" in Ref. 3GPP TS 25.466 [9]. | See "Antenna bearing" in3GPP TS 25.466 [9]. |
| confOutputPower | It defines the allowed total power to use for all cells together in this sector. It may be set by the operator and/or limited by HW limitation or licensed power, e.g.: 20, 40, 60, 80,120 watts |  |
| eUTRANFqBands | This is the list of LTE frequency bands supported by the hardware associated with the SectorEquipmentFunction. The earfcnDl and earfcnUl or earfcn of LTE cells associated with the SectorEquipmentFunction must be assigned with value within one of the specified eUTRANFqBands values. | A list of frequency bands expressed as strings. Valid frequency band values are specified in sub-clause 5.7.3 in 36.104 [7]. For HW not supporting LTE frequency bands, the list shall be empty. |
| fqBand | This is the LTE frequency band supported by the hardware associated with the SectorEquipmentFunction, for the case when only one frequency band is supported. The earfcnDl and earfcnUl of cells associated with the SectorEquipmentFunction must be assigned with value within this fqBand value. | See clause 5 .7.3 of TS 36.104 [7]. |
| height | The height of an antenna above sea level. Note: The value of this attribute has no operational impact on the network, e.g. the NE behavior is not affected by the value setting of this attribute. Note as well that this attribute is not supported over the Iuant interface according to Ref. 3GPP TS 25.466 [9]. | An integral value representing a number of metres in 0.1 meter increments. |
| horizBeamwidth | The 3 dB power beamwidth of the antenna pattern in the horizontal plane. A value of 360 indicates an omni-directional antenna. Note: The value of this attribute has no operational impact on the network, e.g. the NE behaviour is not affected by the value setting of this attribute. Note as well that this attribute is not supported over the Iuant interface according to Ref. 3GPP TS 25.466 [9]. | A single integral value corresponding to an angle in degrees between 0 and 360. |
| id | An attribute whose "name+value" can be used as an RDN when naming an instance of the object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance |  |
| maxAzimuthValue | The maximum amount of change of azimuth the RET system can support. This is the change in degrees clockwise from bearing. Note: The value of this attribute has no operational impact on the network, e.g. the NE behaviour is not affected by the value setting of this attribute. Note as well that this attribute is not supported over the Iuant interface according to Ref. 3GPP TS 25.466 [9]. | A single integral value corresponding to an angle in degrees between 0 and 360 with a resolution of 0.1 degrees, see Note. |
| minAzimuthValue | The minimum amount of change of azimuth the RET system can support. This is the change in degrees counter-clockwise from bearing. Note: The value of this attribute has no operational impact on the network, e.g. the NE behaviour is not affected by the value setting of this attribute. Note as well that this attribute is not supported over the Iuant interface according to Ref. 3GPP TS 25.466 [9]. | A single integral value corresponding to an angle in degrees between 0 and 360 with a resolution of 0.1 degrees. |
| retGroupName | The group name is a textual, alpha-numeric string to define a logical grouping of antennas which may be in different cells. This attribute permits the definition of a logical grouping of the antennas. This may be defined either at installation time, or by management activity to provisioning the group name via the Itf-N. | String size is bounded to 80 characters. |
| retTiltValue | The electrical tilt setting of the antenna, "Tilt value" in Ref. 3GPP TS 25.466 [9]. | See "Tilt value" in Ref. 3GPP TS 25.466 [9]. |
| sharedTechnologies | This attribute defines the radio access technologies sharing the common functionalities of a Base Station (BS) | Legal Values: GSM, UMTS, LTE, or any combination thereof |
| theAntennaList | This attribute contains the DNs of one or more AntennaFunction |  |
| theCellList | This attribute contains the DNs of cells (derivates of EUtranGenericCell or UtranGenericCell) if association A2 is used. This attribute contains the DNs of GSMCellPart if association A6 is used.. |  |
| theSectorEquipment | This attribute contains the DN of one SectorEquipmentFunction. |  |
| theTmaList | This attribute contains the DNs of one or more TmaFunction | A list of DNs as defined in TS 32.300 [6]. |
| tmaAntennaBearing | A data field defined in Table B.3 of 3GPP TS 25.466 [9]. | Defined in TS 25.466 [9] |
| tmaBaseStationId | A data field defined in Table B.3 of 3GPP TS 25.466 [9] | Defined in TS 25.466 [9] |
| tmaFunctionFlag | Defined in 3GPP TS 25.466 [9] | Defined in TS 25.466 [9] |
| tmaGainFigure | Defined in 3GPP TS 25.466 [9] | Defined in TS 25.466 [9] |
| tmaGainResolution | A data field defined in Table B.3 of 3GPP TS 25.466 [9]. | Defined in TS 25.466 [9] |
| tmaInstalledMechanicalTilt | A data field defined in Table B.3 of 3GPP TS 25.466 [9]. | Defined in TS 25.466 [9] |
| tmaMaxGain | Defined in 3GPP TS 25.466 [9] | Defined in TS 25.466 [9] |
| tmaMinGain | Defined in 3GPP TS 25.466 [9] | Defined in TS 25.466 [9] |
| tmaNumberOfSubunits | Defined in 3GPP TS 25.466 [9] | Defined in TS 25.466 [9] |
| tmaResolution | Defined in 3GPP TS 25.466 [9] | Defined in TS 25.466 [9] |
| tmaSectorId | A data field defined in Table B.3 of 3GPP TS 25.466 [9]. | Defined in TS 25.466 [9] |
| tmaStateFlag | Defined in 3GPP TS 25.466 [9] | Defined in TS 25.466 [9] |
| tmaSubunitNumber | Defined in 3GPP TS 25.466 [9] | Defined in TS 25.466 [9] |
| tmaSubunitRxFrequencyBand | A data field defined in Table B.3 of 3GPP TS 25.466 [9]. | Defined in TS 25.466 [9] |
| tmaSubunitType | A data field defined in Table B.3 of 3GPP TS 25.466 [9]. | Defined in TS 25.466 [9] |
| tmaSubunitTxFrequencyBand | A data field defined in Table B.3 of 3GPP TS 25.466 [9]. | Defined in TS 25.466 [9] |
| tsc | This attribute has the same definition as the one used in GsmCell IOC. The presence of GSMCellPart means the tsc attribute in GsmCell IOC instance is irrelevant (not applicable). |  |
| uTRANFDDFqBands | This is the list of UTRAN FDD frequency bands supported by the hardware associated with the SectorEquipmentFunction. The arfcnDl and arfcnUl of UTRAN FDD cells associated with the SectorEquipmentFunction must be assigned with value within one of the specified uTRANFDDFqBands values. | A list of frequency bands expressed as strings. Valid frequency band values are specified in sub-clause 5.2 of TS 25.104 [16]. |
| uTRANTDDFqBands | This is the list of UTRAN TDD frequency bands supported by the hardware associated with the SectorEquipmentFunction. The earfcn of UTRAN TDD cells associated with the SectorEquipmentFunction must be assigned with value within one of the specified uTRANTDDFqBands values. | A list of frequency bands expressed as strings. Valid frequency band values are specified in sub-clause 5.2 of TS 25.105 [17]. |
| vertBeamwidth | The 3 dB power beamwidth of the antenna pattern in the vertical plane. Note: The value of this attribute has no operational impact on the network, e.g. the NE behaviour is not affected by the value setting of this attribute. Note as well that this attribute is not supported over the Iuant interface according to Ref. 3GPP TS 25.466 [9]. | A single integral value corresponding to an angle in degrees between 0 and 180. |

## 4.6	Common Notifications

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAckStateChanged | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyChangedAlarm | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyClearedAlarm | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyNewAlarm | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyComments | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyAlarmListRebuilt | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyPotentialFaultyAlarmList | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyAttributeValueChange | O |  |
| notifyObjectCreation | O |  |
| notifyObjectDeletion | O |  |

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2011-03 | SP-51 | SP-110118 | -- | -- |  | Presentation to SA for Information | 1.0.0 |
| 2011-05 | SP-52 | SP-110273 | -- | -- |  | Presentation to SA for Approval | 2.0.0 |
| 2011-06 | SP-52 | --- | -- | -- |  | Publication | 10.0.0 |
| 2012-09 | SP-57 | SP-120563 | 001 | -- |  | Add/Correct support for multi frequency HW | 10.1.0 |
| 2016-06 | SA#72 | SP-160409 | 0002 | 1 | F | Correcting references | 10.2.0 |