3GPP TSG-SA5 Meeting #122	S5-187210

# Spokane,US, 12-16 November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0033 | rev | <Rev#> | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add PDCP data volume measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2018-11-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In 5GS, Cell PDCP data volume related measurements are useful measurements which represent the real data traffic for each cell. Monitoring the Cell PDCP SDU data volume could provide operators the load information and offload information in DC scenarios. It is helpful for operators to do cell congestion evaluation, load balancing and cell planning. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add PDCP data volume related measurements in TS 28.552. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The volume of Cell PDCP data can’t be monitored by the operators to do cell planning in 5GS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.3.x(new), A.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

5.1.3.X	PDCP data volume measurements

5.1.3.X.1 DL PDCP Data Volume

5.1.3.X.1.1	DL Cell PDCP SDU Data Volume

- This measurement provides the Data Volume (amount of PDCP SDU bits) in the downlink delivered to PDCP layer. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3).

The unit is Mbit.

b)	CC

c)	This measurement is obtained by counting the number of bits entering the NG-RAN PDCP layer. The measurement is performed at the PDCP SDU level. The measurement is performed per configured PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3).

d)	Each measurement is an integer value representing the number of bits measured in Mbits. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels.
[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 200.

e)	The measurement name has the form QosFlow.PdcpSduVolumeDl_Filter.

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

5.1.3.X.1.2	DL Cell PDCP PDU Data Volume

- This measurement provides the Data Volume (amount of PDCP PDU bits) in the downlink delivered from PDCP layer to RLC layer. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3). 
The unit is Mbit.

b)	CC

c)	This measurement is obtained by counting the number of bits entering the NG-RAN RLC layer. The measurement is performed at the PDCP PDU level. The measurement is performed per configured PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3).

d)	Each measurement is an integer value representing the number of bits measured in Mbits. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels.
[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 200.

e)	The measurement name has the form QosFlow.PdcpPduVolumeDL_Filter.

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

5.1.3.X.1.3	DL Cell PDCP PDU Data Volume in X2 Interface

- This measurement provides the Data Volume (amount of PDCP PDU bits) in the downlink delivered on X2 interface in NSA scenarios. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3). 
The unit is Mbit.

b)	CC

c)	This measurement is obtained by counting the number of bits entering the eNB RLC layers. The measurement is performed at the PDCP PDU level. The measurement is performed per configured PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3).

d)	Each measurement is an integer value representing the number of bits measured in Mbits. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels.
[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 200.

e)	The measurement name has the form QosFlow.PdcpPduVolumeX2DL_Filter.

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

5.1.3.X.1.4	DL Cell PDCP PDU Data Volume in Xn Interface

- This measurement provides the Data Volume (amount of PDCP PDU bits) in the downlink delivered on Xn interface in SA scenarios. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3). 
The unit is Mbit.

b)	CC

c)	This measurement is obtained by counting the number of bits entering the NG-RAN RLC layers. The measurement is performed at the PDCP PDU level. The measurement is performed per configured PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3).

d)	Each measurement is an integer value representing the number of bits measured in Mbits. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels.
[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 200.

e)	The measurement name has the form QosFlow.PdcpPduVolumeXnDL_Filter.

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

5.1.3.X.2 UL Cell PDCP PDU Data Volume

5.1.3.X.2.1	UL Cell PDCP SDU Data Volume

- This measurement provides the Data Volume (amount of PDCP SDU bits) in the uplink delivered from PDCP layer to SDAP layer or UPF. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3). 
The unit is Mbit.

b)	CC

c)	This measurement is obtained by counting the number of bits leaving the NG-RAN PDCP layer. The measurement is performed at the PDCP SDU level. The measurement is performed per configured PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3).

d)	Each measurement is an integer value representing the number of bits measured in Mbits. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels.
[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 200.

e)	The measurement name has the form QosFlow.PdcpSduVolumeUL_Filter.

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

5.1.3.X.2.2	UL Cell PDCP PDU Data Volume

- This measurement provides the Data Volume (amount of PDCP PDU bits) in the uplink delivered from RLC layer to PDCP layer. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3).

The unit is Mbit.

b)	CC

c)	This measurement is obtained by counting the number of bits entering the NG-RAN PDCP layer. The measurement is performed at the PDCP PDU level. The measurement is performed per configured PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3).

d)	Each measurement is an integer value representing the number of bits measured in Mbits. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels.
[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 200.

e)	The measurement name has the form QosFlow.PdcpPduVolumeUl_Filter.

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

5.1.3.X.2.3	UL Cell PDCP PDU Data Volume in X2 Interface

- This measurement provides the Data Volume (amount of PDCP PDU bits) in the uplink delivered on X2 interface in NSA scenarios. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3). 
The unit is Mbit.

b)	CC

c)	This measurement is obtained by counting the number of bits entering the NG-RAN PDCP layers through X2 interface. The measurement is performed at the PDCP PDU level. The measurement is performed per configured PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3).

d)	Each measurement is an integer value representing the number of bits measured in Mbits. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels.
[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 200.

e)	The measurement name has the form QosFlow.PdcpPduVolumeX2UL_Filter.

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

5.1.3.X.2.4	UL Cell PDCP PDU Data Volume in Xn Interface

- This measurement provides the Data Volume (amount of PDCP PDU bits) in the uplink delivered on Xn interface in SA scenarios. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3). 
The unit is Mbit.

b)	CC

c)	This measurement is obtained by counting the number of bits entering the NG-RAN PDCP layers through Xn interface. The measurement is performed at the PDCP PDU level. The measurement is performed per configured PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3).

d)	Each measurement is an integer value representing the number of bits measured in Mbits. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels.
[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 200.

e)	The measurement name has the form QosFlow.PdcpPduVolumeXnUL_Filter.

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

Second change

A.X	Monitoring of PDCP data volume measurements

In 5GS, Cell PDCP data volume is a useful measurement which represents the real data traffic for each cell. The monitor of the Cell PDCP data volume could provide operators the traffic information and is useful for operators to do cell congestion evaluation, load balancing and cell capacity planning.

In addition, in scenarios of dual connectivity, Cell PDCP data volume is useful for optimization of data offload.

End of changes