3GPP TSG-SA5 Meeting #123 	S5-191489

# Montreal,Canada, 21-25 January 2019											Revision of S5-191467

# Source:	Huawei

# Title:	Management aspects of NSIs shared by multiple communication services

# Document for:	Approval

Agenda Item:	6.5.6

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]	3GPP TS 28.530 “Concepts, use cases and requirements, v15.0.0”

[2]	3GPP TR 28.805 “Study on management aspects of communication services, v0.2.0”

# 3	Rationale

As described in 3GPP TS 28.530 [1] Clause 4.1.3, network slices and network slice subnets can be shared by services using them, or different customers. The following is an extract from 3GPP TS 28.530 [1] to explain sharing more clearly.

***************************** Extract from TS 28.530 v15.0.0 [1]************************************

Figure 4.1.3.1: A variety of communication services instances provided by multiple NSIs

Different NSIs (NSI A, NSI B and NSI C) contains NFs (e.g. belonging to NSSI AN-1, NSSI AN-2, NSSI CN-1, NSSI CN-2 and NSSI CN-3) as well as all information relevant to the interconnections between these NFs. Different Communication Service Instances are provided by CSP through using different NSIs. For example, Communication Service Instance 1 and Communication Service Instance 2 use NSI A which contains dedicated NSSIs (NSSI CN-1 and NSSI AN-1), Communication Service Instance 3 uses NSI C which contains a dedicated NSSI (NSSI CN-3) and an NSSI (NSSI AN-2) shared with NSI B.

*************************** End of extract from TS 28.530 v15.0.0 [1]*********************************

In 3GPP TS 28.805 [2], communication service management is described from customer, service and resource views. Accordingly, CSMF has functionalities regarding customer aspects and service and resource aspects as shown in the following:

***************************** Extract from TR 28.805 v0.2.0 [2] *************************************

Figure 4.4.2.1: Customer aspects as well as, service and resource aspects regarding CSMF

*************************** End of extract from TR 28.805 v0.2.0 [2]**********************************

When providing a network slice as a service, to a customer, the customer may obtain it as an NSI satisfying customer service requirements or as an NSI satisfying network resource requirement

This document provide the use case, where a a network provider provides a network slice to satisfy the service requirements provided by the customers, i.e. network provider does service-based sharing of the network slice among its customers.

# 4	Detailed proposal

This document proposes to add following text to TR 28.805 [2].

1st change

# 5	Potential use cases and requirements

## 5.X	Management aspects of NSIs utilized by multiple communication services

### 5.X.1	Description

Network slices and network slice subnets can be shared by different communication services as described in Clause 4.1.3 of 3GPP TS 28.530 [Y]. Also, same NSI can be utilized to provide communication services to different customers. In both cases, multiple CSIs may share a single NSI. In this use case, a communication service provider uses a network slice to provide multiple CSIs, i.e. network provider performs service-based sharing of the network slice among its customers.

On the contrary to the cases where a single NSI is utilized for a single CSI, the network management system is responsible for maintaining QoS guarantees for each service or customer over the resource facing aspects of a single NSI. For service-based sharing, the customer provides service requirements. Therefore, the NSI resources are shared based on service requirements to satisfy SLA requirements. In this case, the network provider determines the policies and rules about how the network resources are utilized. In other words, the network itself is transparent to the customer of the CSI service. Therefore, the network management system can utilize a database including available services.

While utilizing NSIs with sharing, effect of all services on that NSI and its constituent should be considered. Effects of loads of services can be evaluated using historical data and analysis provided by MDAF, as a part of feasibility check for provisioning CSIs. Similarly, for modifications on NSIs and NSSIs with sharing, effect of the modification on all services must be considered.

### 5.X.2	Potential requirements

REQ-XX-CON-01 The 3GPP management system shall have the capability to assure the QoS guarantees for each service when a network slice is utilized by multiple services.

End of change