# 3GPP TSG SA WG5 (Telecom Management) Meeting #123	S5-191496

# 21 - 25 January 2019, Montreal, Canada	Rev. of S5-191381

# Source:	ETRI, KT

# Title:	Add intent driven NSI performance assurance scenario

# Document for:	Approval

Agenda Item:	6.4.6

1	Decision/action requested

The group is asked to discuss and approve the contribution.

2	References

[1]	3GPP TR 28.812 Study on Scenarios for Intent driven management services for mobile networks v0.2.0.

[2]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[3]	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[4]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[5]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

3	Rationale

One of NSI consumers’ main concern when they purchase NSI(s) from NSI providers is how to ensure the performance of the NSI provided.  NSI consumers do not need to know the details of how the provided NSIs’ performance is guaranteed.  Instead, they just need to specify in high-level terms on the desired performance by mapping with the services they want to deploy over the provided NSIs such as performance level for fixed wireless, enhanced mobile broadband, massive IoT, ultra-reliable low-latency communications, enhanced event experience, and/or legacy best effort services, etc.  Thus, NSI consumers’ requests target NSI performance assurance with a simple intent and NSI providers must translate the intent of their language and ensure the required performance with periodic reports of the performance status to the NSI consumers.

This pCR proposes to add a scenario of intent-driven network slice instance (NSI) performance assurance.

4	Detailed proposal

It is proposed to make the following changes to TR 28.812 [1].

1st Change

5.2.X	Scenario X: Intent driven NSI performance assurance scenario

5.2.X.1	Pre-condition

The NSI consumer (e.g. vertical slice customer etc.) wants the NSI provider to assure the performance of the provided NSI based on its intent (e.g., type of services that need performance assurance, granularity of performance assurance report notifications, etc.).

The NSI provider translates the intent and triggers the requested NSI performance assurance operations including resource provisioning based on the performance requirements, performance monitoring, and performance notificattion.

5.2.X.2	Description

The NSI consumer as performance assurance MnS consumer expresses his intent for a NSI performance assurance (e.g., high-level abstraction of the SLA for performance assurance, granularity of performance assurance report notifications, etc.) to performance assurance MnS provider. The NSI consumer doesn’t care about what the details of mechanisms to assure the performance.

The MnS provider translates the intent from the MnS consumer to the implementable NSI performance assurance related requirements and provisions target resources, deploy performance monitoring management services, and perform periodic optimization operations to ensure the target KPIs through the associated performance assurance related management services.  It also generates performance assurance reports and notifies to the MnS consumers.

All implementable NSI performance assurance related requirements and associated procedures are defined in TS 28.531[2], 28.550[3], 28.552[4], 28.554[5].

Figure 5.2.X.2-1 An example of intent driven NSI performance assurance scenario

5.2.X.3	Post-condition

The requested NSI performance assurance requirements as expressed in the intent from the NSI consumer is fulfilled.

End