3GPP TSG-SA5 Meeting #125 	S5-193285

# Newport Beach, US, 8-12 April 2019												Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.290 | CR | 0046 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of result code classification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SBI_CH |  |  |  |  |  | Date: |  |  | 2019-04-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Correcting how the failure and retry handling should be handled att different reslut codes |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding how the NF (CTF) is expected to handle different types of errors. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The interworking between CHF and any NF in case of error would be undefined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

### 5.5.x	Error and Result definition

Table 5.5.x-1: Error and result types

| Error or result types | Description | NF Consumer (CTF) handling |
| --- | --- | --- |
| Session or event level errors or results |  |  |
| Charging Failed | The session request is rejected because the session or subscriber information needed by the CHF for charging or CDR creation is incomplete, erroneous or not available. | - Permanent result - Terminate charging session - Termination handling applicable |
| Charging not Applicable | The session request is granted by the CHF since it has been determined that the service can be allowed to the end user without any charging or CDR creation. | - Permanent result - Terminate charging session - Termination handling not applicable |
|  | cr |  |
| End User Request Denied | The session request is denied by the CHF due to restrictions or limitations related to the end-user. | - Permanent result - Terminate charging session - Termination handling applicable |
| Quota Limit Reached | The session request is denied by the CHF because the end user's account could not cover the requested service. If the request contained used units they are deducted, if applicable | - Transient result - Terminate charging session - Termination handling applicable |
| End User Request Rejected | The session request is rejected by the CHF due to end-user restrictions or limitations. | - Transient result - Terminate charging session - Termination handling applicable |
| Service level errors or results |  |  |
| End User Service Denied | The service request is denied by the CHF due to end-user service restrictions or limitations related to the end-user. | - Permanent result - Continue charging session - Termination handling applicable |
| Quota Management not Applicable | The service request is granted by the CHF without quota management and with used units reporting | - Permanent result - Continue charging session - Termination handling not applicable |
| Quota Limit Reached | The service request is denied by the CHF since the end user's account could not cover the requested service (i.e. terminate services for the rating group). | - Transient result - Continue charging session - Termination handling applicable |
| End User Service Rejected | The service request is rejected by the CHF due to end-user restrictions or limitations. | - Transient result - Continue charging session - Termination handling applicable |
| Rating Failed | The service request is rejected by the CHF because the service cannot be rated due to insufficient rating input, incorrect parameter combination or unrecognized parameter, or parameter value. | - Permanent result - Continue charging session - Termination handling applicable |
| Final Unit Indication | The service request is to be denied by the CHF since the end user's account will only cover the granted quota. | - Transient result - Continue charging session - Termination handling applicable |
| Zero Units Granted | The service request is denied by the CHF since no quota could be granted cover the requested service usage. | - Transient result - Continue charging session - Termination handling applicable |

End of change