3GPP TSG-SA5 Meeting #125Adhoc 	S5-194131

# Sapporo, Japan 25-28 June 2019												Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0109 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct NR NRM model to be applicable for all NG-RAN architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE-5GNRM |  |  |  |  |  | Date: |  |  | 2019-06-14 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Currently, the NR NRM model can’t be applicable for management of all NG-RAN architecture for the following reasons: - The definition for gNBDUFuntion in clause 4.3.1 (This IOC represents the logical function DU of gNB or en-gNB defined in 3GPP TS 38.401 [4]) is only appllicable for 3-split and 2-split NG-RAN architecture, beasuce for non-split NG-RAN arcgitecture, there is no logical function DU of gNB or en-gNB in TS 38.401. Similar issues for gNBCUCPFunction and gNBCUUPFunction. - Currently, in clause 4.3.1.2, the “gNBDUId” is mandatory attribute for gNBDUFuntion, however, for non-split NG-RAN architecture, there is no “gNBDUId” used in the signalling and it is not necessary for operator to assign the “gNBDUId”. Similar issue for the attribute “gNBDUName” and “gNBDUName”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the definition for gNBDUFunction, gNBCUCPFunction and gNBCUUPFunction to applicale for all NG-RAN architectures defined in TS 38.401. - Change Qualifier of the attribute “gNBDUId”， “gNBDUName” and“gNBDUName” from M to CM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NR NRM can’t be applicable for all NG-RAN architectures defined in TS 38.401. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1, 4.3.1.3, 4.3.2.1, 4.3.2.3, 4.3.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Change

4.3.1	GNBDUFunction

4.3.1.1	Definition

For non-split NG-RAN architecture, this IOC together with GNBCUCPFunction IOC and GNBCUUPFunction IOC provide the management of gNB or en-gNB function as defined in clause 6.1.1 in 3GPP TS 38.401 [4].

For 2-split and 3-split NG-RAN architecture, this IOC provides the management representation of the logical function DU of gNB or en-gNB defined in clause 6.1.1 and 6.1.2 in 3GPP TS 38.401 [4].

The following table identifies the necessary end points required for the representation of gNB and en-gNB, of all deployment scenarios.

| Req Role | End point requirement for 3-split deployment scenario | End point requirement for 2-split deployment scenario | End point requirement for Non-split deployment scenario |
| --- | --- | --- | --- |
| gNB | <<IOC>>EP_F1C, <<IOC>>EP_F1U | <<IOC>>EP_F1C, <<IOC>>EP_F1U | None. |
| en-gNB | <<IOC>>EP_F1C, <<IOC>>EP_F1U | <<IOC>>EP_F1C, <<IOC>>EP_F1U | None. |

4.3.1.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBDUId | CM | T | T | F | T |
| gNBDUName | CO | T | T | F | T |
| gNBId | M | T | F | F | T |
| gNBIdLength | M | T | T | F | T |
| pLMNId | M | T | T | T | T |

4.3.1.3	Attribute constraints

| Name | Definition |
| --- | --- |
| gNBDUId Support Qualifier | Condition: 2-split or 3-split NG-RAN architecture is supported |
| gNBDUName Support Qualifier | Condition: 2-split or 3-split NG-RAN architecture is supported |

4.3.1.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

4.3.2	GNBCUCPFunction

4.3.2.1	Definition

For non-split NG-RAN architecture, this IOC together with GNBCUUPFunction IOC and GNBDUFunction IOC provide the management representation of gNB or en-gNB function as defined in clause 6.1.1 in 3GPP TS 38.401 [4].

For 2-split NG-RAN architecture, this IOC together with GNBCUUPFunction IOC provide management representation of the logical function CU of gNB and en-gNB defined in clause 6.1.1 in 3GPP TS 38.401 [4].

For 3-split NG-RAN architecture, this IOC provides management representation of the logical function CU-CP of gNB and en-gNB defined in clause 6.1.2 in 3GPP TS 38.401 [4].

The following table identifies the necessary end points required for the representation of gNB and en-gNB, of all deployment scenarios.

| Req Role | End point requirement for 3-split deployment scenario | End point requirement for 2-split deployment scenario | End point requirement for Non-split deployment scenario |
| --- | --- | --- | --- |
| gNB | <<IOC>>EP_XnC, <<IOC>>EP_NgC, <<IOC>>EP_F1C, <<IOC>>EP_E1. | <<IOC>>EP_XnC, <<IOC>>EP_NgC, <<IOC>>EP_F1C <<IOC>>EP_F1U. | <<IOC>>EP_XnC, <<IOC>>EP_NgC. |
| en-gNB | <<IOC>>EP_X2C, <<IOC>>EP_F1C, <<IOC>>EP_E1. | <<IOC>>EP_X2C, <<IOC>>EP_F1C. | <<IOC>>EP_X2C. |

4.3.2.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | F | T |
| gNBIdLength | M | T | T | F | T |
| gNBCUName | CO | T | T | F | T |
| pLMNId | M | T | T | T | T |

4.3.2.3	Attribute constraints

| Name | Definition |
| --- | --- |
| gNBCUName Support Qualifier | Condition: 2-split NG-RAN architecture or 3-split NG-RAN architecture is supported |

4.3.2.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

4.3.3	GNBCUUPFunction

4.3.3.1	Definition

For non-split NG-RAN architecture, this IOC together with GNBCUCPFunction IOC and GNBDUFunction IOC provide the management representation of gNB or en-gNB function as defined in clause 6.1.1 in 3GPP TS 38.401 [4].

For 2-split NG-RAN architecture, this IOC together with GNBCUCPFunction IOC provide management representation of the logical function CU of gNB and en-gNB defined in clause 6.1.1 in 3GPP TS 38.401 [4].

For 3-split NG-RAN architecture, this IOC provides management representation of  the logical function CU-UP of gNB or en-gNB defined in clause 6.1.2 in 3GPP TS 38.401 [4].

The following table identifies the necessary end points required for the representation of gNB and en-gNB, of all deployment scenarios.

| Req Role | End point requirement for 3-split deployment scenario | End point requirement for 2-split deployment scenario | End point requirement for Non-split deployment scenario |
| --- | --- | --- | --- |
| gNB | <<IOC>>EP_XnU, <<IOC>>EP_NgU, <<IOC>>EP_F1U, <<IOC>>EP_E1. | <<IOC>>EP_XnU, <<IOC>>EP_NgU, <<IOC>>EP_F1U. | <<IOC>>EP_XnU, <<IOC>>EP_NgU. |
| en-gNB | <<IOC>>EP_X2U, <<IOC>>EP_S1U, <<IOC>>EP_F1U, <<IOC>>EP_E1. | <<IOC>>EP_X2U, <<IOC>>EP_S1U, <<IOC>>EP_F1U. | <<IOC>>EP_X2U, <<IOC>>EP_S1U. |

4.3.3.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| gNBId | M | T | T | F | T |
| gNBIdLength | M | T | T | F | T |

4.3.3.3	Attribute constraints

None.

4.3.3.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

End of Change