3GPP TSG-SA5 Meeting #126	S5-195249

# Bruges, Belgium, 19th Aug 2019 - 23rd Aug 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.254 | CR | 0009 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR 32.254 Update of NEF API Charging Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1_NEFCH |  |  |  |  |  | Date: |  |  | 2019-08-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Addition of NEF API Charging Informtion |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding all attributes from the APIs supported by the NEF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Recording of the information passed on the APIs will not be possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

##### 6.3.1.4.1	Definition of the NEF API Charging Information

Network Exposure Function API specific charging information is provided within the NEF API Charging Information. The detailed structure of the NEF API Charging Information can be found in table 6.3.1.4.1.

Table 6.3.1.4.1: Structure of the NEF API Charging Information

| Information Element | Category | Description | NEF API Usage |
| --- | --- | --- | --- |
| External Identifier | OC | This parameter holds the external Identifier identifying the served party associated to the SUPI or GPSI, if available. |  |
| API Direction | OC | This field holds the direction to indicate if it is an API invocation from an AF or notification to an AF. |  |
| API Target Network Function | OC | This field holds the identifier of the network function that either is the destination of the API invocation or triggers the notification. |  |
| API Result Code | OC | This parameter holds the result of API Invocation. |  |
| Ethernet Flow Info | OC | Identifies Ethernet packet flows. | AsSessionWithQoS, ChargeableParty |
| Flow Info | OC | Describe the IP data flow (which requires QoS). | AsSessionWithQoS, ChargeableParty |
| MAC Address | OC | Identifies the MAC address. | AsSessionWithQoS, ChargeableParty, TrafficInfluence |
| QoS Reference | OC | Identifies a pre-defined QoS information | AsSessionWithQoS |
| Request Test Notification | OC | Set to true by the AF to request the NEF to send a test notification. Set to false or omitted otherwise. | AsSessionWithQoS, ChargeableParty, DeviceTriggering, MonitoringEvent, TrafficInfluence |
| Sponsor Information | OC | Indicates a sponsor information | AsSessionWithQoS, ChargeableParty |
| UE IPv4 Address | OC | The Ipv4 address of the UE. | AsSessionWithQoS |
| UE Ipv6 Address | OC | The Ipv6 address of the UE. | AsSessionWithQoS |
| Usage Threshold | OC | Time period and/or traffic volume. | AsSessionWithQoS, ChargeableParty |
| Ipv4 Address | OC | Identifies the Ipv4 address. | ChargeableParty, MonitoringEvent, TrafficInfluence |
| Ipv6 Address | OC | Identifies the Ipv6 address. | ChargeableParty, MonitoringEvent, TrafficInfluence |
| Reference ID | OC | The reference ID for a previously selected policy of background data transfer. | ChargeableParty |
| Sponsoring Enabled | OC | Indicates sponsoring status. | ChargeableParty |
| CP Parameter Sets | OC | Identifies a set of CP parameter information that may be part of this CpInfo structure. | CpProvisioning |
| CP Reports | OC | Contains the CP set identifiers for which CP parameter(s) are not added or modified successfully. The failure reason is also included. | CpProvisioning |
| External Group Id | OC | Identifies a user group as defined in Clause 4.6.2 of 3GPP TS 23.682 [x]. | CpProvisioning, MonitoringEvent, TrafficInfluence |
| External Id | OC | Uniquely identifies a user as defined in Clause 4.6.2 of 3GPP TS 23.682 [x]. | CpProvisioning, DeviceTriggering, MonitoringEvent |
| MSISDN | OC | Identifies the MS internal PSTN/ISDN number allocated for a UE. | CpProvisioning, DeviceTriggering, MonitoringEvent |
| MTC Provider Id | OC | Identifies the MTC Service Provider and/or MTC Application. (NOTE 3) | CpProvisioning, MonitoringEvent |
| Application Port Id | OC | This is used to uniquely identify the triggering application addressed in the device. | DeviceTriggering |
| Delivery Result | OC | The delivery result shall be included in the HTTP responses that indicate the delivery status of the device triggering. | DeviceTriggering |
| Notification Destination | OC | An URI indicating the notification destination for T8 notifications. | DeviceTriggering, MonitoringEvent |
| Priority | OC | Identifies the priority of the device trigger. | DeviceTriggering |
| Result | OC | OK, unknown or diverse failures | DeviceTriggering |
| Transaction | OC | Link to the related device triggering transaction resource to which this notification is related. | DeviceTriggering |
| Trigger Payload | OC | The device triggering payload. | DeviceTriggering |
| Validity Period | OC | The validity time in seconds for the specific action requested. | DeviceTriggering |
| Accuracy | OC | Identifies the desired level of accuracy of the requested location information, as described in subclause 4.9.2 of 3GPP TS 23.682 [x]. | MonitoringEvent |
| Add External Group Ids | OC | Identifies user groups as defined in Clause 4.6.2 of 3GPP TS 23.682 [x]. | MonitoringEvent |
| Association Type | OC | Identifies whether the change of IMSI-IMEI or IMSI-IMEISV association shall be detected. | MonitoringEvent |
| Cancel Indication | OC | Indicates whether to request to cancel the corresponding monitoring subscription. Set to false or omitted otherwise. | MonitoringEvent |
| Config Results | OC | Identifies a notification of grouping configuration result. | MonitoringEvent |
| Group Report Guard Time | OC | Identifies the time for which the NEF can aggregate the monitoring event reports detected by the UEs in a group and report them together to the AF. | MonitoringEvent |
| Idle Status Indication | OC | Indicate the notification of when a UE, for which PSM is enabled, transitions into idle mode. | MonitoringEvent |
| Location Area | OC | Indicates the area within which the AF requests the number of UEs. | MonitoringEvent, ResourceManagementOfBdt |
| Location Area 5G | OC | Indicates the 5G area within which the AF requests the number of UEs. | MonitoringEvent, ResourceManagementOfBdt |
| Location Type | OC | Identifies whether the request is for Current Location or Last known Location. | MonitoringEvent |
| Maximum Detection Time | OC | Identifies the maximum period of time after which the UE is considered to be unreachable. | MonitoringEvent |
| Maximum Latency | OC | Identifies the maximum delay acceptable for downlink data transfers. | MonitoringEvent |
| Maximum Number Of Reports | OC | Identifies the maximum number of event reports to be generated by the HSS, MME/SGSN as specified in subclause 5.6.0 of 3GPP TS 23.682 [2]. | MonitoringEvent |
| Maximum Response Time | OC | Identifies the length of time for which the UE stays reachable to allow the AF to reliably deliver the required downlink data. | MonitoringEvent |
| Minimum Report Interval | OC | Identifies a minimum time interval between Location Reporting notifications. | MonitoringEvent |
| Monitor Expire Time | OC | Identifies the absolute time at which the related monitoring event request is considered to expire. | MonitoringEvent |
| Monitoring Event Report | OC | Identifies a monitoring event report which is sent from the NEF to the AF. | MonitoringEvent |
| Monitoring Event Reports | OC | Each element identifies a monitoring event report. | MonitoringEvent |
| Monitoring Type | OC | Enumeration of monitoring type. | MonitoringEvent |
| PLMN Indication | OC | Indicates the notification of UE’s Serving PLMN ID. | MonitoringEvent |
| Reachability Type | OC | Identifies whether the request is for “Reachability for SMS” or “Reachability for Data”. | MonitoringEvent |
| Subscription | OC | Link to the subscription resource to which this notification is related. | MonitoringEvent |
| Suggested Number Of Dl Packets | OC | Identifies the number of packets that the serving gateway shall buffer in case that the UE is not reachable. | MonitoringEvent |
| PFD Datas | OC | Uniquely identifies the PFDs for an external application identifier. | PfdManagement |
| PDF Reports | OC | Contains the external application identifiers for which PFD(s) are not added or modified successfully. The failure reason is also included. | PfdManagement |
| Desired Time Window | OC | Identifies the time interval. | ResourceManagementOfBdt |
| Number Of UEs | OC | Identifies the number of UEs. | ResourceManagementOfBdt |
| Reference Id | OC | Identifies a selected policy of background data transfer. | ResourceManagementOfBdt |
| Selected Policy | OC | Identity of the selected background data transfer policy. | ResourceManagementOfBdt |
| Transfer Policies | OC | Identifies an offered transfer policy. | ResourceManagementOfBdt |
| Volume Per UE | OC | Identifies the data volume expected to be transferred per UE. | ResourceManagementOfBdt |
| AF Application Id | OC | Identifies an application. | TrafficInfluence |
| AF Service Id | OC | Identifies a service on behalf of which the AF is issuing the request. | TrafficInfluence |
| AF Transaction Id | OC | Identifies an NEF Northbound interface transaction, generated by the AF. | TrafficInfluence |
| Any UE Indication | OC | Identifies whether the AF request applies to any UE. | TrafficInfluence |
| Application Relocation Indication | OC | Identifies whether an application can be relocated once a location of the application has been selected. | TrafficInfluence |
| DNAI Change Type | OC | Identifies a type of notification regarding UP path management event. | TrafficInfluence |
| DNN | OC | Identifies a DNN. | TrafficInfluence |
| Ethernet Traffic Filters | OC | Identifies Ethernet packet filters. | TrafficInfluence |
| GPSI | OC | Identifies a user. | TrafficInfluence |
| S-NSSAI | OC | Identifies an S-NSSAI. | TrafficInfluence |
| Subscribed Events | OC | Identifies the requirement to be notified of the event(s). | TrafficInfluence |
| Temp Validities | OC | Indicates the time interval(s) during which the AF request is to be applied. | TrafficInfluence |
| Traffic Filters | OC | Identifies IP packet filters. | TrafficInfluence |
| Traffic Routes | OC | Identifies the N6 traffic routing requirement. | TrafficInfluence |
| Valid Geographic Zone Ids | OC | Identifies a geographic zone that the AF request applies only to the traffic of UE(s) located in this specific zone. | TrafficInfluence |

End of changes