3GPP TSG-SA5 Meeting #126	S5-195720

# Bruges, Belgium, 19th Aug 2019 - 23rd Aug 2019   					Revision of S5-195315

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.290 | CR | 0065 | rev | 1 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of notify response |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SBI_CH |  |  |  |  |  | Date: |  |  | 2019-08-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1)In diameter credit control application, there is two different procedure for successful server-initiated re-authorization senario. As defined in subclause 5.5 in RFC 8506, If a credit re-authorization is not already ongoing (i.e., the credit-control session is in Open state), a credit control client that receives an RAR message with Session-Id equal to a currently active credit-control session MUST acknowledge the request by sending the Re-Auth-Answer (RAA) message and MUST initiate the credit re-authorization toward the server by sending a Credit-Control-Request message with the CC-Request-Type AVP set to the value UPDATE_REQUEST. The Result-Code 2002 (DIAMETER_LIMITED_SUCCESS) SHOULD be used in the RAA message to indicate that an additional message (i.e., CCR message with the value UPDATE_REQUEST) is required to complete the procedure. If a quota was allocated to the service, the credit-control client MUST report the used quota in the Credit-Control-Request. …… If credit re-authorization is ongoing at the time when the RAR message is received (i.e., RAR-CCR collision), the credit-control client successfully acknowledges the request but does not initiate a new credit re-authorization. The Result-Code 2001 (DIAMETER_SUCCESS) SHOULD be used in the RAA message to indicate that a credit re-authorization procedure is already ongoing (i.e., the client was in PendingU state when the RAR was received). The credit-control server SHOULD process the Credit-Control-Request as if it was received in answer to the server initiated credit re-authorization, and should consider the server initiated credit re-authorization process successful upon reception of the Re-Auth-Answer message. It’s better to remain this functionality for 5G charging for backward compatible, i.e. add support for above 1st procedure since 2nd procedure already be covered in 5G charging specification. And following definition of HTTP Response status code 202 in RFC 7233 is suitable for above 1st re-authorization procedure: 6.3.3. 202 Accepted The 202 (Accepted) status code indicates that the request has been accepted for processing, but the processing has not been completed. The request might or might not eventually be acted upon, as it might be disallowed when processing actually takes place. There is no facility in HTTP for re-sending a status code from an asynchronous operation. 2) RFC6733 subclause 8.5.2 has following description for ASA Result-Code in different scenario, which also should be kept for 5G charging for backward compatible: If the session identified by Session-Id in the ASR was successfully terminated, the Result-Code is set to DIAMETER_SUCCESS. If the session is not currently active, the Result-Code is set to DIAMETER_UNKNOWN_SESSION_ID. If the access device does not stop the session for any other reason, the Result-Code is set to DIAMETER_UNABLE_TO_COMPLY. |  |  |  |  |  |  |  |  |
| Summary of change: |  | supplement processing mechnism for: - re-authorization notification successful ackonwledgment scenario. - abort notification unsuccessful ackonwledgment scenario. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | charging notify request is not correctly supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1,5.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

5.4.1	Re-authorization

The CHF (NF Service Producer) may trigger a re-authorization request and the NF Service Consumer shall report quota usage. The reason for the quota being reported shall be notified to the CHF (NF Service Producer). This is described under charging notification procedure in clause 5.3.2.4.

If a pending request between CHF and NF Service Consumer is not already ongoing, the NF Service Consumer returns the Charging Notify Response with status code 202 (Accepted) to indicate that an Charging Data Request [Update] is required and then sends Charging Data Request [Update] to the CHF.

If a pending request between CHF and NF Service Consumer is ongoing at the time when the Charging Notify Request is received, the NF Service Consumer returns the Charging Notify Response with status code 204 (No Content) to indicate an Charging Data Request [Update] is already ongoing but does not send a new Charging Data Request [Update].

Next change

5.4.4	Service termination

The CHF (NF Service Producer) may determine that a service requires termination. The NF Service Producer may perform this termination synchronously if it has a request pending processing by returning response.

If the CHF (NF Service Producer) does not have a pending request (asynchronous), the NF Service Producer may trigger an abort notification to terminate the charging session. On reception of an abort notification, the NF consumer shall terminate the associated charging session by sending a Nchf_ConvergedCharging_Release.

If charging session identified by the abort notification is not currently active, a Charging Notify Response with status code 400 (Bad Request) is returned.

If the NF consumer does not terminate the charging session for any other reason, a Charging Notify Response with status code 500 (Internal Server Error) is returned.

End of change