3GPP TSG-SA5 Meeting #127	S5-196048

# Sophia-Antipolis (FR), 14 - 18 October 2019 									Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0055 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RESTful CM notifications for integration with ONAP VES |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T, Deutsche Telekom, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ONAP3GPP |  |  |  |  |  | Date: |  |  | 2019-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Stage 3 for the integration of provisioning report management service (CM) notifications with ONAP VES is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Stage 3 enabling the integration of provisioning report management service notifications with ONAP VES. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Integration of provisioning report management service notifications with ONAP VES would not be possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.X (New), A, A.1, A.1.X (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

# 12	Management services – Stage 3

## 12.1	Generic provisioning management service

### 12.1.X	RESTful HTTP-based solution set for integration with ONAP VES API

#### 12.1.X.1	Mapping of operations

NOTE: this mapping is not part of the present document.

#### 12.1.X.2	Mapping of notifications

##### 12.1.X.2.1	Introduction

###### 12.1.X.2.1.1	General

The 3GPP IS notifications are mapped to SS equivalents according to table 12.1.X.2.1.1-1.

Table 12.1.X.2.1.1-1: Mapping of 3GPP IS notifications to SS equivalents

| 3GPP IS notifications | HTTP Method | Resource URI | SQ |
| --- | --- | --- | --- |
| notifyMOICreation | POST | /eventListener | M |
| notifyMOIDeletion | POST | /eventListener | M |
| notifyMOIAttributeValueChange | POST | /eventListener | M |

###### 12.1.X.2.1.2	Notification parameter mapping principles

3GPP fault supervision alarm notification parameters are mapped either to:

- ONAP VES API commonEventHeader fields [21] – see Table 12.1.X.2.1.2-1, or to

- ONAP VES API prov3gpp fields – see mapping tables in the following clauses.

Table 12.1.X.2.1.2-1: Mapping of 3GPP IS notification parameters to ONAP VES common event header fields

| 3GPP IS notification parameter name | SQ | Corresponding ONAP VES common event header field name | SQ in ONAP VES common event header | Remark |
| --- | --- | --- | --- | --- |
| no corresponding IS parameter | N/A | sourceId | O |  |
| notificationId | M | eventId | M |  |
| no corresponding IS parameter | N/A | eventName | M | See [23]. |
| eventTime | M | lastEpochMicrosec | M | Type conversion needed [21]. |
| no corresponding IS parameter | N/A | reportingEntityName | M | See [24]. |
| eventTime | M | startEpochMicrosec | M | Same value as lastEpochMicrosec. Type conversion needed [21]. |
| no corresponding IS parameter | N/A | domain | M | “prov3GPP” |
| no corresponding IS parameter | N/A | priority | M | Field not used by this solution set. Any type-compatible value [21] to be provided by the Provisioning data report MnS producer. Actual value out of scope of this document. |
| no corresponding IS parameter | N/A | sequence | M | Field not used by this solution set. Actual value out of scope of this document. Value ‘0’ [21] to be provided by the Provisioning data report MnS producer. |
| no corresponding IS parameter | N/A | sourceName | M | Field not used by this solution set. Any type-compatible value [21] to be provided by the Provisioning data report MnS producer. Actual value out of scope of this document. |
| no corresponding IS parameter | N/A | version | M | Version of the event header [21]. |
| no corresponding IS parameter | N/A | vesEventListenerVersion | M | Version of the VES Event Listener API [21]. |

The ONAP VES API commonEventHeader datatype consists of fields common to all events, as specified in [21].

NOTE 1: Table 12.1.X.2.1.2-1 has the following columns:

- 3GPP IS notification parameter name: name of the input parameter of the notification as specified in clauses of clause 11.1.1 of the present document;

- SQ: Support Qualifier for the notification input parameter, as specified in clauses of clause 11.1.1 of the present document;

- Corresponding VES common event header field name: name of the field of the VES common event header [21];

- SQ in ONAP VES common event header: specified in [21], where ‘M’ corresponds to ‘Required = Yes’ in [21], and ‘O’ corresponds to ‘Required = No’ in [21];

NOTE 2: In Table 12.1.X.2.1.2-1, some VES common event header fields have no corresponding IS parameter. In case those VES common event header field Support Qualifier is ‘M’ (as specified in [21]), the producer of the notification shall provide a value to these parameters. See column ‘Remark’ in Table 12.1.X.2.1.2-1.

NOTE 3: ONAP VES common event header fields which are both optional [21] and have no corresponding IS parameter are not listed in Table 12.1.X.2.1.2-1.

NOTE 4: The mapping tables in following clauses of clause 12.1.X have the following columns:

- 3GPP IS notification parameter name: name of the input parameter of the notification as specified in clauses of clause 11.1.1 of the present document;

- SQ: Support Qualifier for the notification input parameter, as specified in clauses of clause 11.1.1 of the present document;

- SS parameter name (Corresponding VES ‘Prov3GPP’ domain field): name of the field in ‘prov3gppFields’;

- SQ: Support Qualifier in ‘prov3gppFields’. When the IS notification parameter Support Qualifier is ‘M’, this Support Qualifier is set to ‘M’. When the IS notification parameter Support Qualifier is ‘C’, this Support Qualifier is set to ‘O’. When the IS notification parameter Support Qualifier is ‘O’, this Support Qualifier is set to ‘O’.

##### 12.1.X.2.2	Notification notifyMOICreation

The 3GPP IS notification parameters are mapped to SS equivalents according to table 12.1.X.2.1.2-1 and to table 12.1.X.2.2-1.

Table 12.1.X.2.2-1: Mapping of IS notification parameters to SS equivalents in ‘prov3gppFields’

| 3GPP IS operation parameter name | SQ | SS parameter name in ‘prov3gppFields’ – clause A.1.X | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| objectClass objectInstance | M | DN | dN-Type | M |
| notificationType | M | notificationType (value = ‘notifyMOICreation’) | notificationType-Type | M |
| systemDN | M | systemDN | systemDN-Type | M |
| correlatedNotifications | CM | correlatedNotifications | array(correlatedNotification-Type) | O |
| attributeList | O | attributes | array(attributeNameValuePair-Type) | O |
| additionalText | O | additionalText | additionalText-Type | O |

##### 12.1.X.2.3	Notification notifyMOIDeletion

The 3GPP IS notification parameters are mapped to SS equivalents according to table 12.1.X.2.1.2-1 and to table 12.1.X.2.3-1.

Table 12.1.X.2.3-1: Mapping of IS notification parameters to SS equivalents in ‘prov3gppFields’

| 3GPP IS operation parameter name | SQ | SS parameter name in ‘prov3gppFields’ – clause A.1.X | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| objectClass objectInstance | M | DN | dN-Type | M |
| notificationType | M | notificationType (value = ‘notifyMOIDeletion’) | notificationType-Type | M |
| systemDN | M | systemDN | systemDN-Type | M |
| correlatedNotifications | CM | correlatedNotifications | array(correlatedNotification-Type) | O |
| attributeList | O | attributes | array(attributeNameValuePair-Type) | O |
| additionalText | O | additionalText | additionalText-Type | O |

##### 12.1.X.2.4	Notification notifyMOIAttributeValueChange

The 3GPP IS notification parameters are mapped to SS equivalents according to table 12.1.X.2.1.2-1 and to table 12.1.X.2.4-1.

Table 12.1.X.2.4-1: Mapping of IS notification parameters to SS equivalents in ‘prov3gppFields’

| 3GPP IS operation parameter name | SQ | SS parameter name in ‘prov3gppFields’ – clause A.1.X | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| objectClass objectInstance | M | DN | dN-Type | M |
| notificationType | M | notificationType (value = ‘notifyMOIAttributeValueChange’) | notificationType-Type | M |
| systemDN | M | systemDN | systemDN-Type | M |
| correlatedNotifications | CM | correlatedNotifications | array(correlatedNotification-Type) | O |
| attributeValueChange | M | attributeValueChange | array(attributeValueChange-Type) | M |
| additionalText | O | additionalText | additionalText-Type | O |

#### 12.1.X.3	Resources

##### 12.1.X.3.1	Resource structure

Figure 12.1.X.3.1-1 shows the resource structure of the provisioning MnS in the context of its integration with ONAP DCAE VES Collector R3 [22].

Figure 12.1.X.3.1-1: Resource URI structure of the provisioning MnS for integration with ONAP DCAE VES R3 [22]

Table 12.1.X.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 12.1.X.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| eventListener | /eventListener | POST | Send notifications |

##### 12.1.X.3.2	Resource definitions

See [21].

#### 12.1.X.4	Data type definitions

See clause 12.1.1.4.

Next change

Annex A (normative): 
OpenAPI specification

## A.0	Introduction

This clause describes the capabilities of the service in the structure of the OpenAPI Specification Version 3.0.1 [A9]. The OpenAPI document is represented in the JSON format option.

## A.1	Provisioning management service

## A.1.1	Generic provisioning management service

{

"openapi": "3.0.1",

"info": {

"title": "TS 28.532 Provisioning Management Service",

"version": "15.1.0",

"description": "OAS 3.0.1 specification of the Provisioning Management Service"

},

"servers": [

{

"url": "http://{DN_prefix_authority_part}/{DN_prefix_remainder}/ProvMnS/v1500",

"variables": {

"DN_prefix_authority_part": {

"description": "See subclause 4.4 of TS 32.158",

"default": "example.com"

},

"DN_prefix_remainder": {

"description": "See subclause 4.4 of TS 32.158",

"default": ""

}

}

}

],

"paths": {

"/{className}/{id}": {

"parameters": [

{

"name": "className",

"in": "path",

"required": true,

"schema": {

"$ref": "#/components/schemas/className-PathType"

}

},

{

"name": "id",

"in": "path",

"required": true,

"schema": {

"$ref": "#/components/schemas/id-PathType"

}

}

],

"put": {

"summary": "Creates a single resource",

"description": "With HTTP PUT resources are created.",

"requestBody": {

"required": true,

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/resourceCreation-RequestType"

}

}

}

},

"responses": {

"201": {

"description": "Success case (\"201 Created\"). The representation of the newly created resource shall be returned.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/resourceCreation-ResponseType"

}

}

}

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

},

"get": {

"summary": "Read resources",

"description": "With HTTP GET resources are read. The resources to be read are identified with the path component (base resource) and the query component (scope, filer) of the URI. The fields query component allows to select the resource properties to be returned.",

"parameters": [

{

"name": "scope",

"in": "query",

"description": "This parameter extends the set of targeted resources beyond the base resource identified with the path component of the URI. No scoping mechanism is specified in the present document.",

"required": true,

"schema": {

"$ref": "#/components/schemas/scope-QueryType"

}

},

{

"name": "filter",

"in": "query",

"description": "This parameter reduces the targeted set of resources by applying a filter to the scoped set of resource representations. Only resources representations for which the filter construct evaluates to \"true\" are targeted. No filter language is specified in the present document.",

"required": true,

"schema": {

"$ref": "#/components/schemas/filter-QueryType"

}

},

{

"name": "fields",

"in": "query",

"description": "This parameter specifies the attributes of the scoped resources that are returned. The value is a comma-separated list of attribute names.",

"required": true,

"schema": {

"$ref": "#/components/schemas/fields-QueryType"

},

"style": "form",

"explode": false

}

],

"responses": {

"200": {

"description": "Success case (\"200 OK\"). The resources identified in the request for retrieval are returned in the response message body. In case the fields query parameter is used, the selected attributes are returned.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/resourceRetrieval-ResponseType"

}

}

}

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

},

"patch": {

"summary": "Modify one or multiple resources",

"description": "With HTTP PATCH resources are modified. The resources to be modified are identified with the path component (base resource) and the query component (scope, filer) of the URI.",

"parameters": [

{

"name": "scope",

"in": "query",

"description": "This parameter extends the set of targeted resources beyond the base resource identified with the path component of the URI. No scoping mechanism is specified in the present document.",

"required": false,

"schema": {

"$ref": "#/components/schemas/scope-QueryType"

}

},

{

"name": "filter",

"in": "query",

"description": "This parameter reduces the targeted set of resources by applying a filter to the scoped set of resource representations. Only resources representations for which the filter construct evaluates to \"true\" are returned. No filter language is specified in the present document.",

"required": false,

"schema": {

"$ref": "#/components/schemas/filter-QueryType"

}

}

],

"requestBody": {

"description": "The request body describes changes to be made to the target resources as defined in RFC 7396 (JSON Merge Patch). The request body is of type object in the present document. No refined schema is defined.",

"required": true,

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/resourceModification-RequestType"

}

}

}

},

"responses": {

"200": {

"description": "Success case (\"200 OK\"). The modified resources identified in the request for modification are returned.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/resourceModification-ResponseType"

}

}

}

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

},

"delete": {

"summary": "Delete one or multiple resources",

"description": "With HTTP DELETE resources are deleted. The resources to be deleted are identified with the path component (base resource) and the query component (scope, filer) of the URI.",

"parameters": [

{

"name": "scope",

"in": "query",

"description": "This parameter extends the set of targeted resources beyond the base resource identified with the path component of the URI. No scoping mechanism is specified in the present document.",

"required": false,

"schema": {

"$ref": "#/components/schemas/scope-QueryType"

}

},

{

"name": "filter",

"in": "query",

"description": "This parameter reduces the targeted set of resources by applying a filter to the scoped set of resource representations. Only resources representations for which the filter construct evaluates to \"true\" are returned. No filter language is specified in the present document.",

"required": false,

"schema": {

"$ref": "#/components/schemas/filter-QueryType"

}

}

],

"responses": {

"200": {

"description": "Success case (\"200 OK\"). The resources URI's deleted are returned.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/resourceDeletion-ResponseType"

}

}

}

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

}

},

"/subscriptions": {

"post": {

"summary": "Create a subscription",

"description": "To create a subscription the representation of the subscription is POSTed on the /subscriptions collection resource.",

"requestBody": {

"required": true,

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/subscription-RequestType"

}

}

}

},

"responses": {

"201": {

"description": "Success case (\"201 Created\"). The representation of the newly created subscription resource shall be returned.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/subscription-ResponseType"

}

}

}

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

},

"delete": {

"summary": "Delete all subscriptions made with a specific consumerReferenceId",

"description": "The subscriptions are deleted by deleting the corresponding subscription resources. The resources to be deleted are identified with the path component of the URI pointing to the /subscription collection resource and filtering on the consumerReferenceId provided in the query part.",

"parameters": [

{

"name": "consumerReferenceId",

"in": "query",

"description": "Identifies the subscriptions to be deleted.",

"required": true,

"schema": {

"$ref": "#/components/schemas/consumerReferenceId-QueryType"

}

}

],

"responses": {

"204": {

"description": "Success case (\"204 No Content\"). The subscription resources have been deleted. The response message body is absent."

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

},

"callbacks": {

"notifyMOICreation": {

"{request.body#/consumerReference}": {

"post": {

"requestBody": {

"required": true,

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/notifyMOICreation-NotifType"

}

}

}

},

"responses": {

"204": {

"description": "Success case (\"204 No Content\"). The notification is successfully delivered. The response message body is absent."

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

}

}

},

"notifyMOIDeletion": {

"{request.body#/consumerReference}": {

"post": {

"requestBody": {

"required": true,

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/notifyMOIDeletion-NotifType"

}

}

}

},

"responses": {

"204": {

"description": "Success case (\"204 No Content\"). The notification is successfully delivered. The response message body is absent."

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

}

}

},

"notifyMOIAttributeValueChange": {

"{request.body#/consumerReference}": {

"post": {

"requestBody": {

"required": true,

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/notifyMOIAttributeValueChange-NotifType"

}

}

}

},

"responses": {

"204": {

"description": "Success case (\"204 No Content\"). The notification is successfully delivered. The response message body is absent."

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

}

}

}

}

}

},

"/subscriptions/{subscriptionId}": {

"delete": {

"summary": "Delete a single subscription",

"description": "The subscription is deleted by deleting the corresponding subscription resource. The resource to be deleted is identified with the path component of the URI.",

"parameters": [

{

"name": "subscriptionId",

"in": "path",

"description": "Identifies the subscription to be deleted.",

"required": true,

"schema": {

"$ref": "#/components/schemas/subscriptionId-PathType"

}

}

],

"responses": {

"204": {

"description": "Success case (\"204 No Content\"). The subscription resource has been deleted. The response message body is absent."

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

}

}

},

"components": {

"schemas": {

"attributeNameValuePair-Type": {

"type": "object",

"properties": {

"attributeName": {

"type": "string"

},

"attributeValue": {}

}

},

"dateTime-Type": {

"type": "string",

"format": "date-Time"

},

"long-Type": {

"type": "string",

"format": "long"

},

"uri-Type": {

"type": "string"

},

"header-Type": {

"description": "Header used in notifications as notification header",

"type": "object",

"properties": {

"uri": {

"$ref": "#/components/schemas/uri-Type"

},

"notificationId": {

"$ref": "#/components/schemas/notificationId-Type"

},

"notificationType": {

"$ref": "#/components/schemas/notificationType-Type"

},

"eventTime": {

"$ref": "#/components/schemas/dateTime-Type"

},

"systemDN": {

"$ref": "#/components/schemas/systemDN-Type"

}

}

},

"className-PathType": {

"type": "string"

},

"id-PathType": {

"type": "string"

},

"subscriptionId-PathType": {

"type": "string"

},

"consumerReferenceId-QueryType": {

"$ref": "#/components/schemas/uri-Type"

},

"fields-QueryType": {

"type": "array",

"items": {

"type": "string"

}

},

"filter-QueryType": {

"type": "string"

},

"scope-QueryType": {

"type": "string"

},

"resourceCreation-RequestType": {

"type": "object",

"properties": {

"data": {

"$ref": "#/components/schemas/resourceRepresentation-Type"

}

}

},

"resourceModification-RequestType": {

"type": "object"

},

"subscription-RequestType": {

"type": "object",

"properties": {

"data": {

"$ref": "#/components/schemas/subscription-ResourceType"

}

}

},

"error-ResponseType": {

"type": "object",

"properties": {

"error": {

"type": "object",

"properties": {

"errorInfo": {

"type": "string"

}

}

}

}

},

"resourceCreation-ResponseType": {

"type": "object",

"properties": {

"data": {

"$ref": "#/components/schemas/resourceRepresentation-Type"

}

}

},

"resourceDeletion-ResponseType": {

"type": "object",

"properties": {

"data": {

"type": "array",

"items": {

"$ref": "#/components/schemas/uri-Type"

}

}

}

},

"resourceModification-ResponseType": {

"type": "object",

"properties": {

"data": {

"type": "array",

"items": {

"$ref": "#/components/schemas/resourceRepresentation-Type"

}

}

}

},

"resourceRetrieval-ResponseType": {

"type": "object",

"properties": {

"data": {

"type": "array",

"items": {

"$ref": "#/components/schemas/resourceRepresentation-Type"

}

}

}

},

"subscription-ResponseType": {

"type": "object",

"properties": {

"data": {

"$ref": "#/components/schemas/subscription-ResourceType"

}

}

},

"resourceRepresentation-Type": {

"type": "object",

"properties": {

"href": {

"$ref": "#/components/schemas/uri-Type"

},

"class": {

"type": "string"

},

"id": {

"type": "string"

},

"attributes": {

"type": "object"

}

}

},

"subscription-ResourceType": {

"type": "object",

"properties": {

"consumerReference": {

"$ref": "#/components/schemas/uri-Type"

},

"timeTick": {

"$ref": "#/components/schemas/long-Type"

},

"filter": {

"$ref": "#/components/schemas/filter-Type"

}

}

},

"notifyMOICreation-NotifType": {

"type": "object",

"properties": {

"header": {

"$ref": "#/components/schemas/header-Type"

},

"body": {

"type": "object",

"properties": {

"correlatedNotifications": {

"type": "array",

"items": {

"$ref": "#/components/schemas/correlatedNotification-Type"

}

},

"additionalText": {

"$ref": "#/components/schemas/additionalText-Type"

},

"sourceIndicator": {

"$ref": "#/components/schemas/sourceIndicator-Type"

},

"attributeList": {

"type": "array",

"items": {

"$ref": "#/components/schemas/attributeNameValuePair-Type"

}

}

}

}

}

},

"notifyMOIDeletion-NotifType": {

"type": "object",

"properties": {

"header": {

"$ref": "#/components/schemas/header-Type"

},

"body": {

"type": "object",

"properties": {

"correlatedNotifications": {

"type": "array",

"items": {

"$ref": "#/components/schemas/correlatedNotification-Type"

}

},

"additionalText": {

"$ref": "#/components/schemas/additionalText-Type"

},

"sourceIndicator": {

"$ref": "#/components/schemas/sourceIndicator-Type"

},

"attributeList": {

"type": "array",

"items": {

"$ref": "#/components/schemas/attributeNameValuePair-Type"

}

}

}

}

}

},

"notifyMOIAttributeValueChange-NotifType": {

"type": "object",

"properties": {

"header": {

"$ref": "#/components/schemas/header-Type"

},

"body": {

"type": "object",

"properties": {

"correlatedNotifications": {

"type": "array",

"items": {

"$ref": "#/components/schemas/correlatedNotification-Type"

}

},

"additionalText": {

"$ref": "#/components/schemas/additionalText-Type"

},

"sourceIndicator": {

"$ref": "#/components/schemas/sourceIndicator-Type"

},

"attributeList": {

"type": "array",

"items": {

"$ref": "#/components/schemas/attributeNameValuePair-Type"

}

}

}

}

}

},

"additionalText-Type": {

"type": "string"

},

"correlatedNotification-Type": {

"type": "object",

"properties": {

"source": {

"$ref": "#/components/schemas/uri-Type"

},

"notificationIds": {

"type": "array",

"items": {

"$ref": "#/components/schemas/notificationId-Type"

}

}

}

},

"filter-Type": {

"type": "string"

},

"notificationId-Type": {

"$ref": "#/components/schemas/long-Type"

},

"notificationType-Type": {

"type": "string",

"enum": [

"notifyMOICreation",

"notifyMOIDeletion",

"notifyMOIAttributeValueChange"

]

},

"sourceIndicator-Type": {

"type": "string",

"enum": [

"resourceOperation",

"mangementOperation",

"sONOperation",

"unknown"

]

},

"systemDN-Type": {

"type": "string"

}

}

}

}

### A.1.X	JSON schema of ‘prov3gppFields’ for integration with ONAP VES

{

"prov3gppFields": {

"description": "fields specific to 3GPP provisioning management service notifications for integration with ONAP VES",

"type": "object",

"properties": {

"prov3gppFieldsVersion": {

"description": "version of the prov3gppFields block",

"type": "string",

"enum": ["16.1"]

},

"provisioningMnSNotifications": {

"oneOf": [

{"$ref": "#/definitions/notifyMOICreation-NotifType"},

{"$ref": "#/definitions/notifyMOIDeletion-NotifType"},

{"$ref": "#/definitions/notifyMOIAttributeValueChange-NotifType"}

]

}

},

"definitions": {

"notifyMOICreation-NotifType": {

"description": "fields specific to 3GPP notification NotifyMOICreation",

"type": "object",

"properties": {

"dN": {

"$ref": "#/definitions/dN-Type"

},

"notificationType": {

"$ref": "#/definitions/notificationType-Type"

},

"systemDN": {

"$ref": "#/components/schemas/systemDN-Type"

},

"correlatedNotifications": {

"type": "array",

"items": {

"$ref": "#/definitions/correlatedNotification-Type"

}

},

"attributeList": {

"type": "array",

"items": {

"$ref": "#/definitions/attributeNameValuePair-Type"

}

},

"additionalText": {

"$ref": "#/definitions/additionalText-Type"

}

},

"additionalProperties": false,

"required": [ "dN", "notificationType", "systemDN" ]

},

"notifyMOIDeletion-NotifType": {

"description": "fields specific to 3GPP notification NotifyMOIDeletion",

"type": "object",

"properties": {

"dN": {

"$ref": "#/definitions/dN-Type"

},

"notificationType": {

"$ref": "#/definitions/notificationType-Type"

},

"systemDN": {

"$ref": "#/components/schemas/systemDN-Type"

},

"correlatedNotifications": {

"type": "array",

"items": {

"$ref": "#/definitions/correlatedNotification-Type"

}

},

"attributeList": {

"type": "array",

"items": {

"$ref": "#/definitions/attributeNameValuePair-Type"

}

},

"additionalText": {

"$ref": "#/definitions/additionalText-Type"

}

},

"additionalProperties": false,

"required": [ "dN", "notificationType", "systemDN" ]

},

"notifyMOIAttributeValueChange-NotifType": {

"description": "fields specific to 3GPP notification NotifyMOIAttributeValueChange",

"type": "object",

"properties": {

"dN": {

"$ref": "#/definitions/dN-Type"

},

"notificationType": {

"$ref": "#/definitions/notificationType-Type"

},

"systemDN": {

"$ref": "#/components/schemas/systemDN-Type"

},

"correlatedNotifications": {

"type": "array",

"items": {

"$ref": "#/definitions/correlatedNotification-Type"

}

},

"attributeValueChange": {

"type": "array",

"items": {

"$ref": "#/definitions/attributeValueChange-Type"

}

},

"additionalText": {

"$ref": "#/definitions/additionalText-Type"

}

},

"additionalProperties": false,

"required": [ "dN", "notificationType", "systemDN", "attributeValueChange" ]

},

"dN-Type": {

"type": "string"

},

"notificationType-Type": {

"type": "string",

"enum": [

"notifyMOICreation",

"notifyMOIDeletion",

"notifyMOIAttributeValueChange"

]

},

"systemDN-Type": {

"type": "string"

},

"correlatedNotification-Type": {

"type": "object",

"properties": {

"source": {

"$ref": "#/definitions/dN-Type"

},

"notificationIds": {

"type": "array",

"items": {

"$ref": "#/definitions/notificationId-Type"

}

}

}

},

"attributeValueChange-Type": {

"type": "object",

"properties": {

"attributeName": {

"type": "string"

},

"oldAttributeValue": {},

"newAttributeValue": {}

}

},

"attributeNameValuePair-Type": {

"type": "object",

"properties": {

"attributeName": {

"type": "string"

},

"attributeValue": {}

}

},

"additionalText-Type": {

"type": "string"

},

"notificationId-Type": {

"$ref": "#/components/schemas/long-Type"

}

}

}

}

End