| 3GPP TR 28.aaa V0.0.0 (2019-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Heartbeat (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions of terms, symbols and abbreviations	5

3.1	Terms	5

3.2	Abbreviations	5

4	General	6

4.1	Overview	6

5	Specification level requirements	6

5.1	Use cases	6

5.1.1	Configuring heartbeat notification periodicity	6

5.1.2	Requesting immediate heartbeat notification	7

5.1.3	Emitting periodic heartbeat notifications	7

5.2	Requirements	8

5.2.1	Requirements for controlling heartbeat	8

5.2.2	Requirements for notifying heartbeat	8

6	Heartbeat management services	8

7	Procedures for heartbeat management services	9

7.1	Procedure for configuring heartbeat notification periodicity	9

7.2	Procedure for requesting immediate heartbeat notification	9

7.3	Procedure for notifying periodic heartbeat notifications	10

A.1	Procedure for configuring heartbeat notification periodicity	11

A.2	Procedure for requesting immediate heartbeat notification	11

A.3	Procedure for notifying periodic heartbeat notifications	12

Annex <B> (informative): Change history	13

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

To ensure the availability and reliability of the management, an automatic surveillance of the communication between management service (MnS) consumers and management service producers is required. The heartbeat management service is defined as a capability to achieve this goal.

# 1	Scope

The present document specifies use cases, requirements, management services and procedures for the heartbeat management service.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[3]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[4]	3GPP TS 32.351: "Telecommunication management; Communication Surveillance (CS) Integration Reference Point (IRP); Requirements".

[5]	3GPP TS 32.352: "Telecommunication management; Communication Surveillance (CS) Integration Reference Point (IRP); Information Service (IS)".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

MnS	Management Service

# 4	General

## 4.1	Overview

The communication between Management Service (MnS) producers and MnS consumers shall be monitored, and communication link breaks between them shall be discovered by MnS consumers as early as possible. The behaviour of the MnS consumers, after detection of communication failure, is outside the scope of this standardization.

# 5	Specification level requirements

## 5.1	Use cases

### 5.1.1	Configuring heartbeat notification periodicity

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To configure the periodicity at which the heartbeat management service producer shall emit heartbeat notifications to its authorized heartbeat management service consumer. |  |
| Actors and Roles | An authorized consumer of the heartbeat management service. |  |
| Telecom resources | The heartbeat management service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | The periodicity requested by the heartbeat management service consumer has a valid value. |  |
| Begins when | The heartbeat management service consumer sends a request to the heartbeat management service producer to set the periodicity at which it shall emit heartbeat notifications. |  |
| Step 1 | The heartbeat management service producer receives the request and sets its internal countdown timer to a value (which can be zero) equal to the periodicity requested by the heartbeat management service consumer. |  |
| Step 2 | The heartbeat management service producer sends a heartbeat notification to all authorized heartbeat management service consumer(s), provided they previously subscribed to heartbeat notifications.. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The heartbeat notification periodicity has been configured according to the heartbeat management service consumer request. A heartbeat notification is sent out to all authorized heartbeat management service consumer(s). |  |
| Traceability | REQ-HB-CTRL-FUN-2. |  |

### 5.1.2	Requesting immediate heartbeat notification

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To trigger the emission of an immediate heartbeat notification by the heartbeat management service producer. |  |
| Actors and Roles | An authorized consumer of the heartbeat management service. |  |
| Telecom resources | The heartbeat management service producer. |  |
| Assumptions | There are many types of notifications. This UC assumes all types of notification travel over the same communication link. |  |
| Pre-conditions | N/A |  |
| Begins when | The soliciting heartbeat management service consumer sends a request to the heartbeat management service producer to emit immediately a heartbeat notification. |  |
| Step 1 | The heartbeat management service producer receives the request and sends immediately a heartbeat notification to: EITHER the soliciting heartbeat management service consumer only, provided he previously subscribed to heartbeat notifications OR all authorized heartbeat management service consumer(s) who had previously subscribed to heartbeat notifications. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The immediate heartbeat notification has been emitted according to the soliciting heartbeat management service consumer request. |  |
| Traceability | REQ-HB-CTRL-FUN-3, REQ-HB-NOTIF-FUN-2. |  |

### 5.1.3	Emitting periodic heartbeat notifications

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To send periodic heartbeat notifications at the periodicity requested by the heartbeat management service consumer. |  |
| Actors and Roles | An authorized producer of the heartbeat management service. |  |
| Telecom resources | The heartbeat management service consumer. |  |
| Assumptions | The heartbeat notification periodicity has been configured according to the heartbeat management service consumer request. There are many types of notifications. This UC assumes all types of notification travel over the same communication link. | Configuring heartbeat notification periodicity |
| Pre-conditions | N/A |  |
| Begins when | The internal countdown timer managed by the heartbeat management service producer has reached the value 0. |  |
| Step 1 | The heartbeat management service producer sends a heartbeat notification to all authorized heartbeat management service consumer(s), provided they previously subscribed to heartbeat notifications.. |  |
| Step 2 | The heartbeat management service producer resets its internal countdown timer to the value initially configured by the heartbeat management service consumer. | Configuring heartbeat notification periodicity |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The periodic heartbeat notification has been emitted to all authorized heartbeat management service consumer(s) at the requested periodicity. |  |
| Traceability | REQ-HB-NOTIF-FUN-1. |  |

## 5.2	Requirements

### 5.2.1	Requirements for controlling heartbeat

REQ-HB-CTRL-FUN-1:  The heartbeat management service provider shall have the capability to allow its authorized consumer to read the heartbeat period.

REQ-HB-CTRL-FUN-2:  The heartbeat management service provider shall have the capability to allow its authorized consumer to set the heartbeat period.

REQ-HB-CTRL-FUN-3:  The heartbeat management service provider shall have the capability to allow its authorized consumer to request the emission of an immediate heartbeat notification.

### 5.2.2	Requirements for notifying heartbeat

REQ-HB-NOTIF-FUN-1:  The heartbeat management service provider shall have the capability to send periodic heartbeat notifications to its authorized consumer at the frequency specified by the heartbeat management service consumer.

REQ-HB-NOTIF-FUN-2:  The heartbeat management service provider shall have the capability to send immediate heartbeat notifications to its authorized consumer, upon request from the heartbeat management service consumer.

# 6	Heartbeat management services

The heartbeat management services are listed in table 6-1.

Table 6-1: Heartbeat management services

| MnS Name | MnS Component Type A (operations and notifications) | MnS Component Type B (information model) | Note |
| --- | --- | --- | --- |
| HeartbeatControl | Operations defined in TS 28.532 [3] clause 10.X.1: - getHeartbeatPeriod operation - setHeartbeatPeriod operation - triggerHeartbeat operation | ManagedFunction / ManagedElement / SubNetwork defined in TS 28.622 [2]. | This management service enables its consumer to control heartbeat. |
| HeartbeatNotification | Notifications defined in TS 28.532 [3] clause 10.X.1: - notifyHeartbeat notification | ManagedFunction / ManagedElement / SubNetwork defined in TS 28.622 [2]. | This management service enables its consumer to receive heartbeat notifications. |

# 7	Procedures for heartbeat management services

## 7.1	Procedure for configuring heartbeat notification periodicity

Figure 7.1-1 illustrates the procedure for configuring the periodicity of heartbeat notifications.

Figure 7.1-1: Procedure for configuring heartbeat notification periodicity

## 7.2	Procedure for requesting immediate heartbeat notification

Figure 7.2-1 illustrates the procedure for requesting immediate heartbeat notification.

Figure 7.2-1: Procedure for requesting immediate heartbeat notification

## 7.3	Procedure for notifying periodic heartbeat notifications

Figure 7.3-1 illustrates the procedure for notifying periodic heartbeat notifications.

Figure 7.3-1: Procedure for notifying periodic heartbeat notifications

Annex <A> (informative): PlantUML source code

## A.1	Procedure for configuring heartbeat notification periodicity

The following PlantUML source code is used to describe the procedure for configuring heartbeat notification periodicity, as depicted by Figure 7.1-1:

@startuml

title "Configuring heartbeat notification periodicity"

actor "Heartbeat MnS Consumer" as CONS

participant "Heartbeat MnS Provider" as PROV

CONS -> PROV: setHeartbeatPeriod  (New HeartbeatPeriod)

rnote over PROV

Internal Countdown Timer := New HeartbeatPeriod

endrnote

PROV -> CONS: notifyHeartbeat notification

note left

Provided he previously

subscribed to heartbeat

notifications

end note

rnote over PROV

Start countdown immediately

endrnote

@enduml

## A.2	Procedure for requesting immediate heartbeat notification

The following PlantUML source code is used to describe the procedure for requesting immediate heartbeat notification, as depicted by Figure 7.2-1:

@startuml

title "Requesting immediate heartbeat notification"

actor "Heartbeat MnS Consumer" as CONS

participant "Heartbeat MnS Provider" as PROV

CONS -> PROV: triggerHeartbeat

rnote over PROV

Send immediate heartbeat notification

endrnote

alt option a

rnote over PROV

only to soliciting Heartbeat MnS Consumer

provided he previously subscribed

to heartbeat notifications

endrnote

else option b

rnote over PROV

to all Heartbeat MnS Consumers

who had previously subscribed

to heartbeat notifications

endrnote

end

PROV -> CONS: notifyHeartbeat

@enduml

## A.3	Procedure for notifying periodic heartbeat notifications

The following PlantUML source code is used to describe the procedure for notifying periodic heartbeat notifications, as depicted by Figure 7.3-1:

@startuml

title "Notifying periodic heartbeat notifications"

participant "Heartbeat MnS Consumer" as CONS

actor "Heartbeat MnS Provider" as PROV

PROV -> CONS: notifyHeartbeat

rnote over PROV

Reinitialize its internal countdown

timer to the value of heartbeatPeriod

endrnote

@enduml

Annex <B> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |