3GPP TSG-SA5 Meeting #129e 	S5-201323

- e-meeting, 24 February – 4 March 2020

- Source:	Huawei, China Mobile，China Unicom, ZTE

- Title:	Update NE deployment scenario example for classification of network autonomy level

- Document for:	Approval

- Agenda Item:	6.6.5

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	3GPP TR 28.810: “Study on concept, requirements and solutions for levels of autonomous network v0.2.0”.

# 3	Rationale

This contribution proposes to update NE provisioning scenario example for classification of network autonomy in following two aspects:

- Add the entire workflow of RAN NE provisioning

- Add the level of network autonomy for RAN NE provisioning

# 4	Detailed proposal

It proposes to make the following changes to TR 28.810[1].

1st Change

## 5.3	RAN NE provisioning scenario example for classification of network autonomy

### 5.3.1 Introduction

RAN NE provisioning scenario refers to the entire workflow of deploying an RAN NE, full autonomy of RAN NE provisioning can help the network operator to reduce OPEX by reducing manual involvement in such tasks. However, full autonomy of RAN NE provisioning is a long term goal, it will be beneficial for operator to achieve this goal step by step and have clear view on which typical issues can be addressed by utilizing network autonomy mechanism in corresponding steps.

### 5.3.2 Entire workflow

Following are the entire workflow for RAN NE provisioning:

-	Task A: Deciding to provision a RAN NE.

-	Task B: Analysing and determining the automatic radio network data and automatic transport network data for the new RAN NE to be provisioned.

-	Task C: Generating the network configuration data based on the automatic network configuration data (i.e. automatic radio configuration data, automatic transport configuration data) for the RAN NE and hardware information collected.

-	Task D: Connecting the RAN NE to the management system (including NE acquire its IP address and corresponding management system IP address) and collecting the RAN NE information (e.g. hardware information).

-	Task E: Downloading and activing network configuration data and software in RAN NE.

### 5.3.3 Potential classification of network autonomy for RAN NE provisioning

Following are the potential classification of network autonomy for RAN NE provisioning:

- Level 1:

  - Network system executes tasks of downloading and activing the available RAN NE network configuration data (i.e. transport and radio configuration data) and software prepared by human (Task E).

  - Human decide to provison a RAN NE and prepares the network configuration data for the RAN NE (Task A, Task B, Task C). Human connect the RAN NE to the management system and collect the RAN NE information (Task D).

- level 2:

  - Compared to Level 1, network system additionally executes the tasks of connecting RAN NE to the management systemand collecting the RAN NE information (Task D).

  - Human decide to provision a RAN NE and prepares the network configuration data  (Task A, Task B, Task C).

- Level 3:

  - Compared to Level 2, network system additionally executes tasks of analysing and determining the network configuration data for the RAN NE based on the automatic network configuration data specified by human (Tack C).

  - Human decide to provision a RAN NE and prepares the automatic network configuration data (Task A and Task B).

- Level 4:

  - Compared to Level 3, network system additionally execute the task of analysing and determining automatic network configuration data for the RAN NE to be provisioned (Task B) (Task A).

  - Human decide to provision a RAN NE (Task A).

- Level 5: network system can autonomously execute the entire workflow of provisioning an RAN NE, which means the network system can achieve the full autonomy for RAN NE provisioning.

End of changes