3GPP TSG-SA5 Meeting #129e 	S5-201383

- e-meeting, 24 February – 4 March 2020

- Source:	Nokia, Nokia Shanghai Bell

- Title:	TD proposal for tolerance indicator attribute

- Document for:	Discussion and Endorsement

Agenda Item:	6.4.14

# 1	Decision/action requested

Please discuss and endorse

# 2	References

[1]	3GPP TS 28.541:  "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and Stage 3".

# 3	Rationale

Delay Tolerance of  GST/NEST Provide the NSC with service delivery flexibility, especially for the vertical services that are not chasing a high system performance. For instance, the service will be delivered once the mobile system has sufficient resources or during the off-peak hours. For this type of traffic, it is not too critical how long it takes to deliver the amount of data, e.g. within hours, days,weeks, etc.

The similar approach can be applied on other performance related requirements also, e.g. thoughput, isolation level, user density and other QoS related parameters. Therefore, we could consider to put this kind “tolerance indicator” to common part of the SLS related attibutes.

# 4	Detailed proposal

Propose to add a tolerance related attribute in common properties of service related attribute to indicate if the requiremet could tolerate incompliance in specific situation.

ServAttrCom <<dataType>>

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| category | M | T | F | F | T |
| tagging | CM | T | F | F | T |
| exposure | M | T | F | F | T |
| toleranceIndicator | O | T | T | F | T |

Note: the value of toleranceIndicator can be support or non support