3GPP TS 28.550 V15.4.0 (2020-03)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Management and orchestration;

Performance assurance

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

performance, assurance, management, orchestration

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	8

1	Scope	9

2	References	9

3	Definitions and abbreviations	10

3.1	Definitions	10

3.2	Abbreviations	10

4	Concepts and overview	10

4.1	Overview	10

4.2	Management data analytics	10

4.3	PM services	11

5	Specification level requirements	11

5.1	Use cases	11

5.1.0	Introduction	11

5.1.1	NF PM services	12

5.1.1.1	NF measurement job control service	12

5.1.1.1.1	Creation of measurement job for NF(s)	12

5.1.1.1.2	Termination of measurement job for NF(s)	12

5.1.1.1.3	Query of measurement jobs for NF(s)	13

5.1.1.2	NF performance data file reporting service	13

5.1.1.2.1	3GPP NF performance data file reporting	13

5.1.1.3	NF performance data streaming service	14

5.1.1.3.1	3GPP NF performance data streaming	14

5.1.2	NSSI PM services	15

5.1.2.1	NSSI measurement job control service	15

5.1.2.1.1	Creation of measurement job for NSSI(s)	15

5.1.2.1.2	Termination of measurement job for NSSI(s)	16

5.1.2.1.3	Query of measurement jobs for NSSI(s)	16

5.1.2.2	NSSI performance data file reporting service	17

5.1.2.2.1	NSSI performance data file reporting	17

5.1.2.3	NSSI performance data streaming service	17

5.1.2.3.1	NSSI performance data streaming	17

5.1.3	NSI PM services	18

5.1.3.1	NSI measurement job control service	18

5.1.3.1.1	Creation of measurement job for NSI(s)	18

5.1.3.1.2	Termination of measurement job for NSI(s)	19

5.1.3.1.3	Query of measurement jobs for NSI(s)	20

5.1.3.2	NSI performance data file reporting service	20

5.1.3.2.1	NSI performance data file reporting	20

5.1.3.3	NSI performance data streaming service	21

5.1.3.3.1	NSI performance data streaming	21

5.1.4	Network/Sub-network PM services	22

5.1.4.1	Network/Sub-network measurement job control service	22

5.1.4.1.1	Creation of measurement job for network(s)/sub-network(s)	22

5.1.4.1.2	Termination of measurement job for network(s)/sub-network(s)	23

5.1.4.1.3	Query of measurement jobs for network(s)	23

5.1.4.2	Network/Sub-network performance data file reporting service	24

5.1.4.2.1	Network/Sub-network performance data file reporting	24

5.1.4.3	Network/Sub-network performance data streaming service	24

5.1.4.3.1	Network/Sub-network performance data streaming	24

5.1.5	Management data analytics	25

5.1.5.1	Management data analytics for NSIs/NSSIs	25

5.1.5.2	Management data analytics for network	26

5.2	Requirements	26

5.2.1	Requirements for NF measurement job control service	26

5.2.2	Requirements for NF performance data file reporting service	27

5.2.3	Requirements for NF performance data streaming service	27

5.2.4	Requirements for NSSI measurement job control service	27

5.2.5	Requirements for NSSI performance data file reporting service	27

5.2.6	Requirements for NSSI performance data streaming service	27

5.2.7	Requirements for NSI measurement job control service	27

5.2.8	Requirements for NSI performance data file reporting service	28

5.2.9	Requirements for NSI performance data streaming service	28

5.2.10	Requirements for network/sub-network measurement job control service	28

5.2.11	Requirements for network/sub-network performance data file reporting service	29

5.2.12	Requirements for network/sub-network performance data streaming service	29

5.2.13	Management data analytics service	29

6.	Performance assurance specific operations and notifications	29

6.1	Measurement job control related operations	29

6.1.1	Operation createMeasurementJob (M)	29

6.1.1.1	Definition	29

6.1.1.2	Input parameters	30

6.1.1.3	Output parameters	32

6.1.1.4	Exceptions	32

6.1.2	Operation stopMeasurementJob (M)	32

6.1.2.1	Definition	32

6.1.2.2	Input parameters	33

6.1.2.3	Output parameters	33

6.1.2.4	Exceptions	33

6.1.3	Operation listMeasurementJobs (M)	33

6.1.3.1	Definition	33

6.1.3.2	Input parameters	33

6.1.3.3	Output parameters	34

6.1.3.4	Exceptions	34

6.2	Performance data streaming related operations	35

6.2.1	Operation establishStreamingConnection (M)	35

6.2.1.1	Definition	35

6.2.1.2	Input parameters	35

6.2.1.3	Output parameters	36

6.2.1.4	Exceptions	36

6.2.2	Operation terminateStreamingConnection (M)	36

6.2.2.1	Definition	36

6.2.2.2	Input parameters	36

6.2.2.3	Output parameters	36

6.2.2.4	Exceptions	37

6.2.3	Operation reportStreamData (M)	37

6.2.3.1	Definition	37

6.2.3.2	Input parameters	37

6.2.3.3	Output parameters	37

6.2.3.4	Exceptions	37

6.2.4	Operation getStreamInfo (M)	38

6.2.4.1	Definition	38

6.2.4.2	Input parameters	38

6.2.4.3	Output parameters	38

6.2.4.4	Exceptions	38

6.2.5	Operation addStreamInfo (M)	39

6.2.5.1	Definition	39

6.2.5.2	Input parameters	39

6.2.5.3	Output parameters	40

6.2.5.4	Exceptions	40

6.2.6	Operation updateStreamInfo (M)	40

6.2.6.1	Definition	40

6.2.6.2	Input parameters	40

6.2.6.3	Output parameters	41

6.2.6.4	Exceptions	41

6.2.7	Operation deleteStreamInfo (M)	41

6.2.7.1	Definition	41

6.2.7.2	Input parameters	41

6.2.7.3	Output parameters	41

6.2.7.4	Exceptions	41

7.	Performance assurance services components	41

7.1	Measurement job control services	41

7.2	Performance data file reporting services	42

7.3	Performance data streaming services	43

7.4	Void	44

8	RESTful HTTP-based solution set of performance measurement job control service specific operations and notifications	44

8.1	Mapping of operations	44

8.1.1	Introduction	44

8.1.2	Operation "createMeasurementJob"	45

8.1.3	Operation "listMeasurementJobs"	45

8.1.4	Operation "stopMeasurementJob"	46

8.2	Resources	46

8.2.0	Resource structure	46

8.2.1	Resource definitions	46

8.2.1.1	Void	46

8.2.1.2	Resource “/measJobs”	46

8.2.1.2.1	Description	46

8.2.1.2.2	URI	47

8.2.1.2.3	HTTP methods	47

8.2.1.3	Resource “/measJobs/{jobId}”	48

8.2.1.3.1	Description	48

8.2.1.3.2	URI	48

8.2.1.3.3	HTTP methods	48

8.3	Data type definitions	50

8.3.1	General	50

8.3.2	Void	50

8.3.3	Void	50

8.3.4	Structured general data types	50

8.3.5	Structured path data types	50

8.3.6	Query, message body and resource data types	51

8.3.6.1	Type measJobCreation-RequestType	51

8.3.6.2	Type measJobCreation-ResponseType	51

8.3.6.3	Type measJobsRetrieval-ResponseType	51

8.3.6.4	Type error-ResponseType	51

8.3.6.5	Type measJobInfo-ResourceType	52

8.3.7	Referenced structured data types	52

8.3.7.1	Type schedule-Type	52

8.3.7.2	Type timeInterval-Type	52

8.3.7.3	Type scheduleOfDay-Type	52

8.3.7.4	Void	53

8.3.7.5	Type unsupportedMeas-Type	53

8.3.8	Simple data types and enumerations	53

8.3.8.1	General	53

8.3.8.2	Simple data types	53

8.3.8.3	Enumeration reportingMethod-Type	53

8.3.8.4	Enumeration priority-Type	53

8.3.8.5	Enumeration scheduleOption-Type	53

8.3.8.6	Enumeration dayOfWeek-Type	54

9	RESTful HTTP-based solution set of performance data streaming operations with WebSocket-based extensions	54

9.1	Introduction	54

9.2	Mapping of operations	54

9.2.1	Introduction	54

9.2.2	Operation "establishStreamingConnection"	54

9.2.3	Operation "terminateStreamingConnection"	56

9.2.4	Operation "reportStreamData"	57

9.2.5	Operation "getStreamInfo"	57

9.2.6	Operation "addStreamInfo"	58

9.2.7	Operation "updateStreamInfo"	58

9.2.8	Operation "deleteStreamInfo"	59

9.3	Resources	59

9.3.1	Resource structure	59

9.3.2	Resource definitions	60

9.3.2.1	Resource “/streamInfoList”	60

9.3.2.1.1	Description	60

9.3.2.1.2	URI	60

9.3.2.1.3	HTTP methods	60

9.3.2.1a	Resource “/streamInfoList/{streamId}”	62

9.3.2.1a.1	Description	62

9.3.2.1a.2	URI	62

9.3.2.1a.3	HTTP methods	62

9.3.2.2	Resource “/streamingConnection”	63

9.3.2.2.1	Description	63

9.3.2.2.2	URI	64

9.3.2.2.3	HTTP methods	64

9.4	Data type definitions	65

9.4.1	General	65

9.4.2	Structured general data types	66

9.4.3	Structured path data types	66

9.4.4	Query, message body and resource data types	66

9.4.4.1	Type streamInfoListPost-RequestType	66

9.4.4.2	 Type error-ResponseType	66

9.4.4.3	Type streamInfoToUpdate-RequestType	66

9.4.4.4	Type streamInfoListPost-ResponseType	66

9.4.4.5	Type streamInfoRetrieval-ResponseType	66

9.4.4.6	Type streamInfoListRetrieval-ResponseType	66

9.4.4.7	Type streamInfoUpdate-ResponseType	67

9.4.4.8	Type streamInfoListUpdate-ResponseType	67

9.4.5	Referenced structured data types	67

9.4.5.1	Type streamInfoIn-Type	67

9.4.5.2	Type streamInfoOut-Type	67

9.4.6	Simple data types and enumerations	67

9.4.6.1	General	67

9.4.6.2	Simple data types	68

Annex A :	 Void	69

Annex B (informative):	 Procedures for performance assurance services	70

B.1	NF measurement job creation	70

B.2	NSSI measurement job creation	71

B.3	NSI measurement job creation	72

B.4	Network measurement job creation	74

B.5	NF measurement job termination	75

B.6	NSSI measurement job termination	76

B.7	NSI measurement job termination	77

B.8	Network measurement job termination	78

Annex C (normative):	 Performance Data Stream Unit content description	79

Annex D (informative):	 Performance data streaming holistic sequence	80

D.1	Performance data streaming for starting measurement collection	80

D.1.1	Sequence flow	80

D.1.2	PlantUML codes	81

D.2	Performance data streaming for stopping measurement collection	82

D.2.1	Sequence flow	82

D.2.2	PlantUML codes	83

Annex E (normative):	 OpenAPI specification	85

E.1	Introduction	85

E.2	Performance assurance service	85

E.3	Performance data streaming service	90

Annex F (normative):	 ASN.1 definition for performance data stream units	98

F.1	ASN.1 definition rule	98

F.2	ASN.1 definition	99

Annex F (informative):	Change history	101

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies the stage 1, 2 and 3 of performance assurance related management services for 5G networks including network slicing.

The present document does not specify the performance measurements.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[3]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[4]	ITU-T Recommendation X.721 (1992): "Information technology - Open Systems Interconnection - Structure of management information: Definition of management information".

[5]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[6]	ISO 8601:2000(E) Data elements and interchange formats – Information interchange – Representation of dates and times".

[7]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[8]	Void

[9]	Void

[10]	Void

[11]	Void

[12]	Void

[13]	3GPP TS 28.628: "Telecommunication management; Self-Organizing Networks (SON) Policy Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[14]	3GPP TS 32.158: "Management and orchestration; Design rules for Representational State Transfer (REST) Solution Sets (SS)".

[15]	ITU-T Recommendation X.680 (08/2015) "Information Technology - Abstract Syntax Notation One (ASN.1): Specification of basic notation" (Same as the ISO/IEC International Standard 8824-1).

[16]	ITU-T Recommendation X.681 (08/2015) "Information Technology - Abstract Syntax Notation One (ASN.1): Information object specification" (Same as the ISO/IEC International Standard 8824-2).

[17]	ITU-T Recommendation X.691 (08/2015) "Information technology - ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)" (Same as the ISO/IEC International Standard 8825-2).

[18]	IETF RFC 6455: "The WebSocket Protocol".

[19]	IETF RFC 793: "TRANSMISSION CONTROL PROTOCOL".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

# 4	Concepts and overview

## 4.1	Overview

The 5G networks and network slicing are designed to support eMBB, URLLC and mIoT services. Some services have ultra-low latency, high data capacity, and strict reliability requirements, as any faults or performance issues in the networks can cause service failure which may result in property damage and body injury. Therefore, it is necessary to collect real-time performance data that can be used by analytic applications (e.g., network optimization, SON, etc.) to detect the potential issues in advance, and take appropriate actions to prevent or mitigate the issues. Also, the performance data shall be able to be consumed by multiple analytic applications with specific purposes.

## 4.2	Management data analytics

The raw performance data of NFs of the mobile network can be analysed, together with other management data (e.g., alarm information, configuration data),and formed into one or more management analytical data for NFs, sub-networks, NSSIs or NSIs. The management analytical data can be used to diagnose ongoing issues impacting the performance of the mobile network and predict any potential issues (e.g., potential failure and/or performance degradation). For example, the analysis of NSI/NSSI resource usage can form a management analytical data  indicating whether a certain resource is deteriorating. The analysis and correlation of the overall performance data of mobile network may indicate overload situation and potential failure(s).

SON Capacity and Coverage Optimization (CCO) is one typical case that may consume the management analytical data. CCO provides optimal coverage and capacity for the E-UTRAN, see clause 4.5 of TS 28.628 [13], which may also be applicable for 5G radio networks. The management analytical data related to coverage and capacity help the SON CCO to realise the situation of coverage and capacity or interference, and  to trigger corresponding optimization if needed.

Note: 	Details of the management analytical data including e.g. format, categorisation and method/algorithm of calculations are to be defined.

## 4.3	PM services

The PM for 5G networks and network slicing is comprised of the management services listed in the table 4.3-1 below:

Table 4.3-1: PM services for 5G networks and network slicing

| Management service | Description |
| --- | --- |
| Measurement job control service for NF | The management service for creating, terminating and querying the measurement job(s) for the NF(s). |
| Performance data file reporting service for NF | The management service for reporting the NF performance data file. |
| Performance data streaming service for NF | The management service for reporting the NF performance data steam. |
| Measurement job control service for NSSI | The management service for creating, terminating and querying the measurement job(s) for the NSSI(s). |
| Performance data file reporting service for NSSI | The management service for reporting the NSSI performance data file. |
| Performance data streaming service for NSSI | The management service for reporting the NSSI performance data stream. |
| Measurement job control service for NSI | The management service for creating, terminating and querying the measurement job(s) for the NSI(s). |
| Performance data file reporting service for NSI | The management service for reporting the NSI performance data file. |
| Performance data streaming service for NSI | The management service for reporting the NSI performance data stream. |
| Measurement job control service for network/sub-network | The management service for creating, terminating and querying the measurement job(s) for the network(s)/subnetwork(s). The measurement job for the network(s)/subnetwork(s) is to collect the network/subnetwork performance data that are not specific to network slicing. |
| Performance data file reporting service for network/sub-network | The management service for reporting the file of the network/subnetwork performance data that is not specific to network slicing. |
| Performance data streaming service for network/sub-network | The management service for reporting the stream of the network/subnetwork performance data that is not specific to network slicing. |

# 5	Specification level requirements

## 5.1	Use cases

### 5.1.0	Introduction

The steps of the use cases are logical illustration on how the management service request can be fulfilled. Depending on the deployment scenario, other steps can be used to fulfil the management service request.

### 5.1.1	NF PM services

#### 5.1.1.1	NF measurement job control service

##### 5.1.1.1.1	Creation of measurement job for NF(s)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to create a measurement job for collecting the performance data of NF(s). |  |
| Actors and Roles | An authorized consumer of NF measurement job control service. |  |
| Telecom resources | NF(s); Producer of the NF measurement job control service. |  |
| Assumptions | N/A |  |
| Pre-conditions | - The NF(s) have been deployed. - The NF measurement job control service producer is in operation. |  |
| Begins when | The authorized consumer needs to create measurement job for collecting the performance data of NF(s). |  |
| Step 1 (M) | The authorized consumer requests the NF measurement job control service producer to create measurement job to collect the performance data of NF(s). The request needs to indicate that the performance data needs to be reported by performance data file or by performance data streaming. |  |
| Step 2 (M) | The NF measurement job control service producer requests the NF(s) to collect the performance data, per the received measurement job creation request. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The measurement job for NF(s) has been created, and the NF measurement job control service producer generates the performance data for the NF measurement job. |  |
| Traceability | REQ-MJCS_NF-FUN-1, REQ-MJCS_NF-FUN-2, REQ-MJCS_NF-FUN-3, REQ-MJCS_NF-FUN-4 and REQ-MJCS_NF-FUN-7 |  |

##### 5.1.1.1.2	Termination of measurement job for NF(s)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to request the NF measurement job control service producer to terminate a NF measurement job. |  |
| Actors and Roles | An authorized consumer of NF measurement job control service. |  |
| Telecom resources | NF(s) NF measurement job control service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | The NF measurement job has been created. |  |
| Begins when | The authorized consumer does not need the NF measurement job that is collecting the performance data of NF(s). |  |
| Step 1 (M) | The authorized consumer requests the NF measurement job control service producer to terminate a measurement job that is collecting the performance data of NF(s). |  |
| Step 2 (M) | The NF measurement job control service producer terminates the measurement job and may request the NF(s) to stop collecting the measurements requested by the measurement job. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The NF measurement job is terminated, or still retained but not does not serve the subject consumer anymore. |  |
| Traceability | REQ-MJCS_NF-FUN-5 |  |

##### 5.1.1.1.3	Query of measurement jobs for NF(s)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to query the ongoing NF measurement jobs (i.e. the NF measurement jobs that have been created by the subject consumer and not terminated). |  |
| Actors and Roles | An authorized consumer of NF measurement job control service. |  |
| Telecom resources | NF(s) NF measurement job control service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | The NF measurement job control service producer is in operation. |  |
| Begins when | The authorized consumer needs to query the ongoing NF measurement jobs. |  |
| Step 1 (M) | The authorized consumer queries the information about the ongoing NF measurement jobs from the NF measurement job control service producer. |  |
| Step 2 (M) | The NF measurement job control service producer provides the information about the ongoing NF measurement jobs to the consumer. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The information about the ongoing NF measurements jobs are available to the consumer. |  |
| Traceability | REQ-MJCS_NF-FUN-6 |  |

#### 5.1.1.2	NF performance data file reporting service

##### 5.1.1.2.1	3GPP NF performance data file reporting

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to get the performance data file of 3GPP NF(s). |  |
| Actors and Roles | An authorized consumer of NF performance data file reporting service. |  |
| Telecom resources | Producer of the NF performance data file reporting service. |  |
| Assumptions | N/A |  |
| Pre-conditions | - The 3GPP NF has been deployed. - The NF performance data file reporting service producer is in operation. - The NF performance data file reporting service consumer has subscribed the notification about NF performance data file ready. |  |
| Begins when | The performance data file of 3GPP NF is ready at the NF performance data file reporting service producer. |  |
| Step 1 (M) | The NF performance data file reporting service producer sends the notification about performance data file ready to the authorized consumer. |  |
| Step 2 (M) | The authorized consumer fetches the performance data file from the NF performance data file reporting service producer. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The performance data file of 3GPP NF have been reported. |  |
| Traceability | REQ-PDFR_NF-FUN-1, REQ-PDFR_NF-FUN-2 |  |

#### 5.1.1.3	NF performance data streaming service

##### 5.1.1.3.1	3GPP NF performance data streaming

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to receive the performance data stream of 3GPP NF(s). |  |
| Actors and Roles | An authorized consumer of NF performance data streaming service. |  |
| Telecom resources | Producer of the NF performance data streaming service. |  |
| Assumptions | N/A |  |
| Pre-conditions | - The 3GPP NF has been deployed. - The NF performance data streaming service producer is in operation. - The NF performance data streaming service consumer has subscribed for receiving the performance data stream from the NF performance data streaming service producer. |  |
| Begins when | The performance data of 3GPP NF is ready at the NF performance data streaming service producer. |  |
| Step 1 (M) | The NF performance data streaming service producer sends the NF performance data stream to the consumer. |  |
| Ends when | The NF performance data streaming service consumer receives the performance data stream. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions |  |  |
| Traceability | REQ-PDS_NF-FUN-1 |  |

### 5.1.2	NSSI PM services

#### 5.1.2.1	NSSI measurement job control service

##### 5.1.2.1.1	Creation of measurement job for NSSI(s)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to create a measurement job for collecting the performance data of NSSI(s). |  |
| Actors and Roles | An authorized consumer of NSSI measurement job control service. |  |
| Telecom resources | NSSI(s); NSSI measurement job control service producer; NF measurement job control service producer; NF performance data file reporting service producer and/or NF performance data streaming service producer; NSSI performance data file reporting service producer and/or NSSI performance data streaming service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | - The NSSI(s) have been deployed. - The NSSI measurement job control service producer is in operation. |  |
| Begins when | The authorized consumer needs to create measurement job for collecting the performance data of NSSI(s). |  |
| Step 1 (M) | The authorized consumer requests the NSSI measurement job control service producer to create a NSSI measurement job to collect the performance data of NSSI(s). The request needs to indicate that the performance data needs to be reported by performance data file or by performance data streaming. |  |
| Step 2 (M) | The NSSI measurement job control service producer decomposes the performance data type(s) of NSSI into performance data type(s) of the constituent NSSI(s) and/or NF(s).The NSSI measurement job control service producer checks whether the decomposed performance data types of the constituent NSSI(s) and NF(s) can be collected by the existing measurement job(s) for NSSI(s) and/or NF(s). If new measurement job(s) for the constituent NSSI(s) and/or NF(s) are required, the NSSI measurement job control service producer consumes the NSSI measurement job control service and/or the NF measurement job control service to create the new measurement job(s) for the constituent NSSI(s) and/or NF(s) respectively (according to the use case "Creation of measurement job for NF" as described in clause 5.1.1.1.1). | Creation of measurement job for NF |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The measurement job for NSSI has been created, and the NSSI measurement job control service producer consumes the NSSI performance data file reporting service and/or NSSI performance data streaming service to get the performance data of the constituent NSSI(s), and/or consumes the NF performance data file reporting service and/or NF performance data streaming service to get the performance data of the constituent NF(s), and generates the performance data for the NSSI measurement job. | NSSI performance data file reporting; NSSI performance data streaming; NF performance data file reporting; NF performance data streaming |
| Traceability | REQ-MJCS_NSSI-FUN-1, REQ-MJCS_NSSI-FUN-2, REQ-MJCS_NSSI-FUN-3, REQ-MJCS_NSSI-FUN-4 and REQ-MJCS_NSSI-FUN-7 |  |

##### 5.1.2.1.2	Termination of measurement job for NSSI(s)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to request the NSSI measurement job control service producer to terminate a NSSI measurement job. |  |
| Actors and Roles | An authorized consumer of NSSI measurement job control service. |  |
| Telecom resources | NSSI(s) NSSI measurement job control service producer. NF measurement job control service producer |  |
| Assumptions | N/A |  |
| Pre-conditions | The NSSI measurement job has been created. |  |
| Begins when | The authorized consumer does not need the NSSI measurement job. |  |
| Step 1 (M) | The authorized consumer requests the NSSI measurement job control service producer to terminate a measurement job that is collecting the performance data of NSSI(s). |  |
| Step 2 (M) | The NSSI measurement job control service producer terminates the NSSI measurement job, and may - request the corresponding NSSI measurement job control service producer(s) to terminate the supporting measurement job(s) of the constituent NSSI(s), and/or - consume the NF measurement job control service to request termination of the supporting measurement job(s) of the constituent NF(s) (according to the use case "Termination of measurement job for NF(s)" as described in clause 5.1.1.1.2). | Termination of measurement job for NF(s) |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The NSSI measurement job is terminated. |  |
| Traceability | REQ-MJCS_NSSI-FUN-5 |  |

##### 5.1.2.1.3	Query of measurement jobs for NSSI(s)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to query the ongoing NSSI measurement jobs (i.e. the NSSI measurement jobs that have been created by the subject consumer and not terminated). |  |
| Actors and Roles | An authorized consumer of NSSI measurement job control service. |  |
| Telecom resources | NSSI measurement job control service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | The NSSI measurement job control service producer is in operation. |  |
| Begins when | The authorized consumer needs to query the ongoing NSSI measurement jobs. |  |
| Step 1 (M) | The authorized consumer queries the information about the ongoing NSSI measurement jobs from the NSSI measurement job control service producer. |  |
| Step 2 (M) | The NSSI measurement job control service producer provides the information about the ongoing NSSI measurement jobs to the consumer. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The information about the ongoing NSSI measurements jobs are available to the consumer. |  |
| Traceability | REQ-MJCS_NSSI-FUN-6 |  |

#### 5.1.2.2	NSSI performance data file reporting service

##### 5.1.2.2.1	NSSI performance data file reporting

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to get the performance data file of 3GPP NSSI(s). |  |
| Actors and Roles | An authorized consumer of NSSI performance data file reporting service. |  |
| Telecom resources | Producer of the NSSI performance data file reporting service. |  |
| Assumptions | N/A |  |
| Pre-conditions | - The 3GPP NSSI has been deployed. - The NSSI performance data file reporting service producer is in operation. - The NSSI performance data file reporting service consumer has subscribed the notification about performance data file ready. |  |
| Begins when | The performance data file of 3GPP NSSI(s) is ready at the NSSI performance data file reporting service producer. |  |
| Step 1 (M) | The NSSI performance data file reporting service producer sends the notification about performance data file ready to the authorized consumer. |  |
| Step 2 (M) | The authorized consumer fetches the performance data file from the NSSI performance data file reporting service producer. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The performance data file of 3GPP NSSI have been reported. |  |
| Traceability | REQ-PDFR_NSSI-FUN-1, REQ-PDFR_NSSI-FUN-2 |  |

#### 5.1.2.3	NSSI performance data streaming service

##### 5.1.2.3.1	NSSI performance data streaming

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to receive the performance data stream of NSSI(s). |  |
| Actors and Roles | An authorized consumer of NSSI performance data streaming service. |  |
| Telecom resources | Producer of the NSSI performance data streaming service. |  |
| Assumptions | N/A |  |
| Pre-conditions | - The 3GPP NSSI has been deployed. - The NSSI performance data streaming service producer is in operation. - The NSSI performance data streaming service consumer has subscribed for receiving the performance data stream from the NSSI performance data streaming service producer. |  |
| Begins when | The performance data of 3GPP NSSI is ready at the NSSI performance data streaming service producer. |  |
| Step 1 (M) | The NSSI performance data streaming service producer sends the NSSI performance data stream to the consumer. |  |
| Ends when | The NSSI performance data streaming service consumer receives the performance data stream. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions |  |  |
| Traceability | REQ-PDS_NSSI-FUN-1 |  |

### 5.1.3	NSI PM services

#### 5.1.3.1	NSI measurement job control service

##### 5.1.3.1.1	Creation of measurement job for NSI(s)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to create a measurement job for collecting the performance data of NSI(s). |  |
| Actors and Roles | An authorized consumer of NSI measurement job control service. |  |
| Telecom resources | NSI(s); NSI measurement job control service producer; The set of NSSI measurement job control service producer, NSSI performance data file reporting service producer and/or NSSI performance data streaming service producer; and/or The set of NF measurement job control service producer, NF performance data file reporting service producer and/or NF performance data streaming service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | - The NSI(s) have been deployed. - The NSI measurement job control service producer is in operation. |  |
| Begins when | The authorized consumer needs to create measurement job for collecting the performance data of NSI(s). |  |
| Step 1 (M) | The authorized consumer requests the NSI measurement job control service producer to create a NSI measurement job to collect the performance data of NSI(s). The request needs to indicate that the performance data needs to be reported by performance data file or by performance data streaming. |  |
| Step 2 (M) | The NSI measurement job control service producer decomposes the performance data type of NSI(s) into performance data type(s) of the constituent NSSI(s) and/or of constituent NF(s). - The NSI measurement job control service producer checks whether the decomposed performance data of the constituent NSSI(s) can be collected by the existing measurement job(s) for NSSI(s). If new measurement job(s) for the constituent NSSI(s) are required, the NSI measurement job control service producer consumes the NSSI measurement job control service to create the new measurement job(s) for the constituent NSSI(s) (according to the use case "Creation of measurement job for NSSI(s)" as described in clause 5.1.2.1.1); or - The NSI measurement job control service producer checks whether the decomposed performance data of the constituent NF(s) can be collected by the existing measurement job(s) for NF(s). If new measurement job(s) for the constituent NF(s) are required, NSI measurement job control service producer requests the NF PM measurement job control service producer to create the new measurement job(s) for the constituent NF(s) (according to the use case "Creation of measurement job for NF" as described in clause 5.1.1.1.1). | Creation of measurement job for NSSI; and/or Creation of measurement job for NF |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The measurement job for NSI has been created, and the NSI measurement job control service producer consumes the NSSI performance data file reporting service, NSSI performance data streaming service, the NF performance data file reporting service and/or NF performance data streaming service to get the performance data of the constituent NSSI(s) and/or NF(s), and generates the performance data for the NSI measurement job. | NSSI performance data file reporting; NSSI performance data streaming NF performance data file reporting; and/or NF performance data streaming |
| Traceability | REQ-MJCS_NSI-FUN-1, REQ-MJCS_NSI-FUN-2, REQ-MJCS_NSI-FUN-3, REQ-MJCS_NSI-FUN-4 and REQ-MJCS_NSI-FUN-7. |  |

##### 5.1.3.1.2	Termination of measurement job for NSI(s)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to request the NSI measurement job control service producer to terminate a NSI measurement job. |  |
| Actors and Roles | An authorized consumer of NSI measurement job control service. |  |
| Telecom resources | NSI(s); NSI measurement job control service producer; NSSI measurement job control service producer; NF measurement job control service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | The NSI measurement job has been created. |  |
| Begins when | The authorized consumer does not need the NSI measurement job. |  |
| Step 1 (M) | The authorized consumer requests the NSI measurement job control service producer to terminate a NSI measurement job that is collecting the performance data of NSI(s). |  |
| Step 2 (M) | The NSI measurement job control service producer terminates the NSI measurement job, and may - consume the NSSI measurement job control service to request termination of the supporting measurement job(s) of the constituent NSSI(s) if any (according to the use case "Termination of measurement job for NSSI(s)" as described in clause 5.1.2.1.2), and - consume the NF measurement job control service to request termination of the supporting measurement job(s) of the constituent NF(s) if any (according to the use case "Termination of measurement job for NF(s)" as described in clause 5.1.1.1.2). | Termination of measurement job for NSSI(s); Termination of measurement job for NF(s) |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The NSI measurement job is terminated, or still retained to serve other consumers according to step 2. |  |
| Traceability | REQ-MJCS_NSI-FUN-5 |  |

##### 5.1.3.1.3	Query of measurement jobs for NSI(s)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to query the ongoing NSI measurement jobs (i.e. the NSI measurement jobs that have been created by the subject consumer and not terminated). |  |
| Actors and Roles | An authorized consumer of NSI measurement job control service. |  |
| Telecom resources | NSI(s) NSI measurement job control service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | The NSI measurement job control service producer is in operation. |  |
| Begins when | The authorized consumer needs to query the ongoing NSI measurement jobs. |  |
| Step 1 (M) | The authorized consumer queries the information about the ongoing NSI measurement jobs from the NSI measurement job control service producer. |  |
| Step 2 (M) | The NSI measurement job control service producer provides the information about the ongoing NSI measurement jobs to the consumer. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The information about the ongoing NSI measurements jobs are available to the consumer. |  |
| Traceability | REQ-MJCS_NSI-FUN-6 |  |

#### 5.1.3.2	NSI performance data file reporting service

##### 5.1.3.2.1	NSI performance data file reporting

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to get the performance data file of 3GPP NSI(s). |  |
| Actors and Roles | An authorized consumer of NSI performance data file reporting service. |  |
| Telecom resources | Producer of the NSI performance data file reporting service. |  |
| Assumptions | N/A |  |
| Pre-conditions | - The 3GPP NSI has been deployed. - The NSI performance data file reporting service producer is in operation. - The NSI performance data file reporting service consumer has subscribed the notification about performance data file ready. |  |
| Begins when | The performance data file of 3GPP NSI(s) is ready at the NSI performance data file reporting service producer. |  |
| Step 1 (M) | The NSI performance data file reporting service producer sends the notification about performance data file ready to the authorized consumer. |  |
| Step 2 (M) | The authorized consumer fetches the performance data file from the NSI performance data file reporting service producer. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The performance data file of 3GPP NSSI have been reported. |  |
| Traceability | REQ-PDFR_NSI-FUN-1, REQ-PDFR_NSI-FUN-2 |  |

#### 5.1.3.3	NSI performance data streaming service

##### 5.1.3.3.1	NSI performance data streaming

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to receive the performance data stream of NSI(s). |  |
| Actors and Roles | An authorized consumer of NSI performance data streaming service. |  |
| Telecom resources | Producer of the NSI performance data streaming service. |  |
| Assumptions | N/A |  |
| Pre-conditions | - The 3GPP NSI has been deployed. - The NSI performance data streaming service producer is in operation. - The NSI performance data streaming service consumer has subscribed for receiving the performance data stream from the NSI performance data streaming service producer. |  |
| Begins when | The performance data of 3GPP NSI is ready at the NSI performance data streaming service producer. |  |
| Step 1 (M) | The NSI performance data streaming service producer sends the NSI performance data stream to the consumer. |  |
| Ends when | The NSI performance data streaming service consumer receives the performance data stream. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions |  |  |
| Traceability | REQ-PDS_NSI-FUN-1 |  |

### 5.1.4	Network/Sub-network PM services

#### 5.1.4.1	Network/Sub-network measurement job control service

##### 5.1.4.1.1	Creation of measurement job for network(s)/sub-network(s)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to create a measurement job for collecting the network/sub-network performance data that are not specific to network slicing. |  |
| Actors and Roles | An authorized consumer of network measurement job control service. |  |
| Telecom resources | Network(s)/sub-network(s); Network measurement job control service producer; NF measurement job control service producer; NF performance data file reporting service producer and/or NF performance data streaming service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | - The network(s)/sub-network(s) have been deployed; - The network measurement job control service producer is in operation. |  |
| Begins when | The authorized consumer needs to create a network measurement job for collecting the network performance data that are not specific to network slicing. |  |
| Step 1 (M) | The authorized consumer requests the network measurement job control service producer to create measurement job to collect the network performance data that are not specific to network slicing. The request needs to indicate that the performance data needs to be reported by performance data file or by performance data streaming. |  |
| Step 2 (M) | The network measurement job control service producer decomposes the performance data type of network/sub-network into performance data type(s) of the constituent 3GPP NF(s). The network measurement job control service producer whether the decomposed performance data type(s) of the constituent NF(s) can be collected by the existing measurement job(s) for NF(s). If new measurement job(s) for the constituent NF(s) are required, the network measurement job control service producer requests the NF measurement job control service producer to create the new measurement job(s) for the constituent NF(s) (according to the use case "Creation of measurement job for NF" as described in clause 5.1.1.1.1). | Creation of measurement job for NF |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The measurement job for network(s)/sub-network(s) has been created, and the network measurement job control service producer consumes the NF performance data file reporting service and/or NF performance data streaming service to get the performance data of the constituent NF(s), and generates the performance data for the network measurement job. | NF performance data file reporting; NF performance data streaming |
| Traceability | REQ-MJCS_NW-FUN-1, REQ-MJCS_NW-FUN-2, REQ-MJCS_NW-FUN-3, REQ-MJCS_NW-FUN-4 and REQ-MJCS_NW-FUN-7 |  |

##### 5.1.4.1.2	Termination of measurement job for network(s)/sub-network(s)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to request the network measurement job control service producer to terminate a network measurement job. |  |
| Actors and Roles | An authorized consumer of network measurement job control service. |  |
| Telecom resources | NSSI(s); Network measurement job control service producer; NF measurement job control service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | The network measurement job has been created. |  |
| Begins when | The authorized consumer does not need the network measurement job. |  |
| Step 1 (M) | The authorized consumer requests the network measurement job control service producer to terminate a network measurement job that is collecting the performance data of network(s)/sub-network(s). |  |
| Step 2 (M) | The network measurement job control service producer terminates the network measurement job, and may consume the NF measurement job control service to request termination of the supporting measurement job(s) of the constituent NF(s) (according to the use case "Termination of measurement job for NF(s)" as described in clause 5.1.1.1.2). | Termination of measurement job for NF(s) |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The network measurement job is terminated, or still retained to serve other consumers according to step 2. |  |
| Traceability | REQ-MJCS_NW-FUN-5 |  |

##### 5.1.4.1.3	Query of measurement jobs for network(s)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to query the ongoing network measurement jobs (i.e. the network measurement jobs that have been created by the subject consumer and not terminated). |  |
| Actors and Roles | An authorized consumer of network measurement job control service. |  |
| Telecom resources | Network(s)/sub-network(s) Network measurement job control service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | The network measurement job control service producer is in operation. |  |
| Begins when | The authorized consumer needs to query the ongoing network measurement jobs. |  |
| Step 1 (M) | The authorized consumer queries the information about the ongoing network measurement jobs from the network measurement job control service producer. |  |
| Step 2 (M) | The network measurement job control service producer provides the information about the ongoing network measurement jobs to the consumer. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The information about the ongoing network measurements jobs are available to the consumer. |  |
| Traceability | REQ-MJCS_NW-FUN-6 |  |

#### 5.1.4.2	Network/Sub-network performance data file reporting service

##### 5.1.4.2.1	Network/Sub-network performance data file reporting

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to get the network/sub-network performance data that are not specific to network slicing. |  |
| Actors and Roles | An authorized consumer of network/sub-network performance data file reporting service. |  |
| Telecom resources | Network/sub-network performance data file reporting service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | - The network/sub-network has been deployed. - The network/sub-network performance data file reporting service producer is in operation. - The network/sub-network performance data file reporting service consumer has subscribed the notification about performance data file ready. |  |
| Begins when | The performance data file of network/sub-network is ready at the network/sub-network performance data file reporting service producer. |  |
| Step 1 (M) | The network/sub-network performance data file reporting service producer sends the notification about performance data file ready to the authorized consumer. |  |
| Step 2 (M) | The authorized consumer fetches the network /sub-network performance data file from the network/sub-network performance data file reporting service producer. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The network/sub-network performance data file have been reported. |  |
| Traceability | REQ-PDFR_NW-FUN-1, REQ-PDFR_NW-FUN-2 |  |

#### 5.1.4.3	Network/Sub-network performance data streaming service

##### 5.1.4.3.1	Network/Sub-network performance data streaming

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to receive the stream of the network/sub-network performance data that are not specific to network slicing. |  |
| Actors and Roles | An authorized consumer of network/sub-network performance data streaming service. |  |
| Telecom resources | Network/Sub-network performance data streaming service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | - The network/sub-network has been deployed. - The network/sub-network performance data streaming service producer is in operation. - The network/sub-network performance data streaming service consumer has subscribed for receiving the performance data stream from the network/sub-network performance data streaming service producer. |  |
| Begins when | The performance data of network is ready at the network/sub-network performance data streaming service producer. |  |
| Step 1 (M) | The network/sub-network performance data streaming service producer sends the network/sub-network performance data stream to the consumer. |  |
| Ends when | The Network/sub-network performance data streaming service consumer receives the network performance data stream. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions |  |  |
| Traceability | REQ-PDS_NW-FUN-1 |  |

### 5.1.5	Management data analytics

#### 5.1.5.1	Management data analytics for NSIs/NSSIs

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to collect management analytical data for NSIs/NSSIs. |  |
| Actors and Roles | An authorized consumer of management data analytics service. |  |
| Telecom resources | NSI(s), NSSI(s), NF(s); Producer of management data analytics service; Producer of measurement job control service for NSI(s); Producer of measurement job control service for NSSI(s); Producer of measurement job control service for NF(s); Producer of performance data file reporting service for NSI(s); Producer of performance data file reporting service for NSSI(s); Producer of performance data file reporting service for NF(s); |  |
| Assumptions | N/A |  |
| Pre-conditions | - The NSI(s) have been deployed. - The management data analytics service producer is in operation. |  |
| Begins when | The authorized consumer subscribes to the management analytical data for NSI(s)/NSSI(s). |  |
| Step 1 (M) | The management data analytics service producer determines what performance measurements of NSI(s), NSSI(s) and NF(s) are needed to generate the subject management analytical data. |  |
| Step 2 (M) | The management data analytics service producer checks whether the required NSI performance measurements can be collected by the existing measurement job(s) for NSI(s), NSSI(s) and NF(s). - If new measurement job(s) for the NSI(s) are required, the management data analytics service producer consumes the NSI measurement job control service to create the new measurement job(s) for the NSI(s) (according to the use case "Creation of measurement job for NSI(s)" as described in clause 5.1.3.1.1); - If new measurement job(s) for the NSSI(s) are required, the management data analytics service producer consumes the NSSI measurement job control service to create the new measurement job(s) for the NSSI(s) (according to the use case "Creation of measurement job for NSSI(s)" as described in clause 5.1.2.1.1); - If new measurement job(s) for the NF(s) are required, the management data analytics service producer consumes the NF measurement job control service to create the new measurement job(s) for the NF(s) (according to the use case "Creation of measurement job for NF(s)" as described in clause 5.1.1.1.1). | Creation of measurement job for NSI(s); Creation of measurement job for NSSI(s); Creation of measurement job for NF(s) |
| Ends when | The consumer unsubscribes to the management analytical data for NSI(s)/NSSI(s). |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The management data analytics service producer consumes the performance data reporting related services to get the required performance measurements for NSI(s), NSSI(s) and NF(s), generate the management analytical data based on the collected performance measurements, and makes the management analytical data available to the management service responsible for reporting the data. |  |
| Traceability | REQ-MDAS-FUN-1 |  |

#### 5.1.5.2	Management data analytics for network

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to collect management analytical data for the network. |  |
| Actors and Roles | An authorized consumer of network management data analytics service. |  |
| Telecom resources | 3GPP network(s); Producer of network management data analytics service; Producer of measurement job control service for NF(s); Producer of performance data file reporting service for NF(s); |  |
| Assumptions | N/A |  |
| Pre-conditions | - The 3GPP network(s) have been deployed. - The network management data analytics service producer is in operation. |  |
| Begins when | The authorized consumer subscribes to the service of management analytical data for network(s). |  |
| Step 1 (M) | The network management data analytics service producer determines what performance measurements of NF(s) are needed to generate the subject network management analytical data. |  |
| Step 2 (M) | The management data analytics service producer checks whether the required network performance measurements can be collected by the existing measurement job(s) for NF(s. - If new measurement job(s) for the constituent NF(s) are required, the management data analytics service producer consumes the NF measurement job control service to create the new measurement job(s) for the NF(s) (according to the use case "Creation of measurement job for NF(s)" as described in clause 5.1.1.1.1). | Creation of measurement job for NF(s) |
| Step 3 (M) | The management data analytics service producer consumes the performance data reporting related services to get the required performance measurements for NF(s) and generates the management analytical KPI(s) based on the collected performance measurements. |  |
| Ends when | The consumer unsubscribes to the management analytical data for network(s). |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The management analytical data is available to the management service responsible for reporting the data to the consumer. |  |
| Traceability | REQ-MDAS-FUN-2 |  |

## 5.2	Requirements

### 5.2.1	Requirements for NF measurement job control service

REQ-MJCS_NF-FUN-1	The management service producer responsible for NF measurement job control shall have the capability allowing its authorized consumer to request creation of a measurement job to collect the performance data of NF(s).

REQ-MJCS_NF-FUN-2	The management service producer responsible for NF measurement job control shall have the capability allowing its authorized consumer to indicate the reporting method (i.e. by performance data file or by performance data streaming) for the performance data when requesting to create a measurement job for NF(s).

REQ-MJCS_NF-FUN-3	The management service producer responsible for NF measurement job control shall have the capability to fulfill the consumer's request to create a measurement job for NF(s).

REQ-MJCS_NF-FUN-4	The management service producer responsible for NF measurement job control shall have the capability to generate the performance data of NF(s) according to the measurement job.

REQ-MJCS_NF-FUN-5	The management service producer responsible for NF measurement job control shall have the capability to fulfill the request from its authorized consumer to terminate a NF measurement job.

REQ-MJCS_NF-FUN-6	The management service producer responsible for NF measurement job control shall have the capability allowing its authorized consumer to query the information about the ongoing NF measurement jobs.

REQ-MJCS_NF-FUN-7	The management service producer responsible for NF measurement job control may reject a NF measurement job creation request.

### 5.2.2	Requirements for NF performance data file reporting service

REQ-PDFR_NF-FUN-1	The management service producer responsible for NF performance data file reporting shall have the capability to send the notification about NF performance data file ready to its authorized consumer.

REQ-PDFR_NF-FUN-2	The management service producer responsible for NF performance data file reporting shall have the capability to allow its authorized consumer to fetch the performance data file of NF(s).

### 5.2.3	Requirements for NF performance data streaming service

REQ-PDS_NF-FUN-1	The management service producer responsible for NF performance data streaming shall have the capability to send the NF performance data stream to its authorized consumer.

### 5.2.4	Requirements for NSSI measurement job control service

REQ-MJCS_NSSI-FUN-1	The management service producer responsible for NSSI measurement job control shall have the capability allowing its authorized consumer to request creation of a measurement job to collect the performance data of NSSI(s).

REQ-MJCS_NSSI-FUN-2	The management service producer responsible for NSSI measurement job control shall have the capability allowing its authorized consumer to indicate the reporting method (i.e. by performance data file or by performance data streaming) for the performance data when requesting to create a measurement job for NSSI(s).

REQ-MJCS_NSSI-FUN-3	The management service producer responsible for NSSI measurement job control shall have the capability to generate the performance data of NSSI(s).

REQ-MJCS_NSSI-FUN-4	The management service producer responsible for NSSI measurement job control shall have the capability to fulfill the consumer's request to create a measurement job for NSSI(s).

REQ-MJCS_NSSI-FUN-5	The management service producer responsible for NSSI measurement job control shall have the capability to fulfill the request from its authorized consumer to terminate a NSSI measurement job.

REQ-MJCS_NSSI-FUN-6	The management service producer responsible for NSSI measurement job control shall have the capability to fulfil the request from its authorized consumer to query the information about the ongoing NSSI measurement jobs.

REQ-MJCS_NSSI-FUN-7	The management service producer responsible for NSSI measurement job control may reject a NSSI measurement job creation request.

### 5.2.5	Requirements for NSSI performance data file reporting service

REQ-PDFR_NSSI-FUN-1	The management service producer responsible for NSSI performance data file reporting shall have the capability to send the notification about NSSI performance data file ready to its authorized consumer.

REQ-PDFR_NSSI-FUN-2	The management service producer responsible for NSSI performance data file reporting shall have the capability to allow its authorized consumer to fetch the performance data file of NSSI(s).

### 5.2.6	Requirements for NSSI performance data streaming service

REQ-PDS_NSSI-FUN-1	The management service producer responsible for NSSI performance data streaming shall have the capability to send the NSSI performance data stream to its authorized consumer.

### 5.2.7	Requirements for NSI measurement job control service

REQ-MJCS_NSI-FUN-1	The management service producer responsible for NSI measurement job control shall have the capability allowing its authorized consumer to request creation of a measurement job to collect the performance data of NSI(s).

REQ-MJCS_NSI-FUN-2	The management service producer responsible for NSI measurement job control shall have the capability allowing its authorized consumer to indicate the reporting method (i.e. by performance data file or by performance data streaming) for the performance data when requesting to create a measurement job for NSI(s).

REQ-MJCS_NSI-FUN-3	The management service producer responsible for NSI measurement job control shall have the capability to generate the performance data of NSI(s).

REQ-MJCS_NSI-FUN-4	The management service producer responsible for NSI measurement job control shall have the capability to fulfill the consumer's request to create a measurement job for NSI(s).

REQ-MJCS_NSI-FUN-5	The management service producer responsible for management service producer responsible for NSI measurement job control shall have the capability to fulfill the request from its authorized consumer to terminate a NSI measurement job.

REQ-MJCS_NSI-FUN-6	The management service producer responsible for NSI measurement job control shall have the capability to fulfill the request from its authorized consumer to query the information about the ongoing NSI measurement jobs.

REQ-MJCS_NSI-FUN-7	The management service producer responsible for NSI measurement job control may reject a NSI measurement job creation request.

### 5.2.8	Requirements for NSI performance data file reporting service

REQ-PDFR_NSI-FUN-1	The management service producer responsible for NSI performance data file reporting shall have the capability to send the notification about NSI performance data file ready to its authorized consumer.

REQ-PDFR_NSI-FUN-2	The management service producer responsible for NSI performance data file reporting shall have the capability to allow its authorized consumer to fetch the performance data file of NSI(s).

### 5.2.9	Requirements for NSI performance data streaming service

REQ-PDS_NSI-FUN-1	The management service producer responsible for NSI performance data streaming shall have the capability to send the NSI performance data stream to its authorized consumer.

### 5.2.10	Requirements for network/sub-network measurement job control service

REQ-MJCS_NW-FUN-1	The management service producer responsible for network/sub-network measurement job control shall have the capability allowing its authorized consumer to request creation of a measurement job to collect the network/sub-network performance data that are not specific to network slicing.

REQ-MJCS_NW-FUN-2	The management service producer responsible for network/sub-network measurement job control shall have the capability allowing its authorized consumer to indicate the reporting method (i.e. by performance data file or by performance data streaming) for the performance data that are not specific to network slicing when requesting to create a measurement job for network(s)/sub-network(s).

REQ-MJCS_NW-FUN-3	The management service producer responsible for network/sub-network measurement job control shall have the capability to generate the network/sub-network performance data that are not specific to network slicing.

REQ-MJCS_NW-FUN-4	The management service producer responsible for network/sub-network measurement job control shall have the capability to fulfill the consumer's request to create a measurement job for network(s)/sub-network(s).

REQ-MJCS_NW-FUN-5	The management service producer responsible for network/sub-network measurement job control shall have the capability to fulfill the request from its authorized consumer to terminate a network/sub-network measurement job.

REQ-MJCS_NW-FUN-6	The management service producer responsible for network/sub-network measurement job control shall have the capability to fulfill the request from its authorized consumer to query the information about the ongoing network measurement jobs.

REQ-MJCS_NW-FUN-7	The management service producer responsible for network/sub-network measurement job control may reject a network/sub-network measurement job creation request.

### 5.2.11	Requirements for network/sub-network performance data file reporting service

REQ-PDFR_NW-FUN-1	The management service producer responsible for network/sub-network performance data file reporting shall have the capability to send the notification about network/sub-network performance data file ready to its authorized consumer.

REQ-PDFR_NW-FUN-2	The management service producer responsible for network/sub-network performance data file reporting shall have the capability to allow its authorized consumer to fetch the performance data file of network(s)/sub-network(s).

### 5.2.12	Requirements for network/sub-network performance data streaming service

REQ-PDS_NW-FUN-1	The management service producer responsible for network/sub-network performance data streaming shall have the capability to send the network/sub-network performance data stream to its authorized consumer.

### 5.2.13	Management data analytics service

REQ-MDAS-FUN-1 The management data analytics service producer shall have the capability allowing its authorized consumer to request collection of management analytical data for NSIs/NSSIs.

REQ-MDAS-FUN-2 The management data analytics service producer shall have the capability allowing its authorized consumer to request collection of management analytical data for network(s).

# 6.	Performance assurance specific operations and notifications

## 6.1	Measurement job control related operations

### 6.1.1	Operation createMeasurementJob (M)

#### 6.1.1.1	Definition

This operation supports the authorized consumer to request the measurement job control related service producer to create a measurement job.

One measurement job can collect the value of one or multiple measurement types. The measurement types are the performance measurements and assurance data defined in TS 28.552 [2].

When a measurement type is collected by one measurement job for a given instance (e.g., an NF instance), another measurement job creation request which wants to collect the same measurement type for the same instance with different granularity period may be rejected. This behaviour shall be consistent for a given implementation by a specific management service producer.

There are two different methods for the performance data to be reported:

-	Performance data file method: In this method the performance data is accumulated for certain time before it is reported; the data will be delivered as a file.

-	Performance data streaming method: In this method, the performance data streaming producer, when the performance data are ready, sends the performance data to the consumer (i.e., stream target). The volume of the performance data reported by streaming is expected to be small, and the Granularity period of the performance data stream needs to be configurable and is expected to be short.

#### 6.1.1.2	Input parameters

| Parameter Name | Qualifier | Information type | Comment |
| --- | --- | --- | --- |
| iOCName | M | The IOC name defined of the NRMs (e.g., as defined in TS 28.541 [3]), or the class name defined locally in the performance measurements specifications (e.g., TS 28.552 [2]). | It specifies one object class name. The consumer requests to collect one or more measurement type(s) of the instances of this class. |
| iOCInstanceList | M | List of DN | It specifies the list of DNs of object instances whose measurements of the corresponding type(s) are to be collected. An empty list means that for all instances (including the object instances existing at the time of measurement job creation, and the instances added later) known by the management service producer the measurements will be collected. |
| measurementCategoryList | M | List of measurement type names (see TS 28.552 [2]). | It specifies the measurement type(s) to be measured. The elements of the measurementCategoryList shall be one of the following forms: - The form "family.measurementName.subcounter" can be used in order to retrieve a specified subcounter of a measurement type. - The form "family.measurementName" can be used in order to retrieve a specific measurement type. In case the measurement type includes subcounters, all subcounters will be retrieved. - The form "family" can be used in order to retrieve all measurement types in this family. |
| reportingMethod | M | The reporting method of the collected performance data. | It specifies the method for the collected performance data to be reported. One of the following methods can be selected: - by performance data file - by performance data streaming (optional). |
| granularityPeriod | M | The period between generation of two successive measurements. | The management service producer will produce the value of the measurements at the end of each granularityPeriod. If the reportingMethod is performance data file reporting: - The value of granularityPeriod can be 5 minutes, 15 minutes, 30 minutes, 1 hour, 12 hours or 24 hours or other values (see Note 1 below). If the reportingMethod is performance data streaming: - The value of granularityPeriod is an integer value in seconds (see Note 1 below). |
| reportingPeriod | M | The period between two successive performance data reporting. | Applicable when the reportingMethod is performance data file reporting. The performance data report(s) are produced when the reporting period arrives. The reportingPeriod shall be one or multiple of granularityPeriod. The measurement value of each granularityPeriod will be made available to the performance data reporting related service producer, who will prepare the performance data file(s) for each reportingPeriod. If the consumer has subscribed to the notifyFileReady and notifyFilePreparationError notifications from the performance data reporting related service producer, the consumer will receive the notifications about the result of the performance data file preparation from that producer with the interval as defined by reportPeriod; |
| startTime | O | It specifies the begin time from which the measurement job will be active. | All values that indicate valid timestamp. Default value is "start now". If startTime is in the past, the current time will be used and the job will start immediately. When a measurement job becomes active, it does not mean that the measurement job immediately starts generation of the measurements of the given type(s). The consumer can set the detailed time frame (e.g. dailySchedule or weeklySchedule) by schedule parameter for a measurement job to generate the measurements. If there is no time frame scheduled, the measurement job immediately starts generation of the measurements when it becomes active. |
| stopTime | O | It specifies the end time after which the measurement job will be stopped. | The value indicates valid timestamp and shall be later than startTime and current time. This attribute may carry the value "indefinitely". Default value is to run indefinitely. |
| schedule | O | It specifies the detailed time frames (within the startTime and stopTime) during which the measurement job is active and monitors the measurement type(s). | Its value is only one of the following, dailyScheduling or weeklyScheduling. The legal values for them refer to ITU-T Recommendation X.721 [4]. The legal values for them are as follows. dailyScheduling: {{ intervalStart {hour 0, minute 0}, intervalEnd {hour 23, minute 59}}} weeklyScheduling: {{ daysOfWeek '1111111'B, intervalsOfDay dailyScheduling}} Default value is "daily". |
| streamTarget | M | It specifies the target of performance data streams carrying the performance data stream unit(s). | Applicable when the reportingMethod is performance data streaming. |
| priority | O | It specifies the priority of measurement job | Its value should be one of the following: Low, Medium, High Default value is “Medium” |
| reliability | O | It specifies the reliability of measurement job | Its value is vendor specific. See NOTE 2. |
| Note 1: The granularityPeriod defines the measurement data production rate. The supported rates are dependent on the capacity of the producer involved (e.g. the processing power of the producer, number of measurements being measured by the producer at the time, the complexity of the measurement type involved etc) and therefore, it cannot be standardized for all producers involved. The supported rates can only reflect the negotiated agreement between producer and the consumer involved. NOTE 2: meaning of “reliability” is not defined in the present document. |  |  |  |

#### 6.1.1.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| jobId | M | It identifies the measurement job instance (and distinguishes it from all other ongoing and stopped measurement job instances that have been created for the subject consumer). | Unique identifier of the measurement job from all the ongoing and stopped Measurement jobs that have been created for the subject consumer. |
| unsupportedList | M | List of < iOC instance, measurement type name, reason > | To create a measurement job, best-effort is required. The parameter of 'unsupportedList' has to be returned if status = PartialSuccess. The reason can be any of: - Measurement type name is unknown. - Measurement type name is invalid. - Measurement type name is not supported in the specific implementation. - Measurement type name is already monitored for the IOC instance with a different granularityPeriod. - The related IOC instance is unknown (e.g. it does not exist at the time of this operation invocation). - Insufficient capacity to monitor the related IOC instance(s). |
| status | M | ENUM (Success, Failure, PartialSuccess) | An operation may fail because of a specified or unspecified reason. |

#### 6.1.1.4	Exceptions

| Exception Name | Definition |
| --- | --- |
| invalidStartTime | Condition: startTime is invalid. Returned Information: Name of the exception; status is set to 'Failure'. |
| invalidStopTime | Condition: stopTime is invalid. Returned Information: Name of the exception; status is set to 'Failure'. |
| invalidSchedule | Condition: schedule is invalid. Returned Information: Name of the exception; status is set to 'Failure'. |
| invalidReportingMethod | Condition: reportingMethod is invalid. Returned Information: Name of the exception; status is set to 'Failure'. |
| invalidGranularityPeriod | Condition: granularityPeriod is invalid. Returned Information: Name of the exception; status is set to 'Failure'. |
| invalidReportingPeriod | Condition: reportingPeriod is invalid. Returned Information: Name of the exception; status is set to 'Failure'. |
| highWorkLoad | Condition: no sufficient capacity Returned Information: Name of the exception and the detailed reason which is one of: CpuBusy; DiskShortage, LowMemory, maxJobReached, otherReason; status is set to 'Failure'. |
| noValidMeasurementType | Condition: all measurement type names are invalid (i.e. none of the measurement type names are valid). Returned information: output parameter status is set to 'Failure'. |
| invalidPriority | Condition: priority is invalid. Returned Information: Name of the exception; status is set to ‘Failure'. |
| invalidReliability | Condition: reliability is invalid. Returned Information: Name of the exception; status is set to ‘Failure'. |

### 6.1.2	Operation stopMeasurementJob (M)

#### 6.1.2.1	Definition

This operation supports the authorized consumer to request the measurement job control related service producer to terminate a measurement job.

Whether the measurement job is removed from the management service producer is vendor specific and out of scope of the present document.

The measurement job shall be stopped at the end of the granularityPeriod.

After the job has been stopped, the performance data reporting related notification (i.e. notifyFileReady or notifyFilePreparationError) and the performance data stream unit(s) for the last granularityPeriod shall be emitted, by the performance data reporting related service producer immediately.

#### 6.1.2.2	Input parameters

| Parameter Name | Qualifier | Information type | Comment |
| --- | --- | --- | --- |
| jobId | M | See subclause 6.1.1.3 | It specifies the measurement job to be stopped. |

#### 6.1.2.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| Status | M | ENUM (Success, Failure) | An operation may fail because of a specified or unspecified reason. |

#### 6.1.2.4	Exceptions

| Exception Name | Definition |
| --- | --- |
| unknownJob | Condition: the jobId does not exist. Returned information: output parameter status is set to 'Failure'. |
| jobCannotBeStopped | Condition: the measurement job cannot be stopped. Returned information: output parameter status is set to 'Failure'. |

### 6.1.3	Operation listMeasurementJobs (M)

#### 6.1.3.1	Definition

This operation supports the authorized consumer to request the measurement job control related service producer to list the information of all or a set of specified ongoing measurement jobs.

#### 6.1.3.2	Input parameters

| Parameter Name | Qualifier | Information type | Comment |
| --- | --- | --- | --- |
| jobIdList | M | List of jobId of the measurement jobs | This parameter specifies the criteria to list the measurement jobs. If the parameter specifies the list of jobId to be retrieved, then the corresponding information of measurement jobs will be returned. If the parameter contains no information, all the measurement jobs are retrieved. |

#### 6.1.3.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| jobInfoList | M | List of <attributes (refer to input and output parameter of operation createMeasurementJob in clause 6.1.1.2 and clause 6.1.1.3) of measurement job: - jobId - iOCName - iOCInstanceList - measurementCategoryList - granularityPeriod - reportingMethod - reportingPeriod - startTime - stopTime - streamTarget - schedule - priority - reliability> | Returned information of corresponding Measurement Jobs matching the input criteria. If no match, then the length of the jobInfolist will be 0 (with status == Success). If the measurement job is created using non-empty iOCInstanceList in createMeasurementJob, then iOCInstanceList here shall contain the DNs of the supported IOC instances. If the measurement job is created using empty iOCInstanceList, then iOCInstanceList here shall be empty as well. |
| status | M | ENUM (Success, Failure) | An operation may fail because of a specified or unspecified reason. |

#### 6.1.3.4	Exceptions

| Exception Name | Definition |
| --- | --- |
| invalidJobIdList | Condition: jobIdList specified in the input parameter is valid. Returned information: output parameter status is set to 'Failure'. |

## 6.2	Performance data streaming related operations

### 6.2.1	Operation establishStreamingConnection (M)

#### 6.2.1.1	Definition

This operation supports the performance data streaming service producer to establish streaming connection with the consumer (i.e., stream target), as illustrated in the Annex D.

One connection supports one or more streams, and the information about the supported streams are sent from the producer to the consumer during the connection establishment.

If the streaming connection is successfully established, the producer sends the Performance Data Stream Units (see annex C) to the consumer on this connection according to the information of the supported streams when the performance data is ready for each granularity period.

#### 6.2.1.2	Input parameters

| Parameter Name | Qualifier | Information type | Comment |
| --- | --- | --- | --- |
| producerReference | M | The reference of the producer who requests to establish the streaming connection. | The format of the reference may have dependency on the solution set. |
| streamInfoList | M | List of < streamId, measObjDn, measTypes, > | It contains the information on the performance data streams: - streamId: unique identifier of the stream between the performance data streaming producer and the stream target; - measObjDn: the DN of the measured object instance; - measTypes: a list of measurement type whose measurement result values are to be reported by the Performance Data Stream Units via this stream. The measurement result values shall be reported following the sequence of the measurement types as presented in the 'measTypes' parameter. |

#### 6.2.1.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| connectionId | M | The identifier of the established streaming connection. | It identifies the streaming connection. The format may have dependency on the solution set. |
| status | M | ENUM (Success, Failure) | An operation may fail because of a specified or unspecified reason. |

#### 6.2.1.4	Exceptions

| Exception Name | Definition |
| --- | --- |
| unexpectedStreams | Condition: Some information in streamInfoList are unexpected by the consumer. Returned Information: Name of the exception; status is set to ‘Failure’. |

### 6.2.2	Operation terminateStreamingConnection (M)

#### 6.2.2.1	Definition

This operation supports the performance data streaming service producer to terminate the streaming connection with the consumer (i.e., stream target).

If the streaming connection is successfully terminated, the producer shall stop sending the Performance Data Stream Units (see annex C) to the consumer on this connection.

#### 6.2.2.2	Input parameters

| Parameter Name | Qualifier | Information type | Comment |
| --- | --- | --- | --- |
| connectionId | M | See clause 6.2.1.3 | It specifies the streaming connection to be terminated. |

#### 6.2.2.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| Status | M | ENUM (Success, Failure) | An operation may fail because of a specified or unspecified reason. |

#### 6.2.2.4	Exceptions

| Exception Name | Definition |
| --- | --- |
| unknownConnection | Condition: the connectionId is invalid. Returned Information: Name of the exception; status is set to ‘Failure’. |

### 6.2.3	Operation reportStreamData (M)

#### 6.2.3.1	Definition

This operation is used by the performance data streaming service producer to send the Performance Data Stream Unit (see annex C) to the consumer on the connection established between the producer and consumer.

#### 6.2.3.2	Input parameters

| Parameter Name | Qualifier | Information type | Comment |
| --- | --- | --- | --- |
| streamId | M | The identifier of the stream. | The streamId of the performance data stream. |
| granularityPeriodEndTime | M | It specifies the time stamp of the end of the granularity period, for which the measResults are reported. | Time stamp referring to the end of the granularity period. |
| measResults | M | The actual measurement results. | This parameter contains the sequence of result values for the observed measurement types. The "measResults" sequence shall have the same number of elements, which follow the same order as the measurement types presented in “measTypes” for the subject stream in the input parameter streamInfoList of the establishStreamingConnection operation (see clause 6.2.1.2). |

#### 6.2.3.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| status | M | ENUM (Success, Failure) | An operation may fail because of a specified or unspecified reason. |

#### 6.2.3.4	Exceptions

| Exception Name | Definition |
| --- | --- |

### 6.2.4	Operation getStreamInfo (M)

#### 6.2.4.1	Definition

This operation supports the performance data streaming service producer (or another authorized entity) to get the information for one or more streams from the streaming consumer (i.e, stream target).

#### 6.2.4.2	Input parameters

| Parameter Name | Qualifier | Information type | Comment |
| --- | --- | --- | --- |
| streamIdList | M | List of streamId | It provides the list of the streamId for which the stream information to be returned. The empty list indicates the stream information for all streams are to be returned. |

#### 6.2.4.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| listOfStreamInfoOut | M | List of < streamId, measObjDn, measTypes, > | It provides the list of the stream information successfully returned. The stream information contains the following information: - streamId: unique identifier of the stream to add; - measObjDn: the DN of the measured object instance; - measTypes: a list of measurement type. |
| status | M | ENUM (Success, Failure, PartialSuccess) | An operation may fail or partially fail because of a specified or unspecified reason. |

#### 6.2.4.4	Exceptions

| Exception Name | Definition |
| --- | --- |
| UnknownstreamIds | Condition: One or more of streamId in the streamIdList are unknown. Returned Information: Name of the exception; status is set to ‘Failure’ or ‘PartialSuccess’. |

### 6.2.5	Operation addStreamInfo (M)

#### 6.2.5.1	Definition

This operation supports the performance data streaming service producer to add the information for one or more new streams to the consumer (i.e., stream target).

#### 6.2.5.2	Input parameters

| Parameter Name | Qualifier | Information type | Comment |
| --- | --- | --- | --- |
| listOfStreamInfoToAdd | M | List of < streamId, measObjDn, measTypes, > | It provides the list of the stream information to be added. The stream information contains the following information: - streamId: unique identifier of the stream to add; - measObjDn: the DN of the measured object instance; - measTypes: a list of measurement type whose measurement result values are to be reported by the Performance Data Stream Units via this stream. The measurement result values shall be reported following the sequence of the measurement types as presented in the 'measTypes' parameter. |

#### 6.2.5.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| listOfStreamInfoAdded | M | List of < streamId, measObjDn, measTypes, > | It provides the list of the stream information successfully added. The stream information contains the following information: - streamId: unique identifier of the stream to add; - measObjDn: the DN of the measured object instance; - measTypes: a list of measurement type whose measurement result values are to be reported by the Performance Data Stream Units via this stream. The measurement result values shall be reported following the sequence of the measurement types as presented in the 'measTypes' parameter. |
| status | M | ENUM (Success, Failure, PartialSuccess) | An operation may fail or partially fail because of a specified or unspecified reason. |

#### 6.2.5.4	Exceptions

| Exception Name | Definition |
| --- | --- |
| streamIdsAleadyExist | Condition: One or more of streamId in the listOfStreamInfoToAdd already exist in the streamInfoList. Returned Information: Name of the exception; status is set to ‘Failure’ or ‘PartialSuccess’. |

### 6.2.6	Operation updateStreamInfo (M)

#### 6.2.6.1	Definition

This operation supports the performance data streaming service producer to update the information for one or more streams to the consumer (i.e., stream target).

#### 6.2.6.2	Input parameters

| Parameter Name | Qualifier | Information type | Comment |
| --- | --- | --- | --- |
| streamIdList | M | List of streamId | It provides the list of the streamId for which the stream information is to update. |
| listOfStreamInfoToUpdate | M | List of < measObjDn, measTypes, > | The list of stream information to be updated. It shall have the same number of entries as streamIdList parameter in this operation. The measObjDn: the updated DN of the measured object instance. Empty value means no update on this parameter. The measTypes: the updated list of measurement type whose measurement result values are to be reported by the Performance Data Stream Units via this stream. The measurement result values shall be reported following the sequence of the measurement types as presented in the 'measTypes' parameter. Empty value means no update on this parameter. At least one of the measObjDn and measTypes shall be non-empty. |

#### 6.2.6.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| listOfStreamInfoUpdated | M | List of < streamId, measObjDn, measTypes, > | It provides the list of the stream information successfully updated. The stream information contains the following information: - streamId: unique identifier of the stream to update; - measObjDn: the DN of the measured object instance; - measTypes: a list of measurement type. |
| status | M | ENUM (Success, Failure, PartialSuccess) | An operation may fail or partially fail because of a specified or unspecified reason. |

#### 6.2.6.4	Exceptions

| Exception Name | Definition |
| --- | --- |
| unknownStreamIds | Condition: One or more streamId are unknown by the consumer. Returned Information: Name of the exception; status is set to ‘Failure’ or “PartialSuccess”. |

### 6.2.7	Operation deleteStreamInfo (M)

#### 6.2.7.1	Definition

This operation supports the performance data streaming service producer to delete the information for one or more streams from the consumer (i.e., stream target).

#### 6.2.7.2	Input parameters

| Parameter Name | Qualifier | Information type | Comment |
| --- | --- | --- | --- |
| streamIdList | M | List of streamId | It provides the list of the streamId for which the stream information is to delete. |

#### 6.2.7.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| status | M | ENUM (Success, Failure) | An operation may fail because of a specified or unspecified reason. |

#### 6.2.7.4	Exceptions

| Exception Name | Definition |
| --- | --- |
| unknownStreamId | Condition: The streamId is unknown by the consumer. Returned Information: Name of the exception; status is set to ‘Failure’. |

# 7.	Performance assurance services components

## 7.1	Measurement job control services

The components of measurement job control services for NFs, NSSIs, NSIs and networks/sub-networks are listed in table 7.1-1.

Table 7.1-1: Components of measurement job control services

| Management service | Management service component type A | Management service component type B | Management service component type C |
| --- | --- | --- | --- |
| Measurement job control services for NFs | createMeasurementJob | IOCs for 5G NFs, as defined in TS 28.541 [3] | Performance measurements and assurance data for 5G NFs, as defined in draft TS 28.552 [2]. |
|  | stopMeasurementJob |  |  |
|  | listMeasurementJobs |  |  |
| Measurement job control services for NSSIs | createMeasurementJob | IOC(s) for NSSI, as defined in TS 28.541 [3]. | Performance measurements and assurance data for NSSI, as defined in draft TS 28.552 [2]. |
|  | stopMeasurementJob |  |  |
|  | listMeasurementJobs |  |  |
| Measurement job control services for NSIs | createMeasurementJob | IOC(s) for NSI, as defined in TS 28.541 [3] | Performance measurements and assurance data for NSI, as defined in draft TS 28.552 [2]. |
|  | stopMeasurementJob |  |  |
|  | listMeasurementJobs |  |  |
| Measurement job control services for sub-networks | createMeasurementJob | IOC(s) for sub-network, as defined in TS 28.541 [3] | Performance measurements and assurance data for sub-network, as defined in draft TS 28.552 [2]. |
|  | stopMeasurementJob |  |  |
|  | listMeasurementJobs |  |  |

## 7.2	Performance data file reporting services

The components of performance data file reporting services for NFs, NSSIs, NSIs and networks/sub-networks are listed in table 7.2-1.

Table 7.2-1: Components of performance data file reporting services

| Management service | Management service component type A | Management service component type B | Management service component type C |
| --- | --- | --- | --- |
| Performance data file reporting services for NFs | notifyFileReady (see TS 28.532 [7] | IOCs for 5G NFs, as defined in TS 28.541 [3] | Performance measurements for 5G NFs, as defined in draft TS 28.552 [2]. |
|  | notifyFilePreparationError (see TS 28.532 [7] |  |  |
|  | listAvailableFiles (see TS 28.532 [7] |  |  |
|  | subscribe (see TS 28.532 [7]) |  |  |
|  | unsubscribe (see TS 28.532 [7]) |  |  |
| Performance data file reporting services for NSSIs | notifyFileReady (see TS 28.532 [7] | IOC(s) for NSSI, as defined in TS 28.541 [3]. | Performance measurements for NSSI, as defined in draft TS 28.552 [2]. |
|  | notifyFilePreparationError (see TS 28.532 [7] |  |  |
|  | listAvailableFiles (see TS 28.532 [7] |  |  |
|  | subscribe (see TS 28.532 [7]) |  |  |
|  | unsubscribe (see TS 28.532 [7]) |  |  |
| Performance data file reporting services for NSIs | notifyFileReady (see TS 28.532 [7] | IOC(s) for NSI, as defined in TS 28.541 [3]. | Performance measurements for NSI, as defined in draft TS 28.552 [2]. |
|  | notifyFilePreparationError (see TS 28.532 [7] |  |  |
|  | listAvailableFiles (see TS 28.532 [7] |  |  |
|  | subscribe (see TS 28.532 [7]) |  |  |
|  | unsubscribe (see TS 28.532 [7]) |  |  |
| Performance data file reporting services for sub-networks | notifyFileReady (see TS 28.532 [7] | IOC(s) for sub-network, as defined in TS 28.622 [5]. | Performance measurements for sub-network, as defined in draft TS 28.552 [2]. |
|  | notifyFilePreparationError (see TS 28.532 [7] |  |  |
|  | listAvailableFiles (see TS 28.532 [7] |  |  |
|  | subscribe (see TS 28.532 [7]) |  |  |
|  | unsubscribe (see TS 28.532 [7]) |  |  |

## 7.3	Performance data streaming services

The components of performance data streaming services for NFs, NSSIs, NSIs and networks/sub-networks are listed in table 7.3-1.

Table 7.3-1: Components of performance data streaming services

| Management service | Management service component type A | Management service component type B | Management service component type C |
| --- | --- | --- | --- |
| Performance data streaming service for NFs | establishStreamingConnection; terminateStreamingConnection; reportStreamData; getStreamInfo; addStreamInfo; updateStreamInfo; deleteStreamInfo.createMeasurementJob; stopMeasurementJob; listMeasurementJobs | IOCs for 5G NFs, as defined in TS 28.541 [3] | Performance measurements for 5G NFs, as defined in TS 28.552 [2]. |
| Performance data streaming service for NSSIs | establishStreamingConnection; terminateStreamingConnection; reportStreamData; getStreamInfo; addStreamInfo; updateStreamInfo; deleteStreamInfo.createMeasurementJob; stopMeasurementJob; listMeasurementJobs | IOC(s) for NSSI, as defined in TS 28.541 [3]. | Performance measurements for NSSI, as defined in TS 28.552 [2]. |
| Performance data streaming service for NSIs | establishStreamingConnection; terminateStreamingConnection; reportStreamData; getStreamInfo; addStreamInfo; updateStreamInfo; deleteStreamInfo.createMeasurementJob; stopMeasurementJob; listMeasurementJobs | IOC(s) for NSI, as defined in TS 28.541 [3]. | Performance measurements for NSI, as defined in TS 28.552 [2]. |
| Performance data streaming service for sub-networks | establishStreamingConnection; terminateStreamingConnection; reportStreamData; getStreamInfo; addStreamInfo; updateStreamInfo; deleteStreamInfo.createMeasurementJob; stopMeasurementJob; listMeasurementJobs | IOC(s) for sub-network, as defined in TS 28.541 [3]. | Performance measurements for sub-network, as defined in TS 28.552 [2]. |

## 7.4	Void

# 8	RESTful HTTP-based solution set of performance measurement job control service specific operations and notifications

## 8.1	Mapping of operations

### 8.1.1	Introduction

The IS operations are mapped to SS equivalents according to table 8.1.1-1.

Table 8.1.1-1: Mapping of IS operations to SS equivalents

| IS operation | HTTP Method | Resource URI | Qualifier |
| --- | --- | --- | --- |
| createMeasurementJob | POST | /measJobs | M |
| listMeasurementJobs | GET | /measJobs | M |
|  |  | /measJobs/{jobId} | M |
| stopMeasurementJob | DELETE | /measJobs/{jobId} | M |

### 8.1.2	Operation "createMeasurementJob"

The IS operation parameters are mapped to SS equivalents according to table 8.1.2-1 and table 8.1.2-2.

Table 8.1.2-1: Mapping of IS operation input parameters to SS equivalents (HTTP POST)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| iOCName | request body | iOCName | string | M |
| iOCInstanceList | request body | iOCInstanceList | array(uri-Type) | M |
| measurementCategoryList | request body | measurementCategoryList | array(string) | M |
| reportingMethod | request body | reportingMethod | reportingMethodType | M |
| granularityPeriod | request body | granularityPeriod | Integer | M |
| reportingPeriod | request body | reportingPeriod | Integer | M |
| startTime | request body | startTime | dateTime-Type | O |
| stopTime | request body | stopTime | dateTime-Type | O |
| schedule | request body | schedule | ScheduleType | O |
| streamTarget | request body | streamTarget | string | M |
| priority | request body | priority | PriorityType | O |
| reliability | request body | reliability | string | O |

Table 8.1.2-2: Mapping of IS operation output parameters to SS equivalents (HTTP POST)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| jobId | Location header | href | uri-Type | M |
| unsupportedList | response body | unsupportedList | array(unsupportedMeas-Type) | M |
| status | response status codes response body | n/a error | n/a error-ResponseType | M |

### 8.1.3	Operation "listMeasurementJobs"

The IS operation parameters are mapped to SS equivalents according to table 8.1.3-1 and table 8.1.3-2.

Table 8.1.3-1: Mapping of IS operation input parameters to SS equivalents (HTTP GET)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| jobIdList | Path Query | MeasJobs/{jobId} jobIdList | jobId: string array(string) | O |

Table 8.1.3-2: Mapping of IS operation output parameters to SS equivalents (HTTP POST)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| jobInfoList | response body | data | measJobsRetrieval-ResponseType | O |
| status | response status codes response body | n/a error | n/a error-ResponseType | M |

### 8.1.4	Operation "stopMeasurementJob"

The IS operation parameters are mapped to SS equivalents according to table 8.1.4-1 and table 8.1.4-2.

Table 8.1.4-1: Mapping of IS operation input parameters to SS equivalents (HTTP DELETE)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| jobId | path | /MeasJobs/{jobId} | jobId:string | M |

Table 8.1.4-2: Mapping of IS operation output parameters to SS equivalents (HTTP DELETE)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| status | response status codes response body | n/a error | n/a error-ResponseType | M |

## 8.2	Resources

### 8.2.0	Resource structure

Figure 8.2.0-1 shows the resource structure of the performance measurement job control  service.

Figure 8.2.0-1: Resource URI structure of the performance measurement job control service

Table 8.2.0-1 provides an overview of the resources and applicable HTTP methods.

Table 8.2.0-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| measJobs | /measJobs | GET | Retrieve all or a list of measurement jobs |
|  |  | POST | Create a measurement job |
| measJob | /measJobs /{jobId} | GET | Retrieve a measurement job |
|  |  | DELETE | Stop a measurement job |

### 8.2.1	Resource definitions

#### 8.2.1.1	Void

#### 8.2.1.2	Resource “/measJobs”

##### 8.2.1.2.1	Description

This resource represents a collection of measurement jobs.

##### 8.2.1.2.2	URI

Resource URI = {DN_prefix_authority_part}/{DN_prefix_remainder}/PerfMeasJobCtrlMnS/v1520/measJobs

The resource URI variables a defined in the following table.

Table 8.2.1.2.2-1: URI variables

| Name | Definition |
| --- | --- |
| DN_prefix_authority_part | See subclause 4.4 of TS 32.158 [14] |
| DN_prefix_remainder | See subclause 4.4 of TS 32.158 [14] |

##### 8.2.1.2.3	HTTP methods

8.2.1.2.3.1	HTTP POST

This method shall support the URI query parameters specified in the following table.

Table 8.2.1.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Description | SQ |
| --- | --- | --- | --- |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 8.2.1.2.3.1-2: Data structures supported by the POST request body on this resource

| Data type | Description | SQ |
| --- | --- | --- |
| measJobCreation-RequestType | The resource representation of the measurement job to be created | M |

Table 8.2.1.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| measJobCreation-ResponseType | 201 Created | In case of success the representation of the created measurement job is returned. | M |
|  | 202 Partially created | In case of partial success the representation of the created measurement job with unsupported list is returned. |  |
| error-Type | 4xx/5xx | Returned in case of an error | M |

8.2.1.2.3.2	HTTP GET

This method shall support the URI query parameters specified in the following table.

Table 8.2.1.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | Description | SQ |
| --- | --- | --- | --- |
| jobIdList | Array (string) | This parameter extends the set of targeted resources beyond the base resource identified with the path component of the URI. No scoping mechanism is specified in the present release. | O |

This method shall support the request data structures, the response data structures and response codes specified in the following tables.

Table 8.2.1.2.3.2-2: Data structures supported by the GET request body on this resource

| Data type | Description | SQ |
| --- | --- | --- |

Table 8.2.1.2.3.2-3: Data structures supported by the GET response body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| measJobsRetrieval-ResponseType | 200 OK | The resource representations of the measurement job list retrieved. | M |
| error-ResponseType | 4xx/5xx | Returned in case of an error | M |

#### 8.2.1.3	Resource “/measJobs/{jobId}”

##### 8.2.1.3.1	Description

This resource represents a measurement job.

##### 8.2.1.3.2	URI

Resource URI = {DN_prefix_authority_part}/{DN_prefix_remainder}/PerfMeasJobCtrlMnS/v1620/measJobs/{jobId}

The resource URI variables a defined in the following table.

Table 8.2.1.3.2-1: URI variables

| Name | Definition |
| --- | --- |
| DN_prefix_authority_part | See subclause 4.4 of TS 32.158 [14] |
| DN_prefix_remainder | See subclause 4.4 of TS 32.158 [14] |
| jobId | The id of the measurement job |

##### 8.2.1.3.3	HTTP methods

8.2.1.3.3.1	HTTP GET

This method shall support the URI query parameters specified in the following table.

Table 8.2.1.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | Description | SQ |
| --- | --- | --- | --- |

This method shall support the request data structures, the response data structures and response codes specified in the following tables.

Table 8.2.1.3.3.1-2: Data structures supported by the GET request body on this resource

| Data type | Description | SQ |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 8.2.1.3.3.1-3: Data structures supported by the GET response body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| measJobsRetrieval-ResponseType | 200 OK | The resource representations of the measurement job retrieved. | M |
| error-ResponseType | 4xx/5xx | Returned in case of an error | M |

8.2.1.3.3.2	HTTP DELETE

This method shall support the URI query parameters specified in the following table.

Table 8.2.1.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | Description | SQ |
| --- | --- | --- | --- |

This method shall support the request data structures, the response data structures and response codes specified in the following tables.

Table 8.2.1.3.3.2-2: Data structures supported by the DELETE request body on this resource

| Data type | Description | SQ |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 8.2.1.3.3.2-3: Data structures supported by the DELETE response body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| n/a | 204 No Content | In case of success no message body is returned | M |
| error-ResponseType | 4xx/5xx | Returned in case of an error | M |

## 8.3	Data type definitions

### 8.3.1	General

Table 8.3.1-1: Data types defined in this specification

| Data type | Reference | Description |
| --- | --- | --- |
| General types |  |  |
| dataTime-Type | 8.3.8.2 | Data type of date and time. |
| uri-Type | 8.3.8.2 | The data type of a URI |
| Types used in paths |  |  |
| Types used in query parts |  |  |
| Types used in request bodies |  |  |
| measJobCreation-RequestType | 8.3.6.1 | Used in the request body of HTTP POST describing the measurement job to be created |
| Types used in response bodies |  |  |
| measJobCreation-ResponseType | 8.3.6.2 | Used in the response body of HTTP POST describing the measurement job created |
| measJobsRetrieval-ResponseType | 8.3.6.3 | Used in the response body of HTTP GET describing the measurement job(s) retrieved |
| error-ResponseType | 8.3.6.4 | Used in the response body describing the error. |
| Types used for resources |  |  |
| measJobInfo-ResourceType | 8.3.6.5 | Used for representation of the measurement job information. |
| Types referenced by the definitions above |  |  |
| reportingMethod-Type | 8.3.8.3 | This defines the data type for reporting method. |
| schedule-Type | 8.3.7.1 | This defines the data type for schedule. |
| priority-Type | 8.3.8.4 | This defines the data type for priority of the measurement job. |
| unsupportedMeas-Type | 8.3.7.5 | This defines the data type for the unsupported measurement types for an IOC instance. |

Table 8.3.1-2: Data types imported

| Data type | Reference | Description |
| --- | --- | --- |

### 8.3.2	Void

### 8.3.3	Void

### 8.3.4	Structured general data types

None.

### 8.3.5	Structured path data types

None.

### 8.3.6	Query, message body and resource data types

#### 8.3.6.1	Type measJobCreation-RequestType

Table 8.3.6.1-1: Definition of type measJobCreation-RequestType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| iOCName | string | The IOC name of the IOC instances for which the measurement job is to be created. | M |
| iOCInstanceList | array(uri-Type) | The URI(s) of the IOC instances for which the measurement job is to be created. | M |
| measurementCategoryList | array(string) | The list of measurement type(s) to be measured. | M |
| reportingMethod | reportingMethod-Type | The reporting method of the measurements to be collected, i.e., by performance data file or by performance data streaming. | M |
| granularityPeriod | Integer | The granularity period of the measurement job. | M |
| reportingPeriod | Integer | The reporting period of the measurement job. | M |
| startTime | dateTime-Type | The begin time from which the measurement job will be active. | O |
| stopTime | dateTime-Type | The end time after which the measurement job will be stopped. | O |
| schedule | schedule-Type | The detailed time frames (within the startTime and stopTime) during which the measurement job is active and monitors the measurement type(s). | O |
| streamTarget | string | The target of performance data streams carrying the performance data stream unit(s). | M |
| priority | priority-Type | The priority of the measurement job. | O |
| reliability | string | The reliability of the measurement job. | O |

#### 8.3.6.2	Type measJobCreation-ResponseType

Table 8.3.6.2-1: Definition of type measJobCreation-ResponseType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| unsupportedList | array(unsupportedMeas-Type) | The list of unsupported IOC instances, unsupported measurement types and reason. | M |

#### 8.3.6.3	Type measJobsRetrieval-ResponseType

Table 8.3.6.3-1: Definition of type measJobsRetrieval-ResponseType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| jobInfoList | array(measJobInfo-ResourceType) | The list of measurement job information. | M |

#### 8.3.6.4	Type error-ResponseType

Table 8.3.6.4-1: Definition of type error-ResponseType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| error | object | Key indicating the response body containing an error | M |
| > errorInfo | string | Attribute allowing to convey error information in string format | M |

#### 8.3.6.5	Type measJobInfo-ResourceType

Table 8.3.6.3-1: Definition of type measJobsRetrieval-ResponseType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| href | uri-Type | The URI of the measurement job. | M |
| iOCName | string | The IOC name of the IOC instances for which the measurement job created. | M |
| iOCInstanceList | array(uri-Type) | The URI(s) of the IOC instances for which the measurement job is created. | M |
| measurementCategoryList | array(string) | The list of measurement type(s) measured. | M |
| reportingMethod | reportingMethod-Type | The reporting method of the measurements, i.e., by performance data file or by performance data streaming. | M |
| granularityPeriod | Integer | The granularity period of the measurement job. | M |
| reportingPeriod | Integer | The reporting period of the measurement job. | M |
| startTime | dateTime-Type | The begin time from which the measurement job is active. | O |
| stopTime | dateTime-Type | The end time after which the measurement job will be stopped. | O |
| schedule | schedule-Type | The detailed time frames (within the startTime and stopTime) during which the measurement job is active and monitors the measurement type(s). | O |
| streamTarget | string | The target of performance data streams carrying the performance data stream unit(s). | M |
| priority | priority-Type | The priority of the measurement job. | O |
| reliability | string | The reliability of the measurement job. | O |

### 8.3.7	Referenced structured data types

#### 8.3.7.1	Type schedule-Type

Table 8.3.7.1-1: Definition of schedule-Type

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| scheduleOption | scheduleOption-Type | It indicates the schedule is daily or weekly | M |
| dailySchedule | array(timeInterval-Type) | It defines the daily schedule. | M |
| weeklySchedule | array(scheduleOfDay-Type) | It defines the weekly schedule. | M |

#### 8.3.7.2	Type timeInterval-Type

Table 8.3.7.2-1: Definition of timeInterval-Type

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| intervalStart | string | It defines the start time of the schedule, by a string in Time format. | M |
| intervalEnd | string | It defines the end time of the schedule, by a string in Time format.s | M |

#### 8.3.7.3	Type scheduleOfDay-Type

Table 8.3.7.3-1: Definition of scheduleOfDay-Type

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| dayOfWeek | dayOfWeek-Type | It defines the day of a week. | M |
| intervalsOfDay | array(timeInterval-Type) | It defines the schedule of the day. | M |

#### 8.3.7.4	Void

#### 8.3.7.5	Type unsupportedMeas-Type

Table 8.3.7.5-1: Definition of unsupportedMeas-Type

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| iOCInstance | uri-Type | The URI of the IOC instance. | M |
| measurementTypeName | string | It defines the measurement type name that the IOC Instance as indicated above does not support | M |
| reason | string | It specifies the reason that measurement type name is not supported by the IOC instance | M |

### 8.3.8	Simple data types and enumerations

#### 8.3.8.1	General

This subclause defines simple data types and enumerations that are used by the data structures defined in the previous subclauses.

#### 8.3.8.2	Simple data types

Table 8.3.8.2-1: Simple data types

| Type name | Type definition | Description |
| --- | --- | --- |
| dataTime-Type | string | The data type for date and time in “date-time” format. |
| uri-Type | string | The type of a URI |

#### 8.3.8.3	Enumeration reportingMethod-Type

Table 8.3.8.3-1: Enumeration reportingMethod-Type

| Enumeration value | Description |
| --- | --- |
| file | It indicates that the performance data are to be reported by performance data file. |
| streaming | It indicates that the performance data are to be reported by performance data streaming. |

#### 8.3.8.4	Enumeration priority-Type

Table 8.3.8.4-1: Enumeration priority-Type

| Enumeration value | Description |
| --- | --- |
| Low | It indicates that the priority of the measurement job is low |
| medium | It indicates that the priority of the measurement job is medium |
| high | It indicates that the priority of the measurement job is high |

#### 8.3.8.5	Enumeration scheduleOption-Type

Table 8.3.8.5-1: Enumeration scheduleOption-Type

| Enumeration value | Description |
| --- | --- |
| daily | It indicates the schedule of the measurement job is daily. |
| weekly | It indicates the schedule of the measurement job is weekly. |

#### 8.3.8.6	Enumeration dayOfWeek-Type

Table 8.3.8.6-1: Enumeration dayOfWeek-Type

| Enumeration value | Description |
| --- | --- |
| Monday | It indicates Monday of a week. |
| Tuesday | It indicates Tuesday of a week. |
| Wednesday | It indicates Wednesday of a week. |
| Thursday | It indicates Thursday of a week. |
| Friday | It indicates Friday of a week. |
| Saturday | It indicates Saturday of a week. |
| Sunday | It indicates Sunday of a week. |

# 9	RESTful HTTP-based solution set of performance data streaming operations with WebSocket-based extensions

## 9.1	Introduction

This clause specifies the stage 3 solution set of the performance data streaming related operations (as defined in clause 6.2) based on RESTful HTTP and WebSocket protocol (see IETF RFC 6455 [18]).

## 9.2	Mapping of operations

### 9.2.1	Introduction

The IS operations are mapped to SS equivalents according to table 9.2.1-1.

Table 9.2.1-1: Mapping of IS operations to SS equivalents

| IS operation | Method/frame | Qualifier |
| --- | --- | --- |
| establishStreamingConnection | HTTP POST (see NOTE) | M |
|  | HTTP GET (Upgrade, see NOTE) | M |
| terminateStreamingConnection | WebSocket Close frame sent (frame with opcode of 0x8), and WebSocket Close frame received (frame with opcode of 0x8 for successful case) | M |
| reportStreamData | WebSocket Data frame sent (frame with opcode of 0x2) | M |
| addStreamInfo | HTTP GET | M |
| updateStreamInfo | HTTP PATCH | M |
| deleteStreamInfo | HTTP DELETE | M |
| Note: the establishStreamingConnection is mapped to a HTTP POST operation followed by a HTTP GET operation. The HTTP POST operation is to send the information in StreamInfoList parameter to the consumer, and the HTTP GET (Upgrade) operation is to establish the WebSocket connection. |  |  |

### 9.2.2	Operation "establishStreamingConnection"

The IS operation parameters are mapped to SS equivalents according to table x.2.2-1 and table x.2.2-4.

Table 9.2.2-1: Mapping of IS operation input parameters to SS equivalents (HTTP POST)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| streamInfoList | request body | streamInfoList | array(streamInfoList -Type) | M |

Table 9.2.2-2: Mapping of IS operation output parameters to SS equivalents (HTTP POST)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| status | response status codes response body | n/a error | n/a error-ResponseType | M |

Table 9.2.2-3: Mapping of IS operation input parameters to SS equivalents (HTTP GET (Upgrade))

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| producerReference | Headers | Origin | String | n/a |
| -- | HTTP-Version (Request-Line) | -- | String (see Note 1) | M |
| -- | Upgrade Header | -- | Constant string: websocket | M |
| -- | Connection Header | -- | Constant string: Upgrade | M |
| -- | Sec-WebSocket-Key Header | -- | String (see Note 2) | M |
| -- | Sec-WebSocket-Version Header | -- | String (see Note 3) | M |
| -- | See Note 4. |  |  |  |
| NOTE 1: The HTTP version shall be not earlier than HTTP/1.1. NOTE 2: The valid value needs to be assigned according to WebSocket protocol (see IETF RFC 6455 [18]). NOTE 3: The valid value needs to be assigned according to WebSocket protocol (see IETF RFC 6455 [18]). NOTE 4: Other SS parameters (not listed in this table) independent from the Stage 2 may be used, according to the WebSocket protocol (see IETF RFC 6455 [18]). |  |  |  |  |

Table 9.2.2-4: Mapping of IS operation output parameters to SS equivalents (HTTP GET (Upgrade))

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| connectionId | n/a | -- | n/a | n/a |
| status | HTTP-Version (Response-Line) | -- | String (see Note 1) | M |
|  | Status-Code | -- | String |  |
|  | response body | error | error-ResponseType |  |
| -- | Upgrade Header |  | Constant string: websocket | M |
| -- | Connection Header | -- | Constant string: Upgrade | M |
|  | Sec-WebSocket-Accept Header | -- | String (see Note 2) | M |
| -- | See Note 3. |  |  |  |
| NOTE 1: The HTTP version shall be not earlier than HTTP/1.1. NOTE 2: The valid value needs to be assigned according to WebSocket protocol (see IETF RFC 6455 [18]). NOTE 3: Other SS parameters (not listed in this table) independent from the Stage 2 may be used, according to the WebSocket protocol (see IETF RFC 6455 [18]). |  |  |  |  |

The message flow for establishing a streaming connection is as follows:

1.	The performance data streaming service producer sends a HTTP POST request to the consumer.

The URI identifies the "…/streamInfoList" collection resource.

The request message body shall carry the information about the streams, by parameter “StreamInfoList”.

2.	The consumer sends a HTTP POST response to the producer.

On success "201 Posted" shall be returned.

On failure, an appropriate error code shall be returned. The response message body may carry an error object.

3.	If step 2 is successful, the performance data streaming service producer sends a HTTP GET (upgrade) request to the consumer to establish the WebSocket connection.

The URI identifies the "…/streamingConnection" resource with the /secure/flag;

The HTTP-version in the Request-line indicates the HTTP version which is no earlier than HTTP/1.1;

The Upgrade header is with value “websocket”;

The Connection header is with value “Upgrade”;

The Sec-WebSocket-Key header is with a valid value according to IETF RFC 6455 [18].

The Sec-WebSocket-Version header is with a valid according to IETF RFC 6455 [18].

4.	The consumer sends a HTTP GET (Upgrade) response to the producer.

On success, "101 Switching Protocols" shall be returned;

On failure, an appropriate error code shall be returned. The response message body may carry an error object.

The HTTP-version in the Response-line indicates the HTTP version which is no earlier than HTTP/1.1;

The Upgrade header is with value “websocket”;

The Connection header is with value “Upgrade”;

The Sec-WebSocket-Accept header is with a valid value according to IETF RFC 6455 [18].

### 9.2.3	Operation "terminateStreamingConnection"

The IS operation parameters are mapped to SS equivalents according to table x.2.3-1 and table x.2.3-2.

Table 9.2.3-1: Mapping of IS operation input parameters to SS equivalents (WebSocket Close frame sent)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| connectionId | n/a | -- | n/a | n/a |
| -- | Opcode (see clause 5 of IETF RFC 6455 [18]) | -- | Constant value: 0x8 | M |

Table 9.2.3-2: Mapping of IS operation output parameters to SS equivalents (WebSocket Close frame received)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| Status | Opcode | -- | For a successful operation, the Opcode is 0x8, and for an unsuccessful operation, the Opcode has a value other than 0x8 (see clause 5 of IETF RFC 6455 [18]). | M |

### 9.2.4	Operation "reportStreamData"

The IS operation parameters are mapped to SS equivalents according to table 9.2.4-1 and table 9.2.4-2.

Table 9.2.4-1: Mapping of IS operation input parameters to SS equivalents (WebSocket Data frame sent with Opcode of 0x1)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| streamId | Payload data | streamId | See Annex X for the ASN.1 definition of Performance Data Stream Units. | M |
| granularityPeriodEndTime | Payload data | granularityPeriodEndTime | See Annex X for the ASN.1 definition of Performance Data Stream Units. | M |
| measResults | Payload data | measResults | See clause Annex X for the ASN.1 of Performance Data Stream Units. | M |

Table 9.2.4-2: Mapping of IS operation output parameters to SS equivalents

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| Status | n/a | -- See Note 1. | n/a | n/a |
| NOTE 1: The delivery of WebSocket Data frame is taken care of by the underlying TCP (see IETF RFC 793 [19]) which provides reliable data transmission and ensures the data delivery. There is no mechanism at WebSocket protocol level to report the delivery status for WebSocket Data frame. |  |  |  |  |

### 9.2.5	Operation "getStreamInfo"

The IS operation parameters are mapped to SS equivalents according to table 9.2.5-1 and table 9.2.5-2.

Table 9.2.5-1: Mapping of IS operation input parameters to SS equivalents (HTTP GET)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| streamIdList | Path query | /streamInfoList/{streamId} streamIdList | streamId: integer array(integer) | M |

Table 9.2.5-2: Mapping of IS operation output parameters to SS equivalents (HTTP GET)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| listOfStreamInfoOut | response body | streamInfoOut, listOfStreamInfoOut | streamInfoRetrieval-ResponseType, listOfStreamInfoRetrieval-ResponseType, | M |
| status | response status codes response body | n/a error | n/a error-ResponseType | M |

### 9.2.6	Operation "addStreamInfo"

The IS operation parameters are mapped to SS equivalents according to table 9.2.6-1 and table 9.2.6-2.

Table 9.2.6-1: Mapping of IS operation input parameters to SS equivalents (HTTP POST)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| ListOfStreamInfoToAdd | request body | streamInfoList | streamInfoListPost-RequestType | M |

Table 9.2.6-2: Mapping of IS operation output parameters to SS equivalents (HTTP POST)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| listOfStreamInfoAdded | response body | ListOfStreamInfoPosted | streamInfoListPost-ResponseType |  |
| status | response status codes response body | n/a error | n/a error-ResponseType | M |

### 9.2.7	Operation "updateStreamInfo"

The IS operation parameters are mapped to SS equivalents according to table 9.2.7-1 and table 9.2.7-2.

Table 9.2.7-1: Mapping of IS operation input parameters to SS equivalents (HTTP PATCH)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| streamIdList | path, query | /streamInfoList/{streamId} streamIdList | streamId: integer array(integer) | M |
| listOfStreamInfoToUpdate | request body | streamInfoToUpdate, listOfStreamInfoToUpdate | streamInfoToUpdate-RequestType, listOfStreamInfoToUpdate-RequestType, | M |

Table 9.2.7-2: Mapping of IS operation output parameters to SS equivalents (HTTP PATCH)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| listOfStreamInfoUpdated | response body | streamInfoUpdated listOfStreamInfoUpdated | streamInfoUpdate-ResponseType, listOfStreamInfoUpdate-ResponseType, | M |
| status | response status codes response body | n/a error | n/a error-ResponseType | M |

### 9.2.8	Operation "deleteStreamInfo"

The IS operation parameters are mapped to SS equivalents according to table 9.2.8-1 and table 9.2.8-2.

Table 9.2.8-1: Mapping of IS operation input parameters to SS equivalents (HTTP DELETE)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| streamId | path | /streamInfoList/{streamId} | streamId: integer | M |

Table 9.2.8-2: Mapping of IS operation output parameters to SS equivalents (HTTP DELETE)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| status | response status codes response body | n/a error | n/a error-ResponseType | M |

## 9.3	Resources

### 9.3.1	Resource structure

Figure 9.3.1-1 shows the resource structure of the Performance Data Streaming Service.

Figure 9.3.1-1: Resource URI structure of the Performance Data Streaming Service

Table 9.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 9.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| streamInfoList | /streamInfoList | POST | Send the information about the streams to the consumer. |
| streamInfo | /streamInfoList/{streamId} | PATCH | Update the stream information |
|  |  | DELETE | Delete the stream information |
| streamingConnection | /streamingConnection | GET (Upgrade) | Establish the WebSocket connection. |

### 9.3.2	Resource definitions

#### 9.3.2.1	Resource “/streamInfoList”

##### 9.3.2.1.1	Description

This resource represents a set of information about the streams.

##### 9.3.2.1.2	URI

Resource URI = http://{streamTarget}/PerfDataStreamingMnS/v1530/streamInfoList

The resource URI variables a defined in the following table.

Table 9.3.2.1.2-1: URI variables

| Name | Definition |
| --- | --- |
| streamTarget | The “streamTarget” part corresponds to the streamTarget parameter provided in the createMeasurementJob operation (see clause 6.1.1.2) or the streamTarget attribute of the MOI of MeasurementControlor MeasurementReader, see 3GPP TS 28.622 [5]). |

##### 9.3.2.1.3	HTTP methods

9.3.2.1.3.1	HTTP POST

This method shall support the URI query parameters specified in the following table.

Table 9.3.2.1.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Description | SQ |
| --- | --- | --- | --- |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 9.3.2.1.3.1-2: Data structures supported by the POST request body on this resource

| Data type | Description | SQ |
| --- | --- | --- |
| streamInfoListPost-RequestType | The resource representation of the set of information about streams to be posted. | M |

Table 9.3.2.1.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| streamInfoListPost-ResponseType | 201 Posted | In case of success the representation of the posted streamInfoList is returned. | M |
|  | 202 Partially posted | In case of partial success the representation of the posted streamInfoList is returned. |  |
| error-ResponseType | 4xx/5xx | Returned in case of an error | M |

9.3.2.1.3.2	HTTP GET

This method shall support the URI query parameters specified in the following table.

Table 9.3.2.1.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | Description | SQ |
| --- | --- | --- | --- |
| streamIdList | array(integer) | The list of streamId for which the stream information are to be returned. |  |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 9.3.2.1.3.2-2: Data structures supported by the GET request body on this resource

| Data type | Description | SQ |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 9.3.2.1.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| listOfStreamInfoRetrieval-ResponseType | 200 OK | In case of success the representation of the retrieved streamInfoList is returned. | M |
|  | 202 Partially retrieved | In case of partial success the representation of the retrieved streamInfoList is returned. | M |
| error-ResponseType | 4xx/5xx | Returned in case of an error | M |

9.3.2.1.3.3	HTTP PATCH

This method shall support the URI query parameters specified in the following table.

Table 9.3.2.1.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | Description | SQ |
| --- | --- | --- | --- |
| streamIdList | array(integer) | The list of streamId for which the stream information are to be updated. |  |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 9.3.2.1.3.3-2: Data structures supported by the PATCH request body on this resource

| Data type | Description | SQ |
| --- | --- | --- |
| listOfStreamInfoToUpdate-RequestType | The information of the streams identified by the streamIdList parameter in the “query” field to be updated. | M |

Table 9.3.2.1.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| listOfStreamInfoUpdate-ResponseType | 200 OK | In case of success the representation of the updated streamInfoList is returned. | M |
|  | 200 Partially updated | In case of partial success the representation of the updated streamInfoList is returned. |  |
| error-ResponseType | 4xx/5xx | Returned in case of an error | M |

#### 9.3.2.1a	Resource “/streamInfoList/{streamId}”

##### 9.3.2.1a.1	Description

This resource represents the information for a stream.

##### 9.3.2.1a.2	URI

Resource URI = http://{streamTarget}/PerfDataStreamingMnS/v1530/streamInfoList/{streamId}

The resource URI variables a defined in the following table.

Table 9.3.2.1a.2-1: URI variables

| Name | Definition |
| --- | --- |
| streamTarget | See table 9.3.2.1.2-1. |

##### 9.3.2.1a.3	HTTP methods

9.3.2.1a.3.1	HTTP GET

This method shall support the URI query parameters specified in the following table.

Table 9.3.2.1a.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | Description | SQ |
| --- | --- | --- | --- |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 9.3.2.1a.3.1-2: Data structures supported by the GET request body on this resource

| Data type | Description | SQ |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 9.3.2.1a.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| streamInfoRetrieval-ResponseType | 200 OK | In case of success the representation of the retrieved stream information is returned. | M |
| error-ResponseType | 4xx/5xx | Returned in case of an error | M |

9.3.2.1a.3.2	HTTP PATCH

This method shall support the URI query parameters specified in the following table.

Table 9.3.2.1a.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | Description | SQ |
| --- | --- | --- | --- |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 9.3.2.1a.3.2-2: Data structures supported by the PATCH request body on this resource

| Data type | Description | SQ |
| --- | --- | --- |
| streamInfoToUpdate-RequestType | Represention of the stream information to be updated. | M |

Table 9.3.2.1a.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| streamInfoUpdate-ResponseType | 200 OK | In case of success the representation of the updated stream information is returned. | M |
| error-ResponseType | 4xx/5xx | Returned in case of an error | M |

9.3.2.1a.3.3	HTTP DELETE

This method shall support the URI query parameters specified in the following table.

Table 9.3.2.1a.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | Description | SQ |
| --- | --- | --- | --- |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 9.3.2.1a.3.3-2: Data structures supported by the DELETE request body on this resource

| Data type | Description | SQ |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 9.3.2.1a.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| n/a | 204 No Content | In case of success no message body is returned | M |
| error-ResponseType | 4xx/5xx | Returned in case of an error | M |

#### 9.3.2.2	Resource “/streamingConnection”

##### 9.3.2.2.1	Description

This resource represents a streamingConnection (to be) upgraded to WebSocket protocol.

##### 9.3.2.2.2	URI

Resource URI = wss://{streamTarget}/PerfDataStreamingMnS/v1530/streamingConnection

The resource URI variables a defined in the following table.

Table 9.3.2.2.2-1: URI variables

| Name | Definition |
| --- | --- |
| streamTarget | The “streamTarget” part corresponds to the streamTarget parameter provided in the createMeasurementJob operation (see clause 6.1.1.2) or the streamTarget attribute of the MOI of MeasurementControlor MeasurementReader, see 3GPP TS 28.622 [5]). |

##### 9.3.2.2.3	HTTP methods

9.3.2.2.3.1	HTTP GET

This method shall support the URI query parameters specified in the following table.

Table 9.3.2.2.3.1-1: Header parameters supported by the GET request on this resource

| Name | Data type | Description | SQ |
| --- | --- | --- | --- |
| Upgrade | Upgrade-HeaderType | To indicate the HTTP GET operation is to upgrade the connection to WebSocket protocol |  |
| Connection | Connection-HeaderType | To indicate the HTTP GET operation is to upgrade the connection to another protocol |  |
| Sec-WebSocket-Key | Sec-WebSocket-Key-HeaderType | The Sec-WebSocket-Key needed for establishing the WebSocket connection. |  |
| Sec-WebSocket-Version | Sec-WebSocket-Version-HeaderType | The Sec-WebSocket-Version needed for establishing the WebSocket connection. |  |

This method shall support the URI query parameters specified in the following table.

Table 9.3.2.2.3.1-2: URI query parameters supported by the GET method on this resource

| Name | Data type | Description | SQ |
| --- | --- | --- | --- |

This method shall support the request data structures, the response data structures and response codes specified in the following tables.

Table 9.3.2.2.3.1-3: Data structures supported by the GET request body on this resource

| Data type | Description | SQ |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 9.3.2.2.3.1-4: Header parameters supported by the GET response on this resource

| Name | Data type | Description | SQ |
| --- | --- | --- | --- |
| Upgrade | Upgrade-HeaderType | To indicate the HTTP GET operation is to upgrade the connection to WebSocket protocol |  |
| Connection | Connection-HeaderType | To indicate the HTTP GET operation is to upgrade the connection to another protocol |  |
| Sec-WebSocket-Accept | Sec-WebSocket-Accept-HeaderType | The Sec-WebSocket-Accept responded when establishing the WebSocket connection. |  |

Table 9.3.2.2.3.1-5: Data structures supported by the GET response body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| n/a | 101 Switching Protocols | The status code indicating the connection has been successfully upgraded to WebSocket. | M |
| error-ResponseType | 4xx/5xx | Returned in case of an error | M |

## 9.4	Data type definitions

### 9.4.1	General

Table 9.4.1-1: Data types defined in this specification

| Data type | Reference | Description |
| --- | --- | --- |
| General types |  |  |
| uri-Type | x.4.6.2 | The data type of a URI |
| Types used in paths |  |  |
| Types used in headers |  |  |
| Upgrade-HeaderType | x.4.6.2 | Used in the headers of HTTP GET request to upgrade the connection to WebSocket. |
| Connection-HeaderType | x.4.6.2 | Used in the headers of HTTP GET request to upgrade the connection to WebSocket. |
| Sec-WebSocket-Key-HeaderType | x.4.6.2 | Used in the headers of HTTP GET request to upgrade the connection to WebSocket. |
| Sec-WebSocket-Version-HeaderType | x.4.6.2 | Used in the headers of HTTP GET request to upgrade the connection to WebSocket. |
| Sec-WebSocket-Accept | x.4.6.2 | Used in the headers of HTTP GET response for upgrading the connection to WebSocket. |
| Types used in query parts |  |  |
| Types used in request bodies |  |  |
| streamInfoListPost-RequestType | x.4.4.1 | Used in the request body of HTTP POST to post the set of information about the streams. |
| streamInfoToUpdate-RequestType | 9.4.4.x | Used in the request body of HTTP PATCH to update the information for one or more streams. |
| Types used in response bodies |  |  |
| error-ResponseType | x.4.4.2 | Used in the response body describing the error. |
| streamInfoListPost-ResponseType | 9.4.4.p | Used in the response body of HTTP POST to return the set of stream information posted. |
| streamInfoRetrieval-ResponseType | 9.4.4.u | Used in the response body of HTTP GET to return the information about one stream. |
| streamInfoListRetrieval-ResponseType | 9.4.4.w | Used in the response body of HTTP GET to return the set of information about the streams. |
| streamInfoUpdate-ResponseType | 9.4.4.y | Used in the response body of HTTP PATCH to return the updated information about one stream. |
| streamInfoListUpdate-ResponseType | 9.4.4.z | Used in the response body of HTTP PATCH to return the set of updated information about the streams. |
| Types used for resources |  |  |
| Types referenced by the definitions above |  |  |
| streamInfoIn-Type | 9.4.5.1 |  |
| streamInfoOut-Type | 9.4.5.x |  |

Table 9.4.1-2: Data types imported

| Data type | Reference | Description |
| --- | --- | --- |

### 9.4.2	Structured general data types

None.

### 9.4.3	Structured path data types

None.

### 9.4.4	Query, message body and resource data types

#### 9.4.4.1	Type streamInfoListPost-RequestType

Table 9.4.4.1-1: Definition of type streamInfoListPost-RequestType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| streamInfoList | Array (streamInfoIn-Type) | The list of information about the streams. | M |

#### 9.4.4.2		Type error-ResponseType

Table 9.4.4.2-1: Definition of type error-ResponseType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| error | object | Key indicating the response body containing an error | M |
| > errorInfo | string | Attribute allowing to convey error information in string format | M |

#### 9.4.4.3	Type streamInfoToUpdate-RequestType

Table 9.4.4.3-1: Definition of type streamInfoToUpdate-RequestType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| iOCInstance | uri-Type | The updated URI of the measured object instance | M |
| measTypes | array(string) | The updated list of measurement type. | M |

#### 9.4.4.4	Type streamInfoListPost-ResponseType

Table 9.4.4.p-1: Definition of type streamInfoListPost-ResponseType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| ListOfStreamInfoPosted | array(streamInfoOut-Type) | The set of stream information posted | M |

#### 9.4.4.5	Type streamInfoRetrieval-ResponseType

Table 9.4.4.u-1: Definition of type streamInfoRetrieval-ResponseType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| streamInfoOut | streamInfoOut-Type | The returned stream information | M |

#### 9.4.4.6	Type streamInfoListRetrieval-ResponseType

Table 9.4.4.w-1: Definition of type streamInfoRetrieval-ResponseType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| listOfStreamInfoOut | array(streamInfoOut-Type) | The returned information for the list of streams | M |

#### 9.4.4.7	Type streamInfoUpdate-ResponseType

Table 9.4.4.u-1: Definition of type streamInfoUpdate-ResponseType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| streamInfoUpdated | streamInfoOut-Type | The updated stream information | M |

#### 9.4.4.8	Type streamInfoListUpdate-ResponseType

Table 9.4.4.8-1: Definition of type streamInfoUpdate-ResponseType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| listOfStreamUpdated | array(streamInfoOut-Type) | The updated information for the list of streams | M |

### 9.4.5	Referenced structured data types

#### 9.4.5.1	Type streamInfoIn-Type

Table 9.4.5.1-1: Definition of streamInfoIn-Type

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| streamId | integer | The id of the stream | M |
| iOCInstance | uri-Type | The URI of the IOC instance whose measurement types as indicated as below are (to be) reported vis this stream. | M |
| measTypes | array(string) | It specifies the measurement type names whose measurement result values are (to be) reported by the Performance Data Stream Units via this stream | M |

#### 9.4.5.2	Type streamInfoOut-Type

Table 9.4.5.2-1: Definition of streamInfoOut-Type

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| streamId | uri-Type | The id of the stream | M |
| iOCInstance | uri-Type | The URI of the IOC instance whose measurement types as indicated as below are (to be) reported vis this stream. | M |
| measTypes | array(string) | It specifies the measurement type names whose measurement result values are (to be) reported by the Performance Data Stream Units via this stream | M |

### 9.4.6	Simple data types and enumerations

#### 9.4.6.1	General

This subclause defines simple data types and enumerations that are used by the data structures defined in the previous subclauses.

#### 9.4.6.2	Simple data types

Table 9.4.6.2-1: Simple data types

| Type name | Type definition | Description |
| --- | --- | --- |
| Upgrade-HeaderType | Constant string “websocket” | The data type of upgrade header of HTTP GET operation (request and response) for establishing the WebSocket connection. |
| Connection-HeaderType | Constant string “Upgrade” | The data type of Connection header of HTTP GET operation (request and response) for establishing the WebSocket connection. |
| Sec-WebSocket-Key-HeaderType | string | The data type of Sec-WebSocket-Key header of HTTP GET request for establishing the WebSocket connection. |
| Sec-WebSocket-Version-HeaderType | string | The data type of Sec-WebSocket-Version of HTTP GET request for establishing the WebSocket connection. |
| Sec-WebSocket-Accept | string | The data type of Sec-WebSocket-Accept of HTTP GET response for establishing the WebSocket connection. |
| uri-Type | string | The type of a URI |

Annex A : 
Void

Annex B (informative): 
Procedures for performance assurance services

# B.1	NF measurement job creation

The Figure B.1-1 illustrates an example of procedure for creating a measurement job for NF(s).

Figure B.1-1: Example of procedure for NF measurement job creation

1a.	The authorized consumer invokes the CreateMeasurementJob operation (see clause 6.1.1) to NF measurement job control service producer to request creation of a measurement job for NF(s).

2.	The NF measurement job control service producer checks if new measurement type(s) need to be collected from the NF(s) to be measured.

3.	For each NF to be measured, if new measurements type(s) need to be collected:

3a.	the NF measurement job control service producer requests NF to collect the performance data;

3b.	the NF measurement job control service producer receives the acknowledgement of the request from NF.

1b.	The NF measurement job control service producer returns the result of CreateMeasurementJob operation (see clause 6.1.1) to the consumer.

If the NF measurement job is successfully created, the NF measurement job control service producer will collect the performance data from the NF(s) accordingly, and make the measurement results available to the NF performance data reporting service producer for each reporting period.

# B.2	NSSI measurement job creation

The Figure B.2-1 illustrates an example of procedure for creating a measurement job for NSSI(s).

This procedure is only applicable for the scenario where the NSSI measurement type(s) can be decomposed into the measurement data type(s) of the constituent NSSI(s) and NF(s).

Figure B.2-1: Example of procedure for NSSI measurement job creation

1a.	The authorized consumer invokes the CreateMeasurementJob operation (see clause 6.1.1) to NSSI measurement job control service producer to request creation of a measurement job for NSSI(s).

2.	The NSSI measurement job control service producer decomposes the measurement type(s) of each NSSI to the measurement type(s) of the constituent NSSI(s) and/or NF(s), and checks if new measurement type(s) need to be collected for the constituent NSSI(s) and/or NF(s).

3.	For each constituent NSSI to be measured, if new measurements type(s) need to be collected, the NSSI measurement job control service producer acts as consumer of another NSSI measurement job control service instance, and requests the corresponding NSSI measurement job control service producer to request creation of measurement job for the NSSI (following the same procedure as illustrated in this figure).

It is also possible to create one measurement job to collect the performance data for multiple NSSI(s).

4.	For each constituent NF to be measured, if new measurements type(s) need to be collected, the NSSI measurement job control service producer acts as consumer of NF measurement job control service, and requests the corresponding NF measurement job control service producer to request creation of measurement job for the NF (according to the NF measurement job creation procedure as illustrated in clause B.1).

It is also possible to create one measurement job to collect the performance data for multiple NF(s).

1b.	The NSSI measurement job control service producer returns the result of CreateMeasurementJob operation (see clause 6.1.1) to the consumer.

If the NSSI measurement job is successfully created, the NSSI measurement job control service producer will collect the performance data for the constituent NSSI(s) and/or NF(s) accordingly, generate the measurement results for the measured NSSI(s) and make the measurement results available to the NSSI performance data reporting service producer for each reporting period.

# B.3	NSI measurement job creation

This Figure B.3-1 illustrates an example of procedure for creating a measurement job for NSI(s).

This procedure is only applicable for the scenario where the NSI measurement type(s) can be decomposed into the measurement data type(s) of the constituent NSSI(s) or NF(s).

Figure B.3-1: Example of procedure for NSI measurement job creation

1a.	The authorized consumer invokes the CreateMeasurementJob operation (see clause 6.1.1) to NSI measurement job control service producer to request creation of a measurement job for NSI(s).

2.	The NSI measurement job control service producer decomposes the measurement type(s) of each NSI to the measurement type(s) of the constituent NSSI(s) and/or NF(s), and checks if new measurement type(s) need to be collected for the constituent NSSI(s) and/or NF(s).

3.	For each constituent NSSI to be measured, if new measurements type(s) need to be collected, the NSI measurement job control service producer acts as consumer of the NSSI measurement job control service, and requests the corresponding NSSI measurement job control service producer to request creation of measurement job for the NSSI (according to the procedure as illustrated in clause B.2).

It is also possible to create one measurement job to collect the performance data for multiple NSSI(s).

4.	For each constituent NF to be measured, if new measurements type(s) need to be collected, the NSI measurement job control service producer acts as consumer of NF measurement job control service, and requests the corresponding NF measurement job control service producer to request creation of measurement job for the NF (according to the NF measurement job creation procedure as illustrated in clause B.1).

It is also possible to create one measurement job to collect the performance data for multiple NF(s).

1b.	The NSI measurement job control service producer returns the result of CreateMeasurementJob operation (see clause 6.1.1) to the consumer.

If the NSI measurement job is successfully created, the NSI measurement job control service producer will collect the performance data for the constituent NSSI(s) and/or NF(s) accordingly, generate the measurement results for the measured NSI(s) and make the measurement results available to the NSI performance data reporting service producer for each reporting period.

# B.4	Network measurement job creation

This Figure B.4-1 illustrates an example of procedure for creating a measurement job for network/subnetwork(s).

This procedure is only applicable for the scenario where the network/subnetwork measurement type(s) can be decomposed into the measurement data type(s) of the constituent NF(s).

Figure B.4-1: Example of procedure for network measurement job creation

1a.	The authorized consumer invokes the CreateMeasurementJob operation (see clause 6.1.1) to network measurement job control service producer to request creation of a measurement job for network/subnetwork(s).

1b.	The network measurement job control service producer returns the result of CreateMeasurementJob operation (see clause 6.1.1) to the consumer.

2.	The network measurement job control service producer decomposes the measurement type(s) of each network/subnetwork to the measurement type(s) of the constituent NF(s), and checks if new measurement type(s) need to be collected for the constituent NF(s).

3.	For each constituent NF to be measured, if new measurements type(s) need to be collected, the network/sub-network measurement job control service producer acts as consumer of NF measurement job control service, and requests the corresponding NF measurement job control service producer to request creation of measurement job for the NF (according to the NF measurement job creation procedure as illustrated in annex B.1).

It is also possible to create one measurement job to collect the performance data for multiple NF(s).

If the network measurement job is successfully created, the network measurement job control service producer will collect the performance data for the constituent NF(s) accordingly, generate the measurement results for the measured network/subnetwork(s) and make the measurement results available to the network performance data reporting service producer for each reporting period.

# B.5	NF measurement job termination

This Figure B.5-1 illustrates an example of procedure for stopping a measurement job for NF(s).

Figure B.5-1: Example of procedure for NF measurement job termination

1a.	The authorized consumer invokes the StopMeasurementJob operation (see clause 6.1.2) to NF measurement job control service producer to request termination of a measurement job for NF(s).

1b.	The NF measurement job control service producer returns the result of StopMeasurementJob operation (see clause 6.1.2) to the consumer.

2.	The NF measurement job control service producer checks if the measurement collection activity needs to be stopped at the NF(s).

3.	For each NF being measured, if the measurement collection activity needs to be stopped at the NF(s),

3a.	the NF measurement job control service producer requests NF to stop collecting the performance data;

3b.	the NF measurement job control service producer receives the acknowledgement of the request from NF.

# B.6	NSSI measurement job termination

This Figure B.6-1 illustrates an example of procedure for stopping a measurement job for NSSI(s).

This procedure is only applicable for the scenario where the NSSI measurement type(s) can be decomposed into the measurement data type(s) of the constituent NSSI(s) and NF(s).

Figure B.6-1: Example of procedure for NSSI measurement job termination

1a.	The authorized consumer invokes the StopMeasurementJob operation (see clause 6.1.2) to NSSI measurement job control service producer to request termination of a measurement job for NSSI(s).

1b.	The NSSI measurement job control service producer returns the result of StopMeasurementJob operation (see clause 6.1.2) to the consumer.

2.	The NSSI measurement job control service producer checks if the measurement job(s) for the constituent NSSI(s) and/or NF(s) need to be stopped.

3.	For each measurement job for the constituent NSSI(s) needs to be stopped, the NSSI measurement job control service producer acts as consumer of another NSSI measurement job control service instance, and requests the corresponding NSSI measurement job control service producer to terminate the measurement job for the constituent NSSI(s) (following the same procedure as illustrated in this figure).

4.	For each measurement job for the constituent NF(s) needs to be stopped, the NSSI measurement job control service producer acts as consumer of NF measurement job control service, and requests the corresponding NF measurement job control service producer to terminate the measurement job for the NF(s) (according to the NF measurement job termination procedure as illustrated in clause B.5).

# B.7	NSI measurement job termination

This Figure B.7-1 illustrates an example of procedure for stopping a measurement job for NSI(s).

This procedure is only applicable for the scenario where the NSI measurement type(s) can be decomposed into the measurement data type(s) of the constituent NSSI(s) and NF(s).

Figure B.7-1: Example of procedure for NSI measurement job termination

1a.	The authorized consumer invokes the StopMeasurementJob operation (see clause 6.1.2) to NSI measurement job control service producer to request creation of a measurement job for NSI(s).

1b.	The NSI measurement job control service producer returns the result of StopMeasurementJob operation (see clause 6.1.2) to the consumer.

2.	The NSI measurement job control service producer checks if the measurement job(s) for the constituent NSSI(s) and/or NF(s) need to be stopped.

3.	For each measurement job for the constituent NSSI(s) needs to be stopped, the NSI measurement job control service producer acts as consumer of the NSSI measurement job control service, and requests the corresponding NSSI measurement job control service producer to terminate the measurement job for the constituent NSSI(s) (according to the NSSI measurement job termination procedure as illustrated in clause B.6).

4.	For each measurement job for the constituent NF(s) needs to be stopped, the NSI measurement job control service producer acts as consumer of NF measurement job control service, and requests the corresponding NF measurement job control service producer to terminate the measurement job for the NF(s) (according to the NF measurement job termination procedure as illustrated in clause B.5).

# B.8	Network measurement job termination

This Figure B.8-1 illustrates an example of procedure for stopping a measurement job for network/subnetwork(s).

This procedure is only applicable for the scenario where the network/subnetwork measurement type(s) can be decomposed into the measurement data type(s) of the constituent NF(s).

Figure B.8-1: Example of procedure for network measurement job creation

1a.	The authorized consumer invokes the StopMeasurementJob operation (see clause 6.1.2) to network measurement job control service producer to request termination of a measurement job for network/subnetwork(s).

1b.	The network measurement job control service producer returns the result of StopMeasurementJob operation (see clause 6.1.2) to the consumer.

2.	The network measurement job control service producer checks if the measurement job(s) for the constituent NF(s) need to be stopped.

3.	For each measurement job for the constituent NF(s) needs to be stopped, the network measurement job control service producer acts as consumer of NF measurement job control service, and requests the corresponding NF measurement job control service producer to terminate the measurement job for the NF(s) (according to the NF measurement job termination procedure as illustrated in clause B.5).

Annex C (normative): 
Performance Data Stream Unit content description

Table C-1 lists all the Performance Data Stream Unit content items. It also provides an explanation of the individual items.

Table C-1: Performance Data Stream Unit content description

| Performance Data Stream Unit Content | Description |
| --- | --- |
| streamId | The streamId of the performance data stream. |
| granularityPeriodEndTime | Time stamp referring to the end of the granularity period. |
| measResults | This parameter contains the sequence of result values for the observed measurement types. The "measResults" sequence shall have the same number of elements, which follow the same order as the measurement types presented in “measTypes” for the subject stream in the output parameter streamInfoList of the establishStreamingConnection operation (see clause 6.2.1.2). |

Annex D (informative): 
Performance data streaming holistic sequence

# D.1	Performance data streaming for starting measurement collection

## D.1.1	Sequence flow

This annex shows the holistic sequence for performance data streaming, starting from starting the measurement collection (by job or configuration) to sending the performance data to the performance data streaming consumer (stream target).

Figure D.1.1-1

1.	The consumer of MnS for measurement control requests the MnS producer to start the measurement collection by the following two alternatives:

1) by the measurement job control service

1a. The MnS consumer invokes the createMeasurementJob operation towards the MnS producer;

2) by the NRM fragment-based measurement control service

1b.	The MnS consumer selects the correct MeasurementControl MOI for new measurement collection, as a system may have multiple MeasurementControl MOIs;

1c.	If necessary, the MnS consumer modifies the MeasurementControl MOI by invoking the modifyMOI operation towards the MnS producer;

1d.	The MnS consumer creates a new MeasurementReader MOI under the selected MeasurementControl MOI for collecting the measurements, by invoking the createMOI operation towards the MnS producer; or

1e.	The MnS consumer modifies an existing MeasurementReader MOI under the selected MeasurementControl MOI to add new measurements to be collected, by invoking the modifyMOI operation towards the MnS producer.

2.	The producer of MnS for measurement control configures the NF to collect the measurements. The mechanism of this step is vendor specific. If producer of MnS for measurement control is in the NF, this step can be skipped.

3.	The NF triggers the producer of MnS for performance data streaming to set up the streaming information for the new measurements to be collected. The mechanism of this step is vendor specific. If producer of MnS for performance data streaming is in the NF, this step can be skipped.

4.	The producer of MnS for performance data streaming communicates with the consumer to:

4a. establish the streaming (WebSocket) connection containing the stream information if it does not exist yet, by invoking the establishStreamingConnection operation;

4b. add the stream information for the new measurements if they will be reported by new streams, by invoking the addStreamInfo operation;

4c. update the stream information for the new measurements if they will be reported by existing streams, by invoking the updateStreamInfo operation.

5.	The NF collects the measurements. This step is the internal behaviour of the NF.

6.	The NF report the collected measurements to the producer of MnS for performance data streaming. The mechanism of this step is vendor specific. If producer of MnS for performance data streaming is in the NF, this step can be skipped.

7.	The producer of MnS for performance data streaming sends the collected measurements to the consumer via performance data streams, by invoking the reportStreamData operation.

## D.1.2	PlantUML codes

@startuml

skinparam shadowing false

skinparam monochrome true

hide footbox

participant "Consumer of MnS for\n measurement control" as MC

participant "Producer of MnS for\n measurement control" as MP

participant "NF" as NF

participant "Producer of MnS for\n performance data streaming" as SP

participant "Consumer of MnS for\n performance data streaming" as SC

alt Measurement job control service

MC -> MP : 1a. createMeasurementJob\n(measurement control parameters)

else NRM fragment based MnS

MC –> MC: 1b. Select MeasurementControl MOI

Opt Update the selected MeasurementControl MOI if necessary

MC -> MP: 1c: modifyMOI\n(for MeasurementControl MOI)

end

alt create new MOI for collecting measurements

MC -> MP : 1d. createMOI\n(for MeasurementReader IOC)

Else update existing MOI for collecting new measurements

MC -> MP : 1e. modifyMOI\n(for MeasurementReader MOI)

end

end

MP --> NF: 2. Configure measurement\n collection (proprietory)

activate NF

NF --> SP: 3. Set up streaming information\n for new measurements (proprietory)

alt No streaming(WebSocket) connection exists

SP->SC: 4a. establishStreamingConnection()

else Streaming (WebSocket) connection exists

alt add new stream(s)

SP->SC: 4b. addStreamInfo()

else update existing stream(s)

SP->SC: 4c. updateStreamInfo()

end

end

Loop while measurement collection\n is active

NF->NF: 5. Collect measurements

NF --> SP: 6. Report measurements\n (proprietory)

SP -> SC: 7. reportStreamData()

end

@enduml

# D.2	Performance data streaming for stopping measurement collection

## D.2.1	Sequence flow

This annex shows the holistic sequence for performance data streaming in connection with the measurement collection termination.

Figure D.2.1-1

1.	The consumer of MnS for measurement control requests the MnS producer to stop the measurement collection by the following two alternatives:

1) by the measurement job control service

1a.  The MnS consumer invokes the stopMeasurementJob operation towards the MnS producer;

2) by the NRM fragment-based measurement control service

1b.	The MnS consumer selects the correct MeasurementControl MOI for terminating the measurement collection, as a system may have multiple MeasurementControl MOIs;

1c.	If necessary, the MnS consumer modifies the MeasurementControl MOI by invoking the modifyMOI operation towards the MnS producer;

1d.	The MnS consumer deletes an MeasurementReader MOI under the selected MeasurementControl MOI for terminating the measurements collection, by invoking the deleteMOI operation towards the MnS producer;

1e.	The MnS consumer modifies an MeasurementReader MOI with deletion of the measurements that do not need to be collected anymore, by invoking the modifyMOI operation towards the MnS producer.

2.	The producer of MnS for measurement control requests the NF to stop collecting the measurements. The mechanism of this step is vendor specific. If producer of MnS for measurement control is in the NF, this step can be skipped.

3.	The NF stops collecting the measurements. This step is the internal behaviour of the NF.

4.	The NF triggers the producer of MnS for performance data streaming to terminate streaming for the measurements. The mechanism of this step is vendor specific. If producer of MnS for performance data streaming is in the NF, this step can be skipped.

5.	The producer of MnS for performance data streaming communicates with the consumer to:

5a. terminate the streaming (WebSocket) connection if no measurements need to be reported to the consumer anymore, by invoking the terminateStreamingConnection operation;

5b. delete the information for the stream(s) obsoleted due to the termination of the measruements collection if the streaming connection still needs to be retained, by invoking the deleteStreamInfo operation;

5c. update the information for the stream(s) partially affected by the termination of the measurements collection, by invoking the updateStreamInfo operation.

## D.2.2	PlantUML codes

@startuml

skinparam shadowing false

skinparam monochrome true

hide footbox

participant "Consumer of MnS for\n measurement control" as MC

participant "Producer of MnS for\n measurement control" as MP

participant "NF" as NF

participant "Producer of MnS for\n performance data streaming" as SP

participant "Consumer of MnS for\n performance data streaming" as SC

alt Measurement job control service

MC -> MP : 1a. stopMeasurementJob\n(jobId)

else NRM fragment based MnS

MC –> MC: 1b. Select MeasurementControl MOI

Opt Update the selected MeasurementControl MOI if necessary

MC -> MP: 1c: modifyMOI\n(for MeasurementControl MOI)

end

alt delete the MOI to terminate measurements collection

MC -> MP : 1d. deleteMOI\n(for MeasurementReader MOI)

Else update existing MOI to terminate measurements collection

MC -> MP : 1e. modifyMOI\n(for MeasurementReader MOI)

end

end

MP --> NF: 2. terminate measurement\n collection (proprietory)

NF -> NF: 3. stop collecting the measurements

NF --> SP: 4. terminate streaming for the\n measurements (proprietory)

alt Streaming (WebSocket) connection is not needed anymore\n (e.g., no measurements need to be reported to the stream target)

SP->SC: 5a. terminateStreamingConnection()

else Streaming (WebSocket) connection is still needed

alt delete the obsolete stream(s)

loop For all of streams need to be deleted

SP->SC: 5b. deleteStreamInfo()

end

else update existing stream(s)

SP->SC: 5c. updateStreamInfo()

end

end

@enduml

Annex E (normative): 
OpenAPI specification

## E.1	Introduction

This clause describes the capabilities of the Management Services in the structure of the OpenAPI Specification Version 3.0.1. The OpenAPI document is represented in the JSON format option.

## E.2	Performance assurance service

{

"openapi": "3.0.1",

"info": {

"title": "TS 28.550 Performance Measurement Job Control Service",

"version": "15.1.0",

"description": "OAS 3.0.1 specification of the Performance Measurement Job Control Service"

},

"servers": [

{

"url": "http://{DN_prefix_authority_part}/{DN_prefix_remainder}/PerfMeasJobCtrlMnS/v1520",

"variables": {

"DN_prefix_authority_part": {

"description": "See subclause 4.4 of TS 32.158",

"default": "example.com"

},

"DN_prefix_remainder": {

"description": "See subclause 4.4 of TS 32.158",

"default": ""

}

}

}

],

"paths": {

"/measJobs": {

"post": {

"summary": "Create a measurement job",

"description": "To create a measurement job the representation of the measurement job is POSTed on the /measJobs collection resource.",

"requestBody": {

"required": true,

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/measJobCreation-RequestType"

}

}

}

},

"responses": {

"201": {

"description": "Success case (\"201 Created\"). The representation of the newly created measurement job resource shall be returned.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/measJobCreation-ResponseType"

}

}

}

},

"202": {

"description": "Partial success case (\"202 Partically created\"). The representation of the newly created measurement job resource with unsupported list shall be returned.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/measJobCreation-ResponseType"

}

}

}

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

},

"get": {

"summary": "Read resources of measurement jobs",

"description": "With HTTP GET, resources of measurement jobs are read. The resources to be read are identified with the path component (base resource) and the query component (jobIdList) of the URI. The fields query component allows to select the resource properties to be returned.",

"parameters": [

{

"name": "jobIdList",

"in": "query",

"description": "This parameter identifies the list of jobId to select the resources from the collection resources identified with the path component of the URI.",

"required": true,

"schema": {

"type": "array",

"items": {

"type": "string"

}

}

},

],

"responses": {

"200": {

"description": "Success case (\"200 OK\"). The resources identified in the request for retrieval are returned in the response message body. In case the fields query parameter is used, the selected resources are returned.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/measJobsRetrieval-ResponseType"

}

}

}

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

}

},

"/measJobs/{jobId}": {

"get": {

"summary": "Read resource of a single measurement job",

"description": "With HTTP GET, resource of a measurement job is read. The resource to be read is identified with the path component of the URI.",

"parameters": [

{

"name": "jobId",

"in": "path",

"description": "Identifies the measurement job to be read.",

"required": true,

"schema": {

"$ref": "#/components/schemas/uri-Type"

}

}

],

"responses": {

"200": {

"description": "Success case (\"200 OK\"). The resource identified in the path for retrieval is returned in the response message body. ",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/measJobsRetrieval-ResponseType"

}

}

}

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

},

"delete": {

"summary": "Delete a single measurement job",

"description": "The measurement job is deleted by deleting the corresponding measurement job resource. The resource to be deleted is identified with the path component of the URI.",

"parameters": [

{

"name": "jobId",

"in": "path",

"description": "Identifies the measurement job to be deleted.",

"required": true,

"schema": {

"$ref": "#/components/schemas/uri-Type"

}

}

],

"responses": {

"204": {

"description": "Success case (\"204 No Content\"). The measurement job resource has been deleted. The response message body is absent."

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

}

}

},

"components": {

"schemas": {

"dateTime-Type": {

"type": "string",

"format": "date-Time"

},

"uri-Type": {

"type": "string"

},

"measJobCreation-RequestType": {

"type": "object",

"properties": {

"iOCName": {

"type": "string"

},

"iOCInstanceList": {

"type": "array",

"items": {

"$ref": "#/components/schemas/uri-Type"

}

},

"measurementCategoryList": {

"type": "array",

"items": {

"type": "string"

}

},

"reportingMethod": {

"$ref": "#/components/schemas/reportingMethod-Type"

},

"granularityPeriod": {

"type": "integer"

},

"reportingPeriod": {

"type": "integer"

},

"startTime": {

"$ref": "#/components/schemas/dateTime-Type"

},

"stopTime": {

"$ref": "#/components/schemas/dateTime-Type"

},

"schedule": {

"$ref": "#/components/schemas/schedule-Type"

},

"streamTarget": {

"type": "string"

},

"priority": {

"$ref": "#/components/schemas/priority-Type"

},

"reliability": {

"type": "string"

}

}

},

"measJobCreation-ResponseType": {

"type": "object",

"properties": {

"unsupportedList": {

"type": "array",

"items": {

"$ref": "#/components/schemas/unsupportedMeas-Type"

}

}

}

},

"measJobsRetrieval-ResponseType": {

"type": "object",

"properties": {

"jobInfoList": {

"type": "array",

"items": {

"$ref": "#/components/schemas/measJobInfo-ResourceType"

}

}

}

},

"error-ResponseType": {

"type": "object",

"properties": {

"error": {

"type": "object",

"properties": {

"errorInfo": {

"type": "string"

}

}

}

}

},

"measJobInfo-ResourceType": {

"type": "object",

"properties": {

"href": {

"$ref": "#/components/schemas/uri-Type"

},

"iOCName": {

"type": "string"

},

"iOCInstanceList": {

"type": "array",

"items": {

"$ref": "#/components/schemas/uri-Type"

}

},

"measurementCategoryList": {

"type": "array",

"items": {

"type": "string"

}

},

"reportingMethod": {

"$ref": "#/components/schemas/reportingMethod-Type"

},

"granularityPeriod": {

"type": "integer"

},

"reportingPeriod": {

"type": "integer"

},

"startTime": {

"$ref": "#/components/schemas/dateTime-Type"

},

"stopTime": {

"$ref": "#/components/schemas/dateTime-Type"

},

"schedule": {

"$ref": "#/components/schemas/schedule-Type"

},

"streamTarget": {

"type": "string"

},

"priority": {

"$ref": "#/components/schemas/priority-Type"

},

"reliability": {

"type": "string"

}

}

},

"schedule-Type": {

"type": "object",

"properties": {

"scheduleOption": {

"$ref": "#/components/schemas/scheduleOption-Type"

},

"dailySchedule": {

"type": "array",

"items": {

"$ref": "#/components/schemas/timeInterval-Type"

}

},

"weeklySchedule": {

"type": "array",

"items": {

"$ref": "#/components/schemas/scheduleOfDay-Type"

}

}

}

},

"timeInterval-Type": {

"type": "object",

"properties": {

"intervalStart": {

"type": "string",

"format": "Time"

},

"intervalEnd": {

"type": "string",

"format": "Time"

}

}

},

"scheduleOfDay-Type": {

"type": "object",

"properties": {

"dayOfWeek": {

"$ref": "#/components/schemas/dayOfWeek-Type"

},

"intervalsOfDay": {

"type": "array",

"items": {

"$ref": "#/components/schemas/timeInterval-Type"

}

}

}

},

"unsupportedMeas-Type": {

"type": "object",

"properties": {

"iOCInstance": {

"$ref": "#/components/schemas/uri-Type"

},

"measurementTypeName": {

"type": "string"

},

"reason": {

"type": "string"

}

}

},

"reportingMethod-Type": {

"type": "string",

"enum": [

"file",

"streaming"

]

},

"priority-Type": {

"type": "string",

"enum": [

"low",

"medium",

"high"

]

},

"scheduleOption-Type": {

"type": "string",

"enum": [

"daily",

"weekly"

]

},

"dayOfWeek-Type": {

"type": "string",

"enum": [

"Monday",

"Tuesday",

"Wednesday",

"Thursday",

"Friday",

"Saturday",

"Sunday"

]

}

}

}

}

## E.3	Performance data streaming service

{

"openapi": "3.0.1",

"info": {

"title": "TS 28.550 Performance Data Streaming Service",

"version": "15.3.0",

"description": "OAS 3.0.1 specification of the Performance Data Streaming Service"

},

"servers": [

{

"url": "http://{streamTarget}/PerfDataStreamMnS/v1530",

"description": "This URL is used for posting the set of information about streams supported on the connection between the producer and the consumer.",

"variables": {

"streamTarget": {

"description": "The open API server of the performance data streaming service is located in the consumer side,and the “streamTarget” part corresponds to the streamTarget parameter provided in the createMeasurementJob operation (see clause 6.1.1.2) or the streamTarget attribute of the MOI of MeasurementControlor MeasurementReader, see 3GPP TS 28.622 [5]).",

"default": "example.com"

}

}

},

{

"url": "wss://{streamTarget}/PerfDataStreamingMnS/v1530/streamingConnection",

"description": "This URL is used for establishing the WebSocket connection for the performance data streaming service.",

"variables": {

"streamTarget": {

"description": "The open API server of the performance data streaming service is located in the consumer side,and the “streamTarget” part corresponds to the streamTarget parameter provided in the createMeasurementJob operation (see clause 6.1.1.2) or the streamTarget attribute of the MOI of MeasurementControlor MeasurementReader, see 3GPP TS 28.622 [5]).",

"default": "example.com"

}

}

}

],

"paths": {

"/streamInfoList": {

"post": {

"summary": "The set of information about the streams sent from the producer to the consumer",

"description": "To send the streamInfoList from the producer to the consumer",

"requestBody": {

"required": true,

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/streamInfoListPost-RequestType"

}

}

}

},

"responses": {

"201": {

"description": "Success case (\"201 Posted\"). The streamInfoList is successfully posted.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/streamInfoListPost-ResponseType"

}

}

}

},

"202": {

"description": "Partial success case (\"202 Partially posted\"). The representation of the posted resource on stream information.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/streamInfoListPost-ResponseType"

}

}

}

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

}

},

"get": {

"summary": "Read resources of stream information from the streaming consumer",

"description": "With HTTP GET, resources of stream information are read. The resources to be read are identified with the path component (base resource) and the query component (streamIdList) of the URI. The fields query component allows to select the resource properties to be returned.",

"parameters": [

{

"name": "streamIdList",

"in": "query",

"description": "This parameter identifies the list of streamId to select from the collection resources identified with the path component of the URI.",

"required": true,

"schema": {

"type": "array",

"items": {

"type": "integer"

}

}

},

],

"responses": {

"200": {

"description": "Success case (\"200 OK\"). The resources identified in the request for retrieval are returned in the response message body. In case the fields query parameter is used, the selected resources are returned.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/listOfStreamInfoRetrieval-ResponseType"

}

}

}

},

"202": {

"description": "Partial success case (\"202 Partially retrieved\"). The representation of the retrieved resources on stream information.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/listOfStreamInfoRetrieval-ResponseType"

}

}

}

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

},

"patch": {

"summary": "Update resources of stream information to the streaming consumer",

"description": "With HTTP PATCH, resources of stream information are to be updated. The resources to be updated are identified with the path component (base resource) and the query component (streamIdList) of the URI. The fields query component allows to select the resource properties to be updated.",

"parameters": [

{

"name": "streamIdList",

"in": "query",

"description": "This parameter identifies the list of streamId to select from the collection resources identified with the path component of the URI.",

"required": true,

"schema": {

"type": "array",

"items": {

"type": "integer"

}

}

},

],

"requestBody": {

"required": true,

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/listOfStreamInfoToUpdate-RequestType"

}

}

}

},

"responses": {

"200": {

"description": "Success case (\"200 OK\"). The resources selected by the query parameter are updated and returned in the response message body.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/listOfStreamInfoUpdate-ResponseType"

}

}

}

},

"202": {

"description": "Partial success case (\"202 Partially updated\"). The representation of the updated resources on stream information",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/listOfStreamInfoUpdate-ResponseType"

}

}

}

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

},

"delete": {

"summary": "The information of streams to be deleted by the producer to the consumer",

"description": "With HTTP DELETE, resources of stream information are to be deleted. The resources to be deleted are identified with the path component (base resource) and the query component (streamIdList) of the URI. The fields query component allows to select the resource properties to be deleted.",

"parameters": [

{

"name": "streamIdList",

"in": "query",

"description": "This parameter identifies the list of streamId to select from the collection resources identified with the path component of the URI.",

"required": true,

"schema": {

"type": "array",

"items": {

"type": "integer"

}

}

},

],

"responses": {

"204": {

"description": "Success case (\"204 No Content\"). The stream information resource has been deleted. The response message body is absent."

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

}

},

"/streamInfoList/{streamId}": {

"get": {

"summary": "Read resource of the stream information from the streaming consumer",

"description": "With HTTP GET, resource of stream information is read. The resource to be read is identified with the path component the URI.",

"parameters": [

{

"name": "streamId",

"in": "path",

"description": "Identifies the stream for which the information is to be retrieved.",

"required": true,

"schema": {

"$ref": "#/components/schemas/uri-Type"

}

}

],

"responses": {

"200": {

"description": "Success case (\"200 OK\"). The resource identified in the request for retrieval is returned in the response message body.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/listOfStreamInfoRetrieval-ResponseType"

}

}

}

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

},

"patch": {

"summary": "Update the resource of stream information to the streaming consumer",

"description": "With HTTP PATCH, resource of stream information is to be updated. The resource to be updated is identified by the path component of the URI. ",

"parameters": [

{

"name": "streamId",

"in": "path",

"description": "Identifies the stream for which the information is to be updated.",

"required": true,

"schema": {

"$ref": "#/components/schemas/uri-Type"

}

}

],

"requestBody": {

"required": true,

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/streamInfoToUpdate-RequestType"

}

}

}

},

"responses": {

"200": {

"description": "Success case (\"200 OK\"). The resources identified by the path of the URI is updated and returned in the response message body.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/streamInfoUpdate-ResponseType"

}

}

}

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}      },

"delete": {

"summary": "The stream information to be deleted by the producer to the consumer",

"description": "With HTTP DELETE, resource of stream information identified by the path component of the URI is to be deleted.",

"parameters": [

{

"name": "streamId",

"in": "path",

"description": "Identifies the stream for which the information is to be deleted",

"required": true,

"schema": {

"$ref": "#/components/schemas/uri-Type"

}

}

],

"responses": {

"204": {

"description": "Success case (\"204 No Content\"). The stream information resource has been deleted. The response message body is absent."

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

}

},

"/streamConnection": {

"get": {

"summary": "The connection for streaming from the producer to the consumer",

"description": "To establish the WebSocket connection between the producer and the consumer. The HTTP version of this operation shall not be earlier than HTTP/1.1",

"parameters": [

{

"in": "header",

"name": "Upgrade",

"required": true,

"schema": {

"$ref": "#/components/schemas/Upgrade-HeaderType"

}

},

{

"in": "header",

"name": "Connection",

"required": true,

"schema": {

"$ref": "#/components/schemas/Connection-HeaderType"

}

},

{

"in": "header",

"name": "Sec-WebSocket-Key",

"required": true,

"schema": {

"$ref": "#/components/schemas/Sec-WebSocket-Key-HeaderType"

}

},

{

"in": "header",

"name": "Sec-WebSocket-Version",

"required": true,

"schema": {

"$ref": "#/components/schemas/Sec-WebSocket-Version-HeaderType"

}

}

],

"responses": {

"101": {

"description": "Success case (\"101 Switching Protocols \"). The connection has been successfully switched to WebSocket. The response message body is absent.",

"headers": {

"Upgrade": {

"schema": {

"$ref": "#/components/schemas/Upgrade-HeaderType"

}

},

"Connection": {

"schema": {

"$ref": "#/components/schemas/Connection-HeaderType"

}

},

"Sec-WebSocket-Accept-HeaderType": {

"schema": {

"$ref": "#/components/schemas/Sec-WebSocket-Accept-HeaderType"

}

}

}

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

}

}

},

"components": {

"schemas": {

"uri-Type": {

"type": "string"

},

"streamInfoIn-Type": {

"type": "object",

"properties": {

"streamId": {

"type": "integer"

},

"iOCInstance": {

"$ref": "#/components/schemas/uri-Type"

},

"measTypes": {

"type": "array",

"items": {

"type": "string"

}

}

}

},

"streamInfoOut-Type": {

"type": "object",

"properties": {

"streamId": {

"$ref": "#/components/schemas/uri-Type"

},

"iOCInstance": {

"$ref": "#/components/schemas/uri-Type"

},

"measTypes": {

"type": "array",

"items": {

"type": "string"

}

}

}

},

"error-ResponseType": {

"type": "object",

"properties": {

"error": {

"type": "object",

"properties": {

"errorInfo": {

"type": "string"

}

}

}

}

},

"streamInfoListPost-RequestType": {

"type": "object",

"properties": {

"streamInfoList": {

"type": "array",

"items": {

"$ref": "#/components/schemas/streamInfoIn-Type"

}

}

}

},

"streamInfoListPost-ResponseType": {

"type": "object",

"properties": {

"streamInfoListPosted": {

"type": "array",

"items": {

"$ref": "#/components/schemas/streamInfoOut-Type"

}

}

}

},

"streamInfoRetrieval-ResponseType": {

"type": "object",

"properties": {

"streamInfoOut": {

"$ref": "#/components/schemas/streamInfoOut-Type"

}

}

},

"listOfStreamInfoRetrieval-ResponseType": {

"type": "object",

"properties": {

"listOfStreamInfoOut": {

"type": "array",

"items": {

"$ref": "#/components/schemas/streamInfoOut-Type"

}

}

}

},

"streamInfoToUpdatePropertyType": {

"type": "object",

"properties": {

"iOCInstance": {

"description": "The updated measured object instance, empty value means no update.",

"allOf": [

"$ref": "#/components/schemas/uri-Type"

]

},

"measTypes": {

"description": "The updated list of measurement type, empty value means no update.",

"type": "array",

"items": {

"type": "string"

}

}

}

},

"streamInfoToUpdate-RequestType": {

"type": "object",

"properties": {

"streamInfoToUpdate": {

"$ref": "#/components/schemas/streamInfoToUpdatePropertyType"

}

}

},

"listOfStreamInfoToUpdate-RequestType": {

"type": "object",

"properties": {

"listOfStreamInfoToUpdate": {

"type": "array",

"items": {

"$ref": "#/components/schemas/streamInfoToUpdatePropertyType"

}

}

}

},

"streamInfoUpdate-ResponseType": {

"type": "object",

"properties": {

"streamInfoUpdated": {

"$ref": "#/components/schemas/streamInfoOut-Type"

}

}

},

"listOfStreamInfoUpdate-ResponseType": {

"type": "object",

"properties": {

"listOfStreamInfoUpdated": {

"type": "array",

"items": {

"$ref": "#/components/schemas/streamInfoOut-Type"

}

}

}

},

"Upgrade-HeaderType": {

"type": "string",

"enum": [

"websocket",

]

},

"Connection-HeaderType": {

"type": "string",

"enum": [

"Upgrade",

]

},

"Sec-WebSocket-Key-HeaderType": {

"type": "string"

},

"Sec-WebSocket-Version-HeaderType": {

"type": "string"

},

"Sec-WebSocket-Accept-HeaderType": {

"type": "string"

}

}

}

}

Annex F (normative): 
ASN.1 definition for performance data stream units

# F.1	ASN.1 definition rule

For performance data streaming, the type of WebSocket Data frame shall be binary (with opcode of 0x2).

This subclause specifies the abstract syntax notation one (ASN.1) definition for the Performance Data Stream Units as Payload data of WebSocket Data frame.

The Performance Data Stream Units are described using ASN.1 as specified in ITU-T Rec. X.680 [15] and X.681 [16]. Transfer syntax for Performance Data Stream Units is derived from their ASN.1 definitions by use of Packed Encoding Rules (PER), aligned as specified in ITU-T Rec. X.691 [17].

The following encoding rules apply in addition to what has been specified in ITU-T Rec. X.691 [17]:

-	When a bit string value is placed in a bit-field as specified in clause 15.6 to 15.11 in ITU-T Rec X.691 [17], the leading bit of the bit string value shall be placed in the leading bit of the bit-field, and the trailing bit of the bit string value shall be placed in the trailing bit of the bit-field;

-	When decoding a BIT STRING or OCTET STRING constrained with a Contents Constraint, PER decoders are required to never report an error if there are extraneous zero or non-zero bits at the end of the BIT STRING or OCTET STRING.

NOTE:	The terms 'leading bit' and 'trailing bit' are defined in ITU-T Rec. X.680 [15]. When using the 'bstring' notation, the leading bit of the bit string value is on the left, and the trailing bit of the bit string value is on the right.

# F.2	ASN.1 definition

-- ASN1START

PerformanceDataStreamUnits-Schema DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- PDSUs-START

PDSUs ::=                      SEQUENCE OF PDSU

PDSU ::=  SEQUENCE {

streamId                   INTEGER,

granularityPeriodEndTime   DATE-TIME,

standardizedMeasResults    SEQUENCE OF MeasValue,

vendorSpecificMeasResults  SEQUENCE OF MeasValue OPTIONAL -- may be omitted

}

MeasValue ::= CHOICE {

integerValue   INTEGER,

realValue      REAL,

stringValue    VisibleString,

subCounters    SubCounterListType,

...                                             -- allow extension in futher version

}

-- uses recursion for the value to support multi-dimensional measurements

SubCounterListType ::= SEQUENCE {

subCounterIndex   SubCounterIndexType,

subCounterValue   MeasValue OPTIONAL            -- "empty" bins are allowed

}

SubCounterIndexType ::= CHOICE {

sum       VisibleString ("SUM"),

binIndex  INTEGER,

qOS-5QI   INTEGER,

qOS-QCI   INTEGER,

cause     INTEGER,

stringIndex   VisibleString,

plMN      OCTET STRING (SIZE(3)),              -- definition from TS 38.413

sNSSAI    SEQUENCE {                           -- definition from TS 38.413

sst   OCTET STRING (SIZE(1)),

sd    OCTET STRING (SIZE(3))

},

...                                            -- allow extension in futher version

}

-- PDSUs-STOP

END

-- ASN1STOP

Annex F (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-03 | SA#83 | SP-190122 | 0003 | 1 | F | Add the missing RESTFul API definitions | 15.1.0 |
| 2019-03 | SA#83 | SP-190122 | 0007 | 1 | F | Correction on MDAS | 15.1.0 |
| 2019-03 | SA#85 | SP-190748 | 0011 | - | F | Correction on performance data file reporting service components | 15.2.0 |
| 2019-03 | SA#85 | SP-190748 | 0014 | 3 | F | Add the missing stage 3 solutions for performance data streaming | 15.2.0 |
| 2019-03 | SA#85 | SP-190748 | 0015 | 1 | F | Remove the PM file format | 15.2.0 |
| 2019-03 | SA#85 | SP-190748 | 0024 | 1 | F | Add the missing stage 2 for performance data streaming | 15.2.0 |
| 2019-12 | SA#86 | SP-191174 | 0032 | 2 | F | Add stream information management related operations | 15.3.0 |
| 2020-03 | SA#87E | SP-200181 | 0043 | 1 | F | Update the peformance data streaming procedure | 15.4.0 |
| 2020-03 | SA#87E | SP-200181 | 0045 | 1 | F | Add streaming procedure for measurement collection termination | 15.4.0 |