3GPP TSG-SA5 Meeting #130e 	S5-202136rev

- e-meeting, 20 April – 24 April 2020

- Source:	Huawei Technologies

- Title:	pCR TS 28.201 Add the NWDAF and MnS producer message flows

- Document for:	Approval

Agenda Item:	7.4.2

# 1	Discussion

This is a pCR to TS 28.201 Adding the NWDAF and MnS producer message flows in clause 5.2.

# 2	References

[1]	3GPP TS 28.201 "Charging management; network slice performance and analytics charging in 5G System; Stage 2".

# 3	Rationale

This pCR proposes to add the NWDAF and MnS producer message flows to clause 5.2 of TS 28.201 "Charging management; network slice performance and analytics charging in 5G System ; Stage 2".

# 4	Detailed propsosal

Propose to incorporate the following change into the TS 28.201 [1].

First changes

#### 5.2.2.3	Network data analytics subscription from NWDAF

##### 5.2.2.3.1	General

The clause below describes network data analytics subscription scenarios.

##### 5.2.2.3.2	Network slice performance and analytics information Subscribe/Notify

The following figure 5.2.2.3.2-1 procedure is used by CSIF to subscribe/unsubscribe at NWDAF to be notified on performance and analytics information, using Nnwdaf_AnalyticsSubscription service defined in TS 23.288 [150].

Figure 5.2.2.3.2-1: Analytics and performance from NWDAF

1)	Subcribe Request: the CSIF decides to start the collection of performance and analytics information and subscribes to performance and analytics information from NWDAF.

2)	Subcribe Respond: the NWDAF responses the subscription successful.

3)	Notification: based on the subscription of performance and analytics information, the NWDAF notifies the CSIF with the performance and analytics information.

NOTE: There might be several “notification” during the subscription.

4)	Notification Acknowledge: the CSIF sends the acknowledge notification if need.

5)	Unsubscribe Request: the CSIF cancels the subscription to performance and analytics information.

6)	Unsubscribe Respond: the NWDAF responses the unsubscription successful.

#### 5.2.2.4	Network slice performance and analytics subscription from MnS producer

##### 5.2.2.4.1	General

CSIF may support subscription to network slice performance information from MnS producer. The clauses below describes message flow of network slice performance and analytics charging from MnS producer.

##### 5.2.2.4.2	Network slice performance and analytics information Subscribe/Notify

The following figure 5.2.2.4.2-1 procedure is used by CSIF to subscribe/unsubscribe at MnS producer to be notified on performance and analytics information, using MnS.

Figure 5.2.2.4.2-1: Performance and analytics information from MnS Producer

1)	Subcribe Request: The CSIF decides to start the collection of performance and analytics information and subscribes to performance and analytics information uMnS.

2)	Subcribe Respond: the MnS producer responses the subscription successful.

3)	Notification: Based on the subscription of performance and analytics information, the MnS producer notifies the CSIF with the performance and analytics information.

NOTE: There might be several “notification” during the subscription.

4)	Notification Acknowledge: the CSIF sends the acknowledge notification if need.

5)	Unsubscribe Request: the CSIF cancels the subscription to performance and analytics information.

6)	Unsubscribe Respond: the MnS producer responses the unsubscription successful.

End of changes