3GPP TSG-SA5 Meeting #130e 	S5-202238

- e-meeting 20-28 April 2020

- Source:	Huawei,China Mobile

- Title:	Move and update Annex A management control loop in different layers from TS 28.535

- Document for:	Approval

- Agenda Item:	6.4.7

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	3GPP TS 28.536: “Management and orchestration; Management services for communication service assurance; Stage2 and Stage3”.

# 3	Rationale

Firstly, the Annnex A in TS 28.535 described an example of management control loops in different layer, which is a deployment scenario of management control, however, TS 28.535 is the requirement specification, so it proposes to move to TS 28.536 stage2 and stage3 specification.

Secondly, Annex A use the term “Observation” which is not align with close loop step description in other places of TS 28.535 and TS 28.536. So it proposes to update Annex A.

Thirdly, add brief introduction for control loop deployed in different layers.

# 4	Detailed proposal

It proposes to make the following changes to TS 28.536[1].

1st Change

Annex A (informative):
Control loop deployed in different layers

### A.1 Introduction

This example gives a high-level view of control loops deployed in different layers, which consists of control loop in communication service layer, control loop in network slice layer, control loop in network slice subnet layer and control loop in NF layer, as described as Figure A.1-1, where the analytic could be leverage MDAS, and different control loops can provide input (interact with) to other control loops (in the same layer or different layers) and obtain the output from other control loops (in the same layer or different layers).

Figure A.1-1 Control loop in different layers

### A.2 Control loop in CS layer

SLA/SLS requirements provided from CSC are translated into serviceProfile, which represents the requirements for communication service assurance to the CSP. Coordination between control loop in CS layer and control loop in NS layer is needed to calculate the CSI resource requirements and to assure the CSI SLA/SLS requirements.

When the CSI is active, network slice performance is monitored and analysed for the CSI according to end user service experiences.

If service degradation occurs or it is predicted, the 3GPP management system could take actions, i.e. the allocated resources are scaled up or the SLS is adjusted based on pre-agreement/interaction between CSP and CSC.

### A.3 Control loop in NS layer

After the network slice provisioning, the network slice performance (e.g. KPI, QoE) are monitored and reported. When network slice performance is not met, the 3GPP management system identifies the root cause and may reconfigure the resources according to analytical report from MDAS. The NSI resources are also modified accordantly in case the network slice performance requirement needs to be changed based on CSI service requirement adjustment.

### A.4 Control loop in NSS layer

The 3GPP management system could have the capability of service observation (e.g., the supervision to the NSSI) based on MDAS. Based on such observation, management actions on the NSSI might be performed if NSSI performance requirements fulfillment indicates a problem. Another possible scenario is that, when the NSSI performance requirement is changed because of the NSI modification management action, the NSSI resources might be also reconfigured.

### A.5 Control loop in NF layer

Editor’s note: the control loop in NF layer is FFS.

End of changes