3GPP TSG-SA5 Meeting #130e 	S5-202240

- e-meeting 20-28 April 2020

- Source:	Huawei, China Mobile

- Title:	Update Clause 5.2 Coverage optimization scenario example for classification of network autonomy level

- Document for:	Approval

- Agenda Item:	6.6.4

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	3GPP TR 28.810: “Study on concept, requirements and solutions for levels of autonomous network v0.3.0”.

# 3	Rationale

This contribution proposes to update Clause 5.2 coverage optimization scenario example for classification of network autonomy.

# 4	Detailed proposal

It proposes to make the following changes to TR 28.810[1].

1st Change

## 5.2	Coverage optimization scenario example for classification of network autonomy

### 5.2.X Introduction

Coverage is critical for good user experience, however, coverage optimization is complexity.  Full autonomy of coverage optimization for whole network is a long term goal, it will be beneficial for operator to achieve this goal step by step and have clear view on which typical issues can be addressed by utilizing network autonomy mechanism in corresponding steps.

### 5.2.Y Entire workflow

Following are the entire workflow for the radio network coverage optimization:

-	Task A: Coverage requirements determination. The tasks of deciding coverage requirements (e.g. coverage area, weak coverage ratio).

-	Task B: Coverage optimization policies determination. The tasks of analysing and determining the coverage optimization policies (e.g, weak coverage analysis policies, weak coverage optimization policies).

-	Task C: Coverage related information collection. The tasks of collecting coverage performance data (e.g. measurement data, MDT data).

-	Task D: Coverage issues identification. The tasks of analysing the coverage performance and identifying the coverage issues (e.g. weak coverage).

-	Task E: Coverage optimization solution analysis and generation. The tasks of analysing the cause of the identified coverage issues and generating the coverage adjustment actions.

-	Task F: Coverage adjustment actions determination. The tasks of deciding the coverage adjustment actions to be executed.

- 	Task G: Coverage adjustment actions execution (e.g. selection of coverageShape, digitalTilt and digitalAzimuth). The tasks of adjusting and configuring the coverage related parameters.

5.2.Z Potential classification of network autonomy for coverage optimization

- Level 0:

  - All the tasks in the radio network coverage optimization workflow (Task A, Task B, Task C, Task D, Task E, Task F, Task G) are accomplished by human.

- Level 1:

  - Telecom system executes the tasks of adjusting and configuring coverage related parameters (e.g. coverageShape, digitalTilt and digitalAzimuth) based on coverage related parameters specified by human (Task G). Telecom system also can execute the tasks of collecting coverage related data (e.g. measurement data, MDT data and trace data) based on collection rule specified by human (Task C).

  - All the other tasks in the radio network coverage optimization workflow (Task A, Task B, Task D, Task E, Task F) are accomplished by human.

- Level 2:

  - Compared to Level 1, telecom system can analyse the coverage performance  and identify the coverage issue based on the coverage issue rule specified by human (Task D).

  - All the other tasks in the radio network coverage optimization workflow (Task A, Task B, Task E, Task F) are accomplished by human.

- Level 3:

  - Compared to Level 2, telecom system additionally executes the tasks of analysing the coverage optimization solution (Task E) and determining the coverage adjustment actions (Task F) to be executed based on coverage requirements and/or optimization policies specified by Human.

  - All the other tasks in the radio network coverage optimization workflow (Task A, Task B) are accomplished by human.

- Level 4 :

  - Compared to Level 3, the telecom system additionally executes the tasks of analysing and determining coverage optimization policies (e.g, weak coverage cause analysis policies, weak coverage optimization policies) (Task B).  For certain scenarios, the telecom system can translate the radio service assurance intent (e.g. traffic distribution and user distribution specified for the radio network service in certain area needs to be assuranced) to coverage requirements (e.g. weak coverage ratio) (Task A) and adjust the coverage optimization policies.

  - The radio service assurance intent translation policies maybe pre-defined and specified by human to assist the telecom system.

- Level 5:

  - Telecom system can autonomously execute the entire workflow of radio network coverage optimization for all scenarios, which means the network system can achieve the full autonomy for radio network coverage optimization for full scenarios.



Fgiure 5.2.3-1 Classification of network autonomy for radio network coverage optimization

End of changes