3GPP TSG-SA5 Meeting #131e	S5-203058rev1

25 May to 03 June 2020, E-meeting

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0215 | rev | 1 | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add measurements related to DL packet delay between NG-RAN and UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2020-05-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The DL packet delay between NG-RAN and UE (including the delay at gNB-CU-UP, on F1-U and on gNB-DU and the delay over Uu interface) is one significant part of the e2e delay that has impact to users’ experience for some types of services (e.g., URLLC). The measurements on DL packet delay between NG-RAN and UE are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added measurements on DL packet delay between NG-RAN and UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The DL packet delay between NG-RAN and UE cannot be monitored. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.1.y (new), A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

##### 5.1.1.1.y	Distribution of DL delay between NG-RAN and UE

a)	This measurement provides the distribution of DL packet delay between NG-RAN and UE, which is the delay incurred in NG-RAN (including the delay at gNB-CU-UP, on F1-U and on gNB-DU) and the delay over Uu interface. This measurement is split into subcounters per 5QI and subcounters per S-NSSAI.

b)	DER (n=1).

c)	The measurement is obtained by the following method:

The gNB performs the GTP PDU packet delay measurement for QoS monitoring per the GTP PDU monitoring packets received from UPF, and records the following time stamps and information included in the GTP-U header of each GTP PDU monitoring response packet (packet i) sent to UPF (see 23.501 [4] and 38.415 [31]):

-	The DL Delay Result from NG-RAN to UE indicating the downlink delay measurement result which is the sum of the delay incurred in NG-RAN (including the delay at gNB-CU-UP, on F1-U and on gNB-DU) and the delay over Uu interface (see 38.415 [31], and the DL Delay Result is denoted by DRdl in the present document);

-	The 5QI and S-NSSAI associated to the GTP PDU monitoring response packet.

The gNB  increments the corresponding bin with the delay range where the DRdl falls into by 1 for the subcounters per 5QI and subcounters per S-NSSAI.

d)	Each measurement is an integer representing the number of GTP PDUs measured with the delay within the range of the bin.

e)	DRB.DelayDlNgranUeDist.5QI.Bin, where Bin indicates a delay range which is vendor specific, and 5QI identifies the 5QI; 
DRB.DelayDlNgranUeDist.SNSSAI.Bin, where Bin indicates a delay range which is vendor specific, and SNSSAI identifies the S-NSSAI.

f)	NRCellCU (for non-split and 2-split scenario);
GNBCUUPFunction (for 3-split scenario).

g)	Valid for packet switched traffic.

h)	5GS.

Next Modified Sections

# A.x	Monitoring of distribution of integrated delay in NG-RAN

The integrated DL/UL packet delay in NG-RAN, i.e., the delay between NG-RAN and UE (including the delay at gNB-CU-UP, on F1-U and on gNB-DU for split scenario and the delay over Uu interface) is one significant part of the e2e delay that has direct impact to users’ experience for some types of services (e.g., URLLC).

Besides the average integrated delay in NG-RAN which can reflect whether or not the users experience can be met on average, it is very useful for operator to know how many (percents of) data packets are with satfistfied delay perfomance, and how many are not with satisfied performance and how far they are from the satisfied performance. Therefore, the distribution of integrated delay in NG-RAN needs to be monitored.

As each S-NSSAI or 5QI has different requirements on the delay, so the distribution of integrated delay in NG-RAN needs to be monitored per S-NSSAI and per 5QI.

End of Modified Sections