3GPP TS 32.322 V10.1.0 (2020-07)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Test management Integration Reference Point (IRP);

Information Service (IS)

(Release 10)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

UMTS, management

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

©2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI  registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions and abbreviations	7

3.1	Definitions	7

3.2	Abbreviations	7

4	System Overview	7

4.1	System Context	7

5	Information Object Classes (IOCs)	8

5.1	Information Entities imported and local Labels	8

5.2	Class Diagram	9

5.2.1	Attributes and Relationships	9

5.2.2	Inheritance	10

5.3	Information Object Classes (IOCs) definition	10

5.3.1	Information Object Class TestManagementIRP	10

5.3.1.1	Definition	10

5.3.1.2	Attributes	11

5.3.2	Information Object Class TestActionPerformer	11

5.3.2.1	Definition	11

5.3.2.2	Attributes	11

5.3.3	Information Object Class TesterObject	11

5.3.3.1	Definition	11

5.3.3.2	Attributes	12

5.3.4	Information Object Class ResourceSelfTestTesterObject	12

5.3.4.1	Definition	12

5.3.4.2	Attributes	12

5.3.5	Proxy Class VSETestCategoryTesterObject	12

5.3.5.1	Definition	12

5.3.5.2	Attributes	12

5.3.6	Proxy Class VSEResourceSelfTestTesterObject	12

5.3.6.1	Definition	12

5.3.6.2	Attributes	13

5.3.7	Proxy Class VSETesterObject	13

5.3.7.1	Definition	13

5.3.7.2	Attributes	13

5.3.8	Proxy Class MORT	13

5.3.8.1	Definition	13

5.3.8.2	Attributes	13

5.3.9	Information Object Class TestInvocation	13

5.3.9.1	Definition	13

5.3.9.2	Attributes	13

5.3.10	Information Object Class NetworkPerformFaultTesterObject	13

5.3.10.1	Definition	13

5.3.10.2	Attributes	14

5.3.11	Proxy Class VSENetworkPerformFaultTesterObject	14

5.3.11.1	Definition	14

5.3.11.2	Attributes	14

5.4	Information relationships definition	14

5.4.1	Relationship between TestManagementIRP and TestActionPerformer	14

5.4.1.1	Definition	14

5.4.1.2	Roles	14

5.4.2	Relationship between TestActionPerformer and TesterObject	14

5.4.2.1	Definition	14

5.4.2.2	Roles	14

5.4.3	Relationship between TestActionPerformer and TestInvocation	15

5.4.3.1	Definition	15

5.4.3.2	Roles	15

5.4.4	Relationship between TesterObject and TestInvocation	15

5.4.4.1	Definition	15

5.4.4.2	Roles	15

5.4.5	Relationship between TesterObject and MORT	15

5.4.5.1	Definition	15

5.4.5.2	Roles	15

5.4.6	Relationship between TestInvocation and MORT	15

5.4.6.1	Definition	15

5.4.6.2	Roles	15

5.5	Information attributes definition	16

5.5.1	Definition and legal Values	16

6	Interface definition	18

6.1	Class diagram representing interfaces	18

6.2	Generic rules	18

6.3	Interface testManagementIRPControlOperations	19

6.3.1	Operation initiateTests (M)	19

6.3.1.1	Definition	19

6.3.1.2	Input parameters	19

6.3.1.3	Output parameters	20

6.3.1.4	Pre-condition	20

6.3.1.5	Post-condition	20

6.3.1.6	Exceptions	20

6.3.2	Operation terminateTests (M)	21

6.3.2.1	Definition	21

6.3.2.2	Input parameters	21

6.3.2.3	Output parameters	21

6.3.2.4	Pre-condition	21

6.3.2.5	Post-condition	21

6.3.2.6	Exceptions	22

6.4	Interface TestManagementIRPMonitorOperations	23

6.4.1	Operation monitorTest (M)	23

6.4.1.1	Definition	23

6.4.1.2	Input parameters	23

6.4.1.3	Output parameters	23

6.4.1.4	Pre-condition	23

6.4.1.5	Post-condition	24

6.4.1.6	Exception	24

6.5	Interface TestManagementIRPNotifications	25

6.5.1	Notification notifyTestResults (M)	25

6.5.1.1	Definition	25

6.5.1.2	Triggering Events for the Test	26

6.5.1.2.1	From-State	27

6.5.1.2.2	To-State	27

Annex A (informative):	Network Performance Measurement Procedure	28

Annex B (informative):	Change history	30

# Foreword

This Technical Specification (TS) has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management; Test management Integration Reference Point (IRP), as identified below:

32.321:	"Test management Integration Reference Point (IRP); Requirements"

32.322:	"Test management Integration Reference Point (IRP): Information Service (IS)"

32.326:	"Test management Integration Reference Point (IRP): Solution Set (SS) definitions"

A 3G telecommunication network is composed of a multitude of different Network Elements (NE). For a successful operation of the network the operator must be provided with mechanisms allowing him to manage the network. These management activities can be grouped into several areas: configuration management, fault management, performance management, accounting management and security management.

A management function assisting in different high level management areas such as fault management and performance management is test management. The purpose of testing is to get information about the functionality and performance of the 3G managed network subject to the test.

The present document is part of a TS-family defining the Telecommunication Management (TM) of 3G systems. 
The TM principles are described in 3GPP TS 32.101 [5]. The TM architecture is described in 3GPP TS 32.102 [6]. 
The other specifications define the interface (Itf-N) between the managing system (manager), which is in general the Network Manager (NM) and the managed system (agent), which is either an Element Manager (EM) or the managed NE itself. The Itf-N is composed of a number of integration reference points (IRPs) defining the information in the agent that is visible for the manager, the operations that the manager may perform on this information and the notifications that are sent from the agent to the manager. One of these IRPs is the Test Management IRP.

Each IRP is specified by the requirements part, the IS part and at least one SS (e.g. CORBA SS).

# 1	Scope

The present document defines the IS part of the Test Management IRP, which describes the semantics of the information and the interactions visible across Itf-N in a protocol independent way. The information is specified by means of information object classes and the interactions by means of operations and notifications. The present document does not specify the syntax (encoding) of the information.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.302: "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP): Information Service (IS)".

[2]	3GPP TS 32.312: "Telecommunication management; Generic Integration Reference Point (IRP) management; Information Service (IS)".

[3]	3GPP TS 32.622: "Telecommunication management; Configuration Management (CM); Generic network resources Integration Reference Point (IRP): Network Resource Model (NRM)".

[4]	ITU-T Recommendation X.733: "Information technology - Open Systems Interconnection - Systems Management: Alarm reporting function".

[5]	ITU-T Recommendation X.745: "Information technology - Open Systems Interconnection - Systems Management: Test management function".

[6]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[7]	3GPP TS 32.102: "Telecommunication management; Architecture".

[8]	3GPP TS 32.321: "Telecommunication management; Test management Integration Reference Point (IRP): Requirements".

[9]	3GPP TS 32.672: "Telecommunication management; Configuration Management (CM); State Management Integration Reference Point (IRP): Information Service (IS)".

[10]	ITU-T Recommendation X.737: "Information technology - Open Systems Interconnection - Systems Management: Confidence and diagnostic test categories.

[11]	3GPP TS 32.150: "Telecommunication management; Integration Reference Point (IRP) Concept and definitions".

[12]	IETF RFC4656: "A One-way Active Measurement Protocol (OWAMP)".

[13]	IETF RFC 5357: "A Two-way Active Measurement Protocol (TWAMP)".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TS 32.101 [6], 3GPP TS 32.102 [7] and 3GPP TS 32.321 [8] apply.

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

EM	Element Manager

IOC	Information Object Class

IRP	Integration Reference Point

IS	Information Service

ME	Element Manager

MORT	Managed Object Referring to Test

NE	Network Element

TM	Telecommunication Management

TO	Tester Object

VSE	Vendor-Specific-Extended

VSE TO	Vendor-Specific-Extended Tester Object

# 4	System Overview

## 4.1	System Context

The general definition of the System Context for the present IRP is found in 3GPP TS 32.150 [11] subclause 4.7.

In addition, the set of related IRP(s) relevant to the present IRP is shown in the two diagrams below.

Figure 1: System Context A

Figure 2: System Context B

# 5	Information Object Classes (IOCs)

## 5.1	Information Entities imported and local Labels

| Label reference | Local label |
| --- | --- |
| 3GPP TS 32.622 [3], information object class, Top | Top |
| 3GPP TS 32.622 [3], information object class, IRPAgent | IRPAgent |
| 3GPP TS 32.312 [2], information object class, managedGenericIRP | managedGenericIRP |

## 5.2	Class Diagram

### 5.2.1	Attributes and Relationships

The following figure shows, for the Test Management IRP, the class definitions and the relationships between the classes.

From the cardinalities can be seen that each instance of TestManagementIRP may have several instances of TestActionPerformer. Each instance of TestActionPerformer can have multiple instances of associated TesterObjects. Each instance of TesterObject in turn has one instance of TestInvocation. For the resource self test, each instance of TesterObject has one instance of MORT. For the connection and loopback test, each instance of TesterObject has two instances of MORT.

### 5.2.2	Inheritance

The following figure depicts the inheritance relationships between the information object classes.

## 5.3	Information Object Classes (IOCs) definition

### 5.3.1	Information Object Class TestManagementIRP

#### 5.3.1.1	Definition

The IOC TestManagementIRP together with the IOC TestActionPerformer represent the test management capabilities defined by this specification. To conduct a test of network resources, this object may require capabilities of other objects such as TesterObject. The IOC TestManagementIRP inherits from the IOC ManagedGenericIRP specified in 3GPP TS 32.312 [2].

#### 5.3.1.2	Attributes

The IOC TestManagementIRP has no own attributes, only those inherited from the IOC ManagedGenericIRP.

### 5.3.2	Information Object Class TestActionPerformer

#### 5.3.2.1	Definition

The IOC TestActionPerformer provides the ability to receive and react upon test requests. This class must also be able to instantiate and delete tester objects or, in case the tester objects are permanently instantiated, to allocate and reserve them for their usage. This specification does not require this IOC to be instantiated. It may be abstract and used for inheritance purposes only. In this way the ability to receive and react upon test requests may be included in any other IOC.

#### 5.3.2.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| supportedTOClasses | + | M | M | - |
| testActionPerformerId | + | M see note | M | - |
| NOTE: This attribute is only mandatory in case the IOC TestActionPerformer is instantiated. In case this IOC is an abstract class and used for inheritance purposes only the attribute shall be omitted. |  |  |  |  |

### 5.3.3	Information Object Class TesterObject

#### 5.3.3.1	Definition

The IOC TesterObject monitors and controls the testing of a MORT instance and reports the outcome of the test execution. Tester Objects (TOs) are instantiated by the IOC TestActionPerformer in response to a valid test initiation request (initiateTests). They are deleted after termination of the test. It is also possible that TOs are permanently instantiated. In this case they are allocated to a certain TestActionPerformer during the test execution. After termination of the test they are released.

The IOC TesterObject defines a generic TO. It shall be used as an abstract class from which more specific tester objects shall be derived by specialisation for each test category. Test categories and the associated test category specific TOs are defined in ITU-T Recommendation X.737 [10]. These test category specific TOs can be specialised further by defining Vendor-Specific-Extended (VSE) TOs. The generic TO defines attributes pertaining to a test and required for all test categories.

Each test invocation shall have only one associated TO.

Only test category specific TOs or VSE TOs shall be instantiated.

For simplicity this specification will often use only the term TO. In this case either the test category specific TO or the VSE TO is referred to depending on which is actually instantiated.

#### 5.3.3.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| testOutcome | + | M | M | - |
| testState | + | M | M | - |
| testInvocationInitiator | - | C | M | - |
| additionalInformation | - | O | - | - |
| proposedRepairActions | - | O | - | - |
| fileReference | - | M see note | - | - |
| fileExpiryDate | - | M see note | - | - |
| testerObjectId | + | M | M | - |
| NOTE: In case the TO does support capturing test results in a file this parameter shall be present and contain information. In case the TO does not support capturing test results in a file this parameter shall contain no information or shall be absent. |  |  |  |  |

### 5.3.4	Information Object Class ResourceSelfTestTesterObject

#### 5.3.4.1	Definition

The IOC ResourceSelfTesterObject is a specialised TO for the resource self test. It inherits from the IOC TesterObject. It specifies the triggering events for the emission of the test result notifications.

#### 5.3.4.2	Attributes

This IOC has no own attributes, only those inherited from the generic IOC TesterObject.

### 5.3.5	Proxy Class VSETestCategoryTesterObject

#### 5.3.5.1	Definition

Certain tests may not fit in any of the test categories defined in ITU-T Recommendation X.737 [10]. In this case vendors may define new (VSE) test categories and the associated test category specific TOs. The Proxy Class VSETestCategoryTesterObject represents the set of these VSE test category tester objects

The IOCs represented by the Proxy Class VSETestCategoryTesterObject shall inherit from the IOC TesterObject.

NOTE:	A vendor may also claim 3GPP compliance to a certain release in case that a specific test fits into one of the ITU-T test categories without that the corresponding ITU-T test category specific TO is supported in this release supposed that this test category specific TO will be added in a later release than the current one. The vendor shall update this specification in due time.

#### 5.3.5.2	Attributes

The attributes of this IOC are vendor specific.

### 5.3.6	Proxy Class VSEResourceSelfTestTesterObject

#### 5.3.6.1	Definition

In case the IOC ResourceSelfTestTesterObject does not fulfil the specific requirements of a certain resource self test, vendors may define proprietary IOCs by further specialisation. The Proxy Class VSEResourceSelfTestTesterObject represents these IOCs.

The IOCs represented by the Proxy Class VSEResourceSelfTestTesterObject shall inherit from the IOC ResourceSelfTestTesterObject.

#### 5.3.6.2	Attributes

Apart from the attributes inherited the attributes of the IOCs represented by this Proxy Class are vendor specific.

### 5.3.7	Proxy Class VSETesterObject

#### 5.3.7.1	Definition

In case an IOC represented by the Proxy Class VSETestCategoryTesterObject does not fulfil the specific requirements of a certain test, vendors may define proprietary IOCs by further specialisation. The Proxy Class VSETesterObject represents these IOCs.

The IOCs represented by the Proxy Class VSETesterObject shall inherit from the associated IOC represented by the Proxy Class VSETestCategoryTesterObject.

#### 5.3.7.2	Attributes

Apart from the attributes inherited the attributes of the IOCs represented by this Proxy Class are vendor specific.

### 5.3.8	Proxy Class MORT

#### 5.3.8.1	Definition

The ProxyClass MORT represents a network resource that is under test. Its class definition shall be one defined in the various 3GPP Network Resource Model specifications or defined by a VSE network resource class.

#### 5.3.8.2	Attributes

This IOC has no attributes.

### 5.3.9	Information Object Class TestInvocation

#### 5.3.9.1	Definition

The IOC TestInvocation is the abstract representation of a test invocation. A test invocation shall aim to test one or more capabilities of a MORT. The IRPManager can request for the establishment of a test invocation using the operation initiateTests.

A MORT can be complex in that there are multiple capabilities that can be subject to test. Therefore, it is possible to have multiple test activities active, all aimed at the same MORT but on its different capabilities. Whether multiple test activities can be testing the same MORT capabilities at the same time is an implementation decision, probably based on the nature and behaviour of the TO, and therefore, is outside the scope of this specification.

#### 5.3.9.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| actualStartTime | + | O | M | - |
| actualStopTime | + | O | M | - |
| maxTestingPhaseDuration | - | O | - | - |
| testInvocationId | + | M | M | - |

### 5.3.10	Information Object Class NetworkPerformFaultTesterObject

#### 5.3.10.1	Definition

The IOC NetworkPerformFaultTesterObject is a specialised TO for the network performance test. It inherits from the IOC TesterObject.

#### 5.3.10.2	Attributes

Apart from the attributes inherited from the generic IOC TesterObject, this IOC has own attributes in the below table.

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| testSourceAddress | + | M | M | M |
| testDestinationAddress | + | M | M | M |
| testLoopbackAddress | + | M | M | M |
| testPacketInformation | + | M | M | M |

### 5.3.11	Proxy Class VSENetworkPerformFaultTesterObject

#### 5.3.11.1	Definition

In case the IOC NetworkPerformFaultTesterObject does not fulfil the specific requirements of a certain connection test, vendors may define proprietary IOCs by further specialisation. The Proxy Class VSENetworkPerformFaultTesterObject represents these IOCs.

The IOCs represented by the Proxy Class VSENetworkPerformFaultTesterObject shall inherit from the IOC NetworkPerformFaultTesterObject.

#### 5.3.11.2	Attributes

Apart from the attributes inherited the attributes of the IOCs represented by this Proxy Class are vendor specific.

## 5.4	Information relationships definition

### 5.4.1	Relationship between TestManagementIRP and TestActionPerformer

#### 5.4.1.1	Definition

This relationship defines a binary association between the IOC TestManagementIRP and the IOC TestActionPerformer.

#### 5.4.1.2	Roles

This relationship has no roles.

### 5.4.2	Relationship between TestActionPerformer and TesterObject

#### 5.4.2.1	Definition

This relationship defines a binary association between the IOC TestActionPerformer and the IOC TesterObject. The association is navigable from the TestActionPerformer to the TesterObject.

#### 5.4.2.2	Roles

| Name | Definition |
| --- | --- |
| theTesterObject | This rolename provides a name allowing to navigate from an instance of TestActionPerformer to the associated instances of TesterObject. If tap is an instance of TestActionPerformert, the expression tap.theTesterObject yields the set of object instances of TesterObject. |

### 5.4.3	Relationship between TestActionPerformer and TestInvocation

#### 5.4.3.1	Definition

This relationship defines a binary association between the IOC TestActionPerformer and the IOC TestInvocation. 
The association is navigable from the TesterObject to the TestInvocation.

#### 5.4.3.2	Roles

| Name | Definition |
| --- | --- |
| theTestInvocation | This rolename provides a name allowing to navigate from an instance of TestActionPerformer to the associated instances of TestInvocation. If tap is an instance of TestActionPerformert, the expression tap.theTestInvocation yields the set of object instances of TestInvocationt. |

### 5.4.4	Relationship between TesterObject and TestInvocation

#### 5.4.4.1	Definition

This relationship defines a binary association between the IOC TesterObject and the IOC TestInvocation. 
The association is navigable in both directions.

#### 5.4.4.2	Roles

| Name | Definition |
| --- | --- |
| theTesterObject | This rolename provides a name allowing to navigate from an instance of TestInvocation to the associated instance of TesterObject. If ti is an instance of TestInvocation, the expression ti.theTesterObject yields an object instance of TesterObject. |
| theTestInvocation | This rolename provides a name allowing to navigate from an instance of TesterObject to the associated instance of TestInvocation. If to is an instance of TesterObject, the expression to.theTestInvocation yields an object instance of TestInvocation. |

### 5.4.5	Relationship between TesterObject and MORT

#### 5.4.5.1	Definition

This relationship defines a binary association between the IOC TesterObject and the Proxy Class MORT. 
The association is navigable from the TesterObject to the MORT.

#### 5.4.5.2	Roles

| Name | Definition |
| --- | --- |
| theMORT | This rolename provides a name allowing to navigate from an instance of TesterObject to the associated instance of MORT. If to is an instance of TesterObject, the expression to.theMORT yields an object instance of MORT. |

### 5.4.6	Relationship between TestInvocation and MORT

#### 5.4.6.1	Definition

This relationship defines an association between the IOC TestInvocation and the IOC MORT. This association specifies that the latter is testing the former.

#### 5.4.6.2	Roles

Instead of roles this relationship has a role name.

## 5.5	Information attributes definition

### 5.5.1	Definition and legal Values

| Attribute Name | Definition | Legal Values |
| --- | --- | --- |
| testInvocationId | This attribute uniquely identifies an instance of a TestInvocation within the TestManagementIRP. The test invocation identifier is assigned by the TestActionPerformer. When a testInvocationId can be reused is outside the scope of this specification. |  |
| testState | This attribute reflects the actual test state (ITU-T Recommendation X.745 [5]). | ENUM {notInitialized, idle, initializing, testing, terminating, disabled} |
| testOutcome | This attribute provides information about the test result, as perceived by the associated TO, in a standardised manner. The information in this parameter is only valid after termination of the test activity. This information shall be present in the last test result notification emitted by a TO prior to its deletion. | ENUM {pass, fail, inconclusive, timed-out, premature-termination} Pass indicates that the test exercise of the test invocation has executed correctly and has found no problem. Fail indicates that the test exercise of the test invocation has executed correctly and has found one or more problems. Inconclusive indicates that the TO has not determined if the execution is Pass or Fail. Timed-out indicates that the TO has terminated its execution because of the expiry of the timer (i.e., the current time – TestSession.sessionStartTime >= TesterObject.timeOut). Premature termination indicates that the TO has (a) never started execution or (b) terminated its execution prematurely, either by TestManagementIRP and its associated objects internal problems or in response to a terminateTests operation. |
| supportedTOClasses | This attribute identifies the TO classes that are supported by a certain managed object instance whose class has inherited from TestActionPerformer or whose class is the TestActionPerformer. | SET OF TO class name |
| testActionPerformerId | This attribute unambiguously identifies an instance of a TestActionPerformer. |  |
| testerObjectId | This attribute unambiguously identifies an instance of a TesterObject. |  |
| testInvocationInitiator | It identifies the IRPManager. | How multiple IRPManagers choose their identifier so that they are distinguishable is outside the scope of this specification. |
| additionalInformation | This attribute holds a set of additional information pertaining to the test. | The semantics of this parameter are outside the scope of this specification |
| proposedRepairActions | This attribute suggests one or more repair actions if the reason for a failure is known. | The semantics of this parameter are outside the scope of this specification |
| actualStartTime | This attribute specifies the time at which the TO will enter or has entered the test state testing. Before the TO enters the testing state this is an estimated time. After entering the testing state this is the actual time. Note that this is not the time of the invocation of the operation initiateTests. | All values indicating a valid time. |
| actualStopTime | This attribute specifies the time at which the TO will leave or has left the test state testing. Before the TO leaves the testing state this is an estimated time. After leaving the testing state this is the actual time. Note that this is not the time of the invocation of the operation terminateTests. | All values indicating a valid time later than the actualStartTime. |
| maxTestingPhaseDuration | This attribute specifies the maximum amount of time that a TO may spend in the testing state. | All values indicating a valid amount of time. |
| fileReference | This attribute carries the reference to a file that contains the test result data set. |  |
| fileExpiryDate | This attribute carries the date and time after which the file, whose reference is carried by the fileReference attribute, may be removed. | All values indicating a valid time. |
| testSourceAddress | This attribute provides the source address of the test. | An IP address indicating source. |
| testDestinationAddress | This attribute provides the destination address of the test. | An IP address indicating destination. In a loopback test, it should be NULL. |
| testLoopbackAddress | This attribute provides the loopback address in a loopback test. | An IP address indicating loopback point in a loopback test. In connection test, it should be NULL. |

# 6	Interface definition

## 6.1	Class diagram representing interfaces

The following diagram depicts the interfaces of the Test Management IRP with their corresponding operations and notifications.

## 6.2	Generic rules

Rule 1: 	Each operation with at least one input parameter supports a pre-condition valid_input_parameter which indicates that all input parameters shall be valid with regards to their information type. Additionally, each such operation supports an exception operation_failed_invalid_input_parameter which is raised when pre-condition valid_input_parameter is false. The exception has the same entry and exit state.

Rule 2: 	Each operation with at least one optional input parameter supports a set of pre-conditions supported_optional_input_parameter_xxx where "xxx" is the name of the optional input parameter and the pre-condition indicates that the operation supports the named optional input parameter. Additionally, each such operation supports an exception operation_failed_unsupported_optional_input_parameter_xxx which is raised when (a) the pre-condition supported_optional_input_parameter_xxx is false and (b) the named optional input parameter is carrying information. The exception has the same entry and exit state.

Rule 3: 	Each operation shall support a generic exception operation_failed_internal_problem that is raised when an internal problem occurs and that the operation cannot be completed. The exception has the same entry and exit state.

## 6.3	Interface testManagementIRPControlOperations

The interface TestManagementIRPControlOperations contains the operations initiateTests and terminateTests. It must be implemented by every object with the ability to receive and react upon test requests, for example by every instance of TestActionPerformer.

### 6.3.1	Operation initiateTests (M)

#### 6.3.1.1	Definition

The IRPManager uses this operation to request the IRPAgent to initiate controlled tests. A single test request may initiate multiple (one or more) tests.

For each test to be initiated the managed object representing the network resource to be tested and the tester object class must be specified.

The initiated tests are independent and not related to each other. This implies that independent test result notifications are sent for each of the tests initiated by s single initiateTests operation.

#### 6.3.1.2	Input parameters

| Parameter Name | Qualifier | Information Type | Comment |
| --- | --- | --- | --- |
| testInvocationInitiator | C | TesterObject. testInvocationInitiator | This parameter identifies the IRPManager.. |
| toBeInitiatedTests | M | SET OF SET { maxTestingStateDuration (O) toBeTestedMORT (O) testerObjectClass (M) testerObjectName (O) testerObjectInitialAttributeList (O) } | This sequence specifies the tests to be initiated. For each test the parameter maxTestingStateDuration specifies the timeout period of the tests to be initiated. One value shall indicate “No limit”. toBeTestedMORT specifies the instance of the MORT to be tested. If the parameter is absent, the MORT is identical to the object instance to which the subject operation is directed to. The parameter testerObjectClass specifies the class of the associated tester object. Optionally, a name for the tester object instance may be specified in the parameter testerObjectName. The parameter testerObjectInitialAttributeList carries some or all the values of the attributes of the TO instance responsible for the test. The syntax and semantics of this attribute value is dependent on the specific TO class definition and is outside the scope of 3GPP. |

#### 6.3.1.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| response | M | SEQUENCE OF CHOICE { testInitiated testNotInitiated } testInitiated = SEQUENCE { TestInvocation.testInvocationId (M) testerObjectName (O) } testNotInitiated = failureReason | The number and the order, related to the tests to be initiated, of elements in this sequence and in the set of the input parameter toBeInitiatedTests shall be identical. For a successfully instantiated test the parameter testInitiated returns the test invocation identifier of the test. In case the tester object name has not been specified in the request it shall be returned in testerObjectName. For a failed test instantiation the parameter testNotInvoked returns the reason why the instantiation of the test failed. Failure reasons are TO class is not existing MORT is not existing MORT is not available others |

#### 6.3.1.4	Pre-condition

The precondition must hold true before the operation is invoked. The pre-condition depends on the test category.

For at least one of the specified tests to be instantiated the following must hold true:

theIndicatedMORTIsExisting AND theIndicatedMORTIsAvailable AND theIndicatedTOClassIsExisting.

| Assertion Name | Definition |
| --- | --- |
| theIndicatedMORTIsExisting | The MORT indicated by the subject operation for this test is existing |
| theIndicatedMORTIsAvailable | The MORT indicated by the subject operation for this test is available. |
| theIndicatedTOClassIsExisting | The TO class indicated by the subject operation for this test is existing. |

#### 6.3.1.5	Post-condition

The post-condition must hold true after the completion of the operation:

allIndicatedTOsInstantiated OR notAllTestsInitiated OR noTestInitiated

| Assertion Name | Definition |
| --- | --- |
| allTestsInitiated | All tests indicated by the subject operation were initiated successfully. |
| notAllTestsInitiated | Not all but at least one test indicated by the subject operation was initiated successfully. |
| noTestInitiated | No test indicated by the subject operation was initiated successfully. |

#### 6.3.1.6	Exceptions

| Exception Name | Definition |
| --- | --- |
| operationFailedEntirely | Condition: noTestInitiated = TRUE Returned information: The response parameter is returned Exit state: Entry state |
| operationFailedPartly | Condition: notAllTestsInitiated = TRUE Returned information: The response parameter is returned Exit state: Entry state |

### 6.3.2	Operation terminateTests (M)

#### 6.3.2.1	Definition

The IRPManager uses this operation to request the IRPAgent to terminate tests during their life time. A single terminateTests operation may terminate multiple (one or more) tests.

The tests to be terminated are identified by their test invocation identifiers. The IRPManager terminating a test may be different from the IRPManager that initiated the test. The terminateTests operation must be invoked on the object which received the corresponding initiateTests operation.

#### 6.3.2.2	Input parameters

| Parameter Name | Qualifier | Information Type | Comment |
| --- | --- | --- | --- |
| toBeTerminatedTests | M | SET OF TestInvocation.testInvocationId | This parameter specifies the tests that shall be terminated. |

#### 6.3.2.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| response | M | SEQUENCE OF CHOICE { testTerminated testNotTerminated } testTerminated = TestInvocation.testInvocationId testNotTerminated = SEQUENCE { TestInvocation.testInvocationId, failureReason } | The number and the order, related to the test invocation identifier, of elements in this sequence and in the set of the input parameter toBeTerminatedTests shall be identical. It specifies the test invocation ids of the tests, that were successfully terminated, and the ids of the tests, that failed to be terminated successfully together with the reason for the failure. Failure reasons are test invocation id is not existing others |

#### 6.3.2.4	Pre-condition

The precondition must hold true before the operation is invoked.

allIndicatedTestInvocationIdsAreExisting OR notAllIndicatedTestInvocationIdsAreExisting

| Assertion Name | Definition |
| --- | --- |
| allIndicatedTestInvocationIdsAreExisting | All test invocation identifiers specified by the subject operation are existing. |
| notAllIndicatedTestInvocationIdsAreExisting | Not all but at least one test invocation identifier specified by the subject operation is existing. |

#### 6.3.2.5	Post-condition

The post-condition must hold true after the completion of the operation.

allIndicatedTestsTerminated OR notAllIndicatedTestsTerminated OR noIndicatedTestTerminated

| Assertion Name | Definition |
| --- | --- |
| allIndicatedTestsTerminated | All tests indicated in the subject operation were terminated successfully. |
| notAllIndicatedTestsTerminated | Not all but at least one test indicated in the subject operation aaws terminated successfully |
| noIndicatedTestTerminated | No test indicated in the subject operation aaws terminated successfully |

#### 6.3.2.6	Exceptions

| Exception Name | Definition |
| --- | --- |
| operationFailedEntirely | Condition: noIndicatedTestTerminated = TRUE Returned information: The response parameter is returned Exit state: Entry state |
| operationFailedPartly | Condition: notAllIndicatedTestInvocationIdsAreExisting = TRUE OR notAllIndicatedTestsTerminated = TRUE Returned information: The response parameter is returned Exit state: Entry state |

## 6.4	Interface TestManagementIRPMonitorOperations

The interface TestManagementIRPMonitorOperations contains the operation monitorTest. It has a realisation relationship with the IOC TestActionPerformer.

### 6.4.1	Operation monitorTest (M)

#### 6.4.1.1	Definition

IRPManager shall be able to retrieve information about the test as observed by the TO during the test execution by reading the relevant attributes of the TO associated to the test. Also after the test execution the manager shall be able to read these attributes as long as the TO exists. Attributes conveying information about the test execution are testState and testOutcome. Depending on the specific test category specific TO or the VSE TO other attributes may also contain information about the test execution. In this case the subject operation may also allow to read the values of these attributes.

#### 6.4.1.2	Input parameters

| Parameter Name | Qualifier | Information Type | Comment |
| --- | --- | --- | --- |
| toBeMonitoredTO | M | tOInstance | This parameter specifies the instance of the tester object, whose attribute values of testState, testOutcome and other attributes shall be retrieved. |

#### 6.4.1.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| monitoredAttributeValues | M | SET { TesterObject.testState (M) TesterObject.testOutcome (M) other attributes (O) } | This parameter shall be returned if all attributes were read successfully and may be returned, if at least one attribute was read successfully. The values to be returned are those prevalent at the time of the reception of the subject operation. |
| error | M | failureReason | This parameter shall be returned if the specified tester object instance is not existing or, in case the tester object instance is existing, at least one attribute could not be read, i. e. if operationFailedEntirely = TRUE OR operationFailedPartly = TRUE The parameter returns the failure reason. |

#### 6.4.1.4	Pre-condition

The precondition must hold true before the operation is invoked.

indicatedTOInstanceIsExisting

| Assertion Name | Definition |
| --- | --- |
| toBeMonitoredTOIsExisting | The TO instance indicated by the subject operation is existing. |

#### 6.4.1.5	Post-condition

The post-condition must hold true after the completion of the operation.

allAttributeValuesRead OR notAllAttributeValuesRead OR noAttributeValueRead

| Assertion Name | Definition |
| --- | --- |
| allAttributeValuesRead | All attributes of the TO indicated by the subject operation were read successfully. |
| notAllAttributeValuesRead | Not all but at least one attribute of the TO indicated by the subject operation were read successfully. |
| noAttributeValueRead | No attribute of the TO indicated by the subject operation was read successfully. |

#### 6.4.1.6	Exception

| Exception Name | Definition |
| --- | --- |
| operationFailedEntirely | Condition: toBeMonitoredTOIsExisting = FALSE OR noAttributeValueRead = TRUE Returned information: The error parameter returns the object identifier of the TO that does not exist or the reasons, why the attributes could not be read Exit state: Entry state |
| operationFailedPartly | Condition: toBeMonitoredTOIsExisting = TRUE AND notAllAttributeValuesRead = TRUE Returned information: The error parameter returns the reason why an attribute could not be read. The attribute that could be read my be returned in the parameter error or the parameter attributeList Exit state: Entry state |

## 6.5	Interface TestManagementIRPNotifications

### 6.5.1	Notification notifyTestResults (M)

#### 6.5.1.1	Definition

Test results are made available to the IRPManager by one or more notifications notifyTestResults emitted by the TO that is related to the test invocation.

Depending on the nature of the test and the specification of the TO behaviour, the TO may need to convey to the IRPManager a test result data set. There are two ways to convey this kind of information. One way is to use the parameter additionalInformation of the notification. In this case, the fileReference and fileExpiryDate shall contain no information or be absent. The other way is to use a file to capture the test result data set. In this case, the additionalInformation parameter may contain no information or be absent and the fileReference and fileExpiryDate shall be present. The file that captures the test result data set shall contain VSE attributes and other 3GPP attributes such as testerObjectClass, testOutcome, etc.

The use of the additionalInformation parameter or a file to capture the test result data set is specified by the class specification of the TO.

In case the TO uses additionalInformation (and not a file) to capture the test result data set, that TO may emit this notification to transfer intermediate (non-final) test results. In this kind of notifications, the testOutcome parameter shall be absent. The TO should emit at least one more notification regarding the subject test invocation in the future. The last notification pertaining to a particular test invocation shall be indicated by including the testOutcome parameter in the notification.

In the case the TO uses a file to capture the test result data set, that TO shall not issue any notifications to transfer intermediate test results. The TO may capture the non-final test results in the file used to capture the final test result data set.

The events triggering the emission of test result notifications depend on the specific test. They shall be specified by the TO that is actually instantiated, i.e. either by the test category specific TO or the VSE TO. Some generic triggering events are included in this specification. It is expected that vendors specify more triggering events.

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| objectClass | M, Y | TesterObject.objectClass | Notification header - see [1]. It shall carry the TO class name. |
| objectInstance | M, Y | TesterObject.objectInstance | Notification header - see [1]. It shall carry the DN of the TO.. |
| notificationId | O, N | -- | Notification header - see [1]. |
| eventTime | M, Y | -- | Notification header - see [1]. |
| systemDN | C, Y | -- | Notification header - see [1]. |
| notificationType | M, Y | "notifyTestResults" | Notification header - see [1]. |
| testInvocationId | O, N | TestInvocation.testInvocationId |  |
| testInvocationInitiator | C, Y | TesterObject.testInvocationInitiator |  |
| testOutcome | O, N (see note) | TesterObject.testOutcome | It shall be included only in the last notification emitted by a TO. In this way the TO indicates that it is sending no more notifications. |
| mORT | O, N | TesterObject.theMORT | It identifies the object instance of the MORT that was subject to the test. |
| proposedRepairActions | O, N | TesterObject.proposedRepairActions |  |
| additionalInformation | O, N (see note) | TesterObject.additionalInformation | It allows the inclusion of any additional information in the notification. As such, it may carry a test result data set. The exact semantics of this parameter is outside the scope of this specification. This parameter may contain no information or be absent, if the test results are captured in a file. It may be present if the test results are not captured in a file. |
| fileReference | M, N (see note) | TesterObject.fileReference | It shall contain no information or be absent if there is no test result captured in a file. It shall contain information if the test results are captured in a file. |
| fileExpiryDate | M, N (see note) | TesterObject.fileExpiryDate | It shall contain no information or be absent if fileReference carries no information or absent. Otherwise, it shall contain a valid future date and time. |
| NOTE: As for the correct interpretation of this qualifier refer to the comment column. |  |  |  |

#### 6.5.1.2	Triggering Events for the Test

For the test the events triggering the emission of test result notifications are:

- Termination of the test execution.

The test may be terminated explicitly by a test termination request. The events triggering an implicit termination are:

- Fulfilment of the conditions for a successful termination of the test.

- Fulfilment of the conditions for a premature termination of the test.

- Occurrence of an error situation.

##### 6.5.1.2.1	From-State

testTerminateRequestReceived OR testCompleted OR prematureTermination OR testTimedOut OR errorSituationOccured.

| Assertion Name | Definition |
| --- | --- |
| testTerminateRequestReceived | The object with the ability to receive and react upon test requests has received a test termination request (see note). |
| testCompleted | The predefined conditions for a successful completion of the test are fulfilled (see note). |
| prematureTermination | The predefined conditions for a premature termination of the test are fulfilled (see note). |
| errorSituationOccured | An error situation has occurred during the test execution and the tester object has aborted the test invocation (see note). |
| NOTE: The conditions to satisfy this trigger are related to the VSE TO definition and therefore, their specifications are outside the scope of 3GPP. |  |

##### 6.5.1.2.2	To-State

testTerminated

| Assertion Name | Definition |
| --- | --- |
| testTerminated | The test has been terminated successfully. |

Annex A (informative):
Network Performance Measurement Procedure

There are four procedures in network performance measurement: initiation, termination, monitor and test result report.

In the initiation, NM sends an initiation request in which there are some configuration parameters to EM with operation initialTests. The EM will store the parameters and send some configuration parameters to the source NE and destination NE (or the loopback NE) to initiate the network performance measurement. For the IP network, the network elements will start the test with the protocols defined in IETF [12] [13]. The following figure illustrates the test initialisation procedure.

Figure A.1 Initiation procedure for Network Performance Measurement

When the NM terminates the test, NM sends a termination request to EM with operation terminateTests. The EM will save and send some parameters to the source NE and destination NE (or the loopback NE) to terminate the network performance measurement. The following figure illustrates the test termination procedure.

Figure A.2 Termination procedure for Network Performance Measurement

When the NM monitors the test, NM sends a monitor request to EM with operation monitorTests. If it is needed, the EM will send request to the NE(s) to retrieve information before it send back the output parameters. Then the EM will send back the output parameters to the NM. The following figure illustrates the test monitor procedure.

Figure A.3 Monitor procedure for Network Performance Measurement

After the test is terminated, the test result is recorded in a test result file in EM. Then the EM sends a notification notifyTestResults to the NM, in which there is some information for the NM to retrieve the file. The following figure illustrates the test monitor procedure.

Figure A.4  Test result report procedure for Network Performance Measurement

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Cat | Old | New |
| Jun 2002 | SA_16 | SP-020328 | -- | -- | Submitted to TSG SA #16 for Information | -- | 1.0.0 |  |
| Sep 2002 | SA_17 | SP-020457 | -- | -- | Submitted to TSG SA #17 for Approval | -- | 2.0.0 | 5.0.0 |
| Dec 2002 | -- | -- | -- | -- | Cosmetics | -- | 5.0.0 | 5.0.1 |
| Jun 2004 | SA_24 | SP-040243 | 0001 | -- | Add missing parameter to the operation initiateTests | F | 5.0.1 | 5.1.0 |
| Sep 2004 | SA_25 | SP-040541 | -- | -- | Automatic upgrade to Rel- 6 (no CR) as per request in SP-040541 SA5_presentation_SA_25.ppt (slide 17) | -- | 5.1.0 | 6.0.0 |
| Jun 2005 | SA_28 | SP-050289 | 0002 | -- | Apply Generic System Context | F | 6.0.0 | 6.1.0 |
| Mar 2006 | SA_31 | SP-060089 | 0003 | -- | Update mapping table to use defined filter qualifier | F | 6.1.0 | 6.2.0 |
| Jun 2007 | SA_36 | -- | -- | -- | Automatic upgrade to Rel-7 (no CR) at freeze of Rel-7. Deleted reference to CMIP SS, discontinued from R7 onwards. | -- | 6.2.0 | 7.0.0 |
| Sep 2007 | SA_37 | SP-070606 | 0005 | -- | Correct granularity of maxTestingStateDuration | A | 7.0.0 | 7.1.0 |
| Mar 2008 | SP-39 | SP-080204 | 0008 | 1 | Remove toBeMonitoredAttributes | F | 7.1.0 | 7.2.0 |
| Jun 2008 | SP-40 | SP-080329 | 0009 | -- | Add new Information Objects for connection and loopback test categories - Align with 32.321 | B | 7.2.0 | 8.0.0 |
| Dec 2009 | - | - | - | - | Update to Rel-9 version (MCC) | - | 8.0.0 | 9.0.0 |
| Mar 2011 | - | - | - | - | Update to Rel-10 version (MCC) | - | 9.0.0 | 10.0.0 |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-07 | SA#88e | SP-200413 | 0012 | - | A | draft-ietf-ippm-twamp has been published as RFC5357 | 10.1.0 |