3GPP TSG-SA5 Meeting #132e 	S5-204067rev1

- e-meeting, 17 - 28 August 2020

- Source:	Orange, AT&T, Deutsche Telekom

- Title:	Key Issue: 5GC EE KPI – Potential solution #1 enhancement

- Document for:	Approval

Agenda Item:	6.6.5

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]	TR 28.813 0.1.0: Study on new aspects of Energy Efficiency (EE) for 5G

# 3	Rationale

This pCR proposes enhancements to the already existing potential solution #1 for DV5GC: User Plane DV based solution.

# 4	Detailed proposal

This document proposes the following changes in TR 28.813 [1].

1st Change

## 4.2	Key Issue #2: EE KPI for 5GC

### 4.2.1	Description

TS 28.554 [5] – clause 6.7 (Energy Efficiency (EE) KPI) contains so far one single KPI definition: NG-RAN data Energy Efficiency (EE) (cf. clause 6.7.1). A corresponding EE KPI is missing for 5GC.

This potential solution proposes to apply the generic EE KPI (see clause 5.3 of ETSI ES 203 228 [6]):

to the 5GC, leading to the definition of a new EE KPI:

Depending on the solution, new measurements may have to be defined in TS 28.552 [4].

### 4.2.2	Potential solutions

#### 4.2.2.1	Potential solution #1 for DV5GC: User Plane DV based solution

##### 4.2.2.1.1	Introduction

This potential solution #1 focuses on the definition of DV5GC.

For this potential solution, it is proposed to apply the same principle as for NG-RAN (cf. [5] clause 6.7.1 ‘NG-RAN data Energy Efficiency (EE)’), i.e. to measure only the User Plane traffic volume. Therefore, the DV part of the EE KPI for 5GC is measured only at the User Plane Functions (UPF) that constitute the 5GC. This potential solution applies only to UPFs with N3 interfaces connected.

5GC Control Plane traffic is not considered.

In this proposed solution, both incoming and outgoing traffic are measured at the GTP-U layer on the N3 interface.

##### 4.2.2.1.2	Description

The considered traffic is the traffic between the 5G-AN and UPFs that constitute the 5GC via the N3 interface. All UL and DL traffic:

- between 5G AN node and UPF, as well as

- between N3IWF and UPF

is considered.

In this potential solution, the following would have to be done: introduce a new EE KPI in TS 28.554 [5] – clause 6.7:

where DV5GC , expressed in bits, would be defined as follows:

and

- GTP.InDataOctN3UPF already defined in TS 28.552 [4) clause 5.4.1.3 (Number of octets of incoming GTP data packets on the N3 interface, from (R)AN to UPF), and

- GTP.OutDataOctN3UPF already defined in TS 28.552 [4) clause 5.4.1.4 (Number of octets of outgoing GTP data packets on the N3 interface, from UPF to (R)AN).

End of change