3GPP TSG-SA5 Meeting #132e 	S5-204080

- e-meeting 17th 28th August 2020

- Source:	China Mobile

- Title:	pCR 28.555 Add high-level use cases

- Document for:	Approval

Agenda Item:	6.5.1

# 1	Decision/action requested

To approve the high-level use cases.

# 2	References

# 3	Rationale

To provide the high-level use cases for TS 28.555

# 4	Detailed proposal

It proposes to make the following changes to TS 28.555

1st Change

### X.4.1	Deploy the network function in specific location

| Use Case Stage | Evolution / Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | According to the network deployment policy, 3GPP system use the policy to deploy the corresponding network. |  |
| Actors and Roles | PAF is responsible for creating the IMS deployment policy. PF is responsible for executing the IMS deployment policy and can store the policy. PSF is responsible for storing the IMS deployment policy. It exists standalone or is embedded in PF. PVF is responsible for evaluating the policy execution. |  |
| Telecom resources | 3GPP management system,NFV MANO (NFVO, VNFM) |  |
| Assumptions | 1. The operator has two types' DCs, core DCs are centralized in some individual region and edge DCs are distributed in most cities. 2. The operator determines core DC supports the control plane network function which handles signalling and edge DC supports the media plane or forwarding plane network function which handles session service and data service. 3. The operator need deploy IMS network and determine CSCF should be deployed in core DC and SBC should be in edge DC, where the hardware resource should support the capability of transcoding and encryption and decryption. |  |
| Pre conditions | 1. The operator designs the IMS network deployment requirement and need deploy the network based on the requirement. 2. Management systems (i.e. 3GPP management system and NFV-MANO system) are running normally. |  |
| Begins when | The operator sets up the IMS network deployment policy through PAF. |  |
| Step 1 (M) | The PAF activates the policy and notifies related PF. |  |
| Step 2 (M) | The PF follows the policy to design the IMS network then the IMS network is instantiated by the management system. |  |
| Step 3 (M) | The PF informs the PAF the policy has been executed. |  |
| Ends when | The PAF requests PVF to evaluate the effectiveness of the policy execution if needed. The PVF evaluates the effectiveness of the policy according to the result of the policy execution. In this case the effectiveness is CSCF(s) and SBC(s) in the IMS network are deployed in appropriate DCs and on appropriate hardware resource. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post Conditions | The IMS network is correctly configured and normally running. |  |
| Traceability | REQ-POM_ND-CON-x |  |

### X.4.2	Deploy the corresponding network functions in different location

| Use Case Stage | Evolution / Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | According to the network deployment policy, 3GPP system use the policy to deploy the corresponding network. |  |
| Actors and Roles | PAF is responsible for creating the IMS deployment policy. PF is responsible for executing the IMS deployment policy and can store the policy. PSF is responsible for storing the IMS deployment policy. It exists standalone or is embedded in PF. PVF is responsible for evaluating the policy execution. |  |
| Telecom resources | 3GPP management system,NFV MANO (NFVO, VNFM), |  |
| Assumptions | The operator determines to deploy one IMS network with 3 load sharing CSCFs. The operator determines it is not allowed to deploy more than 2 CSCFs in the same core DC. |  |
| Pre conditions | 1. The operator designs the IMS network deployment requirement and need deploy the network based on the requirement. 2. Management systems (i.e. 3GPP management system and NFV-MANO system) are running normally. Note: The relation between PAF/PF/PSF and management system is out of scope of the present document. |  |
| Begins when | The operator sets up the IMS network deployment policy through PAF. |  |
| Step 1 (M) | The PAF activates the policy and notifies related PF. |  |
| Step 2 (M) | The PF follows the deployment policy to design the IMS network then the IMS network is instantiated by the management system. Note: Whether and how NSD or VNFD supports the policy is out of scope of the present document. |  |
| Step 3 (M) | The PF informs the PAF the policy has been executed. |  |
|  | The PAF requests PVF to evaluate the effectiveness of the policy execution if needed. |  |
| Ends when | The PAF requests PVF to evaluate the effectiveness of the policy execution if needed.The PVF evaluates the effectiveness of the policy according to the result of the policy execution. In this case the effectiveness is all CSCFs in the IMS network are deployed in appropriate DCs. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post Conditions | The IMS network is correctly configured and normally running. |  |
| Traceability | REQ-POM_ND-CON-x |  |

### X.4.3	Deploy the network function based on certain location constraints

| Use Case Stage | Evolution / Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | According to the network deployment policy, 3GPP system use the policy to deploy the corresponding network. |  |
| Actors and Roles | PAF is responsible for creating the IMS deployment policy. PF is responsible for executing the IMS deployment policy and can store the policy. PSF is responsible for storing the IMS deployment policy. It exists standalone or is embedded in PF. PVF is responsible for evaluating the policy execution. |  |
| Telecom resources | 3GPP management system,NFV MANO (NFVO, VNFM), |  |
| Assumptions | 1. The operator has two types' DCs, core DCs are centralized in some individual region and edge DCs are distributed in most cities. 2. The operator need deploy IMS network in one region and the EPC network has already been running. |  |
| Pre conditions | 1. The operator designs the IMS network deployment requirement and need deploy the network based on the requirement. 2. Management systems (i.e. 3GPP management system and NFV-MANO system) are running normally. 3. The PF has the adjacent location information of the data centre(s) and central office(s). Editor's note: How the PF gets the adjacent location information is out of scope of the present document. |  |
| Begins when | The operator sets up the IMS network deployment policy through PAF. |  |
| Step 1 (M) | The PAF delivers the policy to PF and activates the policy if needed. |  |
| Step 2 (M) | The PF follows the policy to design the IMS network and determine the virtualized SBC location after getting the SAE GW location. |  |
| Step 3 (M) | The management systems follow the location constraint to design template (e.g. NSD or VNFD) and instantiate the IMS network. |  |
| Ends when | The PF informs the PAF the policy has been executed. |  |
| Exceptions | The PAF requests PVF to evaluate the effectiveness of the policy execution if needed. The PVF evaluates the effectiveness of the policy according to the result of the policy execution. In this case the effectiveness is the virtualized SBC is deployed adjacently to the running SAE GW. |  |
| Post Conditions | The IMS network is correctly configured and normally running. |  |
| Traceability | REQ-POM_ND-CON-x |  |

End of changes